3GPP TSG-CT Meeting #102	CP-233074

Edinburgh, GB, 11th – 12th December 2023	C4-234662

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0706 | rev | 4 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Describe HR-SBO related procedures and Offload Identifiers in VPLMN offload policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | HR-SBO is an important feature in the edge_ph2 work item.3GPP TS 29.502 has introduced data type supporting such a feature. However, there is a lack of descriptions of HR-SBO related. It is good to have a corresponding procedure description, which provides a better flow for the spec and allows for reader more friendly reading and understanding. Therefore, we introduce the procedure description for HR-SBO related. TS 23.548 has stated that : The VPLMN Specific Offloading Policy may refer to either allowed or not allowed traffic for HR-SBO (the latter is being used when the HPLMN would like to ensure that certain traffic are not allowed for HR-SBO). The VPLMN Specific Offloading Policy for the allowed and not allowed traffic should be mutually exclusive, i.e., either a list of allowed or a list of not allowed traffic descriptors should be sent, but not both. The VPLMN Specific Offloading Policy can be configured in the H-SMF and tagged with Offload Identifier(s), and which VPLMN Specific Offloading Informationto be sent to V-SMF can be indicated by these Offload Identifier(s) received from UDM/H-PCF. So: - The H-SMF shall respond to the V-SMF with the offload identifier and the VPLMN specific offloading policy for the first time or if the VPLMN specific offloading policy need to be updated. - The V-SMF shall store the offload identifiers from H-SMF and also keep in memory which offload identifiers have been received. These offload identifiers are so called stored offload identifiers. - The V-SMF shall indicate the stored offload identifiers to the H-SMF for any subsequent request to the H-SMF in the Home PLMN for any HR-PDU Session towards that H-PLMN. - For the V-SMF HR-PDU Session request with the stored offload identifiers, if the policies for these received offload identifiers are not changed, the H-SMF shall only send the Offload Identifiers in response. Otherwise, the H-SMF return both offload identifier and offload policy. - The H-SMF may use one of HR-SBO PDU session to update the offload policy if it is changed, the V-SMF shall apply the change for all HR-SBO PDU session with the corresponding offload identifier authorized. - For the V-SMF changes HR PDU session, the target V-SMF provides a list of stored offload identifiers, if the policies for these offload identifiers are not changed, the source V-SMF shall only send the Offload Identifiers in response during the SM context retrieve procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete the missing attribute descriptions and procedure related description. Update the HrsboInfoFromVplmn and HrsboInforFromHplmn to support the offloading structure between H-SMF and V-SMF. Update the SMContextRetrieveData and SmContext to support the offloading structure between the V-SMF(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not align with stage 2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.1, 5.2.2.6.1, 5.2.2.7.1, 5.2.2.8.2.1, 5.2.2.8.3.1, 6.1.6.1, 6.1.6.2.7, 6.1.6.2.39, 6.1.6.2.72, 6.1.6.2.73, 6.1.6.2. A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new feature on Open API Nsmf_PDUsession API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: adding Nokia, Nokia Shanghai Bell and Huawei as supporting company. Update description in 5.2.2.6.1 and some wording improvement. Rev2: correct some change over changes, and update NOTE Y. Update cardinality from 1..N to 0..N for vplmnOffloadingPolicyList and offloadIds Rev3: correct a few minor typos Rev4: a. the VplmnOffloadingInfo is used instead of using VplmnOffloadingPolicy, in clauses 5.2.2.6.1, 6.1.6.2.39, 6.1.6.2.73. b. the storedOffloadIds IE is corrected as an array of OffloadIdentifier instead of being an array in clause 6.1.6.2.72. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** First Change ***

##### 5.2.2.2.1	General

The Create SM Context service operation shall be used to create an individual SM context, for a given PDU session, in the SMF, in the V-SMF for HR roaming scenarios, or in the I-SMF for a PDU session with an I-SMF.

It is used in the following procedures:

-	UE requested PDU Session Establishment (see clauses 4.3.2 and 4.23.5.1 of 3GPP TS 23.502 [3]);

-	EPS to 5GS Idle mode mobility, EPS to 5GS Idle mode mobility with data forwarding or handover using N26 interface (see clauses 4.11.1, 4.23.12.3, 4.23.12.5 and 4.23.12.7 of 3GPP TS 23.502 [3]);

-	EPS to 5GS mobility without N26 interface (see clause 4.11.2.3 3GPP TS 23.502 [3]);

-	Handover of a PDU session between 3GPP access and non-3GPP access, when the target AMF does not know the SMF resource identifier of the SM context used by the source AMF, e.g. when the target AMF is not in the PLMN of the N3IWF (see clause 4.9.2.3.2 of 3GPP TS 23.502 [3]), or when the UE is roaming and the selected N3IWF is in the HPLMN (see clause 4.9.2.4.2 of 3GPP TS 23.502 [3]);

-	Handover from EPS to 5GC-N3IWF (see clause 4.11.3.1 of 3GPP TS 23.502 [3]);

-	Handover from EPC/ePDG to 5GS (see clause 4.11.4.1 of 3GPP TS 23.502 [3]);

-	Xn based or N2 based handover with I-SMF or V-SMF insertion and change (see clauses 4.23.7.3, 4.23.11 and 4.23.12 of 3GPP TS 23.502 [3]);

-	UE Triggered Service Request with I-SMF insertion/change/removal or V-SMF change (see clause 4.23.4.3 of 3GPP TS 23.502 [3]);

-	Registration procedure for a UE with a PDU session with I-SMF or V-SMF insertion, change and removal (see clause 4.23.3 of 3GPP TS 23.502 [3]);

-	Handover from EPC/ePDG to 5GS with I-SMF insertion (see clause 4.23 of 3GPP TS 23.502 [3]);

-	Handover from non-3GPP to 3GPP access with I-SMF insertion or V-SMF change, and Handover from 3GPP to non-3GPP access with I-SMF removal (see clause 4.23.16 of 3GPP TS 23.502 [3]);

-	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]);

-	I-SMF Context Transfer procedure (see clause 4.26.5.2 of 3GPP TS 23.502 [3]);

-	5G-RG requested PDU Session Establishment via W-5GAN (see clause 7.3.1 of 3GPP TS 23.316 [36]);

-	FN-RG related PDU Session Establishment via W-5GAN (see clause 7.3.4 of 3GPP TS 23.316 [36]);

-	Non-5G capable device behind 5G-CRG and FN-CRG requested PDU Session Establishment via W-5GAN (see clause 4.10a of 3GPP TS 23.316 [36]);

-	Handover from 3GPP access/EPS to W-5GAN/5GC (see clause 7.6.4.1 of 3GPP TS 23.316 [36]);

-	SMF triggered I-SMF selection or removal (see clause 4.23.5.4 of 3GPP TS 23.502 [3]);

-	Multicast Session join and session establishment procedure in clause 7.2.1.3 of 3GPP TS 23.247 [44].

There shall be only one individual SM context per PDU session.

The NF Service Consumer (e.g. AMF) shall create an SM context by using the HTTP POST method as shown in Figure 5.2.2.2.1-1.

Figure 5.2.2.2.1-1: SM context creation

1.	The NF Service Consumer shall send a POST request to the resource representing the SM contexts collection resource of the SMF. The payload body of the POST request shall contain:

-	a representation of the individual SM context resource to be created;

-	the requestType IE, if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing Emergency PDU session; the requestType IE shall not be included for a MA-PDU session establishment request; it may be included otherwise;

-	the Old PDU Session ID, if it is received from the UE (i.e. for a PDU session establishment for the SSC mode 3 operation);

-	the indication that the UE is inside or outside of the LADN (Local Area Data Network) service area, if the DNN corresponds to a LADN;

-	the perLadnDnnSnssaiInd IE, indicating that the PDU Session is subject to LADN per LADN DNN and S-NSSAI, if the AMF enforces the LADN Service Area per LADN DNN and S-NSSAI;

-	the indication that a MA-PDU session is requested if a MA-PDU session is requested to be established by the UE, or the indication that the PDU session is allowed to be upgraded to a MA-PDU session if so indicated by the UE;

-	the n3gPathSwitchSupportInd IE if the AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the selected SMF supports non-3GPP path switching and if the UE supports non-3GPP access path switching as specified in clause 4.22.2.1 of 3GPP TS 23.502 [3];

-	the indication that the same PCF is required for the requested DNN and S-NSSAI, if it is received by the AMF from UE Subscription data in the UDM, together with the PCF ID selected by the AMF;

-	the alternative S-NSSAI, if the NF service consumer and UE support the network slice replacement and it is requested to replace the S-NSSAI with the alternative S-NSSAI (see clause 5.15.19 of 3GPP TS 23.501 [2]);

-	the alternative HPLMN S-NSSAI, if the NF service consumer and UE support the network slice replacement and it is requested to replace the HPLMN S-NSSAI with the alternative HPLMN S-NSSAI for a HR PDU session (see clause 5.15.19 of 3GPP TS 23.501 [2]);

-	the anType;

-	the additionalAnType, if the UE is registered over both 3GPP and Non-3GPP accesses;

-	the cpCiotEnabled IE with the value "True", if the NF service consumer (e.g. the AMF) has verified that the CIOT feature is supported by the SMF (and for a home-routed session, that it is also supported by the H-SMF), and Control Plane CIoT 5GS Optimisation is enabled for this PDU session;

-	the cpOnlyInd IE with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation;

-	the Invoke NEF indication with the value "True" for a home-routed PDU session, if the cpCiotEnabled IE is set to "True" and data delivery via NEF is selected for the PDU session;

-	a subscription for SM context status notification;

-	the servingNfId identifying the serving AMF;

-	trace control and configuration parameters, if trace is to be activated (see 3GPP TS 32.422 [22]);

-	identifiers (i.e. FQDN or IP address) of N3 terminations at the W-AGF, TNGF or TWIF, if available;

-	a subscription for DDN failure notification, if the Availability after DDN failure event is subscribed by the UDM;

-	the upipSupported IE set to "true", if the UE supports User Plane Integrity Protection with EPS and if the AMF supports the related functionality.

For the UE requested PDU Session Establishment procedure in home routed roaming scenario (see clause 4.3.2.2.2 of 3GPP TS 23.502 [3]), the NF Service Consumer shall provide the URI of the Nsmf_PDUSession service of the H-SMF in the hSmfUri IE and optionally the corresponding SMF ID, and may provide the URI of the Nsmf_PDUSession service of additional H-SMF(s) with the corresponding SMF ID(s). The V-SMF shall try to create the PDU session using the hSmfUri IE. If due to communication failure on the N16 interface the V-SMF does not receive any response from the H-SMF, then:

-	depending on operator policy, the V-SMF may try reaching the hSmfUri via an alternate path; or

-	if additional H-SMF URI is provided, the V-SMF may try to create the PDU session on one of the additional H-SMF(s) provided.

For a PDU session establishment with an I-SMF (see clause 4.23.5.1 of of 3GPP TS 23.502 [3]), the NF Service Consumer shall provide the URI of the Nsmf_PDUSession service of the SMF in the smfUri IE and optionally the corresponding SMF ID, and may provide the URI of the Nsmf_PDUSession service of additional SMF(s) with the corresponding SMF ID(s). The I-SMF shall try to create the PDU session using the smfUri IE. If due to communication failure on the N16a interface the I-SMF does not receive any response from the SMF, then:

-	depending on operator policy, the I-SMF may try reaching the smfUri via an alternate path; or

-	if additional SMF URI is provided, the I-SMF may try to create the PDU session on one of the additional SMF(s) provided.

For the UE requested PDU Session Establishment procedure, if the AMF determines that the RAT type is NB-IoT and the UE has already 2 PDU Sessions with user plane resources activated, the AMF may continue with the PDU Session establishment and include the cpCiotEnabled IE or cpOnlyInd IE with the value "True" to the SMF as specified in clause 4.3.2.2.1 of 3GPP TS 23.502 [3].

The payload body of the POST request may further contain:

-	the name of the AMF service to which SM context status notification are to be sent (see clause 6.5.2.2 of 3GPP TS 29.500 [4]), encoded in the serviceName attribute;

-	the remote provisioning server information, if both the AMF and SMF support the Remote Provisioning of UEs in Onboarding Network procedures and the AMF received the information from AUSF for remote provisioning of the UE via user plane;

-	the Onboarding Indication, if the UE is registered for onboarding in an SNPN;

-	the indication of Notification for SM Policy Association events with the value "true" and the callback information of the PCF for the UE (i.e. the PCF for AM Policy and possibly UE Policy) to receive the notification, if both NF service consumer and the SMF support the "SPAE" feature and if the SM Policy Association Establishment and Termination events should be reported for the PDU session by the PCF for SM Policy to the PCF for the UE. See clause 4.3.2.2.1 of 3GPP TS 23.502 [3];

-	the satelliteBackhaulCat IE indicating the category of the satellite backhaul used towards the 5G AN serving the UE, if the AMF is aware of that a satellite backhaul is used towards the 5G AN;

-	the disasterRoamingInd IE set to true during the PDU session establishment if the UE is registered for Disaster Roaming service.

-	the hrsboAllowedInd IE set to true if the Session Breakout for HR session in VPLMN is allowed.

2a.	On success, "201 Created" shall be returned, the payload body of the POST response shall contain the representation describing the status of the request and the "Location" header shall be present and shall contain the URI of the created resource. The authority and/or deployment-specific string of the apiRoot of the created resource URI may differ from the authority and/or deployment-specific string of the apiRoot of the request URI received in the POST request.

If the requestType IE was received in the request and set to EXISTING_PDU_SESSION or EXISTING_EMERGENCY_PDU_SESSION (i.e. indicating that this is a UE request for an existing PDU session or an existing emergency PDU session), the SMF shall identify the existing PDU session or emergency PDU session based on the PDU Session ID; in this case, the SMF shall not create a new SM context but instead update the existing SM context and provide the representation of the updated SM context in the "201 Created" response to the NF Service Consumer.

The POST request shall be considered as colliding with an existing SM context if:

-	this is a request to establish a new PDU session, i.e.:

-	the RequestType IE is present in the request and set to INITIAL_REQUEST or INITIAL_EMERGENCY_REQUEST (e.g. single access PDU session establishment request);

-	the RequestType IE and the maRequestInd IE are both absent in the request (e.g. EPS to 5GS mobility); or

-	the maRequestInd IE is present in the request (i.e. MA-PDU session establishment request) and the access type indicated in the request corresponds to the access type of the existing SM context.

and either of the following conditions is met:

-	this is a request to establish a non-emergency PDU session and the request includes the same SUPI and the same PDU Session ID as for an existing SM context; or

-	this is a request to establish an emergency PDU session and the request includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, as for an existing SM context for an emergency PDU session.

A POST request that collides with an existing SM context shall be treated as a request for a new SM context. Before creating the new SM context, the SMF should delete the existing SM context locally and any associated resources in the UPF, PCF, CHF, and UDM. If the UP connection of the existing PDU session is active, the SMF should also request (R)AN to release associated resources. See also clause 5.2.3.3.1 for the handling of requests which collide with an existing SM context. If the smContextStatusUri of the existing SM context differs from the smContextStatusUri received in the POST request, the SMF shall also send an SM context status notification (see clause 5.2.2.5) targeting the smContextStatusUri of the existing SM context to notify the release of the existing SM context. For a HR PDU session, if the H-SMF URI in the request is different from the H-SMF URI of the existing PDU session, the V-SMF should also delete the existing PDU session in the H-SMF by invoking the Release service operation (see clause 5.2.2.9). For a PDU session with an I-SMF, if the SMF URI in the request is different from the SMF URI of the existing PDU session, the I-SMF should also delete the existing PDU session in the SMF by invoking the Release service operation (see clause 5.2.2.9).

If the requestType IE was received in the request and indicates this is a request for a new PDU session (i.e. INITIAL_REQUEST) and if the Old PDU Session ID was also included in the request, the SMF shall identify the existing PDU session to release and to which the new PDU session establishment relates, based on the Old PDU Session ID.

If no GPSI IE is provided in the request, e.g. for a PDU session moved from another access or another system, and the SMF knows that a GPSI is already associated with the PDU session (or a GPSI is received from h-SMF for a HR PDU session), the SMF shall include the GPSI in the response.

If the indication of Notification for SM Policy Association events was received with the value "true" together with the callback information of the PCF for the UE in the request and SM Policy Association is to be established for the PDU session, the SMF shall provide the callback information of the PCF for the UE to the PCF for SM Policy during SM Policy Association Establishment.

2b.	If the request does not include the "UE presence in LADN service area" indication and the SMF determines that the DNN corresponds to a LADN, then the SMF shall consider that the UE is outside of the LADN service area. The SMF shall reject the request if the UE is outside of the LADN service area.

On failure, or redirection during a UE requested PDU Session Establishment, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain an SmContextCreateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3;

-	N1 SM information (PDU Session Reject), if the request included N1 SM information, except if the error prevents the SMF from generating a response to the UE (e.g. invalid request format).

For the UE requested PDU Session Establishment, the SMF shall reject the request with "EXCEEDED_SLICE_DATA_RATE" application error if the SMF receives from the PCF that the maximum bit rate per S-NSSAI is exceeded, or with "EXCEEDED_UE_SLICE_DATA_RATE" application error if the SMF receives from the PCF that the maximum bit rate per S-NSSAI per UE is exceeded.

# *** Next Change ***

##### 5.2.2.6.1	General

The Retrieve SM Context service operation shall be used to retrieve an individual SM context, for a given PDU session, from the (H-)SMF, from the V-SMF during change or removal of V-SMF, or from the I-SMF during change or removal of I-SMF.

It is used in the following procedures:

-	5GS to EPS handover using N26 interface (see clause 4.11.1.2.1 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	5GS to EPS Idle mode mobility using N26 interface (see clause 4.11.1.3.2 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	UE Triggered Service Request with I-SMF insertion/change/removal or with V-SMF insertion/change/removal (see clause 4.23.4.3 of of 3GPP TS 23.502 [3]);

-	Xn based inter NG-RAN handover with insertion of intermediate SMF (see clause 4.23.11 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	Inter NG-RAN node N2 based handover, preparation phase, with I-SMF or V-SMF insertion/change (see clause 4.23.7.3.2 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	I-SMF selection per DNAI (see clause 4.23.5.4 of 3GPP TS 23.502 [3]);

-	Change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions (see clause 4.3.5.2 of 3GPP TS 23.502 [3]).

-	N2 Handover with V-SMF insertion/change/removal in HR-SBO case (see clause 6.7.2.6 of 3GPP TS 23.548 [39]).

-	Inter V-SMF mobility registration update procedure in HR-SBO case (see clause 6.7.2.7 of 3GPP TS 23.548 [39]).

-	Xn Handover with V-SMF change in HR-SBO case (see clause 6.7.2.9 of 3GPP TS 23.548 [39]).

The NF Service Consumer (e.g. AMF or SMF) shall retrieve an SM context by using the HTTP POST method (retrieve custom operation) as shown in Figure 5.2.2.6.1-1.

Figure 5.2.2.6.1-1: SM context retrieval

1.	The NF Service Consumer shall send a POST request to the resource representing the individual SM context to be retrieved. The POST request may contain a payload body with the following parameters:

-	target MME capabilities, if available, to allow the SMF to determine whether to include EPS bearer contexts for Ethernet PDN Type, non-IP PDN type, or requiring UP integrity protection or not;

-	SM context type:

-	indicating that this is a request to retrieve the complete SM context (i.e. 5G SM context including EPS context information as defined in clause 6.1.6.2.39), during scenarios with an I-SMF or V-SMF insertion/change/removal or SMF Context Transfer procedure; or

-	indicating that this is a request to retrieve the AF Coordination Information as defined in clause 6.1.6.2.69, during the change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions, if the runtime coordination between old SMF and AF is enabled.

-	serving core network operator PLMN ID of the new V-SMF, when the procedure is triggered by a new V-SMF, if the new V-SMF supports inter-PLMN V-SMF change or insertion. Or the serving core network operator PLMN ID of the new I-SMF during the procedure with an I-SMF insertion;

-	notToTransferEbiList IE, if the SM context type IE is absent or indicate a request to retrieve the EPS PDN connection, to request the SMF to not transfer EPS bearer context(s) corresponding to EBIs in the list, during an 5GS to EPS mobility when the target MME does not support 15 EPS bearers;

-	ranUnchangedInd IE, if the NG-RAN Tunnel info is required in scenario of I-SMF/V-SMF change/insertion during registration procedure after EPS to 5GS handover or I-SMF selection/removal per DNAI, when the UE is in CM-CONNECTED state as specified in clauses 5.2.2.2.7 and 5.2.2.2.12.

-	hrsboSupportInd IE, if the procedure is triggered by a target V-SMF supporting the HR-SBO feature.

-	storedOffloadIds IE, if the target V-SMF has a list of stored offload identifiers from the previous procedures for the HPLMN of the PDU session.

2a.	On success, "200 OK" shall be returned; the payload body of the POST response shall contain the mapped EPS bearer contexts if this is a request for the UE EPS PDN connection, or the complete SM context if this is a request for retrieving the complete SM context, or the AF Coordination Information if this is a request for retrieving the AF Coordination Information.

If this is a request for the UE EPS PDN connection and the target MME capabilities were provided in the request parameters:

-	if the target MME supports the non-IP PDN type, the SMF shall return, for a PDU session with PDU session type "Unstructured", an EPS bearer context with the "non-IP" PDN type;

-	if the target MME supports the Ethernet PDN type, the SMF shall return, for a PDU session with PDU session type "Ethernet", an EPS bearer context with the "Ethernet" PDN type;

-	if the target MME does not support the Ethernet PDN type but supports the non-IP PDN type, the SMF shall return, for a PDU session with PDU session type "Ethernet", an EPS bearer context with the "non-IP" PDN type.

If the notToTransferEbiList IE was included in the request, the SMF shall not provide EPS bearer context(s) corresponding to EBIs in the list.

If this is a request for retrieving the complete SM context and there are downlink data packets buffered at I-UPF, the SMF shall include the "forwardingInd" attribute with value "true" in the response body to indicate downlink data packets are buffered at the I-UPF. The NF Service Consumer receiving the "forwardingInd" attribute with the value "true" shall setup a forwarding tunnel for receiving the buffered downlink data packets.

If this is a request for retrieving the complete SM context for an inter-PLMN V-SMF change, i.e. if the request contains the serving core network operator PLMN ID indicating a different PLMN than the PLMN of the SMF (acting as the old V-SMF), the latter shall not include the chargingInfo IE and the roamingChargingProfile IE in the SM context returned in the response.

During a procedure with an I-SMF or V-SMF insertion, the anchor SMF should use the servingNetwork IE received in the Retrieve SM Context Request to determine whether the inserted entity is an I-SMF or V-SMF, and if so, encode in the SM Context returned in the response the applicable set of attributes (e.g. hsmfUri, hSmfInstanceId, hSmfServiceInstanceId to a V-SMF, or smfUri, smfInstanceId, smfServiceInstanceId to an I-SMF) and the applicable URI in the pduSessionRef if different URIs are used for intra-PLMN and inter-PLMN signaling requests targeting the PDU session context.

NOTE 1:	During an inter-PLMN procedure with an I-SMF or V-SMF change, the old V-SMF or I-SMF returns the attributes of the SM context as were received from the anchor SMF.

If the UE, target MME and AMF support User Plane integrity protection with EPS, the SMF shall include the UP Security Policy IE in the UE EPS PDN connection context if User Plane integrity protection has been enabled by the SMF as specified in clauses 4.11.1.2.1 and 4.11.1.3.2 of 3GPP TS 23.502 [3].

NOTE 2:	During a 5GS to EPS handover, if User Plane Integrity Protection is used in 5GS and QoS flows are associated with EPS bearer IDs, the I-SMF/V-SMF considers that the UE supports User Plane Integrity Protection in EPS.

If this is a request for retrieving the complete SM context for an I-SMF or V-SMF insertion, and the smfUri IE or hSmfUri IE is provided by the AMF in the Create SM Context request and is different from the smfUri IE or hSmfUri IE in the SM context returned in the Retrieve SM Context response, the latter (i.e. the IEs received in the Retrieve SM Context response) shall prevail and be used by the I-SMF or V-SMF to trigger the create service operation to the (H-)SMF.

If the target V-SMF supports the HR-SBO feature, for a HR-SBO session, the content of the POST response message may also contain the HR-SBO related information, e.g., hrsboAuthResult, hDnsAddr, hPlmnAddr, vplmnOffloadingInfoList.

The source V-SMF shall determine if the target V-SMF has already received the corresponding VPLMN Offloading Information for an offload identifier based on the storedOffloadIds for the HPLMN of the PDU session provided by the target V-SMF in the Retrieve SM context request message for an intra PLMN V-SMF change, the source V-SMF shall include:

-	the Offload Identifier(s) only, if the Offload Identifier(s) are known by the target V-SMF, i.e., the Offload Identifier(s) are part of the storedOffloadIds; otherwise

-	a list of vplmnOffloadingInfo attributes where each vplmnOffloadingInfo contains an offload identifier and the corresponding VPLMN Offloading Information.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.4.4.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.4.4.2-2.

If the EBI value of the QoS Flow associated with the default QoS Rule is included in the notToTransferEbiList IE, the SMF shall set the "cause" attribute in the ProblemDetails structure to "DEFAULT_EBI_NOT_TRANSFERRED".

If a request for the UE EPS PDN connection is rejected due to the target MME not being capable to support the PDU session, e.g. if the PDU session requires UP integrity protection but the target MME does not support User Plane Integrity Protection with EPS, the SMF shall return a 403 Forbidden response with the "cause" attribute in the ProblemDetails structure set to " TARGET_MME_CAPABILITY".

# *** Next Change ***

##### 5.2.2.7.1	General

The Create service operation shall be used to create an individual PDU session in the H-SMF for HR roaming scenarios, or in the SMF for PDU sessions involving an I-SMF.

It is used in the following procedures:

-	UE requested PDU Session Establishment for home-routed roaming or with an I-SMF (see clauses 4.3.2.2.2 and 4.23.5.1 of 3GPP TS 23.502 [3]);

-	when an I-SMF is inserted during the Registration, Service Request, Inter NG-RAN node N2 based handover, Xn based handover and Handover from non-3GPP to 3GPP access procedures (see clauses 4.23.3, 4.23.4, 4.23.7.3, 4.23.11.2 and 4.23.16 of 3GPP TS 23.502 [3]);

-	when a V-SMF is inserted during the Registration and Inter NG-RAN node N2 based handover (see clauses 4.23.3 and 4.23.7.3 of 3GPP TS 23.502 [3]);

-	EPS to 5GS Idle mode mobility or handover using N26 interface (see clauses 4.11, 4.23.12.3, 4.23.12.5 and 4.23.12.7 of 3GPP TS 23.502 [3]);

-	EPS to 5GS mobility without N26 interface (see clause 4.11.2.3 of 3GPP TS 23.502 [3]);

-	Handover of a PDU session between 3GPP access and non-3GPP access, when the target AMF does not know the SMF resource identifier of the SM context used by the source AMF, e.g. when the target AMF is not in the PLMN of the N3IWF (see clause 4.9.2.3.2 of 3GPP TS 23.502 [3]);

-	Handover from EPS to 5GC-N3IWF (see clause 4.11.3.1 of 3GPP TS 23.502 [3]);

-	Handover from EPC/ePDG to 5GS (see clause 4.11.4.1 of 3GPP TS 23.502 [3]);

-	Support of Network Slice Replacement (see clause 5.15.19 of 3GPP TS 23.501 [2]).

-	PDU Session establishment for supporting HR-SBO in VPLMN (see clause 6.7.2.2 of 3GPP TS 23.548 [39]).

The NF Service Consumer (e.g. V-SMF or I-SMF) shall create a PDU session in the SMF (i.e. H-SMF for a HR PDU session, or SMF for a PDU session involving an I-SMF) by using the HTTP POST method as shown in Figure 5.2.2.7.1-1.

Figure 5.2.2.7.1-1: PDU session creation

1.	The NF Service Consumer shall send a POST request to the resource representing the PDU sessions collection resource of the SMF. The payload body of the POST request shall contain:

-	a representation of the individual PDU session resource to be created;

-	the requestType IE, if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing Emergency PDU session; the requestType IE shall not be included for a MA-PDU session establishment request; it may be included otherwise;

-	the indication that a MA-PDU session is requested if a MA-PDU session is requested to be established by the UE, or the indication that the PDU session is allowed to be upgraded to a MA PDU session if the UE indicated so;

-	the n3gPathSwitchSupportInd IE if the AMF, V-SMF and the UE support non-3GPP access path switching as specified in clause 4.22.2.2.1 of 3GPP TS 23.502 [3];

-	the vsmfId IE or ismfId IE identifying the V-SMF or I-SMF respectively;

-	the cpCiotEnabled IE with the value "True", if Control Plane CIoT 5GS Optimisation is enabled for this PDU session;

-	the cpOnlyInd IE with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation;

-	the Invoke NEF indication with the value "True", if the cpCiotEnabled IE is set to "True" and data delivery via NEF is selected for the PDU session;

-	the vcnTunnelInfo IE or icnTunnelInfo IE with the N9 tunnel information of the UPF controlled by the V-SMF or I-SMF respectively, except for EPS to 5GS handover using N26 interface and when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session;

-	the additionalCnTunnelInfo IE with additional N9 tunnel information, if a MA PDU session is requested or if the PDU session is allowed to be upgraded to a MA PDU session, and if the UE is registered over both 3GPP and Non-3GPP accesses;

-	the alternative S-NSSAI, if the NF service consumer and UE support the network slice replacement and it is requested to replace the S-NSSAI with the alternative S-NSSAI (see clause 5.15.19 of 3GPP TS 23.501 [2]);

-	the anType IE, indicating the access network type (3GPP or non-3GPP access) associated to the PDU session;

-	the additionalAnType IE indicating an additional access network type associated to the PDU session, for a MA PDU session, if the UE is registered over both 3GPP and Non-3GPP accesses;

-	the n9ForwardingTunnelInfo IE indicating the allocated N9 tunnel endpoints information for receiving the buffered downlink data packets, when downlink data packets are buffered at I-UPF controlled by the SMF during I-SMF insertion;

-	a callback URI ({vsmfPduSessionUri} or {ismfPduSessionUri}) representing the PDU session resource in the V-SMF or I-SMF. The SMF shall construct the callback URIs based on the received {vsmfPduSessionUri} or {ismfPduSessionUri} as defined in clause 6.1, e.g. the callback URI "{vsmfPduSessionUri}/modify" to modify a PDU session in the V-SMF;

-	the list of DNAIs supported by the I-SMF, for a PDU session with an I-SMF;

-	the QoS constraints from the VPLMN for the QoS Flow associated with the default QoS rule and/or for the Session-AMBR if any, for the HR PDU session, if the VQOS feature is supported by the V-SMF;

-	the upipSupported IE set to "true", if the UE supports User Plane Integrity Protection with EPS and if the AMF supports the related functionality.

The payload body of the POST request may further contain:

-	the satelliteBackhaulCat IE indicating the category of the satellite backhaul used towards the 5G AN serving the UE, if the V-SMF/I-SMF received this information from the AMF;

-	the disasterRoamingInd IE set to true during the PDU session establishment if the UE is registered for Disaster Roaming service.

-	the hrsboInfo IE including the HR-SBO information sent from the VPLMN when the V-SMF requests HR SBO authorization.

-	the ecsAddrConfigInfos IE including the ECS Address Configuration information Parameters received from the NEF.

As specified in clause 4.3.2.2.2 of 3GPP TS 23.502 [3], the NF Service Consumer shall be able to receive an Update request before receiving the Create Response, e.g. for EPS bearer ID allocation (see clause 4.11.1.4.1 of 3GPP TS 23.502 [3]) or Secondary authorization/authentication (see clause 4.3.2.3 of 3GPP TS 23.502 [3]).

NOTE:	If the H-SMF supports the VQOS feature, when QoS constraints are received from the VPLMN and PCF is deployed, the H-SMF provides the QoS constraints from the VPLMN to the PCF; otherwise, in case dynamic PCC is not deployed, the SMF takes them into account when generating the default QoS rule.

2a.	On success, "201 Created" shall be returned, the payload body of the POST response shall contain:

-	the representation describing the status of the request;

-	the QoS flow(s) to establish for the PDU session, except when Control Plane CIoT 5GS Optimisation is enabled for this PDU session;

-	the epsPdnCnxInfo IE and, for each EPS bearer, an epsBearerInfo IE, if the PDU session is associated to (or handed over to) the 3GPP access type and may be moved to EPS during its lifetime;

-	a MA PDU Session Accepted indication, if a MA PDU session is established;

-	the smallDataRateControlEnabled indication set to "true" if small data rate control is applicable on the PDU session;

-	the "Location" header containing the URI of the created resource;

-	the n3gPathSwitchSupportInd IE if the n3gPathSwitchSupportInd was set to true in the request and the SMF supports non-3GPP access path switching as specified in clause 4.22.2.2.1 of 3GPP TS 23.502 [3].

Editor' Note: The need to populate n3gPathSwitchSupportInd between the V-SMF and the H-SMF is FFS.

The payload body of the POST response may further contain:

-	the pending update information list, indicating the information elements whose change(s) are not required to be updated in real-time to the (H-)SMF, i.e. the change(s) of the indicated information elements may be piggybacked in a subsequent update to the (H-)SMF together with other information elements.

The payload body of the POST response may also contain the upSecurity, maxIntegrityProtectedDataRateUl and maxIntegrityProtectedDataRateDl IEs, if the PDU session is associated to (or handed over to) the 3GPP access type.

The SMF may provide alternative QoS profiles for each GBR QoS flow with Notification control enabled, to allow the NG-RAN to accept the setup of the QoS flow if the requested QoS parameters or at least one of the alternative QoS parameters sets can be fulfilled at the time of setup.

The authority and/or deployment-specific string of the apiRoot of the created resource URI may differ from the authority and/or deployment-specific string of the apiRoot of the request URI received in the POST request.

If an Update Request was sent to the NF Service Consumer before the Create Response, the URI in the "Location" header and in the hsmfPduSessionUri IE (carrying the PDU session resource URI of a HR PDU session or a PDU session with an I-SMF) of the SMF initiated Update Request shall be the same. If the requestType IE was received in the request and set to EXISTING_PDU_SESSION or EXISTING_EMERGENCY_PDU_SESSION (i.e. indicating that this is a UE request for an existing PDU session or an existing emergency PDU session), the SMF shall identify the existing PDU session or emergency PDU session based on the PDU Session ID; in this case, the SMF shall not create a new PDU session or emergency PDU session but instead update the existing PDU session or emergency PDU session and provide the representation of the updated PDU session or emergency PDU session in the response to the NF Service Consumer.

The POST request shall be considered as colliding with an existing PDU session context if:

-	this is a request to establish a new PDU session, i.e.:

-	the RequestType IE is present in the request and set to INITIAL_REQUEST or INITIAL_EMERGENCY_REQUEST (e.g. single access PDU session establishment request);

-	the RequestType IE and the maRequestInd IE are both absent in the request (e.g. EPS to 5GS mobility); or

-	the maRequestInd IE is present in the request (i.e. MA-PDU session establishment request) and the access type indicated in the request corresponds to the access type of the existing PDU session context.

and either of the following conditions is met:

-	this is a request to establish a non-emergency PDU session and the request includes the same SUPI and the same PDU Session ID as for an existing PDU session context; or

-	this is a request to establish an emergency PDU session and the request includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, as for an existing PDU session context for an emergency PDU session.

A POST request that collides with an existing PDU session context shall be treated as a request for a new PDU session context. The SMF shall assign a new PDU session reference, i.e. {pduSessionRef} (see clause 6.1.3.6.2), which is different from the existing PDU session context. Before creating the new PDU session context, the SMF should delete the existing PDU session context locally and any associated resources in the UPF, PCF, CHF and UDM. See also clause 5.2.3.3.1 for the handling of requests which collide with an existing PDU session context. If the vsmfPduSessionUri or ismfPduSessionUri of the existing PDU session context differs from the vsmfPduSessionUri or ismfPduSessionUri received in the POST request, the SMF shall also send a status notification (see clause 5.2.2.10) targeting the vsmfPduSessionUri or ismfPduSessionUri of the existing PDU session context to notify the release of the existing PDU session context. The SMF should include a cause IE with value "REL_DUE_TO_DUPLICATE_SESSION_ID" in such a status notification. Upon receipt of such a status notification, the V-SMF or I-SMF shall not send SM context status notification to the AMF.

If the requestType IE was received in the request and indicates this is a request for a new PDU session (i.e. INITIAL_REQUEST) and if the Old PDU Session ID was also included in the request, the SMF shall identify the existing PDU session to be released and to which the new PDU session establishment relates, based on the Old PDU Session ID.

The NF Service Consumer shall store any epsPdnCnxInfo and EPS bearer information received from the SMF.

If the response received from the SMF contains the alwaysOnGranted attribute set to true, the NF Service Consumer shall check and determine whether the PDU session can be established as an always-on PDU session based on local policy.

If no GPSI IE is provided in the request, e.g. for a PDU session moved from another access or another system, and the SMF knows that a GPSI is already associated with the PDU session, the SMF shall include the GPSI in the response.

If one or more requested QoS flow(s) fail to be established, the V-SMF or I-SMF shall send an Update Request including the qosFlowsRelNotifyList attribute to report the failure to the H-SMF or SMF (see clause 5.2.2.8.2.2), or a Release Request to release the PDU session if no QoS flow can be established (see clause 5.2.2.9).

For UE mobility with I-SMF/V-SMF insertion procedure, if a requested functionality is not supported for a PDU session with an I-SMF/V-SMF, the SMF shall accept the POST request and release the PDU Session after the mobility procedure, as specified in clause 4.23.1 of 3GPP TS 23.502 [3].

For a HR-SBO session, the content of the POST response may also contain the hrsboInfo IE including the HR-SBO information sent from the HPLMN, e.g., hrsboAuthResult.

2b.	On failure, or redirection during a UE requested PDU Session Establishment, one of the HTTP status code listed in Table 6.1.3.5.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a PduSessionCreateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.5.3.1-3. The application error shall be set to "NOT_SUPPORTED_WITH_ISMF" during a UE requested PDU Session Establishment, if a requested functionality is not supported for a PDU session with an I-SMF/V-SMF.

-	the n1SmCause IE with the 5GSM cause that the SMF proposes the NF Service Consumer to return to the UE, if the request included n1SmInfoFromUe;

-	n1SmInfoToUe with any information to be sent to the UE (in the PDU Session Establishment Reject).

# *** Next Change ***

###### 5.2.2.8.2.1	General

The NF Service Consumer (i.e. the V-SMF for a HR PDU session, or the I-SMF for a PDU session with an I-SMF) shall update a PDU session in the H-SMF or SMF and/or provide the H-SMF or SMF with information received by the  NF Service Consumer in N1 SM signalling from the UE, by using the HTTP POST method (modify custom operation) as shown in Figure 5.2.2.8.2-1.

Figure 5.2.2.8.2-1: PDU session update towards H-SMF or SMF

1.	The NF Service Consumer shall send a POST request to the resource representing the individual PDU session resource in the H-SMF or SMF. The payload body of the POST request shall contain:

-	the requestIndication IE indicating the request type. Unless specified otherwise in clause 5.2.2.8.2, the value of the requestIndication IE shall be set to NW_REQ_PDU_SES_MOD;

-	the modification instructions and/or the information received by the NF Service Consumer in N1 signalling from the UE.

The NF service consumer shall not include the hoPreparationIndication IE with the value "false" in procedures other than handover execution, cancel and failure procedures.

If the pending update information list was previously received from the (H-)SMF, the NF service consumer should not send a request to the (H-)SMF including only the information elements indicated by the pending update information list. The change(s) of the indicated information elements may be piggybacked in a subsequent update to the (H-)SMF together with other information elements.

For a HR-SBO session, the content of the POST request may further contain:

-	the hrsboInfo IE including the HR-SRO information sent from the VPLMN when the V-SMF requests HR SBO authorization.

-	the ecsAddrConfigInfos IE including the ECS Address Configuration information if the information is modified by the NEF.

2a.	On success, "204 No Content" or "200 OK" shall be returned; in the latter case, the payload body of the POST response shall contain the representation describing the status of the request and/or information necessary for the NF Service Consumer to send N1 SM signalling to the UE. If the PDU session may be moved to EPS with N26 and the EPS PDN Connection Context information of the PDU session is changed, e.g. due to a new anchor SMF is reselected, the payload shall include the "epsPdnCnxInfo" IE including the updated EPS PDN Connection Context information. The NF Service consumer shall overwrite the locally stored EPS PDN Connection Context information with the new one if received.

For a HR-SBO session, the content of the POST response may also contain the hrsboInfo IE including the HR-SBO information sent from the HPLMN, e.g., hrsboAuthResult.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.1.3.6.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain an HsmfUpdateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.6.4.2.2-2;

-	the n1SmCause IE with the 5GSM cause the H-SMF or SMF proposes the NF Service Consumer to return to the UE, if the request included n1SmInfoFromUe;

-	n1SmInfoToUe binary data, if the H-SMF or SMF needs to return NAS SM information which the NF Service Consumer does not need to interpret;

-	the procedure transaction id that was received in the request, if this is a response sent to a UE requested PDU session modification.

If the H-SMF or SMF receives the hoPreparationIndciation IE set to "false" value in step 1, while it did not receive the hoPreparationIndication IE set to "true" value in previous steps (see clauses 5.2.2.7.3 and 5.2.2.7.4), the H-SMF or SMF shall ignore the hoPreparationIndication IE with "false" value and proceed with the processing of the request.

# *** Next Change ***

###### 5.2.2.8.3.1	General

The NF Service Consumer (i.e. the H-SMF for a HR PDU session, or the SMF for a PDU session with an I-SMF) shall update a PDU session in the V-SMF or I-SMF and/or provide information necessary for the V-SMF or I-SMF to send N1 SM signalling to the UE, or request to allocate or revoke EPS Bearer ID(s) for the PDU session, by using the HTTP "modify" custom operation as shown in Figure 5.2.2.8.3.1-1.

Figure 5.2.2.8.3.1-1: PDU session update towards V-SMF or I-SMF

1.	The NF Service Consumer shall send a POST request to the resource representing the individual PDU session resource in the V-SMF or I-SMF. The payload body of the POST request shall contain:

-	the requestIndication IE indicating the request type, which is set to NW_REQ_PDU_SES_MOD;

-	the modification instructions and/or the information necessary for the V-SMF or I-SMF to send N1 SM signalling to the UE;

-	the hsmfPduSessionUri IE if the Update Request is sent to the V-SMF or I-SMF before the Create Response, and the H-SMF or SMF PDU session resource URI has not been previously provided to the V-SMF or I-SMF; in this case, the supportedFeatures IE shall also be included if at least one optional feature defined in clause 6.1.8 is supported.

The payload body of the POST request may further contain:

-	the (updated) pending update information list.

If the PDU session may be moved to EPS with N26 and the EPS PDN Connection Context information of the PDU session is changed, e.g. due to a new anchor SMF is reselected, the payload shall include the "epsPdnCnxInfo" IE including the updated EPS PDN Connection Context information. The NF Service consumer shall overwrite the locally stored EPS PDN Connection Context information with the new one if received.

If the service operation is invoked after the reselection of the anchor SMF and the change of anchor SMF has not been signalled to the V-SMF or I-SMF previously, the request may carry the SMF Instance ID of the new anchor SMF and the updated PDU session resource URI in the HTTP payload body, and/or carry an updated binding indication in the HTTP headers to indicate the change of anchor SMF (as per step 6 of clause 6.5.3.3 of 3GPP TS 29.500 [4]).

For a HR-SBO session, the content of the POST request may also contain the hrsboInfo IE including the HR-SBO information sent from the HPLMN, e.g., due to a change of the user subscription data or policy information. If the VPLMN Specific Offloading Information for a given Offload Identifier is changed, for each V-SMF service instance using the Offload Identifier, the H-SMF shall choose one existing HR-SBO PDU Session served by the V-SMF service instance and using the Offload Identifier to update VPLMN Specific Offloading Information and corresponding Offload Identifier to the V-SMF service instance.

2a.	On success, "204 No Content" or "200 OK" shall be returned; in the latter case, the payload body of the POST response shall contain the representation describing the status of the request and/or information received by the V-SMF or I-SMF in N1 signalling from the UE.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.7.4.2.2-1 shall be returned. For a 4xx/5xx response, the message body shall contain a VsmfUpdateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.7.4.2.2-1;

-	the n1SmCause IE with the 5GSM cause returned by the UE, if available;

-	n1SmInfoFromUe and/or unknownN1SmInfo binary data, if NAS SM information has been received from the UE that needs to be transferred to the H-SMF or SMF, or that the V-SMF or I-SMF does not comprehend;

-	the procedure transaction id received from the UE, if available.

# *** Next Change ***

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service-based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Data within Create SM Context Request |
| SmContextCreatedData | 6.1.6.2.3 | Data within Create SM Context Response |
| SmContextUpdateData | 6.1.6.2.4 | Data within Update SM Context Request |
| SmContextUpdatedData | 6.1.6.2.5 | Data within Update SM Context Response |
| SmContextReleaseData | 6.1.6.2.6 | Data within Release SM Context Request |
| SmContextRetrieveData | 6.1.6.2.7 | Data within Retrieve SM Context Request |
| SmContextStatusNotification | 6.1.6.2.8 | Data within Notify SM Context Status Request |
| PduSessionCreateData | 6.1.6.2.9 | Data within Create Request |
| PduSessionCreatedData | 6.1.6.2.10 | Data within Create Response |
| HsmfUpdateData | 6.1.6.2.11 | Data within Update Request towards H-SMF, or from I-SMF to SMF |
| HsmfUpdatedData | 6.1.6.2.12 | Data within Update Response from H-SMF, or from SMF to I-SMF |
| ReleaseData | 6.1.6.2.13 | Data within Release Request |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF, or from SMF to I-SMF |
| VsmfUpdateData | 6.1.6.2.15 | Data within Update Request towards V-SMF, or from SMF to I-SMF |
| VsmfUpdatedData | 6.1.6.2.16 | Data within Update Response from V-SMF, or from I-SMF to SMF |
| StatusNotification | 6.1.6.2.17 | Data within Notify Status Request |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |
| SmContextRetrievedData | 6.1.6.2.27 | Data within Retrieve SM Context Response |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF, or from I-SMF to SMF |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF, or from SMF to I-SMF |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF, or from SMF to I-SMF |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |
| SmContext | 6.1.6.2.39 | Complete SM Context |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |
| PsaInformation | 6.1.6.2.41 | PSA Information |
| DnaiInformation | 6.1.6.2.42 | DNAI Information |
| N4Information | 6.1.6.2.43 | N4 Information |
| IndirectDataForwardingTunnelInfo | 6.1.6.2.44 | Indirect Data Forwarding Tunnel Information |
| SmContextReleasedData | 6.1.6.2.45 | Data within Release SM Context Response |
| ReleasedData | 6.1.6.2.46 | Data within Release Response |
| SendMoDataReqData | 6.1.6.2.47 | Data within Send MO Data Request |
| CnAssistedRanPara | 6.1.6.2.48 | SMF derived CN assisted RAN parameters tuning |
| UlclBpInformation | 6.1.6.2.49 | UL CL or BP Information |
| TransferMoDataReqData | 6.1.6.2.50 | Data within Transfer MO Data Request |
| TransferMtDataReqData | 6.1.6.2.51 | Data within Transfer MT Data Request |
| TransferMtDataError | 6.1.6.2.52 | Transfer MT Data Error Response |
| TransferMtDataAddInfo | 6.1.6.2.53 | Transfer MT Data Error Response Additional Information |
| VplmnQos | 6.1.6.2.54 | VPLMN QoS |
| DdnFailureSubs | 6.1.6.2.55 | DDN Failure Subscription |
| RetrieveData | 6.1.6.2.56 | Data within Retrieve Request |
| RetrievedData | 6.1.6.2.57 | Data within Retrieve Response |
| SecurityResult | 6.1.6.2.58 | Security Result |
| UpSecurityInfo | 6.1.6.2.59 | User Plane Security Information |
| DdnFailureSubInfo | 6.1.6.2.60 | DDN Failure Subscription Information |
| AlternativeQosProfile | 6.1.6.2.61 | Alternative QoS Profile |
| ProblemDetailsAddInfo | 6.1.6.2.62 | Problem Details Additional Information |
| ExtProblemDetails | 6.1.6.2.63 | Extended Problem Details |
| QosMonitoringInfo | 6.1.6.2.64 | QoS Monitoring Information |
| IpAddress | 6.1.6.2.65 | IP Address |
| RedundantPduSessionInformation | 6.1.6.2.66 | Redundant PDU Session Information |
| QosFlowTunnel | 6.1.6.2.67 | Tunnel Information per QoS Flow |
| TargetDnaiInfo | 6.1.6.2.68 | Target DNAI Information |
| AfCoordinationInfo | 6.1.6.2.69 | AF Coordination Information |
| NotificationInfo | 6.1.6.2.70 | Notification Correlation ID and Notification URI provided by the NF service consumer |
| AnchorSmfFeatures | 6.1.6.2.71 | Anchor SMF supported features |
| HrsboInfoFromVplmn | 6.1.6.2.72 | HR-SBO Information signaled from VPLMN |
| HrsboInfoFromHplmn | 6.1.6.2.73 | HR-SBO Information signaled from HPLMN |
| Teid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |
| EpsBearerContextStatus | 6.1.6.3.2 | EPS Bearer context status |
| DrbId | 6.1.6.3.2 | Data Radio Bearer Identifier |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |
| HoState | 6.1.6.3.4 | Handover State |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |
| Cause | 6.1.6.3.8 | Cause information |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication |
| SmContextType | 6.1.6.3.15 | Type of SM Context information |
| PsaIndication | 6.1.6.3.16 | Indication of whether a PSA is inserted or removed |
| N4MessageType | 6.1.6.3.17 | N4 Message Type |
| QosFlowAccessType | 6.1.6.3.18 | Access type associated with the QoS Flow |
| UnavailableAccessIndication | 6.1.6.3.19 | Indicates the access type of a MA PDU session that is unavailable |
| ProtectionResult | 6.1.6.3.20 | Protection Result of the security policy indicated as "preferred" |
| QosMonitoringReq | 6.1.6.3.21 | Indicates to measure UL, or DL, or both UL/DL delays, or to stop on-going measurements. |
| Rsn | 6.1.6.3.22 | Redundancy Sequence Number |
| SmfSelectionType | 6.1.6.3.23 | SMF Selection Type |
| PduSessionContextType | 6.1.6.3.24 | PDU Session Context Type |
| PendingUpdateInfo | 6.1.6.3.25 | Pending Update Information |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |
| PduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |
| PduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |
| UserLocation | 3GPP TS 29.571 [13] | User Location |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |
| RatType | 3GPP TS 29.571 [13] | RAT Type |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |
| Dnai | 3GPP TS 29.571 [13] | Data Network Access Identifier |
| PlmnIdNid | 3GPP TS 29.571 [13] | PLMN Identity and, for SNPN, Network Identity |
| SmallDataRateStatus | 3GPP TS 29.571 [13] | Small Data Rate Control Status |
| ApnRateStatus | 3GPP TS 29.571 [13] | APN Rate Control Status |
| StationaryIndication | 3GPP TS 29.571 [13] | Stationary Indication |
| ScheduledCommunicationTime | 3GPP TS 29.571 [13] | Scheduled Communication Time |
| ScheduledCommunicationType | 3GPP TS 29.571 [13] | Scheduled Communication Type |
| TrafficProfile | 3GPP TS 29.571 [13] | Traffic Profile |
| BatteryIndication | 3GPP TS 29.571 [13] | Battery Indication |
| NfSetId | 3GPP TS 29.571 [13] | NF Set Identifier |
| MoExpDataCounter | 3GPP TS 29.571 [13] | MO Exception Data Counter |
| DddTrafficDescriptor | 3GPP TS 29.571 [13] | Traffic Descriptor |
| NfServiceSetId | 3GPP TS 29.571 [13] | NF Service Set ID |
| RedirectResponse | 3GPP TS 29.571 [13] | Response body of the redirect response message |
| ServerAddressingInfo | 3GPP TS 29.571 [13] | Information of a Provisioning Server |
| PcfUeCallbackInfo | 3GPP TS 29.571 [13] | The callback information of the PCF for the UE to allow the PCF for the PDU session to send SM Policy Association Establishment and Termination events notification |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [13] | Satellite backhaul category |
| Uint16 | 3GPP TS 29.571 [13] | Unsigned 16-bit integer |
| GeoSatelliteId | 3GPP TS 29.571 [13] | GEO satellite ID (string) |
| Uinteger | 3GPP TS 29.571 [13] | Unsigned Integer common data type |
| VplmnOffloadingInfo | 3GPP TS 29.571 [13] | VPLMN Specific Offloading Information |
| GroupId | 3GPP TS 29.571 [13] | Internal Group Id |
| OffloadIdentifier | 3GPP TS 29.571 [13] | Offload identifier uniquely identifying a VPLMN offloading policy information instance from the HPLMN. |
| EcsAddrConfigInfo | 3GPP TS 29.503 [46] | ECS Address Configuration Information |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |
| WAgfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the W-AGF |
| TngfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TNGF |
| TwifInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TWIF |
| ChargingInformation | 3GPP TS 29.512 [30] | CHF addresses |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |
| SbiBindingLevel | 3GPP TS 29.518 [20] | SBI Binding Level |
| IpIndex | 3GPP TS 29.519 [38] | Information that identifies which IP pool or external server is used to allocate the IP address. |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |

# *** Next Change ***

##### 6.1.6.2.7	Type: SmContextRetrieveData

Table 6.1.6.2.7-1: Definition of type SmContextRetrieveData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| targetMmeCap | MmeCapabilities | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the target MME capabilities. |  |
| smContextType | SmContextType | C | 0..1 | This IE shall be present if this is a request to retrieve the complete SM context, during scenarios with an I-SMF or V-SMF insertion/change/removal, or during SMF Context Transfer procedure for LBO or non-roaming PDU session without I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]). This IE shall also be present if this is a request to retrieve the AF Coordination Information during the change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions, if the runtime coordination between old SMF and AF is enabled (see clause 4.3.5.2 of 3GPP TS 23.502 [3]). | DTSSA, CTXTR EnEDGE |
| servingNetwork | PlmnId | C | 0..1 | This IE shall be present when the procedure is triggered by a new V-SMF, if the new V-SMF supports inter-PLMN V-SMF change or insertion. This IE shall also be present during the procedure with an I-SMF insertion. When present, this IE shall contain the serving core network operator PLMN ID of the NF Service Consumer (i.e. new V-SMF or new I-SMF). | DTSSA |
| notToTransferEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present, if the SM context type IE is absent or indicate a request to retrieve the EPS PDN connection, and the AMF determines that certain EPS bearers shall not to be transferred to EPS during a 5GS to EPS mobility procedure, as specified in clause 4.11.1 of 3GPP TS 23.502 [3]. When present, it shall contain the EBI list not to be transferred. |  |
| ranUnchangedInd | boolean | C | 0..1 | This IE shall be present if AN Tunnel is required, in scenario of I-SMF/V-SMF change/insertion during registration procedure after EPS to 5GS handover or I-SMF selection per DNAI, when UE is in CM-CONNECTED state (see clause 5.2.2.6.1). When present, it shall be set as follows: - true: NG-RAN is not changed and the tunnel information is required; - false (default):NG-RAN is changed and the tunnel information is not required. | DTSSA |
| hrsboSupportInd | boolean | C | 0..1 | This IE shall be present if the procedure is triggered by a new V-SMF which supports the HR-SBO feature. When present, it shall be set as follows: - true: HR-SBO is supported; - false (default): HR-SBO is not supported. |  |
| storedOffloadIds | array(OffloadIdentifier) | C | 1...N | The IE shall be present when the target V-SMF has a list of the stored offload identifiers for the HPLMN of the PDU session from previous procedures (for the same or different PDU sessions) with the HPLMN. When present, this IE shall contain the list of offload identifiers known by the target V-SMF for the HPLMN of the PDU session. (NOTE) | HR-SBO |
| NOTE: The stored VPLMN Offload identifiers shall be included in any subsequent request message to the source V-SMF for other HR-PDU sessions. |  |  |  |  |  |

# *** Next Change ***

##### 6.1.6.2.39	Type: SmContext

Table 6.1.6.2.39-1: Definition of type SmContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pduSessionId | PduSessionId | M | 1 | This IE shall contain the PDU Session ID. |  |
| dnn | Dnn | M | 1 | This IE shall contain the UE requested DNN of the PDU session. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| sNssai | Snssai | M | 1 | This IE shall contain the S-NSSAI for the serving PLMN. |  |
| hplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the S-NSSAI for the HPLMN. |  |
| pduSessionType | PduSessionType | M | 1 | This IE shall indicate the PDU session type. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |  |
| hSmfUri | Uri | C | 0..1 | This IE shall be present in HR roaming scenarios. When present, it shall contain the API URI of the Nsmf_PDUSession service of the H-SMF. The API URI shall be formatted as specified in clause 6.1.1. |  |
| smfUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the API URI of the Nsmf_PDUSession service of the SMF. The API URI shall be formatted as specified in clause 6.1.1. |  |
| pduSessionRef | Uri | C | 0..1 | This IE shall be present for a HR PDU session or a PDU session with an I-SMF. When present, this IE shall include the absolute URI of the PDU Session in H-SMF or SMF, including apiRoot (see clause 6.1.3.6.2) |  |
| interPlmnApiRoot | Uri | C | 0..1 | This IE shall be present, if available. When present, it shall contain the apiRoot of the PDU session context to be used in inter-PLMN signalling request targeting the PDU session context. (NOTE 2) |  |
| intraPlmnApiRoot | Uri | C | 0..1 | This IE shall be present, if available. When present, it shall contain the apiRoot of the PDU session context to be used in intra-PLMN signalling request targeting the PDU session context. (NOTE 2) |  |
| pcfId | NfInstanceId | O | 0..1 | When present, this IE shall contain the identifier of: - the H-PCF selected by the AMF (for UE Policy), for a HR PDU session; or - the V-PCF selected by the AMF (for Access and Mobility Policy), for a PDU session in LBO roaming scenarios; or - the PCF selected by the AMF (for Access and Mobility Policy and/or UE Policy), for a PDU session in non-roaming scenarios. |  |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |  |
| pcfSetId | NfSetId | O | 0..1 | This IE may be present if the pcfId IE is present. When present, it shall contain the NF Set ID of the PCF indicated by the pcfId IE. |  |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |  |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |  |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |  |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE 1) |  |
| sessionAmbr | Ambr | M | 1 | This IE shall contain the Session AMBR granted to the PDU session. |  |
| qosFlowsList | array(QosFlowSetupItem) | M | 1..N | This IE shall contain the set of QoS flow(s) established for the PDU session. It shall contain at least the Qos flow associated to the default Qos rule. The qosRules attribute of each QosFlowSetupItem shall be set to an empty string. |  |
| hSmfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the identifier of the home SMF. |  |
| smfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the identifier of the SMF. |  |
| pduSessionSmfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the home SMF as identified by hSmfInstanceId, or the SMF as identified by the smfInstanceId. |  |
| pduSessionSmfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the PDUSession service instance (for this PDU session) in the home SMF or the SMF. |  |
| pduSessionSmfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the SBI binding level of the PDU session resource in the home SMF or the SMF. |  |
| enablePauseCharging | boolean | C | 0..1 | This IE shall be present for a HR PDU session, if available. When present, it shall indicate whether the use of Pause of Charging is enabled for the PDU session (see clause 4.4.4 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: enable Pause of Charging; - false (default): disable Pause of Charging. |  |
| ueIpv4Address | Ipv4Addr | C | 0..1 | This IE shall be present if a UE IPv4 address to the PDU session. |  |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | This IE shall be present if a UE IPv6 prefix to the PDU session. |  |
| epsPdnCnxInfo | EpsPdnCnxInfo | C | 0..1 | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| epsBearerInfo | array(EpsBearerInfo) | C | 1..N | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| maxIntegrityProtectedDataRate | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for uplink. If the maxIntegrityProtectedDataRateDl IE is absent, this IE applies to both uplink and downlink. |  |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE may be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for downlink. |  |
| alwaysOnGranted | boolean | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether this is an always On PDU session and it shall be set as follows: - true: always-on PDU session granted. - false (default): always-on PDU session not granted. |  |
| upSecurity | UpSecurity | O | 0..1 | When present, this IE shall indicate the security policy for integrity protection and encryption for the user plane of the PDU session. |  |
| hSmfServiceInstanceId | string | O | 0..1 | This IE may be present for a HR PDU session. When present, this IE shall contain the serviceInstanceId of the H-SMF service instance serving the PDU session. This IE may be used by the V-SMF to identify PDU sessions affected by a failure or restart of the H-SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). |  |
| smfServiceInstanceId | string | O | 0..1 | This IE may be present for a PDU session with an I-SMF. When present, this IE shall contain the serviceInstanceId of the SMF service instance serving the PDU session. This IE may be used by the I-SMF to identify PDU sessions affected by a failure or restart of the SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). |  |
| recoveryTime | DateTime | O | 0..1 | This IE may be present if available. When present, this IE shall indicate the timestamp when the H-SMF or SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| forwardingInd | boolean | C | 0..1 | This IE shall be present, when downlink data packets are buffered at I-UPF. The SMF or I-SMF shall use this IE to inform the NF service consumer that a forwarding tunnel is needed for receiving the buffered downlink data packets, as specified in clause 4.23.4 of 3GPP TS 23.502 [3]. When present, this IE shall be set as follows: - true: a forwarding tunnel is needed for sending buffered downlink data packets; - false (default): forwarding tunnel is not needed |  |
| psaTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if available. When present, this IE shall contain the N9 tunnel information of PDU Session Anchor UPF controlled by SMF or H-SMF. |  |
| chargingId | string | C | 0..1 | This IE shall be present for a HR PDU session, in scenarios with a V-SMF insertion/change/removal. When present, it shall contain the Charging ID of the PDU session (see 3GPP TS 32.255 [25]). The string shall encode the Charging ID (32-bit unsigned integer value, with maximum value "4294967295") in decimal representation. Pattern: '^(0¦([1-9]{1}[0-9]{0,9}))$' (NOTE 4) |  |
| chargingInfo | ChargingInformation | C | 0..1 | This IE shall be present for a HR PDU session, if available and if the NF Service Consumer requesting the SM Context pertains to the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID). When present, it shall contain the addresses of the V-CHF used for the PDU session. |  |
| roamingChargingProfile | RoamingChargingProfile | C | 0..1 | This IE shall be present for a HR PDU session, if available and if the NF Service Consumer requesting the SM Context pertains to the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID). When present, it shall contain the Roaming Charging Profile selected by the HPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| nefExtBufSupportInd | boolean | C | 0..1 | This IE shall be present with value "true", if the anchor NEF has indicated support of Extended Buffering for mobile terminated data during SMF-NEF connection establishment. When present, this IE shall be set as following: - true: Extended Buffering supported by NEF - false (default): Extended Buffering not supported by NEF |  |
| ipv6Index | IpIndex | C | 0..1 | This IE shall be present during I-SMF change scenarios, if IPv6 Index has previously been received by old I-SMF. |  |
| dnAaaAddress | IpAddress | O | 0..1 | When present, this IE shall contain the address of DN-AAA server for UE IP Address allocation previously received by old I-SMF. |  |
| redundantPduSessionInfo | RedundantPduSessionInformation | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, if this information has been received previously from the UE, the anchor SMF or the old I-SMF. |  |
| ranTunnelInfo | QosFlowTunnel | C | 0..1 | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the N2 tunnel information of NG-RAN with associated QoS flows (see "DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |  |
| addRanTunnelInfo | array(QosFlowTunnel) | C | 1..N | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the additional N2 tunnel information of NG-RAN together with associated QoS flows for split PDU session (see "Additional DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |  |
| redRanTunnelInfo | QosFlowTunnel | C | 0..1 | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the additional N2 tunnel information of NG-RAN together with associated QoS flows for Redundant QoS Flow(s) (see "Redundant DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |  |
| addRedRanTunnelInfo | array(QosFlowTunnel) | C | 1..N | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the additional N2 tunnel information of NG-RAN together with associated QoS flows for Redundant QoS Flow(s) with split PDU session (see "Additional Redundant DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |  |
| nspuSupportInd | boolean | C | 0..1 | This IE shall be present and set to "true" if the enablePauseCharging in the SmContext data type is set to "true" and if the (H-)SMF and PSA UPF support Notify Start Pause of Charging via user plane feature as specified in clause 5.30 of 3GPP TS 29.244 [29]. When present, it shall be set as follows: - true: Notify Start Pause of Charging via user plane feature is supported. |  |
| smfBindingInfo | string | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the Binding indications of the PDU session resource in the home SMF or the SMF and shall be set to the value of the 3gpp-Sbi-Binding header defined in clause 5.2.3.2.6 of 3GPP TS 29.500 [4], without the header name. |  |
| satelliteBackhaulCat | SatelliteBackhaulCategory | O | 0..1 | When present, this IE shall indicate the satellite backhaul category information last signalled towards the anchor SMF, if any. |  |
| sscMode | string | C | 0..1 | This IE shall be present, if available. When present, this IE shall indicate the SSC mode applicable to the PDU session. When present, it shall be encoded as one character in hexadecimal representation, taking a value of "0" to "7", representing the 3 bits of the SSC mode value of the SSC mode IE specified in clause 9.11.4.16 of 3GPP TS 24.501 [7]. Pattern: "^[0-7]$" Example: SSC mode 3 shall be encoded as "3". |  |
| dlsetSupportInd | boolean | C | 0..1 | This IE shall be present and set to "true" if the (H-)SMF supports the "DLSET" feature as specified in clause 6.1.8. When present, it shall be set as follows: - true: the (H-)SMF supports the "DLSET" feature. - false: the (H-)SMF does not support the "DLSET" feature |  |
| n9fscSupportInd | boolean | C | 0..1 | This IE shall be present and set to "true" if the SMF supports the "N9FSC" feature as specified in clause 6.1.8. When present, it shall be set as follows: - true: "N9FSC" feature is supported. |  |
| disasterRoamingInd | boolean | O | 0..1 | When present, this IE shall be set as follows: - true: the UE is registered for Disaster Roaming service - false (default): the UE is not registered for Disaster Roaming service |  |
| anchorSmfOauth2Required | boolean | O | 0..1 | This IE may be present when the NF consumer (i.e. new I-SMF or new V-SMF) and the NF producer (i.e. the old I-SMF, V-SMF or SMF) belong to the same PLMN. When present, this IE shall indicate whether the H-SMF or SMF for a PDU session with an I-SMF requires Oauth2-based authorization for accessing its Nsmf_PDUSession service. - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of the anchor SMF's Nsmf_PDUSession service. (NOTE 3) |  |
| fullDnaiList | array(Dnai) | O | 1..N | This IE may be present to contain the full list of DNAIs of interest for PDU session, including DNAIs that may not be supported by the (source) I-SMF and excluding the ones supported by the Anchor SMF. | DTSSA-Ext1 |
| hrsboAuthResult | boolean | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. When present, it shall Indicate whether HR-SBO request is authorized - true: authorized. - false: Not authorized. | HR-SBO |
| hDnsAddr | IpAddress | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. When present, this IE shall contain the DNS server address of HPLMN. | HR-SBO |
| hPlmnAddr | IpAddress | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. When present, this IE shall contain the HPLMN address information (e.g. H-UPF IP address on N6). The new V-SMF may configure the new V-EASDF to build EDNS Client Subnet option based on this HPLMN address information for target FQDN of DNS queries which are not authorized for HR-SBO. | HR-SBO |
| vplmnOffloadingInfoList | array(VplmnOffloadingInfo) | C | 1..N | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. When present, it shall contain the list of V-PLMN Offloading policies that apply to the PDU session and whose offload identifiers are not yet known by the target V-SMF. (NOTE X) | HR-SBO |
| offloadIds | array(OffloadIdentifier) | C | 1...N | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO, and if the offloadIds are part of the storedOffloadIds included in the the Retrieve SM Context Request. When present, this IE shall contain a list of offload identifiers that apply to the PDU session and that are already known by the target V-SMF. (NOTE X) | HR-SBO |
| pendingUpdateInfoList | array(PendingUpdateInfo) | O | 1..N | This IE should be included by the old V-SMF/I-SMF if received from the (H-)SMF. When present, this IE shall indicate the list of information that are not required to be updated in real-time to the (H-)SMF, i.e. the change of the listed information (e.g. UE location or Timezone) may be piggybacked in a subsequent essential update (e.g. to exchange the N1 message from the UE) to the (H-)SMF. The NF service consumer (i.e. I-SMF/V-SMF) should not trigger an Update to the (H-)SMF including only the change(s) of the listed information. |  |
| NOTE 1: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 2: See NOTE 7 of Table 6.1.6.2.10-1. NOTE 3: If the anchorSmfOauth2Required IE was received in SmContextCreateData from the AMF, this IE shall be ignored by the new I-SMF or V-SMF. NOTE 4: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. NOTE X: The same offloadId should not appear in both the vplmnOffloadingInfoList and offloadIds attributes. Both vplmnOffloadingInfoList and offloadIds attributes may be present when multiple offload identifers apply to the PDU session and some of them are already known by the target V-SMF while others are not yet known by the target V-SMF. |  |  |  |  |  |

# *** Next Change ***

##### 6.1.6.2.72	Type: HrsboInfoFromVplmn

Table 6.1.6.2.72-1: Definition of type HrsboInfoFromVplmn

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hrsboAuthReqInd | boolean | C | 0..1 | This IE shall be present, for a HR PDU session, if the V-SMF requests HR SBO authorization: - in a Create Request; and - in an Update Request sent during a V-SMF insertion (i.e. H-PLMN to V-PLMN mobility) and during an Inter-PLMN V-SMF change (i.e. mobility between different V-PLMNs). When present, this IE shall be set to true to indicate that authorization of Session Breakout for HR Session in VPLMN is requested. |
| vEasdfAddr | IpAddress | C | 0..1 | This IE shall be present during a request for HR-SBO authorization, when using the EAS Discovery procedure with V-EASF for HR-SBO (see clause 6.7.2.3 of 3GPP TS 23.548 [39]) and if available, or if the V-EASDF IP address previously signaled to the H-SMF needs to be changed e.g. as a result of a V-SMF change. When present, this IE shall contain the IP Address of the (new) V-EASDF. |
| localDnsAddr | IpAddress | O | 0..1 | This IE may be present during the EAS Discovery Procedure with Local DNS for HR-SBO (see clause 6.7.2.4 of 3GPP TS 23.548 [39]) after UL-CL/BP insertion. When present, it shall be set to the address of the Local DNS Server/Resolver. |
| easRediscoveryInd | boolean | C | 0..1 | This IE shall be present, for a HR PDU session, if the UE indicates the support of EAS rediscovery and the V-SMF requests EAS rediscovery in an Update Request sent during a V-SMF insertion (i.e. H-PLMN to V-PLMN mobility) and during an Inter-PLMN V-SMF change (i.e. mobility between different V-PLMNs). When present, this IE shall be set to true to indicate that EAS rediscovery is requested. |
| storedOffloadIds | array(OffloadIdentifier) | C | 1...N | The IE shall be present when the V-SMF service instance has stored a list of offload identifiers which were received from any H-SMF from the Home PLMN during previous signalling procedures for this HR-SBO PDU session and also for other HR-SBO PDU sessions. When present, this IE shall contain the list of offload identifiers of the HPLMN that are known by the V-SMF service instance. (NOTE) |
| NOTE: The H-SMF assumes that the V-SMF does not know any Offload Identifiers of the HPLMN if this IE is absent. |  |  |  |  |

# *** Next Change ***

##### 6.1.6.2.73	Type: HrsboInfoFromHplmn

Table 6.1.6.2.73-1: Definition of type HrsboInfoFromHplmn

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hrsboAuthResult | boolean | C | 0..1 | This IE shall be present, for a HR PDU session, if the H-SMF received a request for HR SBO authorization: - in a Create Response; and - in an Update Response sent during a V-SMF insertion (i.e. H-PLMN to V-PLMN mobility) and during an Inter-PLMN V-SMF change (i.e. mobility between different V-PLMNs). When present, this IE shall indicate whether HR-SBO request is authorized as follows: - true: authorized. - false: Not authorized. |
| hDnsAddr | IpAddress | C | 0..1 | This IE shall be present, when using the EAS Discovery procedure with V-EASF for HR-SBO (see clause 6.7.2.3 of 3GPP TS 23.548 [39]) or the EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO (see clause 6.7.2.5 of 3GPP TS 23.548 [39]), if available. When present, this IE shall contain the DNS server address of HPLMN to be used: - for DNS requests related with traffic not to be subject to HR-SBO, to configure the V-EASDF when using the EAS Discovery procedure with V-EASF for HR-SBO (see clause 6.7.2.3 of 3GPP TS 23.548 [39]), or - for DNS requests related to traffic subject to HR-SBO, to configure the UPF in VPLMN to perform IP replacement when using the EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO (see clause 6.7.2.5 of 3GPP TS 23.548 [39]). (NOTE 3). |
| hPlmnAddr | IpAddress | O | 0..1 | This IE may be present when using the EAS Discovery procedure with V-EASF for HR-SBO (see clause 6.7.2.3 of 3GPP TS 23.548 [39]). When present, this IE shall contain the HPLMN address information (e.g. H-UPF IP address on N6) to be used by the V-SMF to configure the V-EASDF to build EDNS Client Subnet option for target FQDN of DNS queries which are not authorized for HR-SBO. |
| vplmnOffloadingInfoList | array(VplmnOffloadingInfo) | O | 0..N | When present, this IE shall include VPLMN specific Offloading Information list applicable to the PDU session and whose offload identifiers are not yet known by the V-SMF service instance or whose offload identifier's version has changed. (NOTE X)(NOTE Y) |
| offloadIds | array(OffloadIdentifier) | C | 0..N | When present, this IE shall include a list of specific Offload Ids applicable for the PDU session and that are part of the StoredOffloadIds included in the corresponding request message. (NOTE X)(NOTE Y) |
| internalGroupIds | array(GroupId) | O | 1..N | When present, this IE shall include the list of internal group identifier if the UE belongs to any subscribed internal group(s). |
| NOTE 1: The H-SMF sends the V-EASDF address received from the V-SMF to the UE in n1SmInfoToUe attribute when using the EAS Discovery procedure with V-EASDF for HR-SBO (see clause 6.7.2.3 of 3GPP TS 23.548 [39]). NOTE 2: The H-SMF sends the Local DNS Server/Resolver address received from the V-SMF to the UE in n1SmInfoToUe attribute when using the EAS Discovery Procedure with Local DNS for HR-SBO (see clause 6.7.2.4 of 3GPP TS 23.548 [39]). NOTE 3: The H-SMF also sends the DNS server IP address of the HPLMN to the UE in n1SmInfoToUe attribute (via PCO) when using the EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO (see clause 6.7.2.5 of 3GPP TS 23.548 [39]). NOTE 4: The H-SMF sends the EAS rediscovery indication received from the V-SMF to the UE in n1SmInfoToUe attribute. NOTE X: When the H-SMF determines that the V-SMF service instance has already received the corresponding Vplmn Offloading Info based on the storedOffloadIds provided by the V-SMF service instance in the corresponding request message and the associated vplmnOffloadInfo has not changed, then in the corresponding response message, the H-SMF shall include the corresponding offload identifier in the offloadIds attribute; otherwise, the H-SMF shall include it in the vplmnOffloadingInfoList attribute. The H-SMF shall always provision a complete list of vplmnOffloadingInfo and/or offloadIds. NOTE Y: The V-SMF service instance shall always overwrite any vplmnOffloadingInfoList and the offloadIds stored for the PDU session with the latest vplmnOffloadingInfo and/or offloadIds received. When the V-SMF receives a VplmnOffloadingInfo containing an offloadId which is known by the V-SMF, it shall consider that the vplmnOffloadingInfo for the offloadId has changed if the offloadId does not contain a version field or if the offload id has an incremented version number, and if so, the V-SMF service instance shall enforce the changed vplmnOffloadingInfo for the HR-SBO PDU session and also for any other HR-SBO PDU sessions with the same offloadId. |  |  |  |  |

# *** Next Change ***

# A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.3.0-alpha.5'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.502 V18.4.0; 5G System; Session Management Services; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.502/

------ skip for clarity-------

SmContextRetrieveData:

description: Data within Retrieve SM Context Request

type: object

properties:

targetMmeCap:

$ref: '#/components/schemas/MmeCapabilities'

smContextType:

$ref: '#/components/schemas/SmContextType'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

notToTransferEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

ranUnchangedInd:

type: boolean

default: false

hrsboSupportInd:

type: boolean

default: false

storedOffloadIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OffloadIdentifier'

minItems: 1

------ skip for clarity-------

SmContext:

description: Complete SM Context

type: object

properties:

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

hplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

hSmfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pduSessionRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

interPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

intraPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

hSmfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pduSessionSmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

pduSessionSmfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

pduSessionSmfBinding:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/SbiBindingLevel'

enablePauseCharging:

type: boolean

default: false

ueIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

maxIntegrityProtectedDataRate:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

alwaysOnGranted:

type: boolean

default: false

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

hSmfServiceInstanceId:

type: string

smfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

forwardingInd:

type: boolean

default: false

psaTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

chargingId:

type: string

pattern: '^(0|([1-9]{1}[0-9]{0,9}))$'

chargingInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ChargingInformation'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

nefExtBufSupportInd:

type: boolean

default: false

ipv6Index:

$ref: 'TS29519_Policy_Data.yaml#/components/schemas/IpIndex'

dnAaaAddress:

$ref: '#/components/schemas/IpAddress'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

ranTunnelInfo:

$ref: '#/components/schemas/QosFlowTunnel'

addRanTunnelInfo:

type: array

items:

$ref: '#/components/schemas/QosFlowTunnel'

minItems: 1

redRanTunnelInfo:

$ref: '#/components/schemas/QosFlowTunnel'

addRedRanTunnelInfo:

type: array

items:

$ref: '#/components/schemas/QosFlowTunnel'

minItems: 1

nspuSupportInd:

type: boolean

smfBindingInfo:

type: string

satelliteBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

sscMode:

type: string

pattern: '^[0-7]$'

dlsetSupportInd:

type: boolean

n9fscSupportInd:

type: boolean

disasterRoamingInd:

type: boolean

default: false

anchorSmfOauth2Required:

type: boolean

fullDnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

hrsboAuthReqInd:

type: boolean

hDnsAddr:

$ref: '#/components/schemas/IpAddress'

hPlmnAddr:

$ref: '#/components/schemas/IpAddress'

vplmnOffloadingInfoList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VplmnOffloadingInfo'

minItems: 1

offloadIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OffloadIdentifier'

minItems: 1

pendingUpdateInfoList:

type: array

items:

$ref: '#/components/schemas/PendingUpdateInfo'

minItems: 1

required:

- pduSessionId

- dnn

- sNssai

- pduSessionType

- sessionAmbr

- qosFlowsList

------ skip for clarity-------

HrsboInfoFromVplmn:

description: HR-SBO Information in request message

type: object

properties:

hrsboAuthReqInd:

type: boolean

enum:

- true

vEasdfAddr:

$ref: '#/components/schemas/IpAddress'

localDnsAddr:

$ref: '#/components/schemas/IpAddress'

easRediscoveryInd:

type: boolean

enum:

- true

storedOffloadIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OffloadIdentifier'

minItems: 1

------ skip for clarity-------

HrsboInfoFromHplmn:

description: HR-SBO Information in response message

type: object

properties:

hrsboAuthResult:

type: boolean

hDnsAddr:

$ref: '#/components/schemas/IpAddress'

hPlmnAddr:

$ref: '#/components/schemas/IpAddress'

vplmnOffloadingInfoList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VplmnOffloadingInfo'

offloadIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OffloadIdentifier'

internalGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

------ skip for clarity-------

*** End of Changes ***