3GPP TSG-CT3 Meeting #130	C3-234149

# Xiamen, China, 9th – 13th October 2023														was C3-233066

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.122 | CR | 0718 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the remaining LOLC related ENs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | NBI17 |  |  |  |  |  | Date: |  | 2023-09-29 |
| Category: | A |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are two remaining Editor's Notes in the definition of the support of Load and Overload Control mecahnisms for NBI APIs. For the first Editor's Note on how the support of these mechanisms is exchanged between the involved entities, this is actually not needed as the receiver of the LCI/OCI header will simply ignore it if it does not support this mechanism, as it is the case for 5GC APIs. For the second Editor's Note on how reselection is performed, this should be left open to implementations, e.g., the AF may reselect using CAPIF or by deployment specific means. |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to: - Remove these remaining Editor's Notes. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The Load and Overload Control functionality may appear incomplete in the specification whereas it is already finalized. |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 5.2.11 |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  |  | This CR does not impact the OpenAPI descriptions defined in this specification. |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

### 5.2.11	Support of Load and Overload Control

The Load Control mechanisms defined in clause 6.3 of 3GPP TS 29.500 [44] may be supported by T8 APIs with the following differences:

-	The "Load Control based on load signalled via the NRF" mechanism defined in clause 6.3.2 of 3GPP TS 29.500 [44] shall not be supported; and

-	The "Load Control based on LCI Header" mechanism defined in clause 6.3.3 of 3GPP TS 29.500 [44] may be supported with the following differences:

-	The Load Control "scope" and all the related provisions are not applicable;

-	The "NF Service Consumer" corresponds to the sending entity (e.g. SCEF, SCS/AS, AF) of the message in T8 APIs and the "NF Service Producer" corresponds to the receiving entity  (e.g. SCEF, SCS/AS, AF) of the message in T8 APIs;

-	The provisions related to the SCP (Service Communication Proxy) and SEPP (Security Edge Protection Proxy) entities are not applicable; and

-	The "3gpp-Sbi-Lci" custom HTTP header corresponds to the "Nbi-Api-Lci" custom HTTP header defined in clause 5.2.8.3.4 for T8 APIs.

-	The provisions and mechanisms using the NRF (i.e. 5GC NF Repository Function) are not applicable.

The Overload Control mechanisms defined in clause 6.4 of 3GPP TS 29.500 [44] may be supported by T8 APIs with the following differences:

-	The "Overload Control based on HTTP status codes" mechanism defined in clause 6.4.2 of 3GPP TS 29.500 [44] may be supported with the following differences:

-	The "NF Service Consumer" corresponds to the sending entity (e.g. SCEF, SCS/AS, AF) of the message in T8 APIs and the "NF Service Producer" corresponds to the receiving entity  (e.g. SCEF, SCS/AS, AF) of the message in T8 APIs;

and

-	The "Overload Control based on OCI Header" mechanism defined in clause 6.4.3 of 3GPP TS 29.500 [44] may be supported with the following differences:

-	The Overload Control "scope" and all the related provisions are not applicable;

-	The "NF Service Consumer" corresponds to the sending entity (e.g. SCEF, SCS/AS, AF) of the message in T8 APIs and the "NF Service Producer" corresponds to the receiving entity  (e.g. SCEF, SCS/AS, AF) of the message in T8 APIs;

-	The provisions related to the SCP (Service Communication Proxy) and SEPP (Security Edge Protection Proxy) entities are not applicable; and

-	The "3gpp-Sbi-Oci" custom HTTP header corresponds to the "Nbi-Api-Oci" custom HTTP header defined in clause 5.2.8.3.5 for T8 APIs.

-	The provisions and mechanisms using the NRF (i.e. 5GC NF Repository Function) are not applicable.

* * * * End of changes * * * *