3GPP TSG-CT WG4 Meeting #121	C4-240703

# Athens, Greece, 26 February - 1 March, 2024										(was C4-240319)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0518 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new data type for MBSR operation allowed |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2024-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 clarified the ‘MBSR operation allowed’ subscripton data in the clause 5.2.3.3.1 of TS 23.502 with their CR, S2-2313764. Table 5.2.3.3.1-1: UE Subscription data types Subscription data type; Field; Description. Access and Mobility Subscription data (data needed for UE Registration and Mobility Management); MBSR Operation allowed; Indicates the subscriber is allowed for MBSR operation as specified in clause 5.35A.4 of TS 23.501 [2]. If present, additional location information (i.e. a list of TAIs or Area Codes that can be interpreted by AMF into TAIs ) and/or time information (including one or more time windows, and/or one or more recurring time periods) may also be present to restrict the MBSR operation to be within the location and time provided.. To implement the ‘one or more recurring time periods’ of time information, it is necessary to define the new data type, MonthOfYear. |
| Summary of change: |  | This CR proposes new simple data type, MonthOfYear. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Subscription data for the MBSR is not fully implemented in stage3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | (rev1) typo in API |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.2.2	Simple Data Types

This clause specifies common simple data types.

Table 5.2.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Binary | string | String with format "binary" as defined in OpenAPI Specification [3] |
| BinaryRm | string | This data type is defined in the same way as the "Binary" data type, but with the OpenAPI "nullable: true" property. |
| Bytes | string | String with format "byte" as defined in OpenAPI Specification [3], i.e, base64-encoded characters, |
| BytesRm | string | This data type is defined in the same way as the "Bytes" data type, but with the OpenAPI "nullable: true" property. |
| Date | string | String with format "date" as defined in OpenAPI Specification [3] |
| DateRm | string | This data type is defined in the same way as the "Date" data type, but with the OpenAPI "nullable: true" property. |
| DateTime | string | String with format "date-time" as defined in OpenAPI Specification [3] |
| DateTimeRm | string | This data type is defined in the same way as the "DateTime" data type, but with the OpenAPI "nullable: true" property. |
| DiameterIdentity | Fqdn | String containing a Diameter Identity (FQDN), according to clause 4.3 of IETF RFC 6733 [18]. DiameterIdentity is defined as a simple data type because Fqdn is also a simple data type (string). |
| DiameterIdentityRm | FqdnRm | This data type is defined in the same way as the "DiameterIdentity" data type, but with the OpenAPI "nullable: true" property. DiameterIdentityRm is defined as a simple data type because FqdnRm is also a simple data type (either a string or null). |
| Double | number | Number with format "double" as defined in OpenAPI Specification [3] |
| DoubleRm | number | This data type is defined in the same way as the "Double" data type, but with the OpenAPI "nullable: true" property. |
| DurationSec | integer | Unsigned integer identifying a period of time in units of seconds. |
| DurationSecRm | integer | This data type is defined in the same way as the "DurationSec" data type, but with the OpenAPI "nullable: true" property. |
| Float | number | Number with format "float" as defined in OpenAPI Specification [3] |
| FloatRm | number | This data type is defined in the same way as the "Float" data type, but with the OpenAPI "nullable: true" property. |
| Uint16 | integer | Integer where the allowed values correspond to the value range of an unsigned 16-bit integer, i.e. 0 to 65535. Minimum = 0. Maximum = 65535. |
| Uint16Rm | integer | This data type is defined in the same way as the "Uint16" data type, but with the OpenAPI "nullable: true" property. |
| Int32 | integer | Integer with format "int32" as defined in OpenAPI Specification [3] |
| Int32Rm | integer | This data type is defined in the same way as the "Int32" data type, but with the OpenAPI "nullable: true" property. |
| Int64 | integer | Integer with format "int64" as defined in OpenAPI Specification [3] |
| Int64Rm | integer | This data type is defined in the same way as the "Int64" data type, but with the OpenAPI "nullable: true" property. |
| Ipv4Addr | string | String identifying a IPv4 address formatted in the "dotted decimal" notation as defined in IETF RFC 1166 [4]. Pattern: '^(([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])\.){3}([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])$' |
| Ipv4AddrRm | string | This data type is defined in the same way as the "Ipv4Addr" data type, but with the OpenAPI "nullable: true" property. |
| Ipv4AddrMask | string | String identifying a IPv4 address mask formatted in the "dotted decimal" notation as defined in IETF RFC 1166 [4]. Pattern: '^(([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])\.){3}([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])(\/([0-9]¦[1-2][0-9]¦3[0-2]))$' |
| Ipv4AddrMaskRm | string | This data type is defined in the same way as the "Ipv4AddrMask" data type, but with the OpenAPI "nullable: true" property. |
| Ipv6Addr | string | String identifying an IPv6 address formatted according to clause 4 of IETF RFC 5952 [5]. The mixed IPv4 IPv6 notation according to clause 5 of IETF RFC 5952 [5] shall not be used. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$' |
| Ipv6AddrRm | string | This data type is defined in the same way as the "Ipv6Addr" data type, but with the OpenAPI "nullable: true" property. |
| Ipv6Prefix | string | String identifying an IPv6 address prefix formatted according to clause 4 of IETF RFC 5952 [5]. IPv6Prefix data type may contain an individual /128 IPv6 address. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])¦([0-9]{2})¦(1[0-1][0-9])¦(12[0-8])))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$' |
| Ipv6PrefixRm | string | This data type is defined in the same way as the "Ipv6Prefix" data type, but with the OpenAPI "nullable: true" property. |
| MacAddr48 | string | String identifying a MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [17]. Pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$' |
| MacAddr48Rm | string | This data type is defined in the same way as the "MacAddr48" data type, but with the OpenAPI "nullable: true" property. |
| SupportedFeatures | string | A string used to indicate the features supported by an API that is used as defined in clause 6.6 in 3GPP TS 29.500 [25]. The string shall contain a bitmask indicating supported features in hexadecimal representation: Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent the support of 4 features as described in table 5.2.2-3. The most significant character representing the highest-numbered features shall appear first in the string, and the character representing features 1 to 4 shall appear last in the string. The list of features and their numbering (starting with 1) are defined separately for each API. If the string contains a lower number of characters than there are defined features for an API, all features that would be represented by characters that are not present in the string are not supported. |
| Uinteger | integer | Unsigned Integer, i.e. only value 0 and integers above 0 are permissible. Minimum = 0. |
| UintegerRm | integer | This data type is defined in the same way as the "Uinteger" data type, but with the OpenAPI "nullable: true" property. |
| Uint32 | integer | Integer where the allowed values correspond to the value range of an unsigned 32-bit integer, i.e. 0 to (2^32)-1. Minimum = 0. Maximum = 4294967295. |
| Uint32Rm | integer | This data type is defined in the same way as the "Uint32" data type, but with the OpenAPI "nullable: true" property. |
| Uint64 | integer | Integer where the allowed values correspond to the value range of an unsigned 64-bit integer, i.e. 0 to (2^64)-1. Minimum = 0. Maximum = 18446744073709551615. |
| Uint64Rm | integer | This data type is defined in the same way as the "Uint64" data type, but with the OpenAPI "nullable: true" property. |
| Uri | string | String providing an URI formatted according to IETF RFC 3986 [6]. If the URI fields intended to convey generic data (e.g., in the value part of a query parameter, or in the URI path segments) contain reserved characters, these reserved characters shall be percent-encoded as defined in clause 5.2.10.2 of 3GPP TS 29.500 [25]. |
| UriRm | string | This data type is defined in the same way as the "Uri" data type, but with the OpenAPI "nullable: true" property. |
| VarUeId | string | String represents the SUPI or GPSI. Pattern: "^(imsi-[0-9]{5,15}¦nai-.+¦msisdn-[0-9]{5,15}¦extid-[^@]+@[^@]+¦gci-.+¦gli-.+¦.+)$". |
| VarUeIdRm | string | This data type is defined in the same way as the "VarUeId" data type, but with the OpenAPI "nullable: true" property. |
| TimeZone | string | String with format "<time-numoffset>" optionally appended by "<daylightSavingTime>", where: - <time-numoffset> shall represent the time zone adjusted for daylight saving time and be encoded as time-numoffset as defined in clause 5.6 of IETF RFC 3339 [10]; - <daylightSavingTime> shall represent the adjustment that has been made and shall be encoded as "+1" or "+2" for a +1 or +2 hours adjustment. Example: "-08:00+1" (for 8 hours behind UTC, +1 hour adjustment for Daylight Saving Time). |
| TimeZoneRm | string | This data type is defined in the same way as the "TimeZone" data type, but with the OpenAPI "nullable: true" property. |
| StnSr | string | String representing the STN-SR as defined in clause 18.6 of 3GPP TS 23.003 [7]. |
| StnSrRm | string | This data type is defined in the same way as the "StnSr" data type, but with the OpenAPI "nullable: true" property. |
| CMsisdn | string | String representing the C-MSISDN as defined in clause 18.7 of 3GPP TS 23.003 [7]). Pattern: "^[0-9]{5,15}$". |
| CMsisdnRm | string | This data type is defined in the same way as the "CMsisdn" data type, but with the OpenAPI "nullable: true" property. |
| MonthOfYear | integer | Integer between and including 1 and 12 denoting a month. "1" shall indicate "January", and the subsequent months shall be indicated with the next higher numbers. "12" shall indicate "December". |
| DayOfWeek | integer | Integer between and including 1 and 7 denoting a weekday. "1" shall indicate "Monday", and the subsequent weekdays shall be indicated with the next higher numbers. "7" shall indicate "Sunday". |
| TimeOfDay | string | String with format "partial-time" or "full-time" as defined in clause 5.6 of IETF RFC 3339 [10]. Examples: "20:15:00", "20:15:00-08:00" (for 8 hours behind UTC). |
| EmptyObject | object | Empty JSON object: { } It is defined with the keyword: "additionalProperties: false". |
| Fqdn | string | Fully Qualified Domain Name Pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$' minLength: 4 maxLength: 253 |
| FqdnRm | string | This data type is defined in the same way as the "Fqdn" data type, but it also allows the null value. |

Table 5.2.2-2: Reused OpenAPI data types

| Type Name | Description |
| --- | --- |
| boolean | As defined in OpenAPI Specification [3] |
| integer | As defined in OpenAPI Specification [3] |
| number | As defined in OpenAPI Specification [3] |
| string | As defined in OpenAPI Specification [3] |
| object | As defined in OpenAPI Specification [3] |
| array | As defined in OpenAPI Specification [3] |
| NOTE Data types defined in OpenAPI Specification [3] do not follow the UpperCamel convention for data types in 3GPP TS 29.501 [2] |  |

Table 5.2.2-3: Meaning of a Hexadecimal Character in SupportedFeatures Type

| Character | Feature n+3 supported | Feature n+2 supported | Feature n+1 supported | Feature n supported |
| --- | --- | --- | --- | --- |
| "0" | no | no | no | no |
| "1" | no | no | no | yes |
| "2" | no | no | yes | no |
| "3" | no | no | yes | yes |
| "4" | no | yes | no | no |
| "5" | no | yes | no | yes |
| "6" | no | yes | yes | no |
| "7" | no | yes | yes | yes |
| "8" | yes | no | no | no |
| "9" | yes | no | no | yes |
| "A" | yes | no | yes | no |
| "B" | yes | no | yes | yes |
| "C" | yes | yes | no | no |
| "D" | yes | yes | no | yes |
| "E" | yes | yes | yes | no |
| "F" | yes | yes | yes | yes |
| NOTE 1 "n" shall be i * 4 + 1, where "i" is zero or a natural number, i.e permissible values of "n" are 1, 5, 9, … NOTE 2 In this table if a feature is not defined, it shall also be indicated with value "no". |  |  |  |  |

For example, if only the first feature defined in the feature list is set to 1, the corresponding SupportedFeatures attribute would have a hexadecimal character value of "1", or a string of hexadecimal characters with value of "001" (any amount of 0's to the left of the 1 would result into an equivalent feature list). If we have 32 features defined, and only the last feature in a feature list is set to 1, the corresponding SupportedFeatures attribute would have a string of hexadecimal characters with value of "80000000"(see the description of the SupportedFeatures encoding in Table 5.2.2-1).

* * * Next Change * * * *

# A.2	Data related to Common Data Types

openapi: 3.0.0

info:

version: '1.5.0-alpha.5'

title: 'Common Data Types'

description: |

Common Data Types for Service Based Interfaces.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.571 Common Data Types for Service Based Interfaces, version 18.4.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.571/'

paths: {}

components:

schemas:

***********text not shown for clarity**********

CMsisdnRm:

type: string

pattern: '^[0-9]{5,15}$'

nullable: true

description: >

String representing the C-MSISDN as defined in clause 18.7 of 3GPP TS 23.003 with

the OpenAPI 'nullable: true' property.

MonthOfYear:

type: integer

minimum: 1

maximum: 12

description: >

integer between and including 1 and 12 denoting a month. 1 shall indicate January,

and the subsequent months shall be indicated with the next higher numbers.

12 shall indicate December.

DayOfWeek:

type: integer

minimum: 1

maximum: 7

description: >

integer between and including 1 and 7 denoting a weekday. 1 shall indicate Monday,

and the subsequent weekdays  shall be indicated with the next higher numbers.

7 shall indicate Sunday.

TimeOfDay:

type: string

description: >

String with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339.

Examples, 20:15:00, 20:15:00-08:00 (for 8 hours behind UTC).

EmptyObject:

description: Empty JSON object { }, it is defined with the keyword additionalProperties false

type: object

additionalProperties: false

Fqdn:

description: Fully Qualified Domain Name

type: string

pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$'

minLength: 4

maxLength: 253

FqdnRm:

description: Fully Qualified Domain Name, but it also allows the null value

anyOf:

- $ref: '#/components/schemas/Fqdn'

- $ref: '#/components/schemas/NullValue'

#

#   COMMON ENUMERATED DATA TYPES

#

***********text not shown for clarity**********

* * * End of Change * * * *