3GPP TSG-CT WG6 Meeting #118	C6-240143

# Athens, Greece; 27th Feb – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.121 | CR | 0539 | rev | 2 | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE recognising the search period of the Higher priority PLMN – NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R17 |  |  |  |  |  | Date: |  |  | 2024-02-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify that the Higher priority PLMN timer is read and the Higher priority PLMN takes precedence over the VPLMN in which the UE is currently registered in. Add test to verify it is correctly handled by the UE for NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR adds new clause 7.4.X that introduces test on search period of the Higher priority PLMN for NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test taking into account the Higher priority PLMN search period timer and the priority for NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.8, 7.4.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was C6-240130 was C6-240100 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.8	Applicability table

Table B.1: Applicability of tests

| Test# | Title | from Rel | up to Rel (see note) | Applicability | Network Dependency | Add. Recommend | Support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Subscription related tests |  |  |  |  |  |  |
| 5.1 | IMSI / TMSI handling |  |  |  |  |  |  |
| 5.1.1 | UE identification by short IMSI | R99 | Rel-7 | M | USS OR SS | AER005 |  |
|  |  | Rel-8 |  | C049 |  |  |  |
| 5.1.2 | UE identification by short IMSI using 2-digit MNC | R99 | Rel-7 | M | USS OR SS | AER005 |  |
|  |  | Rel-8 |  | C049 |  |  |  |
| 5.1.3 | UE identification by "short" TMSI | R99 |  | C004 | USS OR SS | AER005 |  |
| 5.1.4 | UE identification by "long" TMSI | R99 |  | C004 | USS OR SS | AER005 |  |
| 5.1.5 | UE identification by long IMSI, TMSI updating after key set identifier assignment | R99 |  | C004 | USS OR SS | AER005 |  |
| 5.1.6 | UE identification by short IMSI when accessing E-UTRAN/EPC | Rel-8 | Rel-12 | C027 | E-USS OR NB-SS (see note 2) |  |  |
|  |  | Rel-13 |  | C045 |  |  |  |
| 5.1.7 | UE identification by short IMSI using 2-digit MNC when accessing E-UTRAN/EPC | Rel-8 | Rel-12 | C027 | E-USS OR NB-SS (see note 2) |  |  |
|  |  | Rel-13 |  | C045 |  |  |  |
| 5.1.8 | UE identification after changed IMSI with service "EMM Information" not available | Rel-8 | Rel-12 | C027 | E-USS OR NB-SS (see note 2) |  |  |
|  |  | Rel-13 |  | C045 |  |  |  |
| 5.1.9 | UE identification by GUTI when using USIM with service "EMM Information" not available | Rel-8 | Rel-12 | C027 | E-USS OR NB-SS (see note 2) |  |  |
|  |  | Rel-13 |  | C045 |  |  |  |
| 5.1.10 | UE identification by GUTI when using USIM with service "EMM Information" available | Rel-8 | Rel-12 | C027 | E-USS OR NB-SS (see note 2) |  |  |
|  |  | Rel-13 |  | C045 |  |  |  |
| 5.2 | Access Control handling |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |
| 7.4 | Higher priority PLMN search handling |  |  |  |  |  |  |
| 7.4.1 | UE recognising the search period of the Higher priority PLMN | R99 |  | C010 | USS OR SS only | AER005 |  |
| 7.4.2 | GSM/UMTS dual mode UEs recognising the search period of the Higher priority PLMN | R99 |  | C003 | USS AND SS |  |  |
| 7.4.3 | UE recognising the search period of the Higher priority PLMN – E-UTRAN | Rel-8 | Rel-12 | C010 AND C027 | E-USS OR NB-SS (see note 2) |  |  |
|  |  | Rel-13 |  | C010 AND C045 |  |  |  |
| 7.4.4 | E-UTRAN/EPC capable UEs recognising the search period of the Higher priority PLMN – GSM/E-UTRAN | Rel-8 |  | C003 AND C027 | E-USS AND SS |  |  |
| 7.4.5 | E-UTRAN/EPC capable UEs recognising the search period of the Higher priority PLMN – UTRAN/E-UTRAN | Rel-8 |  | C011 AND C027 | E-USS AND USS |  |  |
| 7.4.X | UE recognising the search period of the Higher priority PLMN – NG-RAN | Rel-15 |  | C056 | NG-SS |  |  |

* * * Next Change * * * *

### 7.4.5	E-UTRAn/EPC capable UEs recognising the search period of the Higher priority PLMN – UTRAN/E-UTRAN

#### 7.4.5.1	Definition and applicability

The Higher priority PLMN list gives in priority order the Higher priority PLMN on which the UE shall register first. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFHPLMNwACT. The Higher priority PLMN search period gives the time interval in which the UE shall search for a possible Higher priority PLMN registration.

To avoid a duplication of tests, this test supersedes test 7.4.x.

#### 7.4.5.2	Conformance requirement

After registered onto a VPLMN the UE shall take into account the Higher priority PLMN search period timer and the priority order of the Higher priority PLMNs in the preferred list on the USIM including the Access Technology Identifier.

-	TS 22.011 [6], clauses 3.2.2 and 3.2.2.5.

#### 7.4.5.3	Test purpose

To verify that the Higher priority PLMN timer is read and the Higher priority PLMN with the higher priority (defined by its position in EFHPLMNwACT) takes precedence over the VPLMN in which the UE is currently registered in. Hereby the new coding for RAT  E-UTRAN has to be handled correctly by the UE.

#### 7.4.5.4	Method of test

##### 7.4.5.4.1	Initial conditions

For this test both a UTRAN USS and an E-UTRAN E-USS are needed.

The USS transmits on BCCH, with the following network parameters:

-	Attach/detach:	disabled.

-	LAI (MCC/MNC/LAC):	244/009/0001.

-	RAI (MCC/MNC/LAC/RAC):	244/009/0001/05.

-	Access control:	unrestricted.

After the registration of UE the USS transmits on a second BCCH, with the following network parameters:

-	Attach/detach:	disabled.

-	LAI (MCC/MNC/LAC):	244/081/0001.

-	RAI (MCC/MNC/LAC/RAC):	244/081/0001/05.

-	Access control:	unrestricted.

At the same time as the SS sends on a second BCCH, the E- USS transmits on BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	244/081/0001.

--	Access control:	unrestricted.

The default E-UTRAN UICC is used with the following exception:

EFHPLMNwACT (HPLMN selector with Access Technology)

Logically:	Set to MCC 244 and MNC 081

Set to

E-UTRAN

| Coding: | B1 | B2 | B3 | B4 | B5 |
| --- | --- | --- | --- | --- | --- |
| Hex | 42 | 14 | 80 | 40 | 00 |

EFHPPLMN (Higher Priority HPLMN Search period)

Logically:	set to 6minutes

| Coding: | B1 |
| --- | --- |
| Hex | 01 |

EFUST (USIM Service Table)

Logically:	Local Phone Book available

User controlled PLMN selector available

Fixed dialling numbers available

Barred dialling numbers available

The GSM Access available

The Group Identifier level 1 and level 2 not available

Service n 33 (Packed Switched Domain) shall be set to '1'

Enabled Services Table available

HPLMN selector with access technology available

EPS Mobility Management Information available

Allowed CSG Lists and corresponding indications not available

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx x1xx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 |  |  |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx |  |  |  |  |  |

The UICC is installed into the Terminal, the UE is set to automatic PLMN selection mode and to auto GPRS attach.

##### 7.4.5.4.2	Procedure

a)	The UE is powered on.

b)	After receipt of a RRC CONNECTION REQUEST from the UE, the USS shall send RRC CONNECTION SETUP to the UE, followed by RRC CONNECTION SETUP COMPLETE sent by the UE to the USS.

c)	After receipt of an ATTACH REQUEST from the UE, the SS sends ATTACH ACCEPT with:

RAI (MCC/MNC/LAC/RAC):	244/009/0001/05

TMSI:	"34567890"

to the UE.

d)	After receipt of a ATTACH COMPLETE from the UE, the USS sends RRC CONNECTION RELEASE to the UE, followed by RRC CONNECTION RELEASE COMPLETE sent by the UE to the USS.

e)	TheUSS starts to send on the second BCCH with the MCC/MNC 244/081 and the E-USS starts to send with the Same MCC/MNC. An internal timer shall start to run.

f)	After receipt of an RRCConnectionRequest from the UE on the E-UTRAN-cell related to the BCCH transmitting MCC/MNC 244/081, the E-USS sends RRCConnectionSetup to the UE, followed by RRCConnectionSetupComplete sent by the UE to the E-USS.

g)	During registration and after receipt of a TrackingAreaUpdateRequest from the UE, the E-USS initiates authentication, starts integrity by using the security procedure and sends TrackingAreaUpdateAccept with to the UE:

TAI (MCC/MNC/TAC):	244/081/ 0001

GUTI:	"24408100010266436587"

h)	After receipt of the TrackingAreaUpdatComplete during registration from the UE, the E-USS sends RRCConnectionRelease.

i)	The UE is soft powered down.

#### 7.4.5.5	Acceptance criteria

1.) After step e) the UE shall send a RRCConnectionRequest on the E-UTRAN-cell related to the BCCH transmitting MCC/MNC 244/081 to the e-USS.

2)	After step f) the terminal shall send TrackingAreaUpdateReques to the E-USS.

3)	After step g) the terminal shall respond with TrackingAreaUpdatComplete during registration.

4)	After step i) the USIM shall contain the following values:

EFEPSLOCI (EPS Information)

Logically:	GUTI:	24408100010266436587

Last visited registered TAI:	244/081/0001

EPS update status:	updated

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 14 | 80 | 00 | 01 | 02 | 66 | 43 | 65 |
|  | B12 | B13 | B14 | B15 | B16 | B17 | B18 |  |  |  |  |
|  | 87 | 42 | 14 | 80 | 00 | 01 | 00 |  |  |  |  |

### 7.4.X	UE recognising the search period of the Higher priority PLMN – NG-RAN

#### 7.4.X.1	Definition and applicability

The Higher priority PLMN list gives in priority order the Higher priority PLMN on which the UE shall register first. The Higher priority PLMN search period gives the time interval in which the UE shall search for a possible Higher priority PLMN registration.

#### 7.4.X.2	Conformance requirement

After registered onto a VPLMN the UE shall take into account the Higher priority PLMN search period timer and the priority order of the Higher priority PLMNs in the preferred lists on the USIM.

-	TS 22.011 [6], clauses 3.2.2 and 3.2.2.5.

-	TS 24.501 [42], clause 5.5.1.2.4.

-	TS 31.102 [4], clause 4.2.6.

#### 7.4.X.3	Test purpose

To verify that the Higher priority PLMN timer is read and the Higher priority PLMN takes precedence over the VPLMN in which the UE is currently registered in. Hereby the new coding for NG-RAN has to be handled correctly by the UE.

#### 7.4.X.4	Method of test

##### 7.4.X.4.1	Initial conditions

For this test a NG-SS is required.

The NG-SS transmits on BCCH with the following network parameters:

--	TAI (MCC/MNC/TAC):	244/008/000001.

-	Access control:	unrestricted.

After the registration of UE the NG-SS transmits on a second BCCH with the following network parameters:

-	TAI (MCC/MNC/TAC):	244/083/000001.

--	Access control:	unrestricted.

The default 5G-NR UICC is used with the following exception:

EFHPLMNwACT (HPLMN selector with Access Technology)

Logically:	Set to MCC 244 and MNC 083

Set to NG-RAN

| Coding: | B1 | B2 | B3 | B4 | B5 |
| --- | --- | --- | --- | --- | --- |
| Hex | 42 | 34 | 80 | 08 | 00 |

EFHPPLMN (Higher Priority HPLMN Search period)

Logically:	set to 6 minutes

| Coding: | B1 |
| --- | --- |
| Hex | 01 |

EFUST (USIM Service Table)

Logically:

| Service n°43: |  | HPLMN selector with access technology | available |
| --- | --- | --- | --- |

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx x1xx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xxxx xxxx | ... | xxxx xxxx |  |  |  |

The UICC shall be installed into the Terminal and the UE shall be set to automatic PLMN selection mode.

##### 7.4.X.4.2	Procedure

a)	The UE is powered on.

b)	After receipt of a RRCConnectionRequest from the UE on the NG-RAN-cell related to the BCCH transmitting MCC/MNC 244/008, the NG-SS sends RRCConnectionSetup to the UE, followed by RRCConnectionSetupComplete sent by the UE to the NG-SS.

c)	During registration and after receipt of a REGISTRATION REQUEST from the UE, the NG-SS initiates authentication, starts integrity by using the security procedure and sends REGISTRATION ACCEPT to the UE with:

TAI (MCC/MNC/TAC):	244/008/ 000001

5G-GUTI:	"24400800010266436587"

d)	After receipt of the REGISTRATION COMPLETE during registration from the UE, the NG-SS sends RRCConnectionRelease.

e)	The NG-SS starts to send on the second BCCH with the MCC/MNC 244/083. An internal timer shall start to run.

f)	After receipt of an RRCConnectionRequest from the UE on the NG-RAN-cell related to the BCCH transmitting MCC/MNC 244/083, the NG-SS sends RRCConnectionSetup  to the UE, followed by RRCConnectionSetupComplete sent by the UE to the NG-SS. The internal timer is stopped.

g)	During registration and after receipt of a REGISTRATION REQUEST from the UE, the NG-SS initiates authentication, starts integrity by using the security procedure and sends REGISTRATION ACCEPT to the UE with:

TAI (MCC/MNC/TAC):	244/083/ 000001

5G-GUTI:	"24408300010266436587"

h)	The UE is soft powered down.

#### 7.4.X.5	Acceptance criteria

1) After step e) the UE shall send a RRCConnectionRequest on the NG-RAN-cell related to the BCCH transmitting MCC/MNC 244/083 to the NG-SS.

2)	The value of the internal timer shall not exceed 6 minutes.

NOTE:	To take the systems processing time into account, the value of the internal timer may allow to be a guard time of 10 % greater than the required 6 minutes.

3)	After step h) the USIM shall contain the following values:

EF5GS3GPPLOCI (5GS 3GPP location information)

Logically:	5G-GUTI:	24408300010266436587

Last visited registered TAI in 5GS for 3GPP access:	244/083/000001

5GS update status for 3GPP access:	5U1 UPDATED

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 0B | F2 | 42 | 34 | 80 | 00 | 01 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 02 | 66 | 43 | 65 | 87 | 42 | 34 | 80 |
|  | B17 | B18 | B19 | B20 |  |  |  |  |
|  | 00 | 00 | 01 | 00 |  |  |  |  |

* * * End of Changes * * * *