3GPP TSG-CT Meeting #103	CP-240155

# Maastricht, Netherlands; 18th – 19th March 2024               Revision of C4-240724, C4-240461

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0225 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add UPUnSubscribe and update UPSubscribe service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CATT, Huawei, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Stage 2 has defined new service operation in TS 23.273: 8.3.2.10 Nlmf_Location_UPUnSubscribe service operation Service operation name: Nlmf_Location_UPUnSubscribe Description: Allow the consumer NF to unsubscribe about status of a secure LCS-UP connection for a target UE. Input, Required: Subscription Correlation ID. Input, Optional: None. Outputs, Required: None. Outputs, Optional: None. This CR updates the existing UPSubscribe operation and adds the UPUnSubscribe operation to meet the requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ update the UPSubscribe operation 2/ add UPUnSubscribe operation 3/ modify the OpenAPI file |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New UPUnSubscribe service operation cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1, 5.2.2.7.1, 5.2.2.7.2, 5.2.2.x(new), 6.1.3.1, 6.1.4.1, 6.1.3.x(new), 6.1.3.y(new), 6.1.4.6, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features to Nlmf_Location API file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Add Huawei and Nokia as cosigners Rev 2: 1/ Correct the figures 5.2.2.x.2-1 and 6.1.3.1-1 2/ Add schema for path variable in the OpenAPI |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.2.2.1	Introduction

The service operations defined for the Nlmf_Location service are as follows:

-	DetermineLocation: It provides UE location information to the consumer NF.

-	EventNotify: It notifies the consumer NF of an event for periodic or triggered location for a target UE, or cumulative event report for the location reporting over user plane from the UE.

-	CancelLocation: It enables a consumer NF to cancel an ongoing periodic or triggered location for a target UE.

-	LocationContextTransfer: It enables a consumer NF to transfer location context information for periodic or triggered location of a target UE to a new LMF.

-	MeasurementData: It enables a consumer NF to request the PRU location measurements from PRU serving LMFs.

-	UPSubscribe: It enables a consumer NF to subscribe the status of a secure LCS-UP connection for a target UE.

-	UPUnSubscribe: It enables the consumer NF to unsubscribe the status of a secure LCS-UP connection for a target UE.

-	UPNotify: It notifies the consumer NF of the status or modification of a secure LCS-UP connection for a target UE.

-	UPConfig: It enables a consumer NF to set up, modify or terminate a secure LCS-UP connection for a target UE.

* * * Next Change * * * *

##### 5.2.2.7.1	General

The following procedures are defined, using the "UPSubscribe" service operation:

-	Subscribe to Notification of LCS-UP connection status

* * * Next Change * * * *

##### 5.2.2.7.2	Subscribe to Notification of LCS-UP connection status

This procedure allows a consumer NF to subscribe the status of a secure LCS-UP connection for a target UE.

Figure 5.2.2.7.2-1: NF service consumer subscribes to notifications

1.	The NF service consumer shall send a POST request to the resource URI representing the "up-subscriptions" collection resource.

The callback URI of the NF service consumer shall be include in the request content to receive notifications of  LCS-UP connection status from the LMF.

2a.	On success, the LMF responds with "201 Created". The response shall contain the data related to the created subscription, and the HTTP Location header shall contain the URI of the created subscription.

2b.	On failure or redirection, one of the HTTP status code listed in Table Table 6.1.3.x.3.1-3 shall be returned. For a 4xx/5xx response, the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table Table 6.1.3.x.3.1-3.

* * * Next Change * * * *

#### 5.2.2.x	UPUnSubscribe

##### 5.2.2.x.1	General

The following procedure is defined using the "UPUnSubscribe" service operation:

-	Unsubscribe to notification of LCS-UP connection status

##### 5.2.2.x.2	Unsubscribe to notification of LCS-UP connection status

This procedure allows the consumer NF to unsubscribe the status of a secure LCS-UP connection for a target UE.

Figure 5.2.2.x.2-1: NF service consumer unsubscribes to notifications

1.	The NF service consumer shall send a DELETE request to the resource identified by the URI previously received during subscription creation.

2a.	On success, the LMF responds with "204 No Content".

2b.	On failure or redirection, one of the HTTP status code listed in Table Table 6.1.3.y.3.1-3 shall be returned. For a 4xx/5xx response, the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table Table 6.1.3.y.3.1-3.

* * * Next Change * * * *

#### 6.1.3.1	Overview

The structure of the Resource URIs of the Nlmf_Location service is shown in figure 6.1.3.1-1.

Figure 6.1.3.1-1: Resource URI structure of the Nlmf_Location API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| up-subscriptions (Collection) | /up-subscriptions | POST | Create a subscription |
| up-subscription (Document) | /up-subscriptions/{subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId} |

* * * Next Change * * * *

#### 6.1.4.1	Overview

The URI structure for Custom Operations without associated resources is included as part of the Figure 6.1.3.1-1

Table 6.1.4.1-1: Custom operations without associated resources

| Operation Name | Custom operation URI | Mapped HTTP method | Description (Service Operation) |
| --- | --- | --- | --- |
| determine-location | /determine-location | POST | Determine Location |
| cancel-location | /cancel-location | POST | Cancel Location |
| location-context-transfer | /location-context-transfer | POST | Transfer Location Context |
| location-measure | /location-measure | POST | Location Measure |
| up-configure | /up-configure | POST | Create, Modify or Terminate the LCS-UP connection |

* * * Next Change * * * *

#### 6.1.3.x	Resource: up-subscriptions (Collection)

##### 6.1.3.x.1	Description

This resource represents a collection of subscriptions to notifications.

##### 6.1.3.x.2	Resource Definition

Resource URI: {apiRoot}/nlmf-loc/v1/up-subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.x.2-1.

Table 6.1.3.x.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.1.3.x.3	Resource Standard Methods

6.1.3.x.3.1	POST

This method creates a new subscription. This method shall support the URI query parameters specified in table 6.1.3.x.3.1-1.

Table 6.1.3.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.x.3.1-2 and the response data structures and response codes specified in table 6.1.3.x.3.1-3.

Table 6.1.3.x.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UpSubscription | M | 1 | Input parameters to the "UPSubscribe" operation |

Table 6.1.3.x.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UpSubscription | M | 1 | 201 Created | This case represents the successful creation of a subscription. Upon success, the HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.x.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nlmf-loc/v1/up-subscriptions/{subscriptionId} |

Table 6.1.3.x.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.x.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

#### 6.1.3.y	Resource: up-subscription (Document)

##### 6.1.3.y.1	Description

This resource represents an individual subscription.

##### 6.1.3.y.2	Resource Definition

Resource URI: {apiRoot}/nlmf-loc/v1/up-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.y.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| subscriptionId | string | Represents a specific subscription |

##### 6.1.3.y.3	Resource Standard Methods

6.1.3.y.3.1	DELETE

This method terminates an existing subscription. This method shall support the URI query parameters specified in table 6.1.3.y.3.1-1.

Table 6.1.3.5.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.y.3.1-2 and the response data structures and response codes specified in table 6.1.3.y.3.1-3.

Table 6.1.3.y.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.y.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.y.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.y.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

#### 6.1.4.6	Void

* * * Next Change * * * *

# A.2	Nlmf_Location API

openapi: 3.0.0

info:

version: '1.3.0-alpha.5'

title: 'LMF Location'

description: |

LMF Location Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.572 V18.4.0; 5G System; Location Management Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.572/'

servers:

- url: '{apiRoot}/nlmf-loc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

paths:

/determine-location:

post:

summary: Determine Location of an UE

operationId: DetermineLocation

tags:

- Determine Location

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

- oAuth2ClientCredentials:

- nlmf-loc

- nlmf-loc:determine-location:invoke

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/InputData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/InputData'

binaryDataLppMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataLppMessage:

contentType:  application/vnd.3gpp.lpp

headers:

Content-Id:

schema:

type: string

binaryDataLppMessageExt1:

contentType:  application/vnd.3gpp.lpp

headers:

Content-Id:

schema:

type: string

binaryDataLppMessageExt2:

contentType:  application/vnd.3gpp.lpp

headers:

Content-Id:

schema:

type: string

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/LocationDataExt'

'204':

description: Expected response for MO-LR requesting location assistance data.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

EventNotify:

'{$request.body#/hgmlcCallBackURI}':

post:

requestBody:

description: UE Event Notification

content:

application/json:

schema:

$ref: '#/components/schemas/EventNotifyDataExt'

responses:

'204':

description: Expected response to a valid notification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/up-subscriptions:

post:

summary: subscribe about status of a secure LCS-UP connection for a target UE

operationId: UpSubscriptions

tags:

- UP Subscribe

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

- oAuth2ClientCredentials:

- nlmf-loc

- nlmf-loc:up-subscriptions:invoke

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UpSubscription'

required: true

responses:

'201':

description: Expected response to successful UP Subscription

content:

application/json:

schema:

$ref: '#/components/schemas/UpSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

UPNotify:

'{$request.body#/upNotifyCallBackURI}':

post:

requestBody:

description: UP Connection Status Notification

content:

application/json:

schema:

$ref: '#/components/schemas/UpNotifyData'

responses:

'204':

description: Expected response to a valid notification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/up-subscriptions/{subscriptionId}:

delete:

summary: Deletes a subscription

operationId: DeleteSubscription

tags:

- UP Unsubscribe

parameters:

- name: subscriptionId

in: path

required: true

description: Unique subscription Id

schema:

type: string

responses:

'204':

description: Expected response to a successful subscription removal

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/cancel-location:

post:

summary: request cancellation of periodic or triggered location

operationId: CancelLocation

tags:

- Cancel Location

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

- oAuth2ClientCredentials:

- nlmf-loc

- nlmf-loc:cancel-location:invoke

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/CancelLocData'

required: true

responses:

'204':

description: Expected response to a successful cancellation

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/location-context-transfer:

post:

summary: transfer context information for periodic or triggered location

operationId: LocationContextTransfer

tags:

- Location Context Transfer

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

- oAuth2ClientCredentials:

- nlmf-loc

- nlmf-loc:location-context-transfer:invoke

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/LocContextData'

required: true

responses:

'204':

description: Expected response to successful location context transfer

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/location-measure:

post:

summary: PRU location measurement

operationId: LocationMeasure

tags:

- Location Measure

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

- oAuth2ClientCredentials:

- nlmf-loc

- nlmf-loc:location-measure:invoke

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/LocMeasurementReq'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/LocMeasurementResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/up-configure:

post:

summary: set up, modify or terminate a secure LCS-UP connection for a target UE

operationId: UpConfig

tags:

- UP Configure

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

- oAuth2ClientCredentials:

- nlmf-loc

- nlmf-loc:up-configure:invoke

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UpConfig'

required: true

responses:

'204':

description: Expected response to successful set up, modify or terminate LCS-UP connection.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nlmf-loc: Access to the Nlmf_Location API

nlmf-loc:determine-location:invoke: Access to invoke Determine Location

nlmf-loc:cancel-location:invoke: Access to invoke Cancel Location

nlmf-loc:location-context-transfer:invoke: Access to invoke Location Context

nlmf-loc:location-measure:invoke: Access to invoke Location Measurement

nlmf-loc:up-subscriptions:invoke: Access to invoke UP Subscription

nlmf-loc:up-configure:invoke: Access to invoke UP Config

schemas:

#

# COMPLEX TYPES

#

InputData:

description: Information within Determine Location Request.

type: object

not:

required: [ ecgi, ncgi ]

properties:

externalClientType:

$ref: '#/components/schemas/ExternalClientType'

correlationID:

$ref: '#/components/schemas/CorrelationID'

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

locationQoS:

$ref: '#/components/schemas/LocationQoS'

supportedGADShapes:

type: array

items:

$ref: '#/components/schemas/SupportedGADShapes'

minItems: 1

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

requestedRangingSlResult:

type: array

items:

$ref: '#/components/schemas/RangingSlResult'

minItems: 1

relatedUEs:

type: array

items:

$ref: '#/components/schemas/RelatedUE'

minItems: 1

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ecgiOnSecondNode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ncgiOnSecondNode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

priority:

$ref: '#/components/schemas/LcsPriority'

velocityRequested:

$ref: '#/components/schemas/VelocityRequested'

ueLcsCap:

$ref: '#/components/schemas/UeLcsCapability'

lcsServiceType:

$ref: '#/components/schemas/LcsServiceType'

ldrType:

$ref: '#/components/schemas/LdrType'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

lirGmlcCallBackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

vgmlcAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

lirReference:

$ref: '#/components/schemas/LirReference'

periodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: '#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: '#/components/schemas/MotionEventInfo'

reportingAccessTypes:

type: array

items:

$ref: '#/components/schemas/ReportingAccessType'

minItems: 1

ueConnectivityStates:

$ref: '#/components/schemas/UeConnectivityState'

ueLocationServiceInd:

$ref: '#/components/schemas/UeLocationServiceInd'

moAssistanceDataTypes:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/LcsBroadcastAssistanceTypesData'

lppMessage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

lppMessageExt:

description: Indicates the lpp message extension.

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

uePositioningCap:

$ref: '#/components/schemas/UePositioningCapabilities'

tnapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TnapId'

twapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TwapId'

ueCountryDetInd:

type: boolean

scheduledLocTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

reliableLocReq:

type: boolean

default: false

evtRptAllowedAreas:

type: array

items:

$ref: '#/components/schemas/ReportingArea'

minItems: 1

maxItems: 250

ueUnawareInd:

type: boolean

enum:

- true

intermediateLocationInd:

type: boolean

default: false

maxRespTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

lpHapType:

$ref: 'TS29518_Namf_Location.yaml#/components/schemas/LpHapType'

ueUpPosCaps:

type: array

items:

$ref: '#/components/schemas/UeUpPositioningCapabilities'

minItems: 1

reportingInd:

allOf:

- $ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/ReportingInd'

default: INSIDE_REPORTING

mbsrInfo:

$ref: '#/components/schemas/MbsrInfo'

integrityRequirements:

$ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/IntegrityRequirements'

upLocRepAddrAf:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/UpLocRepAddrAfRm'

upCumEvtRptCriteria:

$ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/UpCumEvtRptCriteria'

mappedQoSEps:

$ref: '#/components/schemas/MappedLocationQoSEps'

additionalUeInfo:

$ref: '#/components/schemas/AdditionalUeInfo'

LocationDataExt:

description: Extended Location Data for UEs

allOf:

- $ref: '#/components/schemas/LocationData'

- $ref: '#/components/schemas/AddLocationDatas'

LocationData:

description: Information within Determine Location Response.

type: object

required:

- locationEstimate

properties:

locationEstimate:

$ref: '#/components/schemas/GeographicArea'

accuracyFulfilmentIndicator:

$ref: '#/components/schemas/AccuracyFulfilmentIndicator'

ageOfLocationEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

timestampOfLocationEstimate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

velocityEstimate:

$ref: '#/components/schemas/VelocityEstimate'

civicAddress:

$ref: '#/components/schemas/CivicAddress'

localLocationEstimate:

$ref: '#/components/schemas/LocalArea'

positioningDataList:

type: array

items:

$ref: '#/components/schemas/PositioningMethodAndUsage'

minItems: 1

gnssPositioningDataList:

type: array

items:

$ref: '#/components/schemas/GnssPositioningMethodAndUsage'

minItems: 1

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

altitude:

$ref: '#/components/schemas/Altitude'

barometricPressure:

$ref: '#/components/schemas/BarometricPressure'

servingLMFIdentification:

$ref: '#/components/schemas/LMFIdentification'

uePositioningCap:

$ref: '#/components/schemas/UePositioningCapabilities'

ueAreaInd:

$ref: '#/components/schemas/UeAreaIndication'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

achievedQos:

$ref: '#/components/schemas/MinorLocationQoS'

directReportInd:

type: boolean

default: false

indoorOutdoorInd:

$ref: '#/components/schemas/IndoorOutdoorInd'

acceptedPeriodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

haGnssMetrics:

$ref: '#/components/schemas/HighAccuracyGnssMetrics'

losNlosMeasureInd:

$ref: '#/components/schemas/LosNlosMeasureInd'

relatedApplicationlayerId:

type: string

rangeDirection:

$ref: '#/components/schemas/RangeDirection'

2dRelativeLocation:

$ref: '#/components/schemas/2DRelativeLocation'

3dRelativeLocation:

$ref: '#/components/schemas/3DRelativeLocation'

relativeVelocity:

$ref: '#/components/schemas/VelocityEstimate'

GeographicArea:

description: Geographic area specified by different shape.

anyOf:

- $ref: '#/components/schemas/Point'

- $ref: '#/components/schemas/PointUncertaintyCircle'

- $ref: '#/components/schemas/PointUncertaintyEllipse'

- $ref: '#/components/schemas/Polygon'

- $ref: '#/components/schemas/PointAltitude'

- $ref: '#/components/schemas/PointAltitudeUncertainty'

- $ref: '#/components/schemas/EllipsoidArc'

GADShape:

description: Common base type for GAD shapes.

type: object

required:

- shape

properties:

shape:

$ref: '#/components/schemas/SupportedGADShapes'

discriminator:

propertyName: shape

mapping:

POINT: '#/components/schemas/Point'

POINT_UNCERTAINTY_CIRCLE: '#/components/schemas/PointUncertaintyCircle'

POINT_UNCERTAINTY_ELLIPSE: '#/components/schemas/PointUncertaintyEllipse'

POLYGON: '#/components/schemas/Polygon'

POINT_ALTITUDE: '#/components/schemas/PointAltitude'

POINT_ALTITUDE_UNCERTAINTY: '#/components/schemas/PointAltitudeUncertainty'

ELLIPSOID_ARC: '#/components/schemas/EllipsoidArc'

LOCAL_2D_POINT_UNCERTAINTY_ELLIPSE: '#/components/schemas/Local2dPointUncertaintyEllipse'

LOCAL_3D_POINT_UNCERTAINTY_ELLIPSOID: '#/components/schemas/Local3dPointUncertaintyEllipsoid'

Point:

description: Ellipsoid Point.

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

PointUncertaintyCircle:

description: Ellipsoid point with uncertainty circle.

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- uncertainty

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

uncertainty:

$ref: '#/components/schemas/Uncertainty'

PointUncertaintyEllipse:

description: Ellipsoid point with uncertainty ellipse.

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- uncertaintyEllipse

- confidence

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

uncertaintyEllipse:

$ref: '#/components/schemas/UncertaintyEllipse'

confidence:

$ref: '#/components/schemas/Confidence'

Polygon:

description: Polygon.

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- pointList

properties:

pointList:

$ref: '#/components/schemas/PointList'

PointAltitude:

description: Ellipsoid point with altitude.

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- altitude

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

altitude:

$ref: '#/components/schemas/Altitude'

PointAltitudeUncertainty:

description: Ellipsoid point with altitude and uncertainty ellipsoid.

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- altitude

- uncertaintyEllipse

- uncertaintyAltitude

- confidence

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

altitude:

$ref: '#/components/schemas/Altitude'

uncertaintyEllipse:

$ref: '#/components/schemas/UncertaintyEllipse'

uncertaintyAltitude:

$ref: '#/components/schemas/Uncertainty'

confidence:

$ref: '#/components/schemas/Confidence'

EllipsoidArc:

description: Ellipsoid Arc.

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- innerRadius

- uncertaintyRadius

- offsetAngle

- includedAngle

- confidence

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

innerRadius:

$ref: '#/components/schemas/InnerRadius'

uncertaintyRadius:

$ref: '#/components/schemas/Uncertainty'

offsetAngle:

$ref: '#/components/schemas/Angle'

includedAngle:

$ref: '#/components/schemas/Angle'

confidence:

$ref: '#/components/schemas/Confidence'

LocalOrigin:

description: Indicates a Local origin in a reference system

type: object

properties:

coordinateId:

type: string

point:

$ref: '#/components/schemas/GeographicalCoordinates'

RelativeCartesianLocation:

description: Relative Cartesian Location

type: object

required:

- x

- y

properties:

x:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

y:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

z:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

LocalArea:

description: Local area specified by different shape

oneOf:

- $ref: '#/components/schemas/Local2dPointUncertaintyEllipse'

- $ref: '#/components/schemas/Local3dPointUncertaintyEllipsoid'

UeAreaIndication:

description: Indicates area (country, area in a country or international area) where UE is located

type: object

oneOf:

- required:

- country

- required:

- internationalAreaInd

properties:

country:

description: Indicates country or area in a country where UE is located

type: string

internationalAreaInd:

description: Indicates international area indication if UE is located in international area

type: boolean

default: false

Local2dPointUncertaintyEllipse:

description: Local 2D point with uncertainty ellipse

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- localOrigin

- point

- uncertaintyEllipse

- confidence

properties:

localOrigin:

$ref: '#/components/schemas/LocalOrigin'

point:

$ref: '#/components/schemas/RelativeCartesianLocation'

uncertaintyEllipse:

$ref: '#/components/schemas/UncertaintyEllipse'

confidence:

$ref: '#/components/schemas/Confidence'

Local3dPointUncertaintyEllipsoid:

description: Local 3D point with uncertainty ellipsoid

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- localOrigin

- point

- uncertaintyEllipsoid

- confidence

properties:

localOrigin:

$ref: '#/components/schemas/LocalOrigin'

point:

$ref: '#/components/schemas/RelativeCartesianLocation'

uncertaintyEllipsoid:

$ref: '#/components/schemas/UncertaintyEllipsoid'

confidence:

$ref: '#/components/schemas/Confidence'

GeographicalCoordinates:

description: Geographical coordinates.

type: object

required:

- lon

- lat

properties:

lon:

type: number

format: double

minimum: -180

maximum: 180

lat:

type: number

format: double

minimum: -90

maximum: 90

UncertaintyEllipse:

description: Ellipse with uncertainty.

type: object

required:

- semiMajor

- semiMinor

- orientationMajor

properties:

semiMajor:

$ref: '#/components/schemas/Uncertainty'

semiMinor:

$ref: '#/components/schemas/Uncertainty'

orientationMajor:

$ref: '#/components/schemas/Orientation'

UncertaintyEllipsoid:

description: Ellipsoid with uncertainty

type: object

required:

- semiMajor

- semiMinor

- vertical

- orientationMajor

properties:

semiMajor:

$ref: '#/components/schemas/Uncertainty'

semiMinor:

$ref: '#/components/schemas/Uncertainty'

vertical:

$ref: '#/components/schemas/Uncertainty'

orientationMajor:

$ref: '#/components/schemas/Orientation'

PointList:

description: List of points.

type: array

items:

$ref: '#/components/schemas/GeographicalCoordinates'

minItems: 3

maxItems: 15

LocationQoS:

description: QoS of Location request.

type: object

properties:

hAccuracy:

$ref: '#/components/schemas/Accuracy'

vAccuracy:

$ref: '#/components/schemas/Accuracy'

verticalRequested:

type: boolean

responseTime:

$ref: '#/components/schemas/ResponseTime'

minorLocQoses:

type: array

items:

$ref: '#/components/schemas/MinorLocationQoS'

minItems: 1

maxItems: 2

lcsQosClass:

$ref: '#/components/schemas/LcsQosClass'

PositioningMethodAndUsage:

description: Indicates the usage of a positioning method.

type: object

required:

- method

- mode

- usage

properties:

method:

$ref: '#/components/schemas/PositioningMethod'

mode:

$ref: '#/components/schemas/PositioningMode'

usage:

$ref: '#/components/schemas/Usage'

methodCode:

type: integer

minimum: 16

maximum: 31

GnssPositioningMethodAndUsage:

description: Indicates the usage of a Global Navigation Satellite System (GNSS) positioning method.

type: object

required:

- mode

- gnss

- usage

properties:

mode:

$ref: '#/components/schemas/PositioningMode'

gnss:

$ref: '#/components/schemas/GnssId'

usage:

$ref: '#/components/schemas/Usage'

CivicAddress:

description: Indicates a Civic address.

type: object

properties:

country:

type: string

A1:

type: string

A2:

type: string

A3:

type: string

A4:

type: string

A5:

type: string

A6:

type: string

PRD:

type: string

POD:

type: string

STS:

type: string

HNO:

type: string

HNS:

type: string

LMK:

type: string

LOC:

type: string

NAM:

type: string

PC:

type: string

BLD:

type: string

UNIT:

type: string

FLR:

type: string

ROOM:

type: string

PLC:

type: string

PCN:

type: string

POBOX:

type: string

ADDCODE:

type: string

SEAT:

type: string

RD:

type: string

RDSEC:

type: string

RDBR:

type: string

RDSUBBR:

type: string

PRM:

type: string

POM:

type: string

usageRules:

type: string

method:

type: string

providedBy:

type: string

VelocityEstimate:

description: Velocity estimate.

oneOf:

- $ref: '#/components/schemas/HorizontalVelocity'

- $ref: '#/components/schemas/HorizontalWithVerticalVelocity'

- $ref: '#/components/schemas/HorizontalVelocityWithUncertainty'

- $ref: '#/components/schemas/HorizontalWithVerticalVelocityAndUncertainty'

HorizontalVelocity:

description: Horizontal velocity.

type: object

required:

- hSpeed

- bearing

properties:

hSpeed:

$ref: '#/components/schemas/HorizontalSpeed'

bearing:

$ref: '#/components/schemas/Angle'

HorizontalWithVerticalVelocity:

description: Horizontal and vertical velocity.

type: object

required:

- hSpeed

- bearing

- vSpeed

- vDirection

properties:

hSpeed:

$ref: '#/components/schemas/HorizontalSpeed'

bearing:

$ref: '#/components/schemas/Angle'

vSpeed:

$ref: '#/components/schemas/VerticalSpeed'

vDirection:

$ref: '#/components/schemas/VerticalDirection'

HorizontalVelocityWithUncertainty:

description: Horizontal velocity with speed uncertainty.

type: object

required:

- hSpeed

- bearing

- hUncertainty

properties:

hSpeed:

$ref: '#/components/schemas/HorizontalSpeed'

bearing:

$ref: '#/components/schemas/Angle'

hUncertainty:

$ref: '#/components/schemas/SpeedUncertainty'

HorizontalWithVerticalVelocityAndUncertainty:

description: Horizontal and vertical velocity with speed uncertainty.

type: object

required:

- hSpeed

- bearing

- vSpeed

- vDirection

- hUncertainty

- vUncertainty

properties:

hSpeed:

$ref: '#/components/schemas/HorizontalSpeed'

bearing:

$ref: '#/components/schemas/Angle'

vSpeed:

$ref: '#/components/schemas/VerticalSpeed'

vDirection:

$ref: '#/components/schemas/VerticalDirection'

hUncertainty:

$ref: '#/components/schemas/SpeedUncertainty'

vUncertainty:

$ref: '#/components/schemas/SpeedUncertainty'

UeLcsCapability:

description: Indicates the LCS capability supported by the UE..

type: object

properties:

lppSupport:

type: boolean

default: true

ciotOptimisation:

type: boolean

default: false

PeriodicEventInfo:

description: Indicates the information of periodic event reporting.

type: object

required:

- reportingAmount

- reportingInterval

properties:

reportingAmount:

$ref: '#/components/schemas/ReportingAmount'

reportingInterval:

$ref: '#/components/schemas/ReportingInterval'

reportingInfiniteInd:

type: boolean

enum:

- true

reportingIntervalMs:

$ref: '#/components/schemas/ReportingIntervalMs'

AreaEventInfo:

description: Indicates the information of area based event reporting.

type: object

required:

- areaDefinition

properties:

areaDefinition:

type: array

items:

$ref: '#/components/schemas/ReportingArea'

minItems: 1

maxItems: 250

occurrenceInfo:

$ref: '#/components/schemas/OccurrenceInfo'

minimumInterval:

$ref: '#/components/schemas/MinimumInterval'

maximumInterval:

$ref: '#/components/schemas/MaximumInterval'

samplingInterval:

$ref: '#/components/schemas/SamplingInterval'

reportingDuration:

$ref: '#/components/schemas/ReportingDuration'

reportingLocationReq:

type: boolean

default: true

ReportingArea:

description: Indicates an area for event reporting.

type: object

required:

- areaType

properties:

areaType:

$ref: '#/components/schemas/ReportingAreaType'

tai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

MotionEventInfo:

description: Indicates the information of motion based event reporting.

type: object

required:

- linearDistance

properties:

linearDistance:

$ref: '#/components/schemas/LinearDistance'

occurrenceInfo:

$ref: '#/components/schemas/OccurrenceInfo'

minimumInterval:

$ref: '#/components/schemas/MinimumInterval'

maximumInterval:

$ref: '#/components/schemas/MaximumInterval'

samplingInterval:

$ref: '#/components/schemas/SamplingInterval'

reportingDuration:

$ref: '#/components/schemas/ReportingDuration'

reportingLocationReq:

type: boolean

default: true

CancelLocData:

description: Information within Cancel Location Request.

type: object

required:

- hgmlcCallBackURI

- ldrReference

properties:

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

LocContextData:

description: Information within Transfer Location Context Request.

type: object

required:

- amfId

- ldrType

- hgmlcCallBackURI

- ldrReference

- eventReportMessage

properties:

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

locationQoS:

$ref: '#/components/schemas/LocationQoS'

supportedGADShapes:

type: array

items:

$ref: '#/components/schemas/SupportedGADShapes'

minItems: 1

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ldrType:

$ref: '#/components/schemas/LdrType'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

periodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: '#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: '#/components/schemas/MotionEventInfo'

eventReportMessage:

$ref: '#/components/schemas/EventReportMessage'

eventReportingStatus:

$ref: '#/components/schemas/EventReportingStatus'

ueLocationInfo:

$ref: '#/components/schemas/UELocationInfo'

cIoT5GSOptimisation:

type: boolean

default: false

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

uePositioningCap:

$ref: '#/components/schemas/UePositioningCapabilities'

scheduledLocTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

indoorOutdoorInd:

$ref: '#/components/schemas/IndoorOutdoorInd'

losNlosMeasureInd:

$ref: '#/components/schemas/LosNlosMeasureInd'

upCumEvtRptCriteria:

$ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/UpCumEvtRptCriteria'

EventReportMessage:

description: Indicates an event report message.

type: object

required:

- eventClass

- eventContent

properties:

eventClass:

$ref: '#/components/schemas/EventClass'

eventContent:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

EventReportingStatus:

description: Indicates the status of event reporting.

type: object

properties:

eventReportCounter:

$ref: '#/components/schemas/EventReportCounter'

eventReportDuration:

$ref: '#/components/schemas/EventReportDuration'

UELocationInfo:

description: Indicates location information of a UE.

type: object

properties:

locationEstimate:

$ref: '#/components/schemas/GeographicArea'

ageOfLocationEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

timestampOfLocationEstimate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

velocityEstimate:

$ref: '#/components/schemas/VelocityEstimate'

ageOfVelocityEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

timestampOfVelocityEstimate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

EventNotifyDataExt:

description: Extended Event Notify Data for UEs

allOf:

- $ref: '#/components/schemas/EventNotifyData'

- $ref: '#/components/schemas/AddEventNotifyDatas'

EventNotifyData:

description: Information within Event Notify Request.

type: object

required:

- reportedEventType

- ldrReference

properties:

reportedEventType:

$ref: '#/components/schemas/ReportedEventType'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

lirReference:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LirReference'

locationEstimate:

$ref: '#/components/schemas/GeographicArea'

ageOfLocationEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

timestampOfLocationEstimate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

civicAddress:

$ref: '#/components/schemas/CivicAddress'

localLocationEstimate:

$ref: '#/components/schemas/LocalArea'

positioningDataList:

type: array

items:

$ref: '#/components/schemas/PositioningMethodAndUsage'

minItems: 1

gnssPositioningDataList:

type: array

items:

$ref: '#/components/schemas/GnssPositioningMethodAndUsage'

minItems: 1

servingLMFidentification:

$ref: '#/components/schemas/LMFIdentification'

terminationCause:

$ref: '#/components/schemas/TerminationCause'

velocityEstimate:

$ref: '#/components/schemas/VelocityEstimate'

altitude:

$ref: '#/components/schemas/Altitude'

achievedQos:

$ref: '#/components/schemas/MinorLocationQoS'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

indoorOutdoorInd:

$ref: '#/components/schemas/IndoorOutdoorInd'

haGnssMetrics:

$ref: '#/components/schemas/HighAccuracyGnssMetrics'

losNlosMeasureInd:

$ref: '#/components/schemas/LosNlosMeasureInd'

upLocRepStatAf:

type: integer

relatedApplicationlayerId:

type: string

rangeDirection:

$ref: '#/components/schemas/RangeDirection'

2dRelativeLocation:

$ref: '#/components/schemas/2DRelativeLocation'

3dRelativeLocation:

$ref: '#/components/schemas/3DRelativeLocation'

relativeVelocity:

$ref: '#/components/schemas/VelocityEstimate'

UeConnectivityState:

description: Indicates the connectivity state of a UE.

type: object

required:

- accessType

properties:

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

connectivitystate:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/CmState'

MinorLocationQoS:

description: Contain Minor Location QoS.

type: object

properties:

hAccuracy:

$ref: '#/components/schemas/Accuracy'

vAccuracy:

$ref: '#/components/schemas/Accuracy'

MbsrInfo:

description: MBSR Information

type: object

properties:

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

LocMeasurementReq:

description: Location Measurement Request.

type: object

properties:

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

preCalcuLocEstimate:

$ref: '#/components/schemas/GeographicArea'

timestampOfPreCalcuLocEstimate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

LocMeasurementResp:

description: Location Measurement Response.

type: object

required:

- locMeasurements

- locationEstimate

properties:

locMeasurements:

$ref: '#/components/schemas/LocMeasurements'

locationEstimate:

$ref: '#/components/schemas/GeographicArea'

ageOfLocationEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

timestampOfLocationEstimate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

velocityEstimate:

$ref: '#/components/schemas/VelocityEstimate'

localLocationEstimate:

$ref: '#/components/schemas/LocalArea'

LocMeasurements:

description: PRU Location Measurements.

type: object

properties:

rstd:

type: integer

HighAccuracyGnssMetrics:

description: High Accuracy GNSS Positioning Metrics.

type: object

properties:

nrOfUsedSatellites:

type: integer

minimum: 0

maximum: 64

hdopi:

type: integer

minimum: 1

maximum: 256

pdopi:

type: integer

minimum: 1

maximum: 256

age:

type: integer

minimum: 0

maximum: 99

fixType:

$ref: '#/components/schemas/FixType'

UpNotifyData:

description: UP Subscription

type: object

required:

- ldrReference

- upConnectionStatus

properties:

ldrReference:

$ref: '#/components/schemas/LdrReference'

upConnectionStatus:

$ref: '#/components/schemas/UpConnectionStatus'

targetLMFIdentification:

$ref: '#/components/schemas/LMFIdentification'

UpSubscription:

description: UP Subscription

type: object

required:

- upNotifyCallBackURI

- supi

properties:

upNotifyCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

RelatedUE:

description: Related UE Information

type: object

required:

- applicationlayerId

- relatedUEType

properties:

applicationlayerId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationlayerId'

relatedUEType:

$ref: '#/components/schemas/RelatedUEType'

UpConfig:

description: UP Config

type: object

required:

- upNotifyCallBackURI

properties:

upNotifyCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

amfReallocationInd:

type: boolean

default: false

lcsUpConnectionInd:

$ref: '#/components/schemas/LcsUpConnectionInd'

targetLMFIdentification:

$ref: '#/components/schemas/LMFIdentification'

RangeDirection:

description: Represents a range and direction from a point A to a point B.

type: object

properties:

range:

type: number

azimuthDirection:

$ref: '#/components/schemas/Angle'

elevationDirection:

$ref: '#/components/schemas/Angle'

2DRelativeLocation:

description: Represents a relative 2D location with uncertainty ellipse.

type: object

properties:

semiMinor:

$ref: '#/components/schemas/Uncertainty'

semiMajor:

$ref: '#/components/schemas/Uncertainty'

orientationAngle:

$ref: '#/components/schemas/Angle'

3DRelativeLocation:

description: Represents a relative 3D location with uncertainty ellipsoid.

type: object

properties:

semiMinor:

$ref: '#/components/schemas/Uncertainty'

semiMajor:

$ref: '#/components/schemas/Uncertainty'

verticalUncertainty:

$ref: '#/components/schemas/Uncertainty'

orientationAngle:

$ref: '#/components/schemas/Angle'

AddLocationDatas:

type: object

properties:

addLocationDatas:

type: array

items:

$ref: '#/components/schemas/LocationData'

minItems: 1

AddEventNotifyDatas:

type: object

properties:

addEventNotifyDatas:

type: array

items:

$ref: '#/components/schemas/EventNotifyData'

minItems: 1

MappedLocationQoSEps:

description: Mapped Location QoS for EPS.

type: object

required:

- hAccuracy

properties:

hAccuracy:

$ref: '#/components/schemas/Accuracy'

vAccuracy:

$ref: '#/components/schemas/Accuracy'

AdditionalUeInfo:

description: MBSR UE Information

type: object

properties:

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

#

#

# SIMPLE TYPES

#

Altitude:

description: Indicates value of altitude.

type: number

format: double

minimum: -32767

maximum: 32767

Angle:

description: Indicates value of angle.

type: integer

minimum: 0

maximum: 360

Uncertainty:

description: Indicates value of uncertainty.

type: number

format: float

minimum: 0

Orientation:

description: Indicates value of orientation angle.

type: integer

minimum: 0

maximum: 180

Confidence:

description: Indicates value of confidence.

type: integer

minimum: 0

maximum: 100

Accuracy:

description: Indicates value of accuracy.

type: number

format: float

minimum: 0

InnerRadius:

description: Indicates value of the inner radius.

type: integer

format: int32

minimum: 0

maximum: 327675

CorrelationID:

description: LCS Correlation ID.

type: string

minLength: 1

maxLength: 255

AgeOfLocationEstimate:

description: Indicates value of the age of the location estimate.

type: integer

minimum: 0

maximum: 32767

HorizontalSpeed:

description: Indicates value of horizontal speed.

type: number

format: float

minimum: 0

maximum: 2047

VerticalSpeed:

description: Indicates value of vertical speed.

type: number

format: float

minimum: 0

maximum: 255

SpeedUncertainty:

description: Indicates value of speed uncertainty.

type: number

format: float

minimum: 0

maximum: 255

BarometricPressure:

description: Specifies the measured uncompensated atmospheric pressure.

type: integer

minimum: 30000

maximum: 115000

LcsServiceType:

description: LCS service type.

type: integer

minimum: 0

maximum: 127

LdrReference:

description: LDR Reference.

type: string

minLength: 2

maxLength: 510

LirReference:

description: LIR Reference.

type: string

minLength: 2

maxLength: 510

ReportingAmount:

description: Number of required periodic event reports.

type: integer

minimum: 1

maximum: 8639999

ReportingInterval:

description: Event reporting periodic interval in seconds.

type: integer

minimum: 1

maximum: 8639999

ReportingIntervalMs:

description: Event reporting periodic interval in milliseconds.

type: integer

minimum: 1

maximum: 999

MinimumInterval:

description: Minimum interval between event reports.

type: integer

minimum: 1

maximum: 32767

MaximumInterval:

description: Maximum interval between event reports.

type: integer

minimum: 1

maximum: 86400

SamplingInterval:

description: Maximum time interval between consecutive evaluations by a UE of a trigger event.

type: integer

minimum: 1

maximum: 3600

ReportingDuration:

description: Maximum duration of event reporting.

type: integer

minimum: 1

maximum: 8640000

LinearDistance:

description: Minimum straight line distance moved by a UE to trigger a motion event report.

type: integer

minimum: 1

maximum: 10000

LMFIdentification:

description: LMF identification.

type: string

EventReportCounter:

description: Number of event reports received from the target UE.

type: integer

minimum: 1

maximum: 8640000

EventReportDuration:

description: Duration of event reporting.

type: integer

minimum: 1

maximum: 8640000

UePositioningCapabilities:

description: Positioning capabilities supported by the UE. A string encoding the "ProvideCapabilities-r9-IEs" IE as specified in clause 6.3 of 3GPP TS 37.355 (start from octet 1).

type: string

format: byte

#

# ENUMS

#

ExternalClientType:

description: Indicates types of External Clients.

anyOf:

- type: string

enum:

- EMERGENCY_SERVICES

- VALUE_ADDED_SERVICES

- PLMN_OPERATOR_SERVICES

- LAWFUL_INTERCEPT_SERVICES

- PLMN_OPERATOR_BROADCAST_SERVICES

- PLMN_OPERATOR_OM

- PLMN_OPERATOR_ANONYMOUS_STATISTICS

- PLMN_OPERATOR_TARGET_MS_SERVICE_SUPPORT

- type: string

SupportedGADShapes:

description: Indicates supported GAD shapes.

anyOf:

- type: string

enum:

- POINT

- POINT_UNCERTAINTY_CIRCLE

- POINT_UNCERTAINTY_ELLIPSE

- POLYGON

- POINT_ALTITUDE

- POINT_ALTITUDE_UNCERTAINTY

- ELLIPSOID_ARC

- LOCAL_2D_POINT_UNCERTAINTY_ELLIPSE

- LOCAL_3D_POINT_UNCERTAINTY_ELLIPSOID

- RANGE_DIRECTION

- RELATIVE_2D_LOCATION_UNCERTAINTY_ELLIPSE

- RELATIVE_3D_LOCATION_UNCERTAINTY_ELLIPSOID

- type: string

ResponseTime:

description: Indicates acceptable delay of location request.

anyOf:

- type: string

enum:

- LOW_DELAY

- DELAY_TOLERANT

- NO_DELAY

- type: string

PositioningMethod:

description: Indicates supported positioning methods.

anyOf:

- type: string

enum:

- CELLID

- ECID

- OTDOA

- BAROMETRIC_PRESSURE

- WLAN

- BLUETOOTH

- MBS

- MOTION_SENSOR

- DL_TDOA

- DL_AOD

- MULTI-RTT

- NR_ECID

- UL_TDOA

- UL_AOA

- NETWORK_SPECIFIC

- type: string

PositioningMode:

description: Indicates supported modes used for positioning method.

anyOf:

- type: string

enum:

- UE_BASED

- UE_ASSISTED

- CONVENTIONAL

- type: string

GnssId:

description: Global Navigation Satellite System (GNSS) ID.

anyOf:

- type: string

enum:

- GPS

- GALILEO

- SBAS

- MODERNIZED_GPS

- QZSS

- GLONASS

- BDS

- NAVIC

- type: string

Usage:

description: Indicates usage made of the location measurement.

anyOf:

- type: string

enum:

- UNSUCCESS

- SUCCESS_RESULTS_NOT_USED

- SUCCESS_RESULTS_USED_TO_VERIFY_LOCATION

- SUCCESS_RESULTS_USED_TO_GENERATE_LOCATION

- SUCCESS_METHOD_NOT_DETERMINED

- type: string

LcsPriority:

description: Indicates priority of the LCS client.

anyOf:

- type: string

enum:

- HIGHEST_PRIORITY

- NORMAL_PRIORITY

- type: string

VelocityRequested:

description: Indicates velocity requirement.

anyOf:

- type: string

enum:

- VELOCITY_IS_NOT_REQUESTED

- VELOCITY_IS_REQUESTED

- type: string

AccuracyFulfilmentIndicator:

description: Indicates fulfilment of requested accuracy.

anyOf:

- type: string

enum:

- REQUESTED_ACCURACY_FULFILLED

- REQUESTED_ACCURACY_NOT_FULFILLED

- type: string

VerticalDirection:

description: Indicates direction of vertical speed.

type: string

enum:

- UPWARD

- DOWNWARD

LdrType:

description: Indicates LDR types.

anyOf:

- type: string

enum:

- UE_AVAILABLE

- PERIODIC

- ENTERING_INTO_AREA

- LEAVING_FROM_AREA

- BEING_INSIDE_AREA

- MOTION

- type: string

ReportingAreaType:

description: Indicates type of event reporting area.

anyOf:

- type: string

enum:

- EPS_TRACKING_AREA_IDENTITY

- E-UTRAN_CELL_GLOBAL_IDENTIFICATION

- 5GS_TRACKING_AREA_IDENTITY

- NR_CELL_GLOBAL_IDENTITY

- type: string

OccurrenceInfo:

description: Specifies occurrence of event reporting.

anyOf:

- type: string

enum:

- ONE_TIME_EVENT

- MULTIPLE_TIME_EVENT

- type: string

ReportingAccessType:

description: Specifies access types of event reporting.

anyOf:

- type: string

enum:

- NR

- EUTRA_CONNECTED_TO_5GC

- NON_3GPP_CONNECTED_TO_5GC

- NR_LEO

- NR_MEO

- NR_GEO

- NR_OTHER_SAT

- EUTRA_CONNECTED_TO_EPC

- type: string

EventClass:

description: Specifies event classes.

anyOf:

- type: string

enum:

- SUPPLEMENTARY_SERVICES

- type: string

ReportedEventType:

description: Specifies type of event reporting.

anyOf:

- type: string

enum:

- PERIODIC_EVENT

- ENTERING_AREA_EVENT

- LEAVING_AREA_EVENT

- BEING_INSIDE_AREA_EVENT

- MOTION_EVENT

- MAXIMUM_INTERVAL_EXPIRATION_EVENT

- LOCATION_CANCELLATION_EVENT

- INTERMEDIATE_EVENT

- DIRECT_REPORT_EVENT

- CUMULATIVE_EVENT_REPORT

- type: string

TerminationCause:

description: Specifies causes of event reporting termination.

anyOf:

- type: string

enum:

- TERMINATION_BY_UE

- TERMINATION_BY_NETWORK

- NORMAL_TERMINATION

- type: string

LcsQosClass:

description: Specifies LCS QoS class.

anyOf:

- type: string

enum:

- BEST_EFFORT

- ASSURED

- MULTIPLE_QOS

- type: string

UeLocationServiceInd:

description: Specifies location service types requested by UE.

anyOf:

- type: string

enum:

- LOCATION_ESTIMATE

- LOCATION_ASSISTANCE_DATA

- type: string

IndoorOutdoorInd:

description: Specifies UE location indoor or outdoor.

anyOf:

- type: string

enum:

- INDOOR

- OUTDOOR

- type: string

FixType:

description: Specifies the positioning fix type.

anyOf:

- type: string

enum:

- CARRIER_PHASE_FLOAT

- CARRIER_PHASE_FIX

- type: string

LosNlosMeasureInd:

description: Specifies LOS measurement or NLOS measurement.

anyOf:

- type: string

enum:

- LOS

- NLOS

- type: string

UpConnectionStatus:

description: UP Connection Status.

anyOf:

- type: string

enum:

- ESTABLISHED

- RELEASED

- MOVE

- type: string

RangingSlResult:

description: Specifies the type of result requested for ranging and sidelink positioning.

anyOf:

- type: string

enum:

- ABSOLUTE_LOCATION

- RELATIVE_LOCATION

- RANGING_DIRECTION

- RANGING

- DIRECTION

- VELOCITY

- RELATIVE_VELOCITY

- type: string

RelatedUEType:

description: Specifies the different roles of UE for ranging and sidelink positioning service.

anyOf:

- type: string

enum:

- LOCATED_UE

- REFERENCE_UE

- type: string

LcsUpConnectionInd:

description: LCS UP Connection Indication.

anyOf:

- type: string

enum:

- TERMINATION

- SETUP

- type: string

UeUpPositioningCapabilities:

description: User plane positioning capabilities supported by the UE.

anyOf:

- type: string

enum:

- LCS-UPP

- SUPL

- type: string

* * * End of Changes * * * *