3GPP TSG-CT Meeting #103	CP-240219

# Maastricht, Netherlands; 18th – 19th March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 5988 | rev | 6 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on Network slice replacement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Lenovo, ZTE |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-03-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 4.6.2.7 for the mobility management based on the network slice replacement contains some inconsistent text and information, which can result in misunderstanding the concept of the network slice replacement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of inconsistency by not dedicating the slice replacement to the allowed S-NSSAI only. Identifying the slice that is to be replaced and the slice that is the alternative slice. Needed miscellenous corrections to avoid misunderstanding the concept. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current text can result in a misunderstanding of the concept. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was agreed in CT1# 147 but introduces an error that the AMF performs registration. Therefore a new revision is submitted to the plenary to undo the related text and reverts the ext from "by performing" back to "during".. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 4.6.2.7	Mobility management based network slice replacement

The support for network slice replacement by a UE or network is optional. If the UE and network support network slice replacement, and the AMF determines that an S-NSSAI included in the allowed NSSAI needs to be replaced with an alternative S-NSSAI, the AMF provides:

a)	the alternative S-NSSAI in the allowed NSSAI, if not included yet;

b)	the alternative S-NSSAI in the configured NSSAI, if not included yet;

c)	the alternative S-NSSAI in the NSAG information, if not included yet and the UE supports NSAG; and

d)	the alternative NSSAI including the mapping information between the S-NSSAI to be replaced and the corresponding alternative S-NSSAI,

to the UE, during the generic UE configuration update procedure or during the registration procedure as follows:

a)	for non-roaming UE, the AMF provides the mapping information between the S-NSSAI included in the allowed NSSAI and the alternative S-NSSAI to the UE; or

NOTE 1:	In non-roaming scenarios, the alternative S-NSSAI does not have to be part of the subscribed S-NSSAI(s) in the UE subscription.

b)	for roaming UE:

1)	if the S-NSSAI included in the allowed NSSAI needs to be replaced (i.e., the S-NSSAI to be replaced is part of the VPLMN S-NSSAIs), the AMF provides the mapping information between the S-NSSAI included in the allowed NSSAI and the alternative S-NSSAI to the UE; and

2)	if the S-NSSAI included in the mapped S-NSSAI(s) for the allowed NSSAI needs to be replaced (i.e., the S-NSSAI to be replaced is part of the HPLMN S-NSSAIs), the AMF provides the mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI to the UE.

NOTE 1A:	In roaming scenarios, the alternative S-NSSAI, being part of the HPLMN S-NSSAIs does not have to be part of the subscribed S-NSSAI(s) in the UE subscription.

NOTE 2:	It is up to AMF local policy to determine when to provide the mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI to the UE, which can be either when the alternative S-NSSAI is available and there is no established PDU session associated with the S-NSSAI to be replaced, or  when the UE has established the first PDU session associated with the S-NSSAI to be replaced.

If the AMF determines that the S-NSSAI which has been replaced is available again, the AMF provides the updated alternative NSSAI excluding the S-NSSAI which has been replaced and the corresponding alternative S-NSSAI to the UE during the UE configuration update procedure or during the registration procedure.

If all the S-NSSAI(s) that were replaced in alternative NSSAI are available again, the AMF provides the alternative NSSAI with Length of Alternative NSSAI contents set to 0 in the UE configuration update procedure or registration procedure. The AMF also provides the updated allowed NSSAI and configured NSSAI to the UE.

NOTE 3:	If there is S-NSSAI location validity information for both the S-NSSAI to be replaced and the alternative S-NSSAI, the NS-AoS of the alternative S-NSSAI is the same as or larger than the NS-AoS of the S-NSSAI to be replaced. If the NS-AoS of the alternative S-NSSAI is larger than the NS-AoS of the S-NSSAI to be replaced, the UE can request the establishment of user plane resources for PDU session(s) associated with the alternative S-NSSAI in the cell(s) outside the NS-AoS of the S-NSSAI to be replaced but within the NS-AoS of the alternative S-NSSAI.

* * * End of Changes * * * *