3GPP TSG-CT Meeting #103	CP-240250

# Maastricht, Netherlands; 18th – 19th March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 5976 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unavailability information IE inclusion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | SUECR, 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-03-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At unavailability indication at deregistration, the requirements are specified in 5.5.2.2.1: If the network indicated support for the unavailability period in the last registration procedure; and an event is triggered in the UE that would make the UE unavailable for a certain period, and: a) the use of unavailability period is not due to NR satellite access discontinuous coverage and the UE is unable to store its 5GMM and 5GSM contexts; or b) the use of unavailability period is due to NR satellite access discontinuous coverage, the UE shall include the Unavailability information IE, and set the De-registration type to "Normal de-registration" in the DEREGISTRATION REQUEST message. The UE shall start the timer T3521 and enter the state 5GMM-DEREGISTERED-INITIATED. The IE inclusion in DEREGISTRATION REQUEST message definition does not correctly reflect the requirements specified in the procedure part. It is therefore proposed to simplify the message definition inclusion to a high-level description to avoid misalignment and avoid repetition of details captured in procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove details from IE description in message definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of implementation differences when to include Unavailability information IE in DEREGISTATION REQUEST message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: WIC “5GSAT_Ph2” added |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.2.12.2	Unavailability information

The UE may include this IE when an event is triggered in the UE that would make the UE unavailable for a certain period.

* * * End of Changes * * * *