3GPP TSG-CT WG4 Meeting #122	C4-241419

Changsha, China, 15th – 19th April 2024	was C4-241154

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.555 | CR | 0021 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on N5g-ddnmf_Discovery API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed CR (S2-2403839), the N5g-ddnmf_Discovery_MonitorAuthorize service operation was updated in stage 2 for "restricted" discovery, i.e., update the required output parameter to ProSe Restricted Code(s)/Prefix. This CR proposes to update the MonitorAuthDataForRestricted data type and the N5g-ddnmf_Discovery API to align the changes in stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the MonitorAuthDataForRestricted data type in clause 6.1.6.2.12. - Updated the N5g-ddnmf_Discovery API. - Corrected error in clause 6.1.6.2.11. - Added feature EnRestrictedDiscovery for feature negotiation to clause 6.2.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The N5g-ddnmf_Discovery API is not align with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.11, 6.1.6.2.12, 6.1.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the N5g-ddnmf_Discovery API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Correct the cover page. CATT added as co-source. |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 6.1.6.2.11	Type: MonitorAuthDataForOpen

Table 6.1.6.2.11-1: Definition of type MonitorAuthDataForOpen

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseAppCodes | array(ProseApplicationCode) | C | 1..N | When present, this IE shall contain a list of the ProSe Application Codes. (NOTE) |  |
| proseAppPrefix | ProseApplicationPrefix | C | 0..1 | When present, this IE shall contain the Prose Application Prefix. (NOTE) |  |
| proseAppMasks | array(ProseApplicationMask) | M | 1..N | This IE shall contain a list of the ProSe Application Masks. |  |
| ttl | integer | M | 1 | This IE shall contain the TTL. If the value sets to zero, it indicates to revoke the authorization for the monitoring in the PLMN. |  |
| NOTE: Either attribute proseAppCodes or proseAppPrefix shall be present if the discovery type is open. |  |  |  |  |  |

# *** Next Change ***

##### 6.1.6.2.12	Type: MonitorAuthDataForRestricted

Table 6.1.6.2.12-1: Definition of type MonitorAuthDataForRestricted

| Attribute name |  |  | Data type |  |  | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| proseRestrictedCode |  |  | ProseRestrictedCode |  |  | M | 1 | This IE shall contain the ProSe Restricted Code. (NOTE) |  |
| addProseRestrictedCodes | array(ProseRestrictedCode) | C | 1..N | This IE shall be present when multiple ProSe Restricted Codes are discovered. When present, this IE shall contain a list of additional ProSe Restricted Codes. (NOTE) | EnRestrictedDiscovery |
| proseRestrictedPrefix |  |  | ProseRestrictedPrefix |  |  | C | 0..1 | When present, this IE shall contain the ProSe Restricted Code Prefix. (NOTE) | EnRestrictedDiscovery |
| validityTime |  |  | DateTime |  |  | M | 1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire. |  |
| NOTE: When the proseRestrictedPrefix IE is present, the proseRestrictedCode IE shall be ignored and the addProseRestrictedCodes IE shall be absent. |  |  |  |  |  |

# *** Next Change ***

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the N5g-ddnmf_Discovery API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PatchReport | If some of the modifications included in the PATCH request are not successfully implemented, the 5G DDNMF reports the result of PATCH request execution to the consumer. See clause 5.2.7.2 of 3GPP TS 29.500 [5]. |
| x | EnRestrictedDiscovery | The 5G-DDNMF and the NF consumer supporting this feature shall support multiple ProSe Restricted Codes or ProSe Restricted Prefix in MonitorAuthorize service operation outpout for "restricted" discovery. |

# *** Next Change ***

# A.2	N5g-ddnmf_Discovery API

openapi: 3.0.0

********************** Text Skipped for Clarity *********************

MonitorAuthDataForRestricted:

type: object

description: Represents the obtained Announce Authorize Data for restricted discovery for a UE

required:

- proseRestrictedCode

- validityTime

properties:

proseRestrictedCode:

$ref: '#/components/schemas/ProseRestrictedCode'

addProseRestrictedCodes:

type: array

items:

$ref: '#/components/schemas/ProseRestrictedCode'

minItems: 1

proseRestrictedPrefix:

$ref: '#/components/schemas/ProseRestrictedPrefix'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

********************** Text Skipped for Clarity *********************

*** End of Changes ***