3GPP TSG-CT3 Meeting #134	C3-242373

# Changsha, China, 15th – 19st April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.486 | CR | 0124 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Essential corrections to the VAE Server APIs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  | 2024-04-08 |
| Category: | A |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  | The AppplicationQosRequirement is written with 3 "p" letters everywhere in the specification, including in the OpenAPI description, except in Table 6.8.6.2.2-1. As per the guidelines agreed during CT3#120-e meeting (see the comments section of C3-221131), such issues are considered as FASMO due to a misalignment between the main body and the OpenAPI description + a misalignment within the main body. Also, the figure in clause 5.6.2.2.2 is incorrect as it shows a POST instead of PUT, in addition to the resource path segments being also incorrect. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Correct the above-detailed issue. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - Incorrect data type name remains in the specification. - Incorrect figure remains in the specification. |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.2.2, 6.8.6.2.2 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions of the APIs defined in this specification. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

##### 5.6.2.2.2	Query service continuity

Figure 5.6.2.2.2-1 depicts a scenario where a service consumer sends a request to the VAE Server to query service continuity information.

Figure 5.6.2.2.2-1: Query service continuity

When the service consumer (e.g. V2X server) needs to query service continuity information (e.g. receives the local service information request from the VAE Client as specified in the 3GPP TS 24.486 [28]), the service consumer shall send an HTTP GET request as step 1 of the figure 5.6.2.2.2-1 to the "Individual Geographical Area" resource with query parameter V2X service id in "service-id". When the VAE Server receives the HTTP GET request from the service consumer, the VAE Server shall perform the query.

On success, "200 OK" shall be returned as step 2 of the figure 5.6.2.2.2-1 to indicate that the VAE server can support the desired V2X service for the target "Individual Geographical Area" resource. The response body shall contain the "Individual Geographical Area" resource including the requested V2X service id.

If errors occur when processing the HTTP POST request, the VAE Server shall apply error handling procedures as specified in clause 6.5.7.

* * * * Next changes * * * *

##### 6.8.6.2.2	Type: V2vConfigurationData

Table 6.8.6.2.2-1: Definition of type V2vConfigurationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| groupId | V2xGroupId | C | 0..1 | Identity of the V2X group for which the V2X application requirement is initiated. (NOTE) |  |
| serviceId | V2xServiceId | C | 1 | The V2X service ID for which application requirement corresponds to. (NOTE) |  |
| canUeIds | array(V2xUeId) | O | 1..N | List of identities of the V2X UEs, which are candidate to serve as application layer relays. |  |
| appQosReq | AppplicationQosRequirement | O | 0..1 | The application QoS requirements for the V2X service. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the features supported by the service consumer. It shall be included in the first interaction. |  |
| NOTE: Either "groupId" attribute or "serviceId" attribute shall be present. |  |  |  |  |  |

* * * * End of changes * * * *