TSG-CT Meeting #104	CP-241268

Shanghai, P.R. China; 17th – 18th June 2024

Title:	Summary for GBA_U Based APIs

Type:	Work Item Summary

Agenda Item:	21 (CT)

Source:	Huo Weijing

Contact:	huoweijing@chinamobile.com

Summary based on the input provided by China Mobile in CP-241268.

| 960004 | Study on GBA_U Based APIs | FS_GBA_U_API | C6 | CP-221185 | Huo Weijing(China Mobile) |
| --- | --- | --- | --- | --- | --- |
| 1000003 | GBA_U Based APIs | GBA_U_APIs | C6 | CP-231353 | Huo Weijing(China Mobile) |

Introduction
- This WI specifies GBA_U_APIs based on Ks_int_NAF which is derived from the Long Term Key of ISIM/USIM applications on the UICC

-“Study on GBA_U Based APIs” proved the feasibility of GBA_U Based APIs with candidate solutions [1]. “GBA_U Based APIs” [2] [3] [4]specifies the methods of GBA_U_APIs and the Access Control scheme to GBA_U Based APIs.

Description

Your description text here.

GBA_U Based APIs include 2 classes: GBAUCipher and GBAUSignature.The GBAUCipher class is the abstract base class for Cipher algorithms.The GBAUSignature class is the base class for Signature algorithms. Ks_int_NAF keys, internal keys generated during GBA_U process (see 3GPP TS 31.102 and 3GPP TS 33.220), are the only ones used in this class. Those keys are 256 bits symmetric keys then all asymmetric key algorithms are not supported.

There difines an Access Control scheme, when an applet is authorized to use the Ks_int_NAF, the NAF_ID associated to the applet should be included in EFAC_GBAUAPI as defined in USIM or ISIM.

References

List of related CRs: select "TSG Status = Approved" in: 
https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=960004,1000003

[1]	TR 31.822: “Study on  GBA_U Based APIs”.

[2]	3GPP TS 31.102:  "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[3]	3GPP TS 31.103:  "Characteristics of the IP Multimedia Services Identity Module (ISIM) application".

[4]	3GPP TS 31.130:  "(U)SIM Application Programming Interface (API); (U)SIM API for Java™ Card".