3GPP TSG-CT Meeting #104	CP-241305

# Shanghai, P.R. China; 17th – 18th June 2024							(revision of C4-242413)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.176 | CR | 0012 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusion of the video and audio media |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.228 clause AC.9.3.2 defined the procedure of network centric procedure of AR communication. In step 5 and step 6, it defines to connect the audio/video media to MF, but the audio/video media is not defined in the MF service. 5 (Optional). IMS AS initiates a Media re-Negotiation request with UE-A, to connect UE-A's audio/video media stream to MF/MRF. 6. IMS initiates a Media re-Negotiation request with UE-B, to connect UE-B's audio/video media stream to MF/MRF. The UE-A can request to change the network rendering content if the status changed in the UE-A, such as low power, low computing power, etc. Based on the TS 26.114, there are lots of attributes for the audio and video media defined for different encoding types, so it is hard to list all the parameters. To make every thing easier and refer to the H.248, it is proposed to send the m=line and a=lines in the SDP directly to the MF to reserve the media resources for audio and video. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the audio and video media in the MF service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Does not support to connect audio/video media to MF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 6.1.6.1, 6.1.6.2.4, 6.1.6.2.x(new), 6.1.6.3.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the OpenAPI files of Nmf_MRM. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Update the description of SdpString - Update 5.2.2.2.2 and 6.1.6.1 to correct the attribute and data types names. - Correct the description of nonDcMedia in 6.1.6.2.4 to indicate that only one media (audio or video) can be contained in one mediaInfo Rev2: Add the missing “#” in the “ref” path of the defintion localNonDcEndpoint, remoteNonDcEndpoint and sdpaLines in the OpenAPI. Rev3: Adding “/” to changes made in in Rev2 Rev4: to add minItems:1 in the yaml |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.2.2	Creation of a new media context

The NF service consumer shall request a new context by using HTTP method POST with "{apiRoot}/nmf/<apiVersion>/contexts" as resource URI representing the "Contexts Collection", see clause 6.1.3.2.

Figure 5.2.2.2.2-1 Creation of a media context

1.	The NF Service Consumer shall send a POST request to create an "Individual Context" resource in the MF. The payload body of the POST request shall contain a representation of the individual context resource to be created.

The NF service consumer shall include list of termination descriptors in the HTTP message body. Each termination descriptor shall include list of media stream descriptors. Each media stream descriptor shall include:

-	Media ID, i.e. a unique identity of the media stream within the media context instance;

-	Remote Mb specifications, i.e. the media stream IP address and port allocated at the remote endpoint, i.e. remote UE, remote network.

-	Media resource description, which includes

1)	Media resource type, e.g. DC, AR.

2)	If media resource type is set to "DC", the DC media specification shall be included. The DC media specification shall include:

a)	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface. It shall include the SCTP stream Id for the DC, and may include subprotocol, order, maxRetry, maxTime and priority may be included.

b)	Maximum Message Size, which represents the maximum size to be expected.

c)	Data Channel Port, which represents the port of SCTP port for the Data Channel.

d)	Security Setup, which identifies the security setup of the DTLS connection.

e)	Security Certificate Fingerprint, which identifies the security certificate fingerprint.

f	Security Transport Identity, which identifies transport layer identity.

For establishing bootstrap data channel or P2A/A2P application data channel, the following parameters shall be included:

a)	media proxy configuration applicable to the media flow;

b)	remote MDC1/MDC2 media specification information to be applied on the media by the MF;

c)	Replacement HTTP URL for each streamId allocated by the application layer representing the application list (e.g. graphical user interface) offered to the IMS subscriber via the MDC1 interface.

3)	If media resource type is set to "AR", the AR media resource description shall be included, which includes:

a)	Media processing specification.

4)	If media resource type is set to "VIDEO" or "AUDIO", the video or audio media resource description as defined as attribute "remoteNonDcMedia" within the data type MediaInfo shall be included.

2a.	Upon the reception of the HTTP POST request, if the request is accepted and no error occur, the MF shall:

-	create a new media context;

-	assign a media contextId;

-	assign a terminationId for each termination descriptor; and

-	reserve media resources for each mediaId.

The MF shall include a HTTP Location header to provide the location of a newly created resource (MediaContext) together with the status code 201 indicating the requested resource is created in the response message.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.2.3.1-3.

On redirection, the MF shall include 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another MF (service) instance.

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nmf_MRM service based interface protocol.

Table 6.1.6.1-1: Nmf_MRM specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MediaContext | 6.1.6.2.2 | Represents an individual context resource on MF. |  |
| TerminationInfo | 6.1.6.2.3 | Represents the termination information. |  |
| MediaInfo | 6.1.6.2.4 | Represents the media Information. |  |
| DcMedia | 6.1.6.2.5 | Represents the DC media descriptor. |  |
| ArMedia | 6.1.6.2.6 | Represents the AR media descriptor. |  |
| NonDcMedia | 6.1.6.2.x | Represents the audio and video media. |  |
| SdpString | 6.1.6.3.2 | Represents the m=line and a=lines in the SDP media description. |  |
| Mdc2Protocol | 6.1.6.3.3 | Represents the type of the transport layer protocols for MDC2 interface. |  |

Table 6.1.6.1-2 specifies data types re-used by the Nmf_MRM service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nmf_MRM service based interface.

Table 6.1.6.1-2: Nmf_MRM re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [16] | Problem Details |  |
| MediaId | 3GPP TS 29.571 [16] | Uniquely identifies one media flow within an IMS session. |  |
| MaxMessageSize | 3GPP TS 29.571 [16] | Max SCTP user message size. |  |
| MediaResourceType | 3GPP TS 29.571 [16] | Media resource types. |  |
| MediaProxy | 3GPP TS 29.571 [16] | Media proxy configuration applicable to the media flow |  |
| SecuritySetup | 3GPP TS 29.571 [16] | Security setup of the DTLS association. |  |
| DcEndpoint | 3GPP TS 29.571 [16] | DC endpoint |  |
| DcStream | 3GPP TS 29.571 [16] | Data channel stream information. |  |
| ReplaceHttpUrl | 3GPP TS 29.571 [16] | Replacement HTTP URL allocated by the application layer. |  |
| Uri | 3GPP TS 29.571 [16] | Uri |  |
| Endpoint | 3GPP TS 29.571 [16] | Represents the IP endpoint. |  |

* * * Next Change * * * *

##### 6.1.6.2.4	Type: MediaInfo

Table 6.1.6.2.4-1: Definition of type MediaInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mediaId | string | M | 1 | Identifies a media stream, e.g. data channel. |  |
| mediaResourceType | MediaResourceType | M | 1 | Represents the media resource type, e.g. DC, AR. |  |
| localMbEndpoint | Endpoint | C | 0..1 | Represents the local IP address and port number of the local endpoint of MF on the Mb interface. It will be allocated by MF. It shall be contained in the response when creating a new media stream, i.e. Nmf_MRM_Create operation or Nmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. (NOTE 1, NOTE 2) |  |
| remoteMbEndpoint | Endpoint | C | 0..1 | Represents the IP address and port number of the remote endpoint on the Mb interface, e.g. remote UE. It shall be contained in the request of creating a new media stream, i.e. Nmf_MRM_Create operation or Nmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. It shall be "Null" when originating a new media. (NOTE 2) |  |
| dcMedia | DcMedia | C | 0..1 | Represents the data channel media descriptors. It shall be contained if the mediaResourceType is set to "DC". |  |
| arMedia | ArMedia | C | 0..1 | Represents the AR media descriptors. It shall be contained if the mediaResourceType is set to "AR". |  |
| remoteNonDcMedia | NonDcMedia | C | 0..1 | Represents the audio or video media. It shall be contained in the request of creating a new media if the mediaResourceType is set to "AUDIO" or "VIDEO". It can be "Null" when originating a new media. |  |
| localNonDcMedia | NonDcMedia | C | 0..1 | Represents the audio or video media. It shall be contained in the response of creating a new media if the mediaResourceType is set to "AUDIO" or "VIDEO". |  |
| mediaProcessingURL | Uri | C | 0..1 | The mediaProcessingURL indicates the address where MF receive service-related media instructions. It shall be contained in the response when creating a new media stream, i.e. Nmf_MRM_Create operation or Nmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. (NOTE 1) |  |
| NOTE 1: The IE cannot be changed once the media has been established. NOTE 2: The attribute "transport" inside Endpoint data type shall be set to value only "UDP". |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.x	Type: NonDcMedia

Table 6.1.6.2.x-1: Definition of type NonDcMedia

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sdpmLine | SdpString | M | 1 | Represents the m=line in the SDP media description. |  |
| sdpaLines | array(SdpString) | M | 1..N | Represents the a=lines in the SDP media description. |  |

* * * Next Change * * * *

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| SdpString | string | Represents the content after "m=" of the "m=" line and the content after "a=" of the asocciated "a=" lines. |  |

* * * Next Change * * * *

# A.2	Nmf_MRM API

openapi: 3.0.0

info:

title: 'MF Media Resource Management (MRM) Service'

version: 1.0.0-alpha.2

description: |

MF Media Resource Management (MRM) Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.176 V18.1.0; IP Multimedia Subsystem (IMS); Media Function (MF) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.176/

servers:

- url: '{apiRoot}/nmf-mrm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nmf-mrm

paths:

/contexts:

post:

summary: Create a new  media context.

operationId: CreateMediaContext

tags:

- MediaContext(Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MediaContext'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/MediaContext'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nmf-mrm/<version>/contexts/{contextId}

required: true

schema:

type: string

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target MF.

required: true

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target MF.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/contexts/{contextId}:

patch:

summary: Update Media Context

operationId: UpdateMediaContext

tags:

- Media Context ID (Document)

parameters:

- name: contextId

in: path

required: true

description: Unique ID of the Media Context to update.

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request.

content:

application/json:

schema:

$ref: '#/components/schemas/MediaContext'

'204':

description: Expected response with empty body

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target MF.

required: true

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target MF.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29571_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'412':

$ref: 'TS29571_CommonData.yaml#/components/responses/412'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes a given Media Context

operationId: DeleteMediaContext

tags:

- Media Context ID (Document)

parameters:

- name: contextId

in: path

required: true

description: Unique ID of the Media Context to delete.

schema:

type: string

responses:

'204':

description: Expected response to a successful deletion.

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target NRF.

required: true

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target NRF.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nmf-mrm: Access to the Nmf_MRM API.

schemas:

MediaContext:

description: Information of a Media Context in a MF.

type: object

required:

- terminations

properties:

contextId:

type: string

terminations:

type: array

items:

$ref: '#/components/schemas/TerminationInfo'

minItems: 1

TerminationInfo:

description: Represents the termination information.

type: object

required:

- terminationId

- medias

properties:

terminationId:

type: string

medias:

type: array

items:

$ref: '#/components/schemas/MediaInfo'

minItems: 1

MediaInfo:

description: Represents the media information.

type: object

required:

- mediaId

- mediaResourceType

properties:

mediaId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaId'

mediaResourceType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaResourceType'

localMbEndpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

remoteMbEndpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

dcMedia:

$ref: '#/components/schemas/DcMedia'

arMedia:

$ref: '#/components/schemas/ArMedia'

localNonDcMedia:

$ref: '#/components/schemas/NonDcMedia'

remoteNonDcMedia:

$ref: '#/components/schemas/NonDcMedia'

mediaProcessingUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

DcMedia:

description: Represents the DC media descriptor.

type: object

required:

- streams

properties:

mediaProxyConfig:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaProxy'

replaceHttpUrl:

type: object

description: >

Contains a list of replacement HTTP URLs. The streamId attribute

within the ReplaceHttpUrl data type is the key of the map.

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ReplaceHttpUrl'

minProperties: 1

remoteMdc1Endpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

remoteMdc2Endpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

localMdc1Endpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

localMdc2Endpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

mdc2Protocol:

$ref: '#/components/schemas/Mdc2Protocol'

streams:

type: object

description: >

Contains a data channel mapping and configuration information. The streamId

attribute within the DcStream data type is the key of the map.

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DcStream'

minProperties: 1

maxMessageSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MaxMessageSize'

localDcEndpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DcEndpoint'

remoteDcEndpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DcEndpoint'

securitySetup:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecuritySetup'

ArMedia:

description: Represents the AR media descriptor.

type: object

required:

- mediaProcessingSpec

properties:

mediaProcessingSpec:

type: string

NonDcMedia:

description: Represents the audio and video media

type: object

required:

- sdpmLine

- sdpaLines

properties:

sdpmLine:

$ref: '#/components/schemas/SdpString'

sdpaLines:

type: array

items:

$ref: '#/components/schemas/SdpString'

minItems: 1

SdpString:

description: The content after "m=" of the "m=" line and the content after "a=" of the asocciated "a=" lines.

type: string

Mdc2Protocol:

description: The type of the transport layer protocols for MDC2 interface.

anyOf:

- type: string

enum:

- UDP/DTLS/SCTP

- TCP

- UDP

- SCTP

- TCP/TLS

- SCTP/DTLS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

* * * End of Changes * * * *