3GPP TSG-CT Meeting #83	CP-190164

# Shenzhen, P. R. China; 18th – 19th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.328 | CR | 0623 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sh XML schema correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-IMSo5G |  |  |  |  |  | Date: |  |  | 2019-03-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The XML schema in annex D contains elements with the names 5GSLocationInformation and 5GSUserState. It is however not possible to start a name with a digit in XML. The element name is of XML Name type (see http://www.w3.org/TR/xml/#NT-Name) and this type cannot start with a digit. Hence the schema validation fails. Further, in the XML schema file "ShDataTyp_Rel15" attached to the document the following errors, also causing the schema to fail, have been corrected: On line 952 "tExtension" removed from " tTADSinformationExtension2tExtension" On line 959 t prepended to "Extension" On line 983 t prepended to "Sh-Data-Extension7" |  |  |  |  |  |  |  |  |
| Summary of change: |  | Prefix the 5GS in the above cases with Sh-. Corrections in the schema file as in Reason for change. Styles of clause C.2 changed to Heading 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The schema validation fails and the resulting XML document is incorrect which makes the receiver reject the document, and then location services will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.7, C.1, C.2, annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.6.7	User state

This information element indicates the state of the User Identity in the domain/node indicated by the Requested-Domain/Requested-Node (see 7.2), with the values specified in 3GPP TS 23.078 [14] for Subscriber State and PS Domain Subscriber State, and with the values specified in 3GPP TS 29.272 [31] for EPS User State and 3GPP TS 29.518 [47] for the 5GS User State.

-	The HSS shall make use of the operation MAP-PROVIDE-SUBSCRIBER-INFO towards the MSC/VLR to obtain this information if the requested domain is CS.

-	The HSS shall make use of the operation S6a-IDR towards the MME to obtain this information if the requested domain is PS and the requested node is MME.

-	The HSS shall make use of the operation MAP-PROVIDE-SUBSCRIBER-INFO or S6d-IDR towards the SGSN to obtain this information if the requested domain is PS and either the requested node is SGSN or the requested node is not present.

-	The HSS shall make use of the operation S6a-IDR towards the MME and MAP-PROVIDE-SUBSCRIBER-INFO or S6d-IDR towards the SGSN to obtain this information if the requested domain is PS and the Requested Nodes is MME and SGSN.

-	The HSS/UDM shall make use of the Location-Report subscription of Namf_EventExposure Service towards the AMF to obtain this information if the requested domain is PS and the requested node is AMF.

This information element shall contain the information as received from the access nodes.The HSS/UDM may retrieve the User States from the MME, SGSN or AMF within one procedure.

The HSS shall include the value "NotProvidedFromSGSN or MME or AMF" in the "EPSUserState" / "PSUserState" / "Sh-5GSUserState" fields, if the MME, SGSN or AMF does not support the retrieval of User State over S6a/S6d-IDR/ Namf_EventExposure Service, or it did not provide any information on subscriber state even though it was requested by HSS.

* * * Next Change * * * *

# C.1	General description

The following picture gives an outline of the UML model of the user profile, which is exchanged between the HSS and an AS:

Figure C.1.1: Sh-Data

Class Sh-Data contains the following attributes:

-	An optional instance of CSUserState attribute.

-	An optional instance of PSUserState attribute.

-	An optional instance of IMSI attribute.

-	Zero or more instances of IMSPrivateUserIdentity attribute.

-	An optional instance of IMEISV attribute.

Class Sh-Data contains the following classes:

-	An optional instance of the class PublicIdentifiers.

-	Zero or moreinstances of the class RepositoryData.

-	An optional instance of the class Sh-IMS-Data.

-	An optional instance of the class CSLocationInformation.

-	An optional instance of the class PSLocationInformation.

-	An optional instance of the class RegisteredIdentities.

-	An optional instance of the class ImplicitIdentities.

-	An optional instance of the class AllIdentities.

-	An optional instance of the class AliasIdentities.

-	An optional instance of the class DeletedIdentities.

If the AS and the HSS both support the Notif-Eff feature and the AS requires to read more than one Identity Sets of a Public Identity, or regardless whether or not the Notif-Eff feature is supported, the AS has subscribed to be notifed of changes to IMSPublicIdentity for more than one Identity Sets (see table 7.6.1), the class PublicIdentifiers shall not be used to convey IMS Public Identities (it shall however still be used to convey the MSISDNs if so requested). Instead the classes RegisteredIdentities, ImplicitIdentities, AllIdentities and AliasIdentities shall be used and they contain the REGISTERED_IDENTITIES, IMPLICIT_IDENTITIES, ALL_IDENTITIES and ALIAS_IDENTITIES associated with the IMS Public Identity included in the request respectively. See subclause 7.6.2 for the detailed information. The class PublicIdentifiers or the one among the four which may be used to contain the corresponding identity set can both be used to convey IMS Public Identities when AS requires only one identity set of a public identity.

If Public Identity(ies) are deleted and the AS has any subscription active for the deleted identity(ies), the HSS shall notify the AS of the deletion of the identity(ies) using the class DeletedIdentities.

Class RepositoryData contains repository data (transparent data) for a given service that are associated to a Public user Identity or a group of alias Public User Identities. It has attributes ServiceIndication, SequenceNumber and ServiceData.

Class CSUserState contains the state of a user in the CS domain. Its only attribute, State, is an enumeration whose possible values are defined in section 7.6.7.

Class PSUserState contains the state of a user in the PS domain (SGSN). Its only attribute, State, is an enumeration whose possible values are defined in section 7.6.7.

NOTE: the fact that attribute State is an enumeration is a difference from what can be carried in the MAP protocol.

Class EPSUserState contains the state of a user in the PS domain (MME). Its only attribute, State, is an enumeration whose possible values are defined in section 7.6.7. .

Class Sh-5GSUserState contains the state of a user in the PS domain (AMF). Its only attribute, State, is an enumeration whose possible values are defined in section 7.6.7.

Class CSLocationInformation has the attributes LocationNumber, ServiceAreaID, CellGlobalId, LocationAreaId, GeographicalInformation, GeodeticInformation, VLRNumber, MSCNumber, AgeOfLocationInformation, CurrentLocationRetrieved, UserCSGInformation, TrackingAreaId, E-UTRANCellGlobalId and LocalTimeZone. They are defined in 7.6.

Class PSLocationInformation has the attributes ServiceAreaId, CellGlobalId, LocationAreaID, RoutingAreaID, GeographicalInformation, GeodeticInformation, SGSNNumber, AgeOfLocationInformation, CurrentLocationRetrieved, UserCSGInformation, VisitedPLMNID, LocalTimeZone and RATtype. They are defined in 7.6.

Class EPSLocationInformation has the attributes E-UTRANCellGlobalId, TrackingAreaID, GeographicalInformation, GeodeticInformation, MMEName, AgeOfLocationInformation, CurrentLocationRetrieved, UserCSGInformation, VisitedPLMNID, LocalTimeZone and RATtype. They are defined in 7.6.

Class TWANLocationInformation has the attributes TWAN-SSID, TWAN-BSSID, TWAN-PLMNID, CivicAddress, TWANOperatorName, LocalTimeZone and LogicalAccessID. They are defined in 7.6.

Class Sh-5GSLocationInformation has the attributes NRCellGlobalId, E-UTRANCellGlobalId, TrackingAreaID, GeographicalInformation, AMFAddress, AgeOfLocationInformation, CurrentLocationRetrieved, VisitedPLMNID, LocalTimeZone and RATtype. They are defined in 7.6.

Class DeletedIdentities contains Public Identities (IMS Public User Identities or Public Service Identities) removed from the HSS.

Class UEReachabilityForIP contains the UE reachability for IP. Its attributes, UEIPReachabilityMME, UEIPReachabilitySGSN, UEIPReachabilityAMF3GPP and UEIPReachabilityAMFnon3GPP are enumerations whose possible values are defined in section 7.6.17.

Class IMSVoiceOverPSSessionsSupport contains the support of IMS voice over PS at the current access. Its only attribute, SupportIndication, is an enumeration whose possible values are defined in section 7.6.18.

Class IMSI contains the UE's IMSI. See subclause 7.6.24.

Class IMSPrivateUserIdentity contains an IMS Private Identity associated with the IMS Public Identity included in the request.

* * * First Change * * * *

## C.2	PublicIdentifiers

The following picture details the UML model of the class PublicIdentifiers:

Figure C.2.1: The UML model of the class PublicIdentifiers

Class PublicIdentifiers contains the following attributes:

-	Zero or more instances of IMSPublicIdentity attribute. Each instance is a Public Identity. See subclause 7.6.2 for information about contents.

-	An optional instance of IdentityType attribute. If only one instance of IMSPublicIdentity attribute is included, then this attribute identifies the identity type in the IMSPublicIdentity attribute, it could be either:

-	A distinct Public User Identity

-	A distinct Public Service Identity

If more than one instance of IMSPublicIdentity attribute is included, this attribute is assumed to apply to all of them.

If IdentityType attribute is not present, it is assumed to be a distinct Public User Identity.

If more than one instance of IMSPublicIdentity shall be returned, if the identities are of different types the EnhancedIMSPublicIdentifiers class should be used instead.

-	An optional instance of WilcardedPSI attribute, if the Public Service Identity in the request matches a Wildcarded PSI.

NOTE:	This attribute can be omitted if this information is conveyed including Wildcarded PSI information element in the response. See subclause 6.1.1 and 6.1.3.

-	An optional instance of WilcardedIMPU attribute, if the Public User Identity in the request matches a Wildcarded IMPU.

NOTE:	This attribute can be omitted if this information is conveyed including Wildcarded Public User Identity information element in the response. See subclause 6.1.1 and 6.1.3.

-	Zero or more instances of MSISDN attribute, that contains an MSISDN.

Class PublicIdentifiers contains the following classes:

-	Zero to more instances of EnhancedIMSPublicIdentifiers. This class is required when more than one instance of IMSPublicIdentity shall be returned and the identities are of different types. See subclause 7.6.2 for more information about contents. It includes the following attributes:

-	One instance of IMSPublicIdentity attribute that contains a Public Identity.

-	One instance of the IdentityType attribute that identifies the identity type in the IMSPublicIdentity attribute, it could be either:

-	A distinct Public User Identity

-	A distinct Public Service Identity

-	A Wildcarded Public Service Identity

-	A Wildcarded Public User Identity

-	Zero or more instances of ExtendedMSISDN, only when Additional-MSISDN feature is enabled. It includes the following attributes:

-	One instance of MSISDN attribute that contains an MSISDN.

-	One instance of MSISDNType attribute that indicates the MSISDN Type.

Figure C.2.2: Void

* * * Next Change * * * *

Annex D (normative):
XML schema for the Sh interface user profile

The file ShDataType_Rel13.xsd, attached to this specification, contains the XML schema for the user profile that is sent over the Sh interface. The user profile XML schema defines the data types that are used in the user profile XML. 
The data that is allowed to be sent in the user profile may vary depending on the features supported by the Diameter end points, see 3GPP TS 29.329 [5]. The user profile XML schema file is intended to be used by an XML parser. 
The version of the Sh application sending the user profile XML shall be the same as the version of the sent user profile XML and thus it implies the version of the user profile XML schema to be used to validate it.

Tables D.1 and D.2 describe the data types and the dependencies among them that configure the user profile XML schema.

Table D.1: XML schema for the Sh user profile interface: simple data types

| Data type | Tag | Base type | Comments |
| --- | --- | --- | --- |
| tPriority | Priority | integer | >= 0 |
| tProfilePartIndicator | ProfilePartIndicator | enumerated | Possible values: 0 (REGISTERED) 1 (UNREGISTERED) |
| tGroupID | Group | integer | >= 0 |
| tRegistrationType | RegistrationType | enumerated | Possible values: 0 (INITIAL_REGISTRATION) 1 (RE-REGISTRATION) 2 (DE-REGISTRATION) |
| tDefaultHandling | DefaultHandling | enumerated | Possible values: 0 (SESSION_CONTINUED) 1 (SESSION_TERMINATED) |
| tDirectionOfRequest | SessionCase | enumerated | Possible values: 0 (ORIGINATING_SESSION) 1 TERMINATING_SESSION 2 (TERMINATING_UNREGISTERED) 3 (ORIGINATING_UNREGISTERED) 4 (ORIGINATING_CDIV) |
| tIMSUserState | IMSUserState | Enumerated | Possible values: 0 (NOT_REGISTERED) 1 (REGISTERED) 2 (REGISTERED_UNREG_SERVICES) 3 (AUTHENTICATION_PENDING) |
| tCSUserState | CSUserState | Enumerated | Possible values (as defined in 3GPP TS 23.078 [14]): 0 (CAMELBusy) 1 (NetworkDeterminedNotReachable) 2 (AssumedIdle) 3 (NotProvidedfromVLR) |
| tPSUserState | PSUserState | Enumerated | Possible values (as defined in 3GPP TS 23.078 [14]): 0 (Detached) 1 (AttachedNotReachableForPaging) 2 (AttachedReachableForPaging) 3 (ConnectedNotReachableForPaging) 4 (ConnectedReachableForPaging) 5 (NotProvidedFromSGSN or MME or AMF) 6 (NetworkDeterminedNotReachable) |
| tLocationNumber | LocationNumber | string | Syntax described in ITU-T Q.763 [9] (Base64 encoded according to IETF RFC 2045 [15]). Length >=4 and <=16 (multiples of 4). |
| tCellGlobalId | CellGlobalId | string | Syntax described in 3GPP TS 29.002 [13] (Base64 encoded according to IETF RFC 2045 [15]). Length = 12. |
| tServiceAreaId | ServiceAreaId | string | Syntax described in 3GPP TS 29.002 [13] (Base64 encoded according to IETF RFC 2045 [15]). Length = 12. |
| tLocationAreaId | LocationAreaId | string | Syntax described in 3GPP TS 29.002 [13] (Base64 encoded according to IETF RFC 2045 [15]). Length = 8. |
| tRoutingAreaId | RoutingAreaId | string | Syntax described in 3GPP TS 29.002 [13] (Base64 encoded according to IETF RFC 2045 [15]). Length = 8. |
| tGeographicalInformation | GeographicalInformation | string | Syntax described in 3GPP TS 29.002 (base 64 encoded according to IETF RFC 2045). Length = 12. |
| tGeodeticInformation | GeodeticInformation | string | Syntax described in 3GPP TS 29.002 [13] (Base64 encoded according to IETF RFC 2045 [15]). Length = 16. |
| tAgeOfLocationInformation | AgeOfLocationInformation | integer | >=0, <=32767 |
| tCSGId | CSGId | string | Syntax described in 3GPP TS 29.002 [13] i.e. 5 octets BER encoded value of 27-bit BIT STRING (Base64 encoded according to IETF RFC 2045 [15]). Length = 8. |
| tAccessMode | AccessMode | string | Syntax described in 3GPP TS 29.002 [13] (Base64 encoded according to IETF RFC 2045 [15]). Length = 4. |
| tTrackingAreaId | TrackingAreaId | string | Syntax described in 3GPP TS 29.002 [13] (Base64 encoded according to IETF RFC 2045 [15]). Length = 8. |
| tE-UTRANCellGlobalId | E-UTRANCellGlobalId | string | Syntax described in 3GPP TS 29.002 [13] (Base64 encoded according to IETF RFC 2045 [15]). Length = 12. |
| tNRCellGlobalId | NRCellGlobalId | string | Syntax described as NrCellId in 3GPP TS 29.571 [46]. Length = 9. |
| tAddressString | Address | string | Syntax described in 3GPP TS 29.002 [13] (Base64 encoded according to IETF RFC 2045 [15]). Length >= 4 and <=28 (multiples of 4). |
| tMSISDN | MSISDN, STN-SR, CSRN | string | Number structure described in 3GPP TS 23.003 [11]. ASCII encoded according to ANSI X3.4 [20]. |
| tSIP_URL | IMSPublicIdentity | anyURI | Syntax described in IETF RFC 3261 [16]. Wildcarded IMPU and Wildcarded PSI syntax described in 3GPP TS 23.003. |
| tTEL_URL | IMSPublicIdentity | anyURI | Syntax described in IETF RFC 3966 [17]. Wildcarded IMPU and Wildcarded PSI syntax described in 3GPP TS 23.003. |
| tDiameterURI | DiameterURI | string | Syntax of a Diameter URI as described in IETF RFC 6733 [44] |
| tIMSPublicIdentity | IMSPublicIdentity | (union) | Union of tSIP_URL and tTEL_URL |
| tIdentityType | IdentityType | enumerated | Possible values: 0 (PUBLIC_USER_IDENTITY) 1 (DISTINCT_PSI) 2 (WILDCARDED_PSI) 3 (WILDCARDED_IMPU) |
| tWildcardedPSI | WildcardedPSI | anyURI | Syntax described in 3GPP TS 23.003 [11]. |
| tWildcardedIMPU | WildcardedIMPU | anyURI | Syntax described in 3GPP TS 23.003 [11]. |
| tServiceInfo | ServiceInfo | string |  |
| tDSAI-Tag | DSAI-Tag | string |  |
| tString | RequestURI, Method, Header, Content, Line, MMEName, AccessType, AccessInfo, AccessValue, IMEISV | string |  |
| tBool | ConditionTypeCNF, ConditionNegated | boolean | Possible values: 0 (false) 1 (true) |
| tSequenceNumber | SequenceNumber | integer | >=0, <=65535 |
| tPSIActivation | PSIActivation | enumerated | Possible Values: 0 (INACTIVE) 1 (ACTIVE) |
| tDSAI-Value | DSAI-Value | enumerated | Possible values are: 0 (ACTIVE) 1 (INACTIVE) |
| tServiceLevelTraceInfo | ServiceLevelTraceInfo | String | Syntax described in 3GPP TS 24.323 [45] |
| tIPv4Address | IPv4Address | String | IPv4 address is represented in dotted-decimal notation (a sequence of four decimal numbers in the range 0 to 255, separated by ".", for example 192.168.1.1. |
| tIPv6Prefix | IPv6Prefix | String | ASCII encoded for the text representation of the address prefix. Syntax described in IETF RFC 4291 [24] and IETF RFC 5952 [41]. |
| tIPv6InterfaceIdentifier | IPv6InterfaceIdentifier | String | ASCII encoded for the text representation of the address. Syntax described in IETF RFC 4291 [24] and IETF RFC 5952 [41]. |
| tServicePriorityLevel | ServicePriorityLevel | enumerated | Possible values: 0 (Highest priority) 1 2 3 4 (Lowest priority) |
| tUEIPReachabilityMME | UEIPReachabilityMME | enumerated | Possible values: 0 (REACHABLE) |
| tUEIPReachabilitySGSN | UEIPReachabilitySGSN | enumerated | Possible values: 0 (REACHABLE) |
| tUEIPReachabilityAMF3GPP | UEIPReachabilityAMF3GPP | enumerated | Possible values: 0 (REACHABLE) |
| tUEIPReachabilityAMFnon3GPP | UEIPReachabilityAMFnon3GPP | enumerated | Possible values: 0 (REACHABLE) |
| tIP-SM-GW-Number | IP-SM-GW-Number | String | IP-SM-GW is an E.164 address where the digits are ASCII encoded according to ANSI X3.4 [20]. Leading indicators for the nature of address and the numbering plan shall not be included. |
| tIMSVoiceOverPSSessionSupport | IMSVoiceOverPSSessionSupport | enumerated | Possible Values: 0 (IMS-VOICE-OVER-PS-NOT-SUPPORTED) 1 (IMS-VOICE-OVER-PS-SUPPORTED) 2 (IMS-VOICE-OVER-PS-SUPPORT-UNKNOWN) |
| tRATtype | RATtype | enumerated | Possible Values are defined in 3GPP TS 29.212 [28], subclause 5.3.31 |
| tAccessType | AccessType | enumerated | Possible Values: 0 (3GPP-ACCESS) 1 (NON-3GPP-ACCESS) |
| tDateTime | LastUEActivityTime | dateTime |  |
| tUE-SRVCC-Capability | UE-SRVCC-Capability | enumerated | Possible Values: 0 (UE-SRVCC-CAPABILITY-NOT-SUPPORTED) 1 (UE-SRVCC-CAPABILITY-SUPPORTED) |
| tPriorityNamespace | PriorityNamespace | string | Possible values are those of the namespaces that are defined in IETF RFC 4412 [25] or defined according to the IANA registration procedure described in IETF RFC 4412 [25] for Resource-Priority Namespaces. |
| tPriorityLevel | PriorityLevel | string | Possible values depend on the PriorityNamespace and are specified with the associated namespace that is defined in IETF RFC 4412 [25] or defined according to the IANA registration procedure described in IETF RFC 4412 [25] for Resource-Priority Namespaces. |
| tSCAddress | SCAddress | string | SCAddress is an E.164 address where the digits are ASCII encoded according to ANSI X3.4 [20]. Leading indicators for the nature of address and the numbering plan shall not be included. |
| tVisitedPLMNID | VisitedPLMNID | string | Consists of Mobile Country Code (MCC) and Mobile Network Code (MNC) as described in 3GPP TS 23.003 [11]. It is a string with 3 digits MCC and 2 or 3 digits MNC. |
| tTimeZone | TimeZone | string | Syntax described in 3GPP TS 29.272 [26] |
| tDaylightSavingTime | DaylightSavingTime | enumerated | Possible values are defined in 3GPP TS 29.272 [26]. |
| tMSISDNType | MSISDNType | enumerated | Possible values: 0 (BASIC) 1 (ADDITIONAL) See 3GPP TS 23.003 [11] for the definition of Additional-MSISDN. |
| tIMSI | IMSI | string | Number structure described in 3GPP TS 23.003 [11]. ASCII encoded according to ANSI X3.4 [20]. |
| tTWAN-SSID | TWAN-SSID | string | Syntax described in 3GPP TS 29.273 [34]. |
| tTWAN-BSSID | TWAN-BSSID | string | Syntax described in 3GPP TS 29.273 [34]. |
| tTWANOperatorName | TWANOperatorName | string | Syntax described in 3GPP TS 23.003 [11]. |
| tCivicAddress | CivicAddress | base64Binary | Syntax described in subclause 3.1 of IETF RFC 4776 [35] excluding the first 3 octets (Base64 encoded according to IETF RFC 2045 [15]). |
| tLogicalAccessID | LogicalAccessID | string | Syntax described in ETSI ES 283 034 [37]. |
| tIMSPrivateUserIdentity | IMSPrivateUserIdentity | string | Syntax described in 3GPP TS 23.003 [11] |

Table D.2: XML schema for the Sh user profile interface: complex data types

| Data type | Tag | Compound of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tag |  |  |  |  |  |  | Type |  |  |  | Cardinality |  |  |  |
| tSh-Data | Sh-Data | PublicIdentifiers |  |  |  |  |  |  | tPublicIdentity |  |  |  | 0 to 1 |  |  |  |
|  |  | RepositoryData |  |  |  |  |  |  | tTransparentData |  |  |  | 0 to n |  |  |  |
|  |  | Sh-IMS-Data |  |  |  |  |  |  | tShIMSData |  |  |  | 0 to 1 |  |  |  |
|  |  | CSLocationInformation |  |  |  |  |  |  | tCSLocationInformation |  |  |  | 0 to 1 |  |  |  |
|  |  | PSLocationInformation |  |  |  |  |  |  | tPSLocationInformation |  |  |  | 0 to 1 |  |  |  |
|  |  | CSUserState |  |  |  |  |  |  | tCSUserState |  |  |  | 0 to 1 |  |  |  |
|  |  | PSUserState |  |  |  |  |  |  | tPSUserState |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tSh-Data-Extension |  |  |  | 0 to 1 |  |  |  |
| tSh-Data-Extension | Extension | RegisteredIdentites |  |  |  |  |  |  | tPublicIdentity |  |  |  | 0 to 1 |  |  |  |
|  |  | ImplicitIdentities |  |  |  |  |  |  | tPublicIdentity |  |  |  | 0 to 1 |  |  |  |
|  |  | AllIdentities |  |  |  |  |  |  | tPublicIdentity |  |  |  | 0 to 1 |  |  |  |
|  |  | AliasIdentities |  |  |  |  |  |  | tPublicIdentity |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tSh-Data-Extension2 |  |  |  | 0 to 1 |  |  |  |
| tSh-Data-Extension2 | Extension | DeletedIdentities |  |  |  |  |  |  | tPublicIdentity |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tSh-Data-Extension3 |  |  |  | 0 to 1 |  |  |  |
| tSh-Data-Extension3 | Extension | TADSinformation |  |  |  |  |  |  | tTADSinformation |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tSh-Data-Extension4 |  |  |  | 0 to 1 |  |  |  |
| tSh-Data-Extension4 | Extension | EPSUserState |  |  |  |  |  |  | tPSUserState |  |  |  | 0 to 1 |  |  |  |
|  |  | EPSLocationInformation |  |  |  |  |  |  | tEPSLocationInformation |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tSh-Data-Extension5 |  |  |  | 0 to 1 |  |  |  |
| tSh-Data-Extension5 | Extension | IMSI |  |  |  |  |  |  | tIMSI |  |  |  | 0 to 1 |  |  |  |
|  |  | TWANLocationInformation |  |  |  |  |  |  | tTWANLocationInformation |  |  |  | 0 to 1 |  |  |  |
|  |  | IMSPrivateUserIdentity |  |  |  |  |  |  | tIMSPrivateUserIdentity |  |  |  | 0 to n |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tSh-Data-Extension6 |  |  |  | 0 to 1 |  |  |  |
| tSh-Data-Extension6 | Extension | IMEISV |  |  |  |  |  |  | tString (NOTE 6) |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tSh-Data-Extension7 |  |  |  | 0 to 1 |  |  |  |
| tSh-Data-Extension7 | Extension | Sh-5GSLocationInformation |  |  |  |  |  |  | tSh-5GSLocationInformation |  |  |  | 0 to 1 |  |  |  |
|  |  | Sh-5GSUserState |  |  |  |  |  |  | tPSUserState |  |  |  | 0 to 1 |  |  |  |
| tTransparentData | RepositoryData | ServiceIndication |  |  |  |  |  |  | string |  |  |  | 1 |  |  |  |
|  |  | SequenceNumber |  |  |  |  |  |  | tSequenceNumber |  |  |  | 1 |  |  |  |
|  |  | ServiceData |  |  |  |  |  |  | tServiceData |  |  |  | 0 to 1 |  |  |  |
| tServiceData | any | any |  |  |  |  |  |  | any |  |  |  | 1 |  |  |  |
| tIFCs | IFCs | InitialFilterCriteria |  |  |  |  |  |  | tInitialFilterCriteria |  |  |  | 0 to n |  |  |  |
| tShIMSData | Sh-IMS-Data | SCSCFName |  |  |  |  |  |  | tSIP_URL |  |  |  | 0 to 1 |  |  |  |
|  |  | IFCs |  |  |  |  |  |  | tIFCs |  |  |  | 0 to 1 |  |  |  |
|  |  | IMSUserState |  |  |  |  |  |  | tIMSUserState |  |  |  | 0 to 1 |  |  |  |
|  |  | ChargingInformation |  |  |  |  |  |  | tChargingInformation |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tShIMSDataExtension |  |  |  | (0 to 1) |  |  |  |
| tShIMSDataExtension | Extension | PSIActivation |  |  |  |  |  |  | tPSIActivation |  |  |  | (0 to 1) |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tShIMSDataExtension2 |  |  |  | 0 to 1 |  |  |  |
| tShIMSDataExtension2 | Extension | DSAI |  |  |  |  |  |  | tDSAI |  |  |  | 0 to n |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tShIMSDataExtension3 |  |  |  | (0 to 1) |  |  |  |
| tShIMSDataExtension3 | Extension | ServiceLevelTraceInfo |  |  |  |  |  |  | tServiceLevelTraceInfo |  |  |  | (0 to 1) |  |  |  |
|  |  | IPv4Address |  |  |  |  |  |  | tIPv4Address |  |  |  | 0 to 1 |  |  |  |
|  |  | IPv6Prefix |  |  |  |  |  |  | tIPv6Prefix |  |  |  | 0 to 1 |  |  |  |
|  |  | IPv6InterfaceIdentifier |  |  |  |  |  |  | tIPv6InterfaceIdentifier |  |  |  | 0 to 1 |  |  |  |
|  |  | ServicePriorityLevel |  |  |  |  |  |  | tServicePriorityLevel |  |  |  | 0 to 1 |  |  |  |
|  |  | UEReachabilityForIP |  |  |  |  |  |  | tUEReachabilityForIP |  |  |  | 0 to 1 |  |  |  |
|  |  | SMSRegistrationInfo |  |  |  |  |  |  | tSMSRegistrationInfo |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tShIMSDataExtension4 |  |  |  | 0 to 1 |  |  |  |
| tShIMSDataExtension4 | Extension | STN-SR |  |  |  |  |  |  | tMSISDN |  |  |  | 0 to 1 |  |  |  |
|  |  | UE-SRVCC-Capability |  |  |  |  |  |  | tUE-SRVCC-Capability |  |  |  | 0 to 1 |  |  |  |
|  |  | ExtendedPriority |  |  |  |  |  |  | tExtendedPriority |  |  |  | 0 to n |  |  |  |
|  |  | CSRN |  |  |  |  |  |  | tMSISDN |  |  |  | 0 to 1 |  |  |  |
|  |  | Extention |  |  |  |  |  |  | tShIMSDataExtension5 |  |  |  | 0 to 1 |  |  |  |
| tShIMSDataExtension5 | Extension | ReferenceLocationInformation |  |  |  |  |  |  | tReferenceLocationInformation |  |  |  | 0 to n (NOTE 7) |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tShIMSDataExtension6 |  |  |  | 0 to 1 |  |  |  |
| tShIMSDataExtension6 | Extension | MTRRIndication |  |  |  |  |  |  | tBool |  |  |  | 0 to 1 |  |  |  |
| tReferenceLocationInformation | ReferenceLocationInformation | AccessType |  |  |  |  |  |  | tString (NOTE 3) |  |  |  | 0 to 1 |  |  |  |
|  |  | AccessInfo |  |  |  |  |  |  | tString (NOTE 3) |  |  |  | 0 to 1 |  |  |  |
|  |  | AccessValue |  |  |  |  |  |  | tString (NOTE 3) |  |  |  | 0 to 1 |  |  |  |
| tCSLocationInformation | CSLocationInformation | LocationNumber |  |  |  |  |  |  | tLocationNumber |  |  |  | 0 to 1 |  |  |  |
|  |  | Choice of |  |  |  | CellGlobalId |  |  |  | tCellGlobalId |  |  |  | 0 to 1 |  |  |
|  |  |  |  |  |  |  | ServiceAreaId |  |  |  | tServiceAreaId |  |  |  | 0 to 1 |  |
|  |  |  |  |  |  |  |  | LocationAreaId |  |  |  | tLocationAreaId |  |  |  | 0 to 1 |
|  |  |  |  |  | GeographicalInformation |  |  |  | tGeographicalInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | GeodeticInformation |  |  |  | tGeodeticInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | VLRNumber |  |  |  | tISDNAddress |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | MSCNumber |  |  |  | tISDNAddress |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | CurrentLocationRetrieved |  |  |  | tBool |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | AgeOfLocationInformation |  |  |  | tAgeOfLocationInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | Extension |  |  |  | tCSLocationInformation-Extension |  |  |  | 0 to 1 |  |  |  |
| tCSLocationInformation-Extension | CSLocationInformation-Extension | UserCSGInformation |  |  |  |  |  |  | tUserCSGInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | Extension |  |  |  | tCSLocationInformation-Extension2 |  |  |  | 0 to 1 |  |  |  |
| tCSLocationInformation-Extension2 | CSLocationInformation-Extension2 | E-UTRANCellGlobalId |  |  |  |  |  |  | tE-UTRANCellGlobalId |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | TrackingAreaId |  |  |  | tTrackingAreaId |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | Extension |  |  |  | tCSLocationInformation-Extension3 |  |  |  | 0 to 1 |  |  |  |
| tCSLocationInformation-Extension3 | CSLocationInformation-Extension3 | LocalTimeZone |  |  |  |  |  |  | tLocalTimeZone |  |  |  | 0 to 1 |  |  |  |
| tPSLocationInformation | PSLocationInformation | Choice of |  |  |  | CellGlobalId |  |  |  | tCellGlobalId |  |  |  | 0 to 1 |  |  |
|  |  |  |  |  |  |  | ServiceAreaId |  |  |  | tServiceAreaId |  |  |  | 0 to 1 |  |
|  |  |  |  |  |  |  |  | LocationAreaId |  |  |  | tLocationAreaId |  |  |  | 0 to 1 |
|  |  |  |  |  | RoutingAreaId |  |  |  | tRoutingAreaId |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | GeographicalInformation |  |  |  | tGeographicalInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | GeodeticInformation |  |  |  | tGeodeticInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | SGSNNumber |  |  |  | tISDNAddress |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | CurrentLocationRetrieved |  |  |  | tBool |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | AgeOfLocationInformation |  |  |  | tAgeOfLocationInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | Extension |  |  |  | tPSLocationInformation-Extension |  |  |  | 0 to 1 |  |  |  |
| tPSLocationInformation-Extension | PSLocationInformation-Extension | UserCSGInformation |  |  |  |  |  |  | tUserCSGInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | Extension |  |  |  | tPSLocationInformation-Extension2 |  |  |  | 0 to 1 |  |  |  |
| tPSLocationInformation-Extension2 | PSLocationInformation-Extension2 | VisitedPLMNID |  |  |  |  |  |  | tVisitedPLMNID |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | LocalTimeZone |  |  |  | tLocalTimeZone |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | RATtype |  |  |  | tRATtype |  |  |  | 0 to 1 |  |  |  |
| tLocalTimeZone | LocalTimeZone | TimeZone |  |  |  |  |  |  | tTimeZone |  |  |  | 1 |  |  |  |
|  |  |  |  |  | DaylightSavingTime |  |  |  | tDaylightSavingTime |  |  |  | 1 |  |  |  |
| tEPSLocationInformation | EPSLocationInformation | E-UTRANCellGlobalId |  |  |  |  |  |  | tE-UTRANCellGlobalId |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | TrackingAreaId |  |  |  | tTrackingAreaId |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | GeographicalInformation |  |  |  | tGeographicalInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | GeodeticInformation |  |  |  | tGeodeticInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | MMEName |  |  |  | tString |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | CurrentLocationRetrieved |  |  |  | tBool |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | AgeOfLocationInformation |  |  |  | tAgeOfLocationInformation |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | UserCSGInformation |  |  |  | tUserCSGInformation |  |  |  |  |  |  |  |
|  |  |  |  |  | Extension |  |  |  | tEPSLocationInformation-Extension |  |  |  | 0 to 1 |  |  |  |
| tEPSLocationInformation-Extension | EPSLocationInformation-Extension | VisitedPLMNID |  |  |  |  |  |  | tVisitedPLMNID |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | LocalTimeZone |  |  |  | tLocalTimeZone |  |  |  | 0 to 1 |  |  |  |
|  |  |  |  |  | Extension |  |  |  | tEPSLocationInformation-Extension2 |  |  |  | 0 to 1 |  |  |  |
| tEPSLocationInformation-Extension2 | EPSLocationInformation-Extension2 | RATtype |  |  |  |  |  |  | tRATtype |  |  |  | 0 to 1 |  |  |  |
| tSh-5GSLocationInformation | Sh-5GSLocationInformation | NRCellGlobalId |  |  |  |  |  |  | tNRCellGlobalId |  |  |  | 0 to 1 |  |  |  |
|  |  | E-UTRANCellGlobalId |  |  |  |  |  |  | tE-UTRANCellGlobalId |  |  |  | 0 to 1 |  |  |  |
|  |  | TrackingAreaId |  |  |  |  |  |  | tTrackingAreaId |  |  |  | 0 to 1 |  |  |  |
|  |  | GeographicalInformation |  |  |  |  |  |  | tGeographicalInformation |  |  |  | 0 to 1 |  |  |  |
|  |  | AMFAddress |  |  |  |  |  |  | tString |  |  |  | 0 to 1 |  |  |  |
|  |  | CurrentLocationRetrieved |  |  |  |  |  |  | tBool |  |  |  | 0 to 1 |  |  |  |
|  |  | AgeOfLocationInformation |  |  |  |  |  |  | tAgeOfLocationInformation |  |  |  | 0 to 1 |  |  |  |
|  |  | VisitedPLMNID |  |  |  |  |  |  | tVisitedPLMNID |  |  |  | 0 to 1 |  |  |  |
|  |  | LocalTimeZone |  |  |  |  |  |  | tLocalTimeZone |  |  |  | 0 to 1 |  |  |  |
|  |  | RATtype |  |  |  |  |  |  | tRATtype |  |  |  | 0 to 1 |  |  |  |
| tUserCSGInformation | UserCSGInformation | CSGId |  |  |  |  |  |  | tCSGId |  |  |  | 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tUserCSGInformation-Extension |  |  |  | 0 to 1 |  |  |  |
| tUserCSGInformation-Extension | UserCSGInformation-Extension | AccessMode |  |  |  |  |  |  | tAccessMode |  |  |  | 0 to 1 |  |  |  |
|  |  | CMI |  |  |  |  |  |  | tBool |  |  |  | 0 to 1 |  |  |  |
| tTADSinformation | TADSinformation | IMSVoiceOverPSSessionSupport |  |  |  |  |  |  | tIMSVoiceOverPSSessionSupport |  |  |  | 1 |  |  |  |
|  |  | RATtype |  |  |  |  |  |  | tRATtype |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tTADSinformationExtension |  |  |  | 0 to 1 |  |  |  |
| tTWANLocationInformation | TWANLocationInformation | TWAN-SSID |  |  |  |  |  |  | tTWAN-SSID |  |  |  | 0 to 1 |  |  |  |
|  |  | TWAN-BSSID |  |  |  |  |  |  | tTWAN-BSSID |  |  |  | 0 to 1 |  |  |  |
|  |  | TWAN-PLMNID |  |  |  |  |  |  | tVisitedPLMNID |  |  |  | 0 to 1 |  |  |  |
|  |  | CivicAddress |  |  |  |  |  |  | tCivicAddress |  |  |  | 0 to n |  |  |  |
|  |  | TWANOperatorName |  |  |  |  |  |  | tTWANOperatorName |  |  |  | 0 to 1 |  |  |  |
|  |  | LocalTimeZone |  |  |  |  |  |  | tLocalTimeZone |  |  |  | 0 to 1 |  |  |  |
|  |  | LogicalAccessID |  |  |  |  |  |  | tLogicalAccessID |  |  |  | 0 to 1 |  |  |  |
| tTADSinformationExtension | TADSinformationExtension | LastUEActivityTime |  |  |  |  |  |  | tDateTime |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tTADSinformationExtension2 |  |  |  | 0 to 1 |  |  |  |
| tTADSinformationExtension2 | TADSinformationExtension2 | AccessType |  |  |  |  |  |  | tAccessType |  |  |  | 0 to 1 |  |  |  |
| tISDNAddress | SGSNNumber, VLRNumber, MSCNumber | Address |  |  |  |  |  |  | tAddressString |  |  |  | 1 |  |  |  |
| tPublicIdentity | PublicIdentifiers, RegisteredIdentites, ImplicitIdentities, AllIdentities, AliasIdentities, DeletedIdentities | IMSPublicIdentity |  |  |  |  |  |  | tIMSPublicIdentity |  |  |  | 0 to n |  |  |  |
|  |  | MSISDN |  |  |  |  |  |  | tMSISDN |  |  |  | 0 to n |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tPublicIdentityExtension |  |  |  | 0 to 1) |  |  |  |
| tPublicIdentityExtension | Extension | IdentityType |  |  |  |  |  |  | tIdentityType |  |  |  | (0 to 1) |  |  |  |
|  |  | WildcardedPSI |  |  |  |  |  |  | tWildcardedPSI |  |  |  | (0 to 1) |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tPublicIdentityExtension2 |  |  |  | (0 to 1) |  |  |  |
| tPublicIdentityExtension2 | Extension | WildcardedIMPU |  |  |  |  |  |  | tWildcardedIMPU |  |  |  | (0 to 1) |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tPublicIdentityExtension3 |  |  |  | 0 to 1 |  |  |  |
| tPublicIdentityExtension3 | Extension | ExtendedMSISDN |  |  |  |  |  |  | tExtendedMSISDN |  |  |  | 0 to n (NOTE 5) |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tPublicIdentityExtension4 |  |  |  | 0 to 1 |  |  |  |
| tPublicIdentityExtension4 | Extension | EnhancedIMSPublicIdentifiers |  |  |  |  |  |  | tEnhancedIMSPublicIdentity |  |  |  | 0 to n |  |  |  |
| tEnhancedIMSPublicIdentity | EnhancedIMSPublicIdentifiers | IMSPublicIdentity |  |  |  |  |  |  | tIMSPublicIdentity |  |  |  | 1 |  |  |  |
|  |  | IdentityType |  |  |  |  |  |  | tIdentityType |  |  |  | 1 |  |  |  |
| tInitialFilterCriteria | InitialFilterCriteria | Priority |  |  |  |  |  |  | tPriority |  |  |  | 1 |  |  |  |
|  |  | TriggerPoint |  |  |  |  |  |  | tTrigger |  |  |  | 0 to 1 |  |  |  |
|  |  | ApplicationServer |  |  |  |  |  |  | tApplicationServer |  |  |  | 1 |  |  |  |
|  |  | ProfilePartIndicator |  |  |  |  |  |  | tProfilePartIndicator |  |  |  | 0 to 1 |  |  |  |
| tTrigger | TriggerPoint | ConditionTypeCNF |  |  |  |  |  |  | tBool |  |  |  | 1 |  |  |  |
|  |  | SPT |  |  |  |  |  |  | tSePoTri |  |  |  | 1 to n |  |  |  |
| tSePoTri | SPT | ConditionNegated |  |  |  |  |  |  | tBool |  |  |  | 0 to 1 |  |  |  |
|  |  | Group |  |  |  |  |  |  | tGroupID |  |  |  | 1 to n |  |  |  |
|  |  | Choice of |  |  |  | RequestURI |  |  |  | tString |  |  |  | 1 |  |  |
|  |  |  |  |  |  |  | Method |  |  |  | tString |  |  |  | 1 |  |
|  |  |  |  |  |  |  |  | SIPHeader |  |  |  | tHeader |  |  |  | 1 |
|  |  |  |  |  |  |  |  | SessionCase |  |  |  | tDirectionOfRequest |  |  |  | 1 |
|  |  |  |  |  |  |  |  | SessionDescription |  |  |  | tSessionDescription |  |  |  | 1 |
|  |  |  |  |  | Extension |  |  |  | tSePoTriExtension |  |  |  | (0 to 1) |  |  |  |
| tSePoTriExtension | Extension | RegistrationType |  |  |  |  |  |  | tRegistrationType |  |  |  | (0 to 2) |  |  |  |
| tHeader | SIPHeader | Header |  |  |  |  |  |  | tString |  |  |  | 1 |  |  |  |
|  |  | Content |  |  |  |  |  |  | tString |  |  |  | 0 to 1 |  |  |  |
| tSessionDescription | SessionDescription | Line |  |  |  |  |  |  | tString |  |  |  | 1 |  |  |  |
|  |  | Content |  |  |  |  |  |  | tString |  |  |  | 0 to 1 |  |  |  |
| tApplicationServer | ApplicationServer | ServerName |  |  |  |  |  |  | tSIP_URL |  |  |  | 1 |  |  |  |
|  |  | DefaultHandling |  |  |  |  |  |  | tDefaultHandling |  |  |  | 0 to 1 |  |  |  |
|  |  | ServiceInfo |  |  |  |  |  |  | tServiceInfo |  |  |  | 0 to 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tApplicationServerExtension |  |  |  | 0 to 1 |  |  |  |
| tApplicationServerExtension | Extension | IncludeRegisterRequest |  |  |  |  |  |  | tIncludeRegisterRequest |  |  |  | 0 to 1 |  |  |  |
|  |  | IncludeRegisterResponse |  |  |  |  |  |  | tIncludeRegisterResponse |  |  |  | 0 to 1 |  |  |  |
| tIncludeRegisterRequest | IncludeRegisterRequest | (NOTE 4) |  |  |  |  |  |  | (NOTE 4) |  |  |  | 0 to 1 |  |  |  |
| tIncludeRegisterResponse | tIncludeRegisterResponse | (NOTE 4) |  |  |  |  |  |  | (NOTE 4) |  |  |  | 0 to 1 |  |  |  |
| tChargingInformation | ChargingInformation | PrimaryEventChargingFunctionName |  |  |  |  |  |  | tDiameterURI |  |  |  | 0 to 1 (NOTE 2) |  |  |  |
|  |  | SecondaryEventChargingFunctionName |  |  |  |  |  |  | tDiameterURI |  |  |  | 0 to 1 |  |  |  |
|  |  | PrimaryCharging CollectionFunctionName |  |  |  |  |  |  | tDiameterURI |  |  |  | 0 to 1 (NOTE 2) |  |  |  |
|  |  | SecondaryCharging CollectionFunctionName |  |  |  |  |  |  | tDiameterURI |  |  |  | 0 to 1 |  |  |  |
| tDSAI | DSAI | DSAI-Tag |  |  |  |  |  |  | tDSAI-Tag |  |  |  | 1 |  |  |  |
|  |  | DSAI-Value |  |  |  |  |  |  | tDSAI-Value |  |  |  | 1 |  |  |  |
| tUEReachabilityForIP | UEReachabilityForIP | UEIPReachabilityMME |  |  |  |  |  |  | tUEIPReachabilityMME |  |  |  | (0 to 1) |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tUEReachabilityForIPExtension |  |  |  | (0 to 1) |  |  |  |
| tUEReachabilityForIPExtension | Extension | UEIPReachabilitySGSN |  |  |  |  |  |  | tUEIPReachabilitySGSN |  |  |  | (0 to 1) |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tUEReachabilityForIPExtension2 |  |  |  | (0 to 1) |  |  |  |
| tUEReachabilityForIPExtension2 | Extension | UEIPReachabilityAMF3GPP |  |  |  |  |  |  | tUEIPReachabilityAMF3GPP |  |  |  | (0 to 1) |  |  |  |
|  |  | UEIPReachabilityAMFnon3GPP |  |  |  |  |  |  | tUEIPReachabilityAMFnon3GPP |  |  |  | (0 to 1) |  |  |  |
| tSMSRegistrationInfo | SMSRegistrationInfo | IP-SM-GW-Number |  |  |  |  |  |  | tIP-SM-GW-Number |  |  |  | 1 |  |  |  |
|  |  | Extension |  |  |  |  |  |  | tSMSRegistrationInfoExtension |  |  |  | (0 to 1) |  |  |  |
| tSMSRegistrationInfoExtension | SMSRegistrationInfoExtension | SCAddress |  |  |  |  |  |  | tSCAddress |  |  |  | (0 to 1) |  |  |  |
| tExtendedPriority | ExtendedPriority | PriorityNamespace |  |  |  |  |  |  | tPriorityNamespace |  |  |  | 1 |  |  |  |
|  |  | PriorityLevel |  |  |  |  |  |  | tPriorityLevel |  |  |  | 1 |  |  |  |
| tExtendedMSISDN | ExtendedMSISDN | MSISDN |  |  |  |  |  |  | tMSISDN |  |  |  | 1 |  |  |  |
|  |  | MSISDNType |  |  |  |  |  |  | tMSISDNType |  |  |  | 1 |  |  |  |
| NOTE 1: "n" shall be interpreted as non-bounded. NOTE 2: At least one of these two information elements (PrimaryEventChargingFunctionName or PrimaryChargingCollectionFunctionName) shall be present. NOTE 3: The syntax of AccessType, AccessInfo and AccessValue is as described in 3GPP TS 29.228 [6]. NOTE 4: empty cells shall be interpreted as complex XML elements without defined content. NOTE 5: PublicIdentifiers contains ExtendedMSISDN in addition to MSISDN when Additional-MSISDN feature is enabled. NOTE 6: Syntax described in 3GPP TS 23.003 [11]; it shall contain either an IMEI value (a string of 14 digits) or an IMEISV value (a string of 16 digits). NOTE 7: The HSS shall not send more than one instance of ReferenceLocationInformation and if the Application Server receives more than one instance of ReferenceLocationInformation it may arbitrarily pick one for further processing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

Changes to file "ShDataType_Rel15.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" elementFormDefault="qualified" attributeFormDefault="unqualified">

<xs:simpleType name="tSIP_URL" final="list restriction">

<xs:restriction base="xs:anyURI"/>

</xs:simpleType>

<xs:simpleType name="tTEL_URL" final="list restriction">

<xs:restriction base="xs:anyURI"/>

</xs:simpleType>

<xs:simpleType name="tDiameterURI" final="list restriction">

<xs:restriction base="xs:anyURI"/>

</xs:simpleType>

<xs:simpleType name="tIdentityType" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="3"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">PUBLIC_USER_IDENTITY</label>

<definition xml:lang="en">Identity is a Public User Identity.</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">DISTINCT_PSI</label>

<definition xml:lang="en">Identity is a distinct Public Service Identity.</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">WILDCARDED_PSI</label>

<definition xml:lang="en">Identity matches a wildcarded Public Service Identity.</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">WILDCARDED_IMPU</label>

<definition xml:lang="en">Identity matches a Wildcarded Public User Identity.</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tIMSPublicIdentity" final="#all">

<xs:union memberTypes="tSIP_URL tTEL_URL"/>

</xs:simpleType>

<xs:simpleType name="tProfilePartIndicator" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">REGISTERED</label>

<definition xml:lang="en">iFC is part of the registered profile</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">UNREGISTERED</label>

<definition xml:lang="en">iFC is part of the unregistered profile</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tServiceInfo" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tDSAI-Tag" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tServiceLevelTraceInfo" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tIPv4Address" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tIPv6Prefix" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tIPv6InterfaceIdentifier" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tIP-SM-GW-Number" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tString" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tServicePriorityLevel" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="4"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">PRIORITY-0</label>

<definition xml:lang="en">Maximum priority level</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en"> PRIORITY-1</label>

<definition xml:lang="en">Priority level 1</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation>

<xs:documentation>

<label xml:lang="en"> PRIORITY-2</label>

<definition xml:lang="en"> Priority level 2</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>

<label xml:lang="en"> PRIORITY-3</label>

<definition xml:lang="en"> Priority level 3</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="4">

<xs:annotation>

<xs:documentation>

<label xml:lang="en"> PRIORITY-4</label>

<definition xml:lang="en">Minimum priority level</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tMSISDN" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tIMSUserState" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="3"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">NOT_REGISTERED</label>

<definition xml:lang="en">Not registered</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">REGISTERED</label>

<definition xml:lang="en">Registered</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">REGISTERED_UNREG_SERVICES</label>

<definition xml:lang="en">Registered, with services for unregistered</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">AUTHENTICATION_PENDING </label>

<definition xml:lang="en">Pending of authentication</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tCSUserState" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="3"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">CAMELBusy</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">NetworkDeterminedNotReachable</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">AssumedIdle</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">NotProvidedFromVLR</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tPSUserState" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="6"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">Detached </label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">AttachedNotReachableForPaging</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">AttachedReachableForPaging</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">ConnectedNotReachableForPaging</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="4">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">ConnectedReachableForPaging</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="5">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">notProvidedFromSGSNorMME</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="6">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">networkDeterminedNotReachable</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tPSIActivation" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">INACTIVE</label>

<definition xml:lang="en"> The PSI is not available for incoming traffic.</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">ACTIVE</label>

<definition xml:lang="en">The PSI is available for incoming traffic.</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tDSAI-Value" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">ACTIVE</label>

<definition xml:lang="en">The service is active</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">INACTIVE</label>

<definition xml:lang="en">The service is inactive</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tLocationNumber" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="4"/>

<xs:maxLength value="16"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tCellGlobalId" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="12"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tServiceAreaId" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="12"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tLocationAreaId" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="8"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tRoutingAreaId" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="8"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tGeographicalInformation" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="12"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tGeodeticInformation" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="16"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tE-UTRANCellGlobalId" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="12"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tNRCellGlobalId" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="9"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tTrackingAreaId" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="8"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tAddressString" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="4"/>

<xs:maxLength value="28"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tSelectedLSAIdentity" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="4"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tPriority" final="list restriction">

<xs:restriction base="xs:int">

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tGroupID" final="list restriction">

<xs:restriction base="xs:int">

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tRegistrationType" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="2"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">INITIAL_REGISTRATION</label>

<definition xml:lang="en">Matches to REGISTER messages that are relater to initial registration.</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">RE-REGISTRATION</label>

<definition xml:lang="en">Matches to REGISTER messages that are relater to re-registration.</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">DE-REGISTRATION</label>

<definition xml:lang="en">Matches to REGISTER messages that are relater to de-registration.</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tID" final="list restriction">

<xs:restriction base="xs:int">

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tUEIPReachabilityMME" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">REACHABLE</label>

<definition xml:lang="en">MME notified that the UE is reachable again</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tUEIPReachabilitySGSN" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">REACHABLE</label>

<definition xml:lang="en">SGSN notified that the UE is reachable again</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tUEIPReachabilityAMF3GPP" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">REACHABLE</label>

<definition xml:lang="en">AMF3GPP notified that the UE is reachable again</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tUEIPReachabilityAMFnon3GPP" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">REACHABLE</label>

<definition xml:lang="en">AMFnon3GPP notified that the UE is reachable again</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tDirectionOfRequest" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="4"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">ORIGINATING_SESSION</label>

<definition xml:lang="en">Originating Session</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">TERMINATING_SESSION</label>

<definition xml:lang="en">Terminating Session</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">TERMINATING_UNREGISTERED</label>

<definition xml:lang="en">Terminating Session for unregistered user</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">ORIGINATING_UNREGISTERED</label>

<definition xml:lang="en">Originating Session for an unregistered user</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="4">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">ORIGINATING_CDIV</label>

<definition xml:lang="en">Originating Session for call diversion</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tDefaultHandling" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">SESSION_CONTINUED</label>

<definition xml:lang="en">Session Continued</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">SESSION_TERMINATED</label>

<definition xml:lang="en">Session Terminated</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tAgeOfLocationInformation" final="list restriction">

<xs:restriction base="xs:int">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="32767"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tBool">

<xs:restriction base="xs:boolean"/>

</xs:simpleType>

<xs:simpleType name="tSequenceNumber" final="list restriction">

<xs:restriction base="xs:int">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="65535"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tIMSVoiceOverPSSessionSupport" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:maxInclusive value="2"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">IMS-VOICE-OVER-PS-NOT-SUPPORTED</label>

<definition xml:lang="en">IMS voice is not supported</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en"> IMS-VOICE-OVER-PS-SUPPORTED </label>

<definition xml:lang="en"> IMS voice is supported </definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">IMS-VOICE-OVER-PS-SUPPORT-UNKNOWN</label>

<definition xml:lang="en">Support of IMS voice is unknown</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tRATtype" final="list restriction">

<xs:restriction base="xs:int">

<xs:annotation>

<xs:documentation>

<label xml:lang="en"> Radio Access Technology Type </label>

<definition xml:lang="en"> See subclause 5.3.31 in 3GPP TS 29.212 </definition>

</xs:documentation>

</xs:annotation>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tAccessType" final="restriction list">

<xs:restriction base="xs:unsignedByte">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">3GPP-ACCESS</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">NON-3GPP-ACCESS</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tCSGId" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="8"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tAccessMode" final="list restriction">

<xs:restriction base="xs:string">

<xs:length value="4"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tDateTime">

<xs:restriction base="xs:dateTime"/>

</xs:simpleType>

<xs:simpleType name="tVisitedPLMNID" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="5"/>

<xs:maxLength value="6"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tTimeZone" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="2"/>

<xs:maxLength value="3"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tDaylightSavingTime" final="restriction list">

<xs:restriction base="xs:unsignedByte">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">NO_ADJUSTMENT</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">PLUS_ONE_HOUR_ADJUSTMENT</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">PLUS_TWO_HOURS_ADJUSTMENT</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tMSISDNType" final="list restriction">

<xs:restriction base="xs:unsignedByte">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">BASIC</label>

<definition xml:lang="en">MSISDN is the MSISDN or the Basic MSISDN in case of multinumbering</definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">ADDITIONAL</label>

<definition xml:lang="en">MSISDN is an Additional MSISDN, when feature Additional MSISDN is enabled </definition>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tIMSI" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tTWAN-SSID" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tTWAN-BSSID" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tTWANOperatorName" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tCivicAddress" final="list restriction">

<xs:restriction base="xs:base64Binary">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tLogicalAccessID" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tIMSPrivateUserIdentity" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tExtension">

<xs:sequence>

<xs:any processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tShIMSDataExtension">

<xs:sequence>

<xs:element name="PSIActivation" type="tPSIActivation" minOccurs="0"/>

<xs:element name="Extension" type="tShIMSDataExtension2" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tShIMSDataExtension2">

<xs:sequence>

<xs:element name="DSAI" type="tDSAI" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Extension" type="tShIMSDataExtension3" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tShIMSDataExtension3">

<xs:sequence>

<xs:element name="ServiceLevelTraceInfo" type="tServiceLevelTraceInfo" minOccurs="0"/>

<xs:element name="IPv4Address" type="tIPv4Address" minOccurs="0"/>

<xs:element name="IPv6Prefix" type="tIPv6Prefix" minOccurs="0"/>

<xs:element name="IPv6InterfaceIdentifier" type="tIPv6InterfaceIdentifier" minOccurs="0"/>

<xs:element name="ServicePriorityLevel" type="tServicePriorityLevel" minOccurs="0"/>

<xs:element name="UEReachabilityForIP" type="tUEReachabilityForIP" minOccurs="0"/>

<xs:element name="SMSRegistrationInfo" type="tSMSRegistrationInfo" minOccurs="0"/>

<xs:element name="Extension" type="tShIMSDataExtension4" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tShIMSDataExtension4">

<xs:sequence>

<xs:element name="STN-SR" type="tMSISDN" minOccurs="0"/>

<xs:element name="UE-SRVCC-Capability" type="tUE-SRVCC-Capability" minOccurs="0"/>

<xs:element name="ExtendedPriority" type="tExtendedPriority" minOccurs="0" maxOccurs="unbounded"/>

<xs:element minOccurs="0" name="CSRN" type= "tMSISDN" />

<xs:element name="Extension" type="tShIMSDataExtension5" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tShIMSDataExtension5">

<xs:sequence>

<xs:element name="ReferenceLocationInformation" type= "tReferenceLocationInformation" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Extension" type="tShIMSDataExtension6" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tShIMSDataExtension6">

<xs:sequence>

<xs:element name="MTRRIndication" type= "tBool" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="tUE-SRVCC-Capability" final="restriction list">

<xs:restriction base="xs:unsignedByte">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="1"/>

<xs:enumeration value="0">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">UE-SRVCC-CAPABILITY-NOT-SUPPORTED</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>

<label xml:lang="en">UE-SRVCC-CAPABILITY-SUPPORTED</label>

</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tExtendedPriority">

<xs:sequence>

<xs:element name="PriorityNamespace" type="tPriorityNamespace"/>

<xs:element name="PriorityLevel" type="tPriorityLevel"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="tPriorityNamespace" final="list restriction">

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="tPriorityLevel" final="list restriction">

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="tSCAddress" final="list restriction">

<xs:restriction base="xs:string">

<xs:minLength value="0"/>

<xs:maxLength value="20"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tReferenceLocationInformation">

<xs:sequence>

<xs:element name="AccessType" type="tString" minOccurs="0"/>

<xs:element name="AccessInfo" type="tString" minOccurs="0"/>

<xs:element name="AccessValue" type="tString" minOccurs="0"/>

<xs:element minOccurs="0" name="Extension" type="tExtension"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tPSLocationInformation-Extension">

<xs:sequence>

<xs:element name="UserCSGInformation" type="tUserCSGInformation" minOccurs="0"/>

<xs:element name="Extension" type="tPSLocationInformation-Extension2" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tPSLocationInformation-Extension2">

<xs:sequence>

<xs:element name="VisitedPLMNID" type="tVisitedPLMNID" minOccurs="0"/>

<xs:element name="LocalTimeZone" type="tLocalTimeZone" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:element name="RATtype" type="tRATtype" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tLocalTimeZone">

<xs:sequence>

<xs:element name="TimeZone" type="tTimeZone"/>

<xs:element name="DaylightSavingTime" type="tDaylightSavingTime"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tUserCSGInformation">

<xs:sequence>

<xs:element name="CSGId" type="tCSGId" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Extension" type="tUserCSGInformation-Extension" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tUserCSGInformation-Extension">

<xs:sequence>

<xs:element name="AccessMode" type="tAccessMode" minOccurs="0"/>

<xs:element name="CMI" type="tBool" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tTWANLocationInformation">

<xs:sequence>

<xs:element name="TWAN-SSID" type="tTWAN-SSID" minOccurs="0"/>

<xs:element name="TWAN-BSSID" type="tTWAN-BSSID" minOccurs="0"/>

<xs:element name="TWAN-PLMNID" type="tVisitedPLMNID" minOccurs="0"/>

<xs:element name="CivicAddress" type="tCivicAddress" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="TWANOperatorName" type="tTWANOperatorName" minOccurs="0"/>

<xs:element name="LocalTimeZone" type="tLocalTimeZone" minOccurs="0"/>

<xs:element name="LogicalAccessID" type="tLogicalAccessID" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSePoTriExtension">

<xs:sequence>

<xs:element name="RegistrationType" type="tRegistrationType" minOccurs="0" maxOccurs="2"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tPublicIdentityExtension2">

<xs:sequence>

<xs:element name="WildcardedIMPU" type="xs:anyURI" minOccurs="0"/>

<xs:element name="Extension" type="tPublicIdentityExtension3" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tPublicIdentityExtension3">

<xs:sequence>

<xs:element name="ExtendedMSISDN" type="tExtendedMSISDN" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Extension" type="tPublicIdentityExtension4" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tPublicIdentityExtension4">

<xs:sequence>

<xs:element name="EnhancedIMSPublicIdentifiers" type="tEnhancedIMSPublicIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tExtendedMSISDN">

<xs:sequence>

<xs:element name="MSISDN" type="tMSISDN" />

<xs:element name="MSISDNType" type="tMSISDNType" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="tPublicIdentityExtension">

<xs:sequence>

<xs:element name="IdentityType" type="tIdentityType" minOccurs="0"/>

<xs:element name="WildcardedPSI" type="xs:anyURI" minOccurs="0"/>

<xs:element name="Extension" type="tPublicIdentityExtension2" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSh-Data-Extension">

<xs:sequence>

<xs:element name="RegisteredIdentities" type="tPublicIdentity" minOccurs="0"/>

<xs:element name="ImplicitIdentities" type="tPublicIdentity" minOccurs="0"/>

<xs:element name="AllIdentities" type="tPublicIdentity" minOccurs="0"/>

<xs:element name="AliasIdentities" type="tPublicIdentity" minOccurs="0"/>

<xs:element name="Extension" type="tSh-Data-Extension2" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSh-Data-Extension2">

<xs:sequence>

<xs:element name="DeletedIdentities" type="tPublicIdentity" minOccurs="0"/>

<xs:element name="Extension" type="tSh-Data-Extension3" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSh-Data-Extension3">

<xs:sequence>

<xs:element name="TADSinformation" type="tTADSinformation" minOccurs="0"/>

<xs:element name="Extension" type="tSh-Data-Extension4" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tTADSinformation">

<xs:sequence>

<xs:element name="IMSVoiceOverPSSessionSupport" type="tIMSVoiceOverPSSessionSupport"/>

<xs:element name="RATtype" type="tRATtype" minOccurs="0"/>

<xs:element name="Extension" type="tTADSinformationExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tTADSinformationExtension">

<xs:sequence>

<xs:element name="LastUEActivityTime" type="tDateTime" minOccurs="0"/>

<xs:element name="Extension" type=" tTADSinformationExtension2" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tTADSinformationExtension2">

<xs:sequence>

<xs:element name="AccessType" type="tAccessType" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSh-Data-Extension4">

<xs:sequence>

<xs:element name="EPSUserState" type="tPSUserState" minOccurs="0"/>

<xs:element name="EPSLocationInformation" type="tEPSLocationInformation" minOccurs="0"/>

<xs:element name="Extension" type="tSh-Data-Extension5" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSh-Data-Extension5">

<xs:sequence>

<xs:element name="IMSI" type="tIMSI" minOccurs="0"/>

<xs:element name="TWANLocationInformation" type="tTWANLocationInformation" minOccurs="0"/>

<xs:element name="IMSPrivateUserIdentity" type="tIMSPrivateUserIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Extension" type="tSh-Data-Extension6" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSh-Data-Extension6">

<xs:sequence>

<xs:element name="IMEISV" type="tString" minOccurs="0"/>

<xs:element name="Extension" type="tSh-Data-Extension7" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSh-Data-Extension7">

<xs:sequence>

<xs:element name="Sh-5GSLocationInformation" type="tSh-5GSLocationInformation" minOccurs="0"/>

<xs:element name="Sh-5GSUserState" type=" tPSUserState" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSh-Data">

<xs:sequence>

<xs:element name="PublicIdentifiers" type="tPublicIdentity" minOccurs="0"/>

<xs:element name="RepositoryData" type="tTransparentData" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Sh-IMS-Data" type="tShIMSData" minOccurs="0"/>

<xs:element name="CSLocationInformation" type="tCSLocationInformation" minOccurs="0"/>

<xs:element name="PSLocationInformation" type="tPSLocationInformation" minOccurs="0"/>

<xs:element name="CSUserState" type="tCSUserState" minOccurs="0"/>

<xs:element name="PSUserState" type="tPSUserState" minOccurs="0"/>

<xs:element name="Extension" type="tSh-Data-Extension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tUEReachabilityForIP">

<xs:sequence>

<xs:element name="UEIPReachabilityMME" type="tUEIPReachabilityMME" minOccurs="0"/>

<xs:element name="Extension" type="tUEReachabilityForIP-Extension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tUEReachabilityForIP-Extension">

<xs:sequence>

<xs:element name="UEIPReachabilitySGSN" type="tUEIPReachabilitySGSN" minOccurs="0"/>

<xs:element name="Extension" type="tUEReachabilityForIP-Extension2" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tUEReachabilityForIP-Extension2">

<xs:sequence>

<xs:element name="UEIPReachabilityAMF3GPP" type="tUEIPReachabilityAMF3GPP" minOccurs="0"/>

<xs:element name="UEIPReachabilityAMFnon3GPP" type="tUEIPReachabilityAMFnon3GPP" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tTransparentData">

<xs:sequence>

<xs:element name="ServiceIndication" type="tString"/>

<xs:element name="SequenceNumber" type="tSequenceNumber"/>

<xs:element name="ServiceData" type="tServiceData" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tServiceData">

<xs:sequence>

<xs:any processContents="lax"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tShIMSData">

<xs:sequence>

<xs:element name="SCSCFName" type="tSIP_URL" minOccurs="0"/>

<xs:element name="IFCs" type="tIFCs" minOccurs="0"/>

<xs:element name="IMSUserState" type="tIMSUserState" minOccurs="0"/>

<xs:element name="ChargingInformation" type="tChargingInformation" minOccurs="0"/>

<xs:element name="Extension" type="tShIMSDataExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tIFCs">

<xs:sequence>

<xs:element name="InitialFilterCriteria" type="tInitialFilterCriteria" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tCSLocationInformation">

<xs:sequence>

<xs:element name="LocationNumber" type="tLocationNumber" minOccurs="0"/>

<xs:choice>

<xs:element name="CellGlobalId" type="tCellGlobalId" minOccurs="0"/>

<xs:element name="ServiceAreaId" type="tServiceAreaId" minOccurs="0"/>

<xs:element name="LocationAreaId" type="tLocationAreaId" minOccurs="0"/>

</xs:choice>

<xs:element name="GeographicalInformation" type="tGeographicalInformation" minOccurs="0"/>

<xs:element name="GeodeticInformation" type="tGeodeticInformation" minOccurs="0"/>

<xs:element name="VLRNumber" type="tISDNAddress" minOccurs="0"/>

<xs:element name="MSCNumber" type="tISDNAddress" minOccurs="0"/>

<xs:element name="CurrentLocationRetrieved" type="tBool" minOccurs="0"/>

<xs:element name="AgeOfLocationInformation" type="tAgeOfLocationInformation" minOccurs="0"/>

<xs:element name="Extension" type="tCSLocationInformation-Extension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tCSLocationInformation-Extension">

<xs:sequence>

<xs:element name="UserCSGInformation" type="tUserCSGInformation" minOccurs="0"/>

<xs:element name="Extension" type="tCSLocationInformationExtension2" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tCSLocationInformationExtension2">

<xs:sequence>

<xs:element name="E-UTRANCellGlobalId" type="tE-UTRANCellGlobalId" minOccurs="0"/>

<xs:element name="TrackingAreaId" type="tTrackingAreaId" minOccurs="0"/>

<xs:element name="Extension" type="tCSLocationInformationExtension3" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tCSLocationInformationExtension3">

<xs:sequence>

<xs:element name="LocalTimeZone" type="tLocalTimeZone" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tPSLocationInformation">

<xs:sequence>

<xs:choice>

<xs:element name="CellGlobalId" type="tCellGlobalId" minOccurs="0"/>

<xs:element name="ServiceAreaId" type="tServiceAreaId" minOccurs="0"/>

<xs:element name="LocationAreaId" type="tLocationAreaId" minOccurs="0"/>

</xs:choice>

<xs:element name="RoutingAreaId" type="tRoutingAreaId" minOccurs="0"/>

<xs:element name="GeographicalInformation" type="tGeographicalInformation" minOccurs="0"/>

<xs:element name="GeodeticInformation" type="tGeodeticInformation" minOccurs="0"/>

<xs:element name="SGSNNumber" type="tISDNAddress" minOccurs="0"/>

<xs:element name="CurrentLocationRetrieved" type="tBool" minOccurs="0"/>

<xs:element name="AgeOfLocationInformation" type="tAgeOfLocationInformation" minOccurs="0"/>

<xs:element name="Extension" type="tPSLocationInformation-Extension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tEPSLocationInformation">

<xs:sequence>

<xs:element name="E-UTRANCellGlobalId" type="tE-UTRANCellGlobalId" minOccurs="0"/>

<xs:element name="TrackingAreaId" type="tTrackingAreaId" minOccurs="0"/>

<xs:element name="GeographicalInformation" type="tGeographicalInformation" minOccurs="0"/>

<xs:element name="GeodeticInformation" type="tGeodeticInformation" minOccurs="0"/>

<xs:element name="MMEName" type="tString" minOccurs="0"/>

<xs:element name="CurrentLocationRetrieved" type="tBool" minOccurs="0"/>

<xs:element name="AgeOfLocationInformation" type="tAgeOfLocationInformation" minOccurs="0"/>

<xs:element name="UserCSGInformation" type="tUserCSGInformation" minOccurs="0"/>

<xs:element name="Extension" type="tEPSLocationInformation-Extension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tEPSLocationInformation-Extension">

<xs:sequence>

<xs:element name="VisitedPLMNID" type="tVisitedPLMNID" minOccurs="0"/>

<xs:element name="LocalTimeZone" type="tLocalTimeZone" minOccurs="0"/>

<xs:element name="Extension" type="tEPSLocationInformation-Extension2" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tEPSLocationInformation-Extension2">

<xs:sequence>

<xs:element name="RATtype" type="tRATtype" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSh-5GSLocationInformation">

<xs:sequence>

<xs:element name="NRCellGlobalId" type="tNRCellGlobalId" minOccurs="0"/>

<xs:element name="E-UTRANCellGlobalId" type="tE-UTRANCellGlobalId" minOccurs="0"/>

<xs:element name="TrackingAreaId" type="tTrackingAreaId" minOccurs="0"/>

<xs:element name="GeographicalInformation" type="tGeographicalInformation" minOccurs="0"/>

<xs:element name="AMFAddress" type="tString" minOccurs="0"/>

<xs:element name="CurrentLocationRetrieved" type="tBool" minOccurs="0"/>

<xs:element name="AgeOfLocationInformation" type="tAgeOfLocationInformation" minOccurs="0"/>

<xs:element name="VisitedPLMNID" type="tVisitedPLMNID" minOccurs="0"/>

<xs:element name="LocalTimeZone" type="tLocalTimeZone" minOccurs="0"/>

<xs:element name="RATtype" type="tRATtype" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tISDNAddress">

<xs:sequence>

<xs:element name="Address" type="tAddressString"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tPublicIdentity">

<xs:sequence>

<xs:element name="IMSPublicIdentity" type="tIMSPublicIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="MSISDN" type="tMSISDN" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Extension" type="tPublicIdentityExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tEnhancedIMSPublicIdentity">

<xs:sequence>

<xs:element name="IMSPublicIdentity" type="tIMSPublicIdentity" />

<xs:element name="IdentityType" type="tIdentityType" />

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tInitialFilterCriteria">

<xs:sequence>

<xs:element name="Priority" type="tPriority"/>

<xs:element name="TriggerPoint" type="tTrigger" minOccurs="0"/>

<xs:element name="ApplicationServer" type="tApplicationServer"/>

<xs:element name="ProfilePartIndicator" type="tProfilePartIndicator" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tTrigger">

<xs:sequence>

<xs:element name="ConditionTypeCNF" type="tBool"/>

<xs:element name="SPT" type="tSePoTri" maxOccurs="unbounded"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSePoTri">

<xs:sequence>

<xs:element name="ConditionNegated" type="tBool" minOccurs="0"/>

<xs:element name="Group" type="tGroupID" maxOccurs="unbounded"/>

<xs:choice>

<xs:element name="RequestURI" type="tString"/>

<xs:element name="Method" type="tString"/>

<xs:element name="SIPHeader" type="tHeader"/>

<xs:element name="SessionCase" type="tDirectionOfRequest"/>

<xs:element name="SessionDescription" type="tSessionDescription"/>

</xs:choice>

<xs:element name="Extension" type="tSePoTriExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSessionDescription">

<xs:sequence>

<xs:element name="Line" type="tString"/>

<xs:element name="Content" type="tString" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tHeader">

<xs:sequence>

<xs:element name="Header" type="tString"/>

<xs:element name="Content" type="tString" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tApplicationServer">

<xs:sequence>

<xs:element name="ServerName" type="tSIP_URL"/>

<xs:element name="DefaultHandling" type="tDefaultHandling" minOccurs="0"/>

<xs:element name="ServiceInfo" type="tServiceInfo" minOccurs="0"/>

<xs:element name="Extension" type="tApplicationServerExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tChargingInformation">

<xs:sequence>

<xs:element name="PrimaryEventChargingFunctionName" type="tDiameterURI" minOccurs="0"/>

<xs:element name="SecondaryEventChargingFunctionName" type="tDiameterURI" minOccurs="0"/>

<xs:element name="PrimaryChargingCollectionFunctionName" type="tDiameterURI" minOccurs="0"/>

<xs:element name="SecondaryChargingCollectionFunctionName" type="tDiameterURI" minOccurs="0"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tDSAI">

<xs:sequence>

<xs:element name="DSAI-Tag" type="tDSAI-Tag"/>

<xs:element name="DSAI-Value" type="tDSAI-Value"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSMSRegistrationInfo">

<xs:sequence>

<xs:element name="IP-SM-GW-Number" type="tIP-SM-GW-Number"/>

<xs:element name="Extension" type="tSMSRegistrationInfoExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSMSRegistrationInfoExtension">

<xs:sequence>

<xs:element name="SCAddress" type="tSCAddress" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tApplicationServerExtension">

<xs:sequence>

<xs:element name="IncludeRegisterRequest" type="tIncludeRegisterRequest" minOccurs="0" maxOccurs="1"/>

<xs:element name="IncludeRegisterResponse" type="tIncludeRegisterResponse" minOccurs="0" maxOccurs="1"/>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tIncludeRegisterRequest">

<xs:sequence>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tIncludeRegisterResponse">

<xs:sequence>

<xs:element name="Extension" type="tExtension" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:element name="Sh-Data" type="tSh-Data"/>

</xs:schema>

* * * End of Changes * * * *