3GPP TSG-CT WG Meeting #86	CP-193134

Itges, SPAIN; 09th – 10th December 2019 	Revision of C4-195496, C4-195184

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.509 | CR | 0069 | rev | 2 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authentication Indication from W-AGF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Orange |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 7.2.1.3 FN-RG Registration via W-5GAN of TS 23.316, The W-AGF performs initial registration on behalf of the FN-RG to the 5GC, shall include the Authenticated Indication to the AMF, and indicates the FN-RG has been authenticated by the access network. AMF sends Nausf_UEAuthentication_Authenticate to the AUSF, the indication that the W-5GAN has authenticated the FN-RG is also included. AUSF sends the Nudm_UEAuthentication_Get Request to the UDM, the indication that the W-5GAN has authenticated the FN-RG is also included. UDM sends a Nudm_UEAuthentication_Get Response to the AUSF, it contains SUPI corresponding to the SUCI, and the indication that authentication is not required for the FN-RG. AUSF sends a Nausf_UEAuthentication_Authenticate Response to the AMF, the indication that authentication is not required for the FN-RG (S3-193684 in SA3#96-Adhoc, agreed in draft CR), and no Kseaf is included. Authentication between FN-RG and W-AGF is out of scope of 3GPP, shall be covered by BBF. AUSF and UDM shall not perform authentication. It is assumed that there is a trust relationship between the wireline operator that manages the W-5GAN and the PLMN operator managing the 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Service operation description on authentication for FN-RG; - Define resource rg-authenticates and the related HTTP method; - Data type definition and OpenAPI updates for the FN-RG authentication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 5.2.2.2.1, 5.2.2.2.x(new), 6.1.3.1, 6.1.3.y(new), 6.1.6.1, 6.1.6.2.x(new), 6.1.6.2.y(new), 6.1.6.3.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features to the OpenAPI file for Nausf_UEAuthentication API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Add the abbreviation of RG - Remove SUPI as UE ID of FN-RG in 5.2.2.2.x Authentication for FN-RG, and remove the attribute supiOrSuci and add suci in data type RgAuthenticationInfo. - Change P column from M to O, and revise the cadinality column and description accordingly in Table 6.1.3.y.3.1-3 . - Define simple data type Suci. - Revise the open api accordingly. Rev2: Baseline of subclause 6.1.6.3.2 is incorrect. |  |  |  |  |  |  |  |  |

* * * * Begin of Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[7]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[8]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[9]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

Editor's Note: This reference may be removed and references to it updated when the IETF publishes the corresponding update version.

[10]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[11]	IETF RFC 7807: "Problem Details for HTTP APIs".

[12]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[15]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[16]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[17]	Internet draft draft-ietf-emu-rfc5448bis: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[18]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[19]	IETF RFC 4648: "The Base16, Base32 and Base64 Data Encodings".

[20]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[21]	3GPP TR 21.900: "Technical Specification Group working methods".

[x]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS); Release 16".

* * * * Next Change * * * *

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AMF	Access and Mobility Management Function

API	Application Programming Interface

AUSF	Authentication Server Function

FN-RG	Fixed Network RG

MAC	Message Authentication Code

NF	Network Function

RG	Residential Gateway

SEAF	SEcurity Anchor Function

SoR	Steering of Roaming

URI	Uniform Resource Identifier

UPU	UE Parameters Update

W-AGF	Wireline Access Gateway Function

* * * * Next Change * * * *

5.2.2.2.1	General

The service operation "Authenticate" permits the requester NF to initiate the Authentication of the UE by providing the following information to the AUSF:

-	UE id (e.g. SUPI)

-	Serving Network Name

The AUSF retrieves the UE's subscribed authentication method from the UDM and depending on the information provided by the UDM, the AUSF enters in one of the following procedures:

-	5G-AKA

-	EAP-based authentication'

For those two different procedures a new resource is generated by the AUSF. The content of the resource will depend on the procedure and will be returned to the AMF.

This service operation "Authenticate" also permits the requester NF to initiate the Authentication of the FN-RG registration via W-AGF by providing the following information to the AUSF:

-	UE id (e.g. SUCI)

-	Indication that the W-AGF has authenticated the FN-RG

The AUSF retrieves the UE's SUPI, indication that authentication is not required for the FN-RG from the UDM, and AUSF shall not perform the authentication.

* * * * Next of Change * * * *

5.2.2.2.x	Authentication for FN-RG

In this procedure, the NF Service Consumer (AMF) requests the authentication of the FN-RG registration via W-AGF by providing the the SUCI of the FN-RG and the authenticated indication.

Figure 5.2.2.2.x-1: Authentication for FN-RG

1.	The NF Service Consumer (AMF) shall send a POST request to the AUSF. The payload of the body shall contain at least the UE Id and the authenticated indication.

2a.	On success, "201 Created" shall be returned. The payload body shall contain the representation of the resource created and the "Location" header shall contain the URI of the created resource (e.g. .../v1/rg-authentications/{authCtxId}).

2b.	On failure, one of the HTTP status code listed in table 6.1.7.3-1 shall be returned with the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1.

* * * * Next of Change * * * *

6.1.3.1	Overview

The structure of the Resource URIs of the "Authenticate" service is shown in Figure 6.1.3.1-1

Figure 6.1.3.1-1: Resource URI structure of the AUSF API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ue-authentications (Collection) | {apiRoot}/nausf-auth/v1/ue-authentications | POST | Initiate the authentication process by providing inputs related to the UE |
| 5g-aka-confirmation (Document) | {apiRoot}/nausf-auth/v1/ue-authentications/{authCtxId}/5g-aka-confirmation | PUT | Put the UE response from the 5G-AKA process. |
| eap-session (Document) | {apiRoot}/nausf-auth/v1/ue-authentications/{authCtxId}/eap-session | POST | Post the EAP response from the UE. See NOTE. |
| rg-authentications (Collection) | {apiRoot}/nausf-auth/v1/rg-authentications | POST | Initiate the authentication process by providing inputs related to the FN-RG |
| NOTE: This POST is used to provide EAP response to the AUSF in a sub-resource (Document) generated by the first POST operation. As this operation is not idempotent (it triggers subsequent EAP operations), a PUT was not adequate. |  |  |  |

* * * * Next of Change * * * *

6.1.3.y	Resource: List of rg-authentications

6.1.3.y.1	Description

This resource represents a collection of the rg-authentication resources generated by the AUSF.

6.1.3.y.2	Resource Definition

Resource URI: {apiRoot}/nausf-auth/v1/rg-authentications

This resource shall support the resource URI variables defined in table 6.1.3.y.2-1.

Table 6.1.3.y.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |

6.1.3.y.3	Resource Standard Methods

6.1.3.y.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.y.3.1-1.

Table 6.1.3.y.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.y.3.1-2 and the response data structures and response codes specified in table 6.1.3.y.3.1-3.

Table 6.1.3.y.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RgAuthenticationInfo | M | 1 | Contains the UE id (i.e. SUCI as specified in 3GPP TS 23.316 [x] or 3GPP TS 33.501 [8]) and the authenticated indication. |

Table 6.1.3.y.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RgAuthCtx | M | 1 | 201 Created | Upon success, the response body will contain the SUPI of the UE and the authentication indication. The HTTP response shall include a "Location" header that contains the resource URI of the created resource. |
| ProblemDetails | O | 0..1 | 400 Bad Request | This case represents the failure to start authentication service because of input parameter error. |
| ProblemDetails | O | 0..1 | 403 Forbidden | This case represents when the UE is not allowed to be authenticated. The "cause" attribute may be used to indicate one of the following application errors: - AUTHENTICATION_REJECTED - INVALID_SCHEME_OUTPUT |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]. |  |  |  |  |

* * * * Next of Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nausf service based interface protocol.

Table 6.1.6.1-1: Nausf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationInfo | 6.1.6.2.2 | Contains the UE id (i.e. SUCI or SUPI) and the Serving Network Name. |
| UEAuthenticationCtx | 6.1.6.2.3 | Contains the information related to the resource generated to handle the UE authentication. It contains at least the UE id, Serving Network, the Authentication Method and related EAP information or related 5G-AKA information. |
| 5gAuthData | 6.1.6.2.4 | Contains 5G authentication related information. |
| AV5gAka | 6.1.6.2.5 | Contains Authentication Vector for method 5G AKA. |
| ConfirmationData | 6.1.6.2.7 | Contains the "RES*" generated by the UE. |
| EapSession | 6.1.6.2.8 | Contains information related to the EAP session. |
| RgAuthenticationInfo | 6.1.6.2.x | Contains the UE id (i.e. SUCI) and the authenticated indication. |
| RgAuthCtx | 6.1.6.2.y | Contains the UE id (i.e. SUPI) and the authentication indication. |

Table 6.1.6.1-2 specifies data types re-used by the Nausf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nausf service based interface.

Table 6.1.6.1-2: Nausf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| LinksValueSchema | 3GPP TS 29.571 [10] | 3GPP Hypermedia link |
| ProblemDetails | 3GPP TS 29.571 [10] | Common Data Type used in response bodies |
| Supi | 3GPP TS 29.571 [10] |  |
| Uri | 3GPP TS 29.571 [10] |  |
| ResynchronizationInfo | 3GPP TS 29.503 [12] |  |
| SupiOrSuci | 3GPP TS 29.503 [12] |  |
| ServingNetworkName | 3GPP TS 29.503 [12] |  |
| Autn | 3GPP TS 29.503 [12] |  |
| Pei | 3GPP TS 29.571 [10] |  |
| TraceData | 3GPP TS 29.571 [10] |  |
| NfGroupId | 3GPP TS 29.571 [10] |  |

* * * * Next of Change * * * *

6.1.6.2.x	Type: RgAuthenticationInfo

Table 6.1.6.2.x-1: Definition of type RgAuthenticationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| suci | Suci | M | 1 | Contains the SUCI of the FN-RG. |
| authenticatedInd | boolean | M | 1 | This IE shall be set as follows: - true: authenticated by the W-AGF; - false (default): unauthenticated by the W-AGF. |

* * * * Next of Change * * * *

6.1.6.2.y	Type: RgAuthCtx

Table 6.1.6.2.y-1: Definition of type RgAuthCtx

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authResult | AuthResult | M | 1 | Indicates the result of the authentication |
| supi | Supi | C | 0..1 | If the authentication is successful and if the AMF had provided a SUCI, this IE shall contain the SUPI of the UE. |
| authInd | boolean | C | 0..1 | When present, this IE shall be set as follows: - true: authentication is not required; - false (default): authentication is required. |

* * * * Next of Change * * * *

6.1.6.3.2	Simple data types

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| EapPayload | string | The EAP packet is encoded using base64 (see IETF RFC 4648 [19]) and represented as a String. |
| ResStar | string | pattern: "[A-Fa-f0-9]{32}"; nullable |
| Kseaf | string | pattern: "[A-Fa-f0-9]{64}" |
| HxresStar | string | pattern: "[A-Fa-f0-9]{32}" |
| Suci | string | String containing a SUCI. Pattern: "^(suci-(0-[0-9]{3}-[0-9]{2,3}¦[1-7]-.+)-[0-9]{1,4}-(0-0-.+¦[a-fA-F1-9]-([1-9]¦[1-9][0-9]¦1[0-9]{2}¦2[0-4][0-9]¦25[0-5])-[a-fA-F0-9]+)¦.+)$" |

* * * * Next of Change * * * *

A.2	Nausf_UEAuthentication API

openapi: 3.0.0

info:

version: 1.1.0.alpha-2

title: AUSF API

description: |

AUSF UE Authentication Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

servers:

- url: '{apiRoot}/nausf-auth/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nausf-auth

paths:

/ue-authentications:

post:

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AuthenticationInfo'

required: true

responses:

'201':

description: UEAuthenticationCtx

content:

application/3gppHal+json:

schema:

$ref: '#/components/schemas/UEAuthenticationCtx'

headers:

Location:

description: 'Contains the URI of the newly created resource according to the structure: {apiRoot}/nausf-auth/v1/ue-authentications/{authCtxId}'

required: true

schema:

type: string

'400':

description: Bad Request from the AMF

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden due to serving network not authorized

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: User does not exist in the HPLMN

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'501':

description: Received protection scheme is not supported by HPLMN

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

/ue-authentications/{authCtxId}/5g-aka-confirmation:

put:

parameters:

- name: authCtxId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ConfirmationData'

responses:

'200':

description: Request processed (EAP success or Failure)

content:

application/json:

schema:

$ref: '#/components/schemas/ConfirmationDataResponse'

'400':

description: Bad Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

/ue-authentications/{authCtxId}/eap-session:

post:

operationId: EapAuthMethod

parameters:

- name: authCtxId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/EapSession'

responses:

'200':

description: Use to handle or close the EAP session

content:

application/json:

schema:

$ref: '#/components/schemas/EapSession'

application/3gppHal+json:

schema:

type: object

properties:

eapPayload:

$ref: '#/components/schemas/EapPayload'

_links:

type: object

description: 'URI : /{eapSessionUri}'

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

minProperties: 1

required:

- eapPayload

- _links

'400':

description: Bad Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

/rg-authentications:

post:

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/RgAuthenticationInfo'

required: true

responses:

'201':

description: RgAuthCtx

content:

application/json:

schema:

$ref: '#/components/schemas/RgAuthCtx'

headers:

Location:

description: 'Contains the URI of the newly created resource according to the structure: {apiRoot}/nausf-auth/v1/rg-authentications/{authCtxId}'

required: true

schema:

type: string

'403':

description: The UE is not allowed to be authenticated

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'400':

description: Bad Request from the AMF

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: User does not exist in the HPLMN

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nausf-auth: Access to Nausf_UEAuthentication API

schemas:

AuthenticationInfo:

type: object

properties:

supiOrSuci:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/SupiOrSuci'

servingNetworkName:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

resynchronizationInfo:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ResynchronizationInfo'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

pattern: '^[0-9]{1,4}$'

required:

- supiOrSuci

- servingNetworkName

UEAuthenticationCtx:

type: object

properties:

authType:

$ref: '#/components/schemas/AuthType'

5gAuthData:

oneOf:

- $ref: '#/components/schemas/Av5gAka'

- $ref: '#/components/schemas/EapPayload'

_links:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

servingNetworkName:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

required:

- authType

- 5gAuthData

- _links

Av5gAka:

type: object

required:

- rand

- hxresStar

- autn

properties:

rand:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

hxresStar:

$ref: '#/components/schemas/HxresStar'

autn:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Autn'

ConfirmationData:

type: object

required:

- resStar

properties:

resStar:

$ref: '#/components/schemas/ResStar'

ConfirmationDataResponse:

type: object

properties:

authResult:

$ref: '#/components/schemas/AuthResult'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

kseaf:

$ref: '#/components/schemas/Kseaf'

required:

- authResult

EapSession:

type: object

properties:

eapPayload:

$ref: '#/components/schemas/EapPayload'

kSeaf:

$ref: '#/components/schemas/Kseaf'

_links:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

authResult:

$ref: '#/components/schemas/AuthResult'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

required:

- eapPayload

RgAuthenticationInfo:

type: object

properties:

suci:

$ref: '#/components/schemas/Suci'

authenticatedInd:

type: boolean

default: false

required:

- suci

- authenticatedInd

RgAuthCtx:

type: object

properties:

authResult:

$ref: '#/components/schemas/AuthResult'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

authInd:

type: boolean

default: false

required:

- authResult

AuthResult:

type: string

enum:

- AUTHENTICATION_SUCCESS

- AUTHENTICATION_FAILURE

- AUTHENTICATION_ONGOING

EapPayload:

type: string

format: base64

description: contains an EAP packet

Kseaf:

type: string

pattern: '[A-Fa-f0-9]{64}'

ResStar:

type: string

pattern: '[A-Fa-f0-9]{32}'

nullable: true

HxresStar:

type: string

pattern: "[A-Fa-f0-9]{32}"

Suci:

type: string

pattern: '^(suci-(0-[0-9]{3}-[0-9]{2,3}|[1-7]-.+)-[0-9]{1,4}-(0-0-.+|[a-fA-F1-9]-([1-9]|[1-9][0-9]|1[0-9]{2}|2[0-4][0-9]|25[0-5])-[a-fA-F0-9]+)|.+)$'

AuthType:

anyOf:

- type: string

enum:

- 5G_AKA

- EAP_AKA_PRIME

- EAP_TLS

- type: string

externalDocs:

description: 3GPP TS 29.509 V16.1.0; 5G System; 3GPP TS Authentication Server services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.509

* * * * End of Change * * * *