3GPP TSG-CT Meeting #86	CP-193165

# Sitges, SPAIN; 09th – 10th December 2019											(was C4-195599)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0042 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of the LMF Broadcast Service Operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.273 clause 6.14.2 defines a procedure to support transfer of ciphering keys and associated ciphering key information from an LMF to a UE via a serving AMF to support broadcast of location assistance data in ciphered form. The procedure includes transferring the ciphering key information from the LMF to the AMF in a first step using an Nlmf_Broadcast_CipheringKeyData service operation. The Nlmf_Broadcast_CipheringKeyData service operation is defined in clause 8.3.3.2 of TS 23.273 as part of an Nlmf_Broadcast service and needs to be added in TS 29.572. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Nlmf_Broadcast service and the Nlmf_Broadcast_CipheringKeyData service operation are added including constituent IEs and YAML API definition. A few small errors for the Nlmf_Location service are also corrected. An additional Request Ciphering Key procedure is included in order to provide a valid context in OpenAPI version 3 for the Provide Ciphering Key Information procedure but it would not be used by an AMF or LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Broadcast of location assistance data in ciphered form for 5GS will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4, 5.1, 5.x (new), 6.1.6.1, 6.y (new), A.n (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backward compatible new feature in the OpenAPI specification file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes in revision 3 (CT#86): - OpenAPI version was changed from “1.0.0” to “1.0.0.alpha-1” in A.n as it is under development - The externalDocs section was moved to the top of the API in A.n - The externalDocs version was changed from “V16.0.0” to “V16.1.0” in A.n - Blank lines were added between sections in A.n for better readability - “binary” was changed to “Binary” in table 6.y.6.3.2-1 and in A.n - The “ldrReference” attribute was removed from the CipheringDataSet data type in A.n - The security field was removed in A.n |  |  |  |  |  |  |  |  |

>>>Start Changes<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[7]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[12]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	OpenAPI Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[15]	IETF RFC 7807: "Problem Details for HTTP APIs".

[16]	3GPP TR 21.900: "Technical Specification Group working methods".

[17]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[18]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[19]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[20]	3GPP TS 24.080: "Mobile radio interface layer 3 Supplementary services specification; Formats and coding".

[aa]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[bb]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

>>>Next Changes<<<

# 4	Overview

The Location Management Function (LMF) is the network entity in the 5G Core Network (5GC) supporting the following functionality:

-	Supports location determination for a UE.

-	Obtains downlink location measurements or a location estimate from the UE.

-	Obtains uplink location measurements from the NG RAN.

-	Obtains non-UE associated assistance data from the NG RAN.

-	Provides broadcast assistance data to UEs and forwards associated ciphering keys to an AMF.

Other functions of an LMF are listed in clause 4.3.8 of 3GPP TS 23.273 [19].

Figure 4-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the LMF:

Figure 4-1: Reference model – LMF

>>>Next Changes<<<

## 5.1	Introduction

The LMF offers to other NFs the following services:

-	Nlmf_Location

-	Nlmf_Broadcast

>>>Next Changes<<<

## 5.x	Nlmf_Broadcast Service

### 5.x.1	Service Description

The Nlmf_Broadcast service enables an NF to obtain ciphering keys and associated parameters applicable to location assistance data that is broadcast to subscribed UEs in ciphered form.

### 5.x.2	Service Operations

#### 5.x.2.1	Introduction

The service operations defined for the Nlmf_Broadcast service are as follows:

-	CipheringKeyData: It provides the ciphering key information to the consumer NF.

#### 5.x.2.2	CipheringKeyData

##### 5.x.2.2.1	General

The following procedures are defined, using the "CipheringKeyData" service operation:

-	Request Ciphering Key Information

-	Provide Ciphering Key Information

NOTE:	The Request Ciphering Key procedure is included in order to provide a valid context in OpenAPI version 3 for the Provide Ciphering Key Information procedure. The Request Ciphering Key procedure is not used for support of ciphering key transfer in 3GPP TS 23.273 [19] and hence need not be supported by an NF Service Consumer or by an LMF.

##### 5.x.2.2.2	Request Ciphering Key Information

This procedure allows a consumer NF to request ciphering key information.

Figure 5.x.2.2.2-1: CipheringKeyData Request

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the "cipher-key-data" custom operation. The request body shall include a notification callback URI.

2a.	On success, "200 OK" shall be returned. The response body shall indicate whether the LMF has ciphering key data. If the LMF has ciphering key data, the Provide Ciphering Key Information procedure is used to provide the ciphering key data to the NF Service Consumer.

##### 2b.	On failure, one of the HTTP status codes listed in Table 6.y.4.4.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.y.7.3-1.

##### 5.x.2.2.3	Provide Ciphering Key Information

This procedure notifies the NF Service Consumer (i.e. AMF) about updated ciphering key information applicable to broadcast of location assistance data in ciphered form to subscribed UEs. The notification is delivered to:

-	the callback URI of an AMF received during an earlier CipheringKeyData request service operation if still available; or

-	a callback URI registered in the NRF, if the AMF registered to the NRF with notification endpoints for ciphering key data notifications;

Otherwise (if not available),

-	an AMF callback URI locally provisioned in the LMF.

The procedure is invoked by issuing a POST request to the callback URI of the NF Service Consumer. See figure 5.x.2.2.3-1.

Figure 5.x.2.2.3-1: CipheringKeyData Notify

1.	The LMF shall send an HTTP POST request to the callback URI for the NF service consumer determined as described above. The request body shall include one or more ciphering keys and for each ciphering key may include a ciphering key value, ciphering key identifier, validity period and set of applicable types of broadcast assistance data.

2a.	On success or partial success, "200 OK" shall be returned. The response body shall indicate which ciphering key information was successfully stored by the NF service consumer.

2b.	On failure to store any ciphering key information, one of the HTTP status codes listed in table 6.y.5.1.3.1-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.y.5.1.3.1-2.

>>>Next Changes<<<

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nlmf_Location service based interface protocol.

Table 6.1.6.1-1: Nlmf_Location specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| InputData | 6.1.6.2.2 |  |
| LocationData | 6.1.6.2.3 |  |
| GeographicalCoordinates | 6.1.6.2.4 |  |
| GeographicArea | 6.1.6.2.5 |  |
| Point | 6.1.6.2.6 |  |
| PointUncertaintyCircle | 6.1.6.2.7 |  |
| PointUncertaintyEllipse | 6.1.6.2.8 |  |
| Polygon | 6.1.6.2.9 |  |
| PointAltitude | 6.1.6.2.10 |  |
| PointAltitudeUncertainty | 6.1.6.2.11 |  |
| EllipsoidArc | 6.1.6.2.12 |  |
| LocationQoS | 6.1.6.2.13 |  |
| CivicAddress | 6.1.6.2.14 |  |
| PositioningMethodAndUsage | 6.1.6.2.15 |  |
| GnssPositioningMethodAndUsage | 6.1.6.2.16 |  |
| VelocityEstimate | 6.1.6.2.17 |  |
| HorizontalVelocity | 6.1.6.2.18 |  |
| HorizontalWithVerticalVelocity | 6.1.6.2.19 |  |
| HorizontalVelocityWithUncertainty | 6.1.6.2.20 |  |
| HorizontalWithVerticalVelocityAndUncertainty | 6.1.6.2.21 |  |
| UncertaintyEllipse | 6.1.6.2.22 |  |
| UeLcsCapability | 6.1.6.2.23 | Indicates the LCS capability supported by the UE. |
| PeriodicEventInfo | 6.1.6.2.24 |  |
| AreaEventInfo | 6.1.6.2.25 |  |
| ReportingArea | 6.1.6.2.26 |  |
| MotionEventInfo | 6.1.6.2.27 |  |
| ReportingAccessTypes | 6.1.6.2.28 |  |
| CancelLocData | 6.1.6.2.29 |  |
| LocContextData | 6.1.6.2.30 |  |
| EventReportMessage | 6.1.6.2.31 |  |
| EventReportingStatus | 6.1.6.2.32 |  |
| UELocationInfo | 6.1.6.2.33 |  |
| EventNotifyData | 6.1.6.2.34 |  |
| Altitude | 6.1.6.3.2 |  |
| Angle | 6.1.6.3.2 |  |
| Uncertainty | 6.1.6.3.2 |  |
| Orientation | 6.1.6.3.2 |  |
| Confidence | 6.1.6.3.2 |  |
| Accuracy | 6.1.6.3.2 |  |
| InnerRadius | 6.1.6.3.2 |  |
| CorrelationID | 6.1.6.3.2 |  |
| AgeOfLocationEstimate | 6.1.6.3.2 |  |
| HorizontalSpeed | 6.1.6.3.2 |  |
| VerticalSpeed | 6.1.6.3.2 |  |
| SpeedUncertainty | 6.1.6.3.2 |  |
| BarometricPressure | 6.1.6.3.2 |  |
| LcsServiceType | 6.1.6.3.2 |  |
| LdrReference | 6.1.6.3.2 |  |
| ReportingAmount | 6.1.6.3.2 |  |
| ReportingInterval | 6.1.6.3.2 |  |
| MinimumInterval | 6.1.6.3.2 |  |
| MaximumInterval | 6.1.6.3.2 |  |
| SamplingInterval | 6.1.6.3.2 |  |
| ReportingDuration | 6.1.6.3.2 |  |
| LinearDistance | 6.1.6.3.2 |  |
| LMFIdentification | 6.1.6.3.2 |  |
| EventReportCounter | 6.1.6.3.2 |  |
| EventReportDuration | 6.1.6.3.2 |  |
| ExternalClientType | 6.1.6.3.3 |  |
| SupportedGADShapes | 6.1.6.3.4 |  |
| ResponseTime | 6.1.6.3.5 |  |
| PositioningMethod | 6.1.6.3.6 |  |
| GnssMethod | 6.1.6.3.7 |  |
| GnssId | 6.1.6.3.8 |  |
| Usage | 6.1.6.3.9 |  |
| LcsPriority | 6.1.6.3.10 |  |
| VelocityRequested | 6.1.6.3.11 |  |
| AccuracyFulfilmentIndicator | 6.1.6.3.12 |  |
| VerticalDirection | 6.1.6.3.13 |  |
| LdrType | 6.1.6.3.14 |  |
| ReportingAreaType | 6.1.6.3.15 |  |
| OccurrenceInfo | 6.1.6.3.16 |  |
| ReportingAccessType | 6.1.6.3.17 |  |
| EventClass | 6.1.6.3.18 |  |
| ReportedEventType | 6.1.6.3.19 |  |
| TerminationCause | 6.1.6.3.20 |  |

Table 6.1.6.1-2 specifies data types re-used by the Nlmf_Location service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf service based interface.

Table 6.1.6.1-2: Nlmf_Location re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Supi | 3GPP TS 29.571 [8] |  |
| Pei | 3GPP TS 29.571 [8] |  |
| Gpsi | 3GPP TS 29.571 [8] |  |
| Ecgi | 3GPP TS 29.571 [8] |  |
| Ncgi | 3GPP TS 29.571 [8] |  |
| NfInstanceId | 3GPP TS 29.571 [8] |  |
| Uri | 3GPP TS 29.571 [8] |  |
| RefToBinaryData | 3GPP TS 29.571 [8] |  |

>>>Next Changes<<<

## 6.y	Nlmf_Broadcast Service API

### 6.y.1	API URI

The Nlmf_Broadcast service shall use the Nlmf_Broadcast API.

The request URI used in HTTP request from the NF service consumer towards the NF service producer shall have the structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nlmf-broadcast".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.y.3.

### 6.y.2	Usage of HTTP

#### 6.y.2.1	General

HTTP/2, as defined in IETF RFC 7540 [12], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nlmf_Location service shall comply with the OpenAPI [14] specification contained in Annex A.

#### 6.y.2.2	HTTP Standard Headers

##### 6.y.2.2.1	General

##### 6.y.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [13], shall be used as content type of the HTTP bodies specified in the present specification as indicated in clause 5.4 of 3GPP TS 29.500 [4].

-	The Problem Details JSON Object (IETF RFC 7807 [15]). The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json".

#### 6.y.2.3	HTTP custom headers

##### 6.y.2.3.1	General

The following HTTP custom headers shall be supported:

-	3gpp-Sbi-Message-Priority: See 3GPP TS 29.500 [4], clause 5.2.3.2.2.

This API does not define any new HTTP custom headers.

### 6.y.3	Resources

#### 6.y.3.1	Overview

The structure of the Resource URIs of the Nlmf_Broadcast service is shown in figure 6.y.3.1-1.

### Figure 6.y.3.1-1: Resource URI structure of the Nlmf_Broadcast API

Table 6.y.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.y.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| CipheringKeyData (Custom operation) | {apiRoot}/nlmf-broadcast/<apiVersion>/cipher-key-data | cipher-key-data (POST) |  |

### 6.y.4	Custom Operations without associated resources

#### 6.y.4.1	Overview

Table 6.y.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/nlmf-broadcast/<apiVersion>/cipher-key-data | POST |  |

#### 6.y.4.4	Operation: cipher-key-data

##### 6.y.4.4.1	Description

This clause describes the custom operation and what it is used for.

##### 6.y.4.4.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 6.y.4.4.2-1 and table 6.y.4.4.2-2.

Table 6.y.4.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CipherRequestData | M | 1 | Input parameters to the "Ciphering Key Data" operation |

Table 6.y.4.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CipherResponseData | M | 1 | 200 OK | This case represents a successful request for ciphering key data. Upon success, a response body is returned indicating whether the LMF has ciphering key data. The ciphering key data is returned separately in a CipheringKeyData notification. |
| ProblemDetails | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application errors: - UNSPECIFIED - BROADCAST_CIPHERING_KEYS_NOT_SUPPORTED See table 6.y.7.3-1 for the description of this error. |

### 6.y.5	Notifications

#### 6.y.5.1	CipheringKeyData

##### 6.y.5.1.1	Description

The CipheringKeyData operation is used to notify the occurrence of new ciphering key information to a consumer NF (e.g. AMF).

##### 6.y.5.1.2	Notification Definition

Callback URI: {amfCallBackURI}

See clause 5.x.2.2.2 for the description of how the LMF obtains the Callback URI of the NF Service Consumer (i.e. AMF).

##### 6.y.5.1.3	Notification Standard Methods

###### 6.y.5.1.3.1	POST

This method sends a ciphering key data notify to the NF Service Consumer.

This method shall support the request and response data structures and response codes specified in table 6.y.5.1.3.1-1 and table 6.y.5.1.3.1-2.

Table 6.y.5.1.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CipheringKeyInfo | M | 1 | Input parameters to the "Ciphering Key Data" operation |

Table 6.y.5.1.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CipheringKeyResponse | M |  | 1 |  | 200 OK |  | This case represents successful or partially successful storage of ciphering key information by the service consumer NF. A response body is returned containing the following parameters: - List of Ciphering Set IDs successfully stored - List of Ciphering Set IDs not successfully stored |  |  |
| ProblemDetails | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application errors: - UNSPECIFIED - UNABLE_TO_STORE_CIPHERING_KEY_DATA See table 6.y.7.3-1 for the description of this error. |

### 6.y.6	Data Model

#### 6.y.6.1	General

This clause specifies the application data model supported by the API.

Table 6.y.6.1-1 specifies the data types defined for the Nlmf_Broadcast service based interface protocol.

Table 6.y.6.1-1: Nlmf_Broadcast specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| CipheringKeyInfo | 6.y.6.2.2 |  |
| CipheringKeyResponse | 6.y.6.2.3 |  |
| CipheringDataSet | 6.y.6.2.4 |  |
| CipheringSetReport | 6.y.6.2.5 |  |
| CipherRequestData | 6.y.6.2.6 |  |
| CipherResponseData | 6.y.6.2.7 |  |
| CipheringSetID | 6.y.6.3.2 |  |
| CipheringKey | 6.y.6.3.2 |  |
| C0 | 6.y.6.3.2 |  |
| ValidityDuration | 6.y.6.3.2 |  |
| StorageOutcome | 6.y.6.3.3 |  |
| DataAvailability | 6.y.6.3.4 |  |

Table 6.y.6.1-2 specifies data types re-used by the Nlmf_Broadcast service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf service based interface.

Table 6.y.6.1-2: Nlmf_Broadcast re-used Data Types

| Data type | Reference | Comments |  |
| --- | --- | --- | --- |
| Binary | 3GPP TS 29.571 [8] |  |  |
| DateTime | 3GPP TS 29.571 [8] |  |  |
| Uri | 3GPP TS 29.571 [8] |  |

#### 6.y.6.2	Structured data types

##### 6.y.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.y.6.2.2	Type: CipheringKeyInfo

Table 6.y.6.2.2-1: Definition of type CipheringKeyInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cipheringData | array(CipheringDataSet) | M | 1..N | This IE contains one or more ciphering data sets, where each ciphering data set contains information for one ciphering key. |

##### 6.y.6.2.3	Type: CipheringKeyResponse

Table 6.y.6.2.3-1: Definition of type CipheringKeyResponse

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cipheringDataReport | Array(CipheringSetReport) | O | 1..N | This IE indicates the ciphering data sets which were successfully stored or not stored. The absence of this IE indicates that all ciphering data sets were successfully stored. |

##### 6.y.6.2.4	Type: CipheringDataSet

Table 6.y.6.2.4-1: Definition of type CipheringDataSet

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cipheringSetID | CipheringSetID | M | 1 | Identification of a ciphering data set |
| cipheringKey | CipheringKey | M | 1 | A ciphering key value |
| c0 | C0 | M | 1 | First component of the initial ciphering counter as defined in clause 7.4.2 of 3GPP TS 36.355 [aa] |
| ltePosSibTypes | Binary | O | 0..1 | This IE contains a bitmap indicating the LTE positioning SIB types for which the ciphering data set is applicable: - a bit set to 0 indicates that the ciphering data set is not applicable to the corresponding LTE positioning SIB type - a bit set to 1 indicates that the ciphering data set is applicable to the corresponding LTE positioning SIB type The mapping of the bits to the LTE positioning SIB types is as follows: -- bit 8 in the first octet maps to positioning SIB Type 1-1 -- bit 7 in the first octet maps to positioning SIB Type 1-2 -- bit 6 in the first octet maps to positioning SIB Type 1-3 -- bit 5 in the first octet maps to positioning SIB Type 1-4 -- bit 4 in the first octet maps to positioning SIB Type 1-5 -- bit 3 in the first octet maps to positioning SIB Type 1-6 -- bit 2 in the first octet maps to positioning SIB Type 1-7 -- bit 1 in the first octet maps to positioning SIB Type 1-8 -- bit 8 in the second octet maps to positioning SIB Type 2-1 -- bit 7 in the second octet maps to positioning SIB Type 2-2 -- bit 6 in the second octet maps to positioning SIB Type 2-3 -- bit 5 in the second octet maps to positioning SIB Type 2-4 -- bit 4 in the second octet maps to positioning SIB Type 2-5 -- bit 3 in the second octet maps to positioning SIB Type 2-6 -- bit 2 in the second octet maps to positioning SIB Type 2-7 -- bit 1 in the second octet maps to positioning SIB Type 2-8 -- bit 8 in the third octet maps to positioning SIB Type 2-9 -- bit 7 in the third octet maps to positioning SIB Type 2-10 -- bit 6 in the third octet maps to positioning SIB Type 2-11 -- bit 5 in the third octet maps to positioning SIB Type 2-12 -- bit 4 in the third octet maps to positioning SIB Type 2-13 -- bit 3 in the third octet maps to positioning SIB Type 2-14 -- bit 2 in the third octet maps to positioning SIB Type 2-15 -- bit 1 in the third octet maps to positioning SIB Type 2-16 -- bit 8 in the fourth octet maps to positioning SIB Type 2-17 -- bit 7 in the fourth octet maps to positioning SIB Type 2-18 -- bit 6 in the fourth octet maps to positioning SIB Type 2-19 -- bit 5 in the fourth octet maps to positioning SIB Type 2-20 -- bit 4 in the fourth octet maps to positioning SIB Type 2-21 -- bit 3 in the fourth octet maps to positioning SIB Type 2-22 -- bit 2 in the fourth octet maps to positioning SIB Type 2-23 -- bit 1 in the fourth octet maps to positioning SIB Type 3-1 Any unassigned bits are spare and shall be coded as zero. Non-included bits shall be treated as being coded as zero. (NOTE 1) |
| nrPosSibTypes | Binary | O | 0..1 | This IE contains a bitmap indicating the NR positioning SIB types for which the ciphering data set is applicable: - a bit set to 0 indicates that the ciphering data set is not applicable to the corresponding NR positioning SIB type - a bit set to 1 indicates that the ciphering data set is applicable to the corresponding NR positioning SIB type The mapping of the bits to the NR positioning SIB types is as follows: -- bit 8 in the first octet maps to positioning SIB Type 1-1 -- bit 7 in the first octet maps to positioning SIB Type 1-2 -- bit 6 in the first octet maps to positioning SIB Type 1-3 -- bit 5 in the first octet maps to positioning SIB Type 1-4 -- bit 4 in the first octet maps to positioning SIB Type 1-5 -- bit 3 in the first octet maps to positioning SIB Type 1-6 -- bit 2 in the first octet maps to positioning SIB Type 1-7 -- bit 1 in the first octet maps to positioning SIB Type 1-8 -- bit 8 in the second octet maps to positioning SIB Type 2-1 -- bit 7 in the second octet maps to positioning SIB Type 2-2 -- bit 6 in the second octet maps to positioning SIB Type 2-3 -- bit 5 in the second octet maps to positioning SIB Type 2-4 -- bit 4 in the second octet maps to positioning SIB Type 2-5 -- bit 3 in the second octet maps to positioning SIB Type 2-6 -- bit 2 in the second octet maps to positioning SIB Type 2-7 -- bit 1 in the second octet maps to positioning SIB Type 2-8 -- bit 8 in the third octet maps to positioning SIB Type 2-9 -- bit 7 in the third octet maps to positioning SIB Type 2-10 -- bit 6 in the third octet maps to positioning SIB Type 2-11 -- bit 5 in the third octet maps to positioning SIB Type 2-12 -- bit 4 in the third octet maps to positioning SIB Type 2-13 -- bit 3 in the third octet maps to positioning SIB Type 2-14 -- bit 2 in the third octet maps to positioning SIB Type 2-15 -- bit 1 in the third octet maps to positioning SIB Type 2-16 -- bit 8 in the fourth octet maps to positioning SIB Type 2-17 -- bit 7 in the fourth octet maps to positioning SIB Type 2-18 -- bit 6 in the fourth octet maps to positioning SIB Type 2-19 -- bit 5 in the fourth octet maps to positioning SIB Type 2-20 -- bit 4 in the fourth octet maps to positioning SIB Type 2-21 -- bit 3 in the fourth octet maps to positioning SIB Type 2-22 -- bit 2 in the fourth octet maps to positioning SIB Type 2-23 -- bit 1 in the fourth octet maps to positioning SIB Type 3-2 -- bit 8 in the fifth octet maps to positioning SIB Type 3-3 -- bit 7 in the fifth octet maps to positioning SIB Type 3-4 Any unassigned bits are spare and shall be coded as zero. Non-included bits shall be treated as being coded as zero. (NOTE 1) |
| validityStartTime | DateTime | M | 1 | This IE contains the UTC time when the ciphering data set becomes valid. |
| validityDuration | ValidityDuration | M | 1 | The validity duration of the ciphering data set. |
| taiList | Binary | O | 0..1 | This IE contains the TAIs of the tracking areas for which the ciphering data set is applicable. It is encoded as octets 2 to n of the 5GS tracking area identity list IE specified in clause 9.11.3.9 of 3GPP TS 24.501 [bb]. If this IE is omitted, the ciphering data set is valid in the entire PLMN. |
| NOTE 1: At least one of ltesibTypes IE and nrsibTypes IE shall be included. |  |  |  |  |

Editor's note [WI: 5G_eLCS, CR#0042]:	The positioning SIBs listed for the IEs lteSibTypes and nrSibTypes include positioning SIBs defined in TS 36.331 and TS 36.355 for E-UTRAN in Rel-15 plus new positioning SIBs for NG-RAN being discussed in RAN2 for Rel-16. These positioning SIBs will need to be aligned later with RAN2 agreements for Rel-16.

##### 6.y.6.2.5	Type: CipheringSetReport

Table 6.y.6.2.5-1: Definition of CipheringSetReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cipheringSetID | CipheringSetID | M | 1 | Identification of a ciphering data set |
| storageOutcome | StorageOutcome | M | 1 | Indication of whether the ciphering data set was successfully stored or was not stored. |

##### 6.y.6.2.6	Type: CipherRequestData

Table 6.y.6.2.6-1: Definition of CipherRequestData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amfCallBackURI | Uri | M | 1 | Callback URI of the NF Service Consumer |

##### 6.y.6.2.7	Type: CipherResponseData

Table 6.y.6.2.7-1: Definition of CipherResponseData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dataAvailability | DataAvailability | M | 1 | An indication of whether the LMF currently has ciphering key data applicable to the NF Service Consumer |

#### 6.y.6.3	Simple data types and enumerations

##### 6.y.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.y.6.3.2	Simple data types

The simple data types defined in table 6.y.6.3.2-1 shall be supported.

Table 6.y.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| CipheringSetID | integer | The ciphering set ID Minimum = 0. Maximum = 65535 |
| CipheringKey | Binary | A 128 bit ciphering key encoded using 16 octets |
| C0 | Binary | A 128 bit value for C0 encoded using 16 octets |
| ValidityDuration | integer | The validity duration in minutes. Minimum = 1. Maximum = 65535 |

##### 6.y.6.3.3	Enumeration: StorageOutcome

The enumeration StorageOutcome represents the outcome of cipher set data storage at the service consumer NF.

Table 6.y.6.3.3-1: Enumeration StorageOutcome

| Enumeration value | Description |
| --- | --- |
| "STORAGE_SUCCESSFUL" |  |
| "STORAGE_FAILED" |  |

##### 6.y.6.3.4	Enumeration: DataAvailability

The enumeration DataAvailability represents the availability of ciphering key data at an LMF.

Table 6.y.6.3.4-1: Enumeration DataAvailability

| Enumeration value | Description |
| --- | --- |
| "CIPHERING_KEY_DATA_AVAILABLE" |  |
| CIPHERING_KEY_DATA_NOT_AVAILABLE" |  |

### 6.y.7	Error Handling

#### 6.y.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.y.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.y.7.3	Application Errors

The application errors defined for the Nlmf_Broadcast service are listed in table 6.y.7.3-1.

Table 6.y.7.3-1: Application errors

| Application Error | HTTP status code |  | Description |  |
| --- | --- | --- | --- | --- |
| UNSPECIFIED | 403 Forbidden |  | The request is rejected due to unspecified reasons. |  |
| UNABLE_TO_STORE_CIPHERING_KEY_DATA | 403 Forbidden |  | The service consumer NF was unable to store ciphering key data. |  |
| BROADCAST_CIPHERING_KEYS_NOT_SUPPORTED | 403 Forbidden | Ciphering keys for broadcast are not supported. |

### 6.y.8	Security

The Nlmf_Broadcast  API does not define service operations for which additional security is needed in this version of the specification.

>>>Next Changes<<<

## A.n	Nlmf_Broadcast API

openapi: 3.0.0

info:

version: '1.0.0.alpha-1'

title: 'LMF Broadcast'

description: |

LMF Broadcast Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.572 V16.1.0; 5G System; Location Management Services; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.572/'

servers:

- url: '{apiRoot}/nlmf-broadcast/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

paths:

/cipher-key-data:

post:

summary: Request ciphering key data

operationId: CipheringKeyData

tags:

- Request Ciphering Key Data

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/CipherRequestData'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CipherResponseData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

CipheringKeyData:

'{$request.body#/amfCallBackURI}':

post:

requestBody:

description: Ciphering Key Data Notification

content:

application/json:

schema:

$ref: '#/components/schemas/CipheringKeyInfo'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CipheringKeyResponse'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

#

# COMPLEX TYPES

#

CipheringKeyInfo:

type: object

required:

- cipheringData

properties:

cipheringData:

type: array

items:

$ref: '#/components/schemas/CipheringDataSet'

minItems: 1

CipheringKeyResponse:

type: object

properties:

cipheringDataReport:

type: array

items:

$ref: '#/components/schemas/CipheringSetReport'

minItems: 1

CipheringDataSet:

type: object

required:

- cipheringSetID

- cipheringKey

- c0

- validityStartTime

- validityDuration

properties:

cipheringSetID:

$ref: '#/components/schemas/CipheringSetID'

cipheringKey:

$ref: '#/components/schemas/CipheringKey'

c0:

$ref: '#/components/schemas/C0'

ltePosSibTypes:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

nrPosSibTypes:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

validityStartTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

validityDuration:

$ref: '#/components/schemas/ValidityDuration'

taiList:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

CipheringSetReport:

type: object

required:

- cipheringSetID

- storageOutcome

properties:

cipheringSetID:

$ref: '#/components/schemas/CipheringSetID'

storageOutcome:

$ref: '#/components/schemas/StorageOutcome'

CipherRequestData:

type: object

required:

- amfCallBackURI

properties:

amfCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

CipherResponseData:

type: object

required:

- dataAvailability

properties:

dataAvailability:

$ref: '#/components/schemas/DataAvailability'

#

#

# SIMPLE TYPES

#

CipheringSetID:

type: integer

minimum: 0

maximum: 65535

CipheringKey:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

C0:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

ValidityDuration:

type: integer

minimum: 1

maximum: 65535

#

# ENUMS

#

StorageOutcome:

anyOf:

- type: string

enum:

- STORAGE_SUCCESSFUL

- STORAGE_FAILED

DataAvailability:

anyOf:

- type: string

enum:

- CIPHERING_KEY_DATA_AVAILABLE

- CIPHERING_KEY_DATA_NOT_AVAILABLE

>>>End of Changes<<<