3GPP TSG-CT WG1 Meeting #128-e	C1-211505

Electronic meeting, 25 February – 5 March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.483 | CR | 0096 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Management object for APN configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT-CT |  |  |  |  |  | Date: |  |  | 2021-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Configuration parameters for APN configuration in the UE initial configuration document are specified in clause A.6 in 23.280 and in 24.484 via the agreed C1-210430.This CR introduces the corresponding configuration for the MO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the APN configuration options in the MO corresponding to the changes in C1-210430. ServiceServerUri has been corrected in Figure 8.1.5 to be consistent with the name used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MO for APN configuration will not be available, inconsistency between 24.483 and 24,484. Stage 2 not fully implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.2.44G3A1 (new), 8.2.44G3A2 (new), 8.2.44G3A3 (new), 8.2.44G3A4 (new), 8.2.44G3A5 (new), 8.2.44G3A6 (new), 8.2.44G3A7 (new), 8.2.44G3A8(new), 8.2.44G6A1 (new), 8.2.44G6A2 (new), 8.2.44G6A3 (new), 8.2.44G6A4 (new), 8.2.44G6A5 (new), 8.2.44G6A6 (new), 8.2.44G6A7 (new), 8.2.44G6A8 (new), 8.2.44G9A1 (new), 8.2.44G9A2 (new), 8.2.44G9A3 (new), 8.2.44G9A4 (new), 8.2.44G9A5 (new), 8.2.44G9A6 (new), 8.2.44G9A7 (new), 8.2.44G9A8 (new), 8.2.44G10 (new), 8.2.44G11 (new), 8.2.44G12 (new), 8.2.44G13 (new), 8.2.44G14 (new), 8.2.44G15 (new), 8.2.44G16 (new), 8.2.44G17 (new), 8.2.44G18 (new), 8.2.44G19 (new), 8.2.44G20 (new), 8.2.44G21 (new), 8.2.44G22 (new), 8.2.44G23 (new), 8.2.44G24 (new), 8.2.44G25(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev 1: Changed capitalization to PassWord for CHAP protocols Changed heading level naming Corrected capitalization in figure 8.1.1of MCIdMPdnInfo |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.1	General

The MCS UE initial configuration Management Object (MO) is used to configure the MCS Client behaviour for the on-network or off-network MCS Service. The MCS UE initial configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

NOTE:	For historical reasons some of the elements in the MCS UE initial MO uses the terminology "MCPTT", however this MO is common to all MCS with some MCPTT specific elements, some MCData specific elements and some MCVideo specific elements. Not all elements that contain the terminology "MCPTT" are just MCPTT specific.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCPTT-UE-initial-configuration:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCPTT user profile MO.

The following nodes and leaf objects are possible under the MCPTT node as described in figure 8.1.1, figure 8.1.2, figure 8.1.3 and figure 8.1.4:

Figure 8.1.1: The MCS UE initial configuration MO (1 of 6)

Figure 8.1.2: The MCS UE initial configuration MO (2 of 6)

Figure 8.1.3: The MCS UE initial configuration MO (3 of 6)

Figure 8.1.4: The MCS UE initial configuration MO (4 of 6)

Figure 8.1.5: The MCS UE initial configuration MO (5 of 6)

Figure 8.1.6: The MCS UE initial configuration MO (6 of 6)

* * * Next Change * * * *

### 8.2.44G3A1	/<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo

Table 8.2.44G10.1: /<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo

| OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for data related to PDN establishment for the MCPTT service. |  |  |  |  |

### 8.2.44G3A2	/<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ApnName

Table 8.2.44G11.1: /<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ApnName

| OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ApnName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the APN to use to access the MCPTT service. |  |  |  |  |

### 8.2.44G3A3	/<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/PapParameters

Table 8.2.44G12.1: /<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/PapParameters

| OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/PapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for PAP access credentials to access the MCPTT PDN. |  |  |  |  |

### 8.2.44G3A4	/<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/PapParameters/UserName

Table 8.2.44G13.1: /<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/PapParameters/UserName

| OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/PapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP user name if needed to access the MCPTT PDN. |  |  |  |  |

### 8.2.44G3A5	/<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/PapParameters/PassWord

Table 8.2.44G14.1: /<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/PapParameters/PassWord

| OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/PapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP password if needed to access the MCPTT PDN. |  |  |  |  |

### 8.2.44G3A6	/<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ChapParameters

Table 8.2.44G15.1: /<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ChapParameters

| OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ChapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for CHAP access credentials to access the MCPTT PDN. |  |  |  |  |

### 8.2.44G3A7	/<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ChapParameters/UserName

Table 8.2.44G16.1: /<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ChapParameters/UserName

| OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ChapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP user name if needed to access the MCPTT PDN. |  |  |  |  |

### 8.2.44G3A8	/<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ChapParameters/PassWord

Table 8.2.44G17.1: /<x>/OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ChapParameters/PassWord

| OnNetwork/MCPTTServiceDetails/MCPTTPdnInfo/ChapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP password if needed to access the MCPTT PDN. |  |  |  |  |

* * * Next Change * * * *

### 8.2.44G6A1	/<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo

Table 8.2.44G18.1: /<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo

| OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for data related to PDN establishment for the MCVideo service. |  |  |  |  |

### 8.2.44G6A2	/<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ApnName

Table 8.2.44G19.1: /<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ApnName

| OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ApnName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the APN to use to access the MCVideo service. |  |  |  |  |

### 8.2.44G6A3	/<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/PapParameters

Table 8.2.44G20.1: /<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/PapParameters

| OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/PapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for PAP access credentials to access the MCVideo PDN. |  |  |  |  |

### 8.2.44G6A4	/<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/PapParameters/UserName

Table 8.2.44G21.1: /<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/PapParameters/UserName

| OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/PapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP user name if needed to access the MCVideo PDN. |  |  |  |  |

### 8.2.44G6A5	/<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/PapParameters/PassWord

Table 8.2.44G22.1: /<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/PapParameters/PassWord

| OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/PapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP password if needed to access the MCVideo PDN. |  |  |  |  |

### 8.2.44G6A6	/<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ChapParameters

Table 8.2.44G23.1: /<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ChapParameters

| OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ChapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for CHAP access credentials to access the MCVideo PDN. |  |  |  |  |

### 8.2.44G6A7	/<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ChapParameters/UserName

Table 8.2.44G24.1: /<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ChapParameters/UserName

| OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ChapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP user name if needed to access the MCVideo PDN. |  |  |  |  |

### 8.2.44G6A8	/<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ChapParameters/PassWord

Table 8.2.44G25.1: /<x>/OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ChapParameters/PassWord

| OnNetwork/MCVideoServiceDetails/MCVideoPdnInfo/ChapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP password if needed to access the MCVideo PDN. |  |  |  |  |

* * * Next Change * * * *

### 8.2.44G9A1	/<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo

Table 8.2.44G26.1: /<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo

| OnNetwork/MCDataServiceDetails/MCDataPdnInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for data related to PDN establishment for the MCData service. |  |  |  |  |

### 8.2.44G9A2	/<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ApnName

Table 8.2.44G27.1: /<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ApnName

| OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ApnName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the APN to use to access the MCData service. |  |  |  |  |

### 8.2.44G9A3	/<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/PapParameters

Table 8.2.44G28.1: /<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/PapParameters

| OnNetwork/MCDataServiceDetails/MCDataPdnInfo/PapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for PAP access credentials to access the MCData PDN. |  |  |  |  |

### 8.2.44G9A4	/<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/PapParameters/UserName

Table 8.2.44G29.1: /<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/PapParameters/UserName

| OnNetwork/MCDataServiceDetails/MCDataPdnInfo/PapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP user name if needed to access the MCData PDN. |  |  |  |  |

### 8.2.44G9A5	/<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/PapParameters/PassWord

Table 8.2.44G30.1: /<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/PapParameters/PassWord

| OnNetwork/MCDataServiceDetails/MCDataPdnInfo/PapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP password if needed to access the MCData PDN. |  |  |  |  |

### 8.2.44G9A6	/<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ChapParameters

Table 8.2.44G31.1: /<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ChapParameters

| OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ChapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for CHAP access credentials to access the MCData PDN. |  |  |  |  |

### 8.2.44G9A7	/<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ChapParameters/UserName

Table 8.2.44G32.1: /<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ChapParameters/UserName

| OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ChapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP user name if needed to access the MCData PDN. |  |  |  |  |

### 8.2.44G9A8	/<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ChapParameters/PassWord

Table 8.2.44G9.33: /<x>/OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ChapParameters/PassWord

| OnNetwork/MCDataServiceDetails/MCDataPdnInfo/ChapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP password if needed to access the MCData PDN. |  |  |  |  |

* * * Next Change * * * *

### 8.2.44G10	/<x>/OnNetwork/MCCommonCorePdnInfo

Table 8.2.44G34.1: /<x>/OnNetwork/MCCommonCorePdnInfo

| OnNetwork/MCCommonCorePdnInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for data related to PDN establishment for the MCCommonCore service. |  |  |  |  |

### 8.2.44G11	/<x>/OnNetwork/MCCommonCorePdnInfo/ApnName

Table 8.2.44G35.1: /<x>/OnNetwork/MCCommonCorePdnInfo/ApnName

| OnNetwork/MCCommonCorePdnInfo/ApnName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the APN to use to access the MCCommonCore services. |  |  |  |  |

### 8.2.44G12	/<x>/OnNetwork/MCCommonCorePdnInfo/PapParameters

Table 8.2.44G36.1: /<x>/OnNetwork/MCCommonCorePdnInfo/PapParameters

| OnNetwork/MCCommonCorePdnInfo/PapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for PAP access credentials to access the MCCommonCore PDN. |  |  |  |  |

### 8.2.44G13	/<x>/OnNetwork/MCCommonCorePdnInfo/PapParameters/UserName

Table 8.2.44G37.1: /<x>/OnNetwork/MCCommonCorePdnInfo/PapParameters/UserName

| OnNetwork/MCCommonCorePdnInfo/PapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP user name if needed to access the MCCommonCore PDN. |  |  |  |  |

### 8.2.44G14	/<x>/OnNetwork/MCCommonCorePdnInfo/PapParameters/PassWord

Table 8.2.44G38.1: /<x>/OnNetwork/MCCommonCorePdnInfo/PapParameters/PassWord

| OnNetwork/MCCommonCorePdnInfo/PapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP password if needed to access the MCCommonCore PDN. |  |  |  |  |

### 8.2.44G15	/<x>/OnNetwork/MCCommonCorePdnInfo/ChapParameters

Table 8.2.44G39.1: /<x>/OnNetwork/MCCommonCorePdnInfo/ChapParameters

| OnNetwork/MCCommonCorePdnInfo/ChapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for CHAP access credentials to access the MCComonCore PDN. |  |  |  |  |

### 8.2.44G16	/<x>/OnNetwork/MCCommonCorePdnInfo/ChapParameters/UserName

Table 8.2.44G40.1: /<x>/OnNetwork/MCCommonCorePdnInfo/ChapParameters/UserName

| OnNetwork/MCCommonCorePdnInfo/ChapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP user name if needed to access the MCCommonCore PDN. |  |  |  |  |

### 8.2.44G17	/<x>/OnNetwork/MCCommonCorePdnInfo/ChapParameters/PassWord

Table 8.2.44G41.1: /<x>/OnNetwork/MCCommonCorePdnInfo/ChapParameters/PassWord

| OnNetwork/MCCommonCorePdnInfo/ChapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP password if needed to access the MCCommonCore PDN. |  |  |  |  |

### 8.2.44G18	/<x>/OnNetwork/MCIdMPdnInfo

Table 8.2.44G42.1: /<x>/OnNetwork/MCIdMPdnInfo

| OnNetwork/MCIdMPdnInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for data related to PDN establishment for the MCIdM service. |  |  |  |  |

### 8.2.44G19	/<x>/OnNetwork/MCIdMPdnInfo/ApnName

Table 8.2.44G43.1: /<x>/OnNetwork/MCIdMPdnInfo/ApnName

| OnNetwork/MCIdMPdnInfo/ApnName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the APN to use to access the MCIdM service. |  |  |  |  |

### 8.2.44G20	/<x>/OnNetwork/MCIdMPdnInfo/PapParameters

Table 8.2.44G44.1: /<x>/OnNetwork/MCIdMPdnInfo/PapParameters

| OnNetwork/MCIdMPdnInfo/PapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for PAP access credentials to access the MCIdM service. |  |  |  |  |

### 8.2.44G21	/<x>/OnNetwork/MCIdMPdnInfo/PapParameters/UserName

Table 8.2.44G45.1: /<x>/OnNetwork/MCIdMPdnInfo/PapParameters/UserName

| OnNetwork/MCIdMPdnInfo/PapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP user name if needed to access the MCIdM service. |  |  |  |  |

### 8.2.44G22	/<x>/OnNetwork/MCIdMPdnInfo/PapParameters/PassWord

Table 8.2.44G46.1: /<x>/OnNetwork/MCIdMPdnInfo/PapParameters/PassWord

| OnNetwork/MCIdMPdnInfo/PapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP password if needed to access the MCIdM service. |  |  |  |  |

### 8.2.44G23	/<x>/OnNetwork/MCIdMPdnInfo/ChapParameters

Table 8.2.44G47.1: /<x>/OnNetwork/MCIdMPdnInfo/ChapParameters

| OnNetwork/MCIdMPdnInfo/ChapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for CHAP access credentials to access the MCIdM service. |  |  |  |  |

### 8.2.44G24	/<x>/OnNetwork/MCIdMPdnInfo/ChapParameters/UserName

Table 8.2.44G48.1: /<x>/OnNetwork/MCIdMPdnInfo/ChapParameters/UserName

| OnNetwork/MCIdMPdnInfo/ChapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP user name if needed to access the MCIdM service. |  |  |  |  |

### 8.2.44G25	/<x>/OnNetwork/MCIdMPdnInfo/ChapParameters/PassWord

Table 8.2.44G49.1: /<x>/OnNetwork/MCIdMPdnInfo/ChapParameters/PassWord

| OnNetwork/MCIdMPdnInfo/ChapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP password if needed to access the MCIdM service. |  |  |  |  |

* * * End of Changes * * * *