3GPP TSG-CT Meeting #91e	CP-210183

# E-Meeting, 18th – 24th March 2021					(revision of CP-201208)

# Source:	3GPP TSG CT WG3

# Title:	Revised WID on PFD management enhancement

# Document for:	Approval

Agenda Item:	17.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Revised WID on PFD management enhancement

## Acronym: pfdManEnh

## Unique identifier: 	 880032

Potential target Release: Rel-17.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 730037 | CT3 Aspect of Sponsored Data Connectivity Improvements | Rel-14 stage 3 work item for EPS |
| 750025 | CT aspects on 5G System - Phase 1 | Rel-15 stage 3 work item for 5GS |

Dependency on non-3GPP (draft) specification: None

## 3	Justification

Pull mode and Push mode have been defined in both EPS and 5GS for the PFD management.

For pull mode, according to stage 2 requirement, when the caching timer elapses, if there are still active PCC/ADC rules that refer to the corresponding application identifier, the PCEF/TDF/SMF reloads the PFD(s) from the PFDF/NEF(PFDF). In stage 3, we define the GET method to retrieve the PFD for the application(s) by the client (i.e. PCEF/TDF in EPS and SMF in 5GS), the server (i.e. PFDF in EPS and NEF (PFDF) in 5GS) responds a whole representation of the resource for the requested application identifier(s) even in the case that the client requests the PFDs when the caching timer expires and the PFD(s) is not changed at the server. It brings a large signalling load if the whole set of the PFD(s) are always returned by considering that the number of the PFD(s) is very large in the deployment.

For push mode, in order to protect the PCEF/TDF from overload, CT3 introduces the notification push for the Push mode in EPS, i.e. the PFDF sends a request with notification flag to the PCEF/TDF and then PCFE/TDF initiates separate pull requests to retrieve the PFDs for different application identifier(s) immediately or within the allowed-delay by considering the load of the PCEF/TDF. But this optimized solution is not defined in 5GS.

## 4	Objective

The objective of this work item is to specify the following stage 3 procedures:

- Define an optimize solution to reduce the load of signalling for pull mode in EPS and 5GS.

- Define a notification push solution for push mode in 5GS.

Above objectives are in the scope of CT3 and SA2 requirement enhancement is not needed.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.251 | Introduce an optimize solution to reduce the load of signalling for pull mode in EPS | CT#92 |  |
| 29.551 | Introduce an optimize solution to reduce the load of signalling for pull mode and a notification push solution in 5GS | CT#92 |  |
| 29.513 | The signalling flows of partial pull and notification push | CT#92 |  |

## 6	Work item Rapporteur(s)

Xiaoyun Zhou, Huawei, zhouxiaoyun8@huawei.com

## 7	Work item leadership

CT3

## 8	Aspects that involve other WGs

None.

## 9	Supporting Individual Members

Supporting IM name

Huawei

China Telecom

China Mobile

China Unicom

ZTE

Nokia

Nokia Shanghai Bell

Ericsson