3GPP TSG-CT Meeting #94-e	CP-213197

# E-Meeting, 13rd – 15th December 2021 	(was C4-216512)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.544 | CR | 0014 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SOR-CMCI support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Work item code: | eCPSOR_CON |  |  |  |  |  | Date: |  |  | 2021-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In stage2 for eCPSOR_CON, concept of SOR-CMCI has been added. The stage3 spec needs to be updated to reflect the corresponding requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of SOR-CMCI is added. - Data type "UiccConfigurationParameter" is updated to include a new data type that includes Steering Container and SOR-CMCI, allowing to create secured packet including SOR-CMCI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Does not meet the stage2 requirement for Workitem eCPSOR_CON |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.6.1, 6.1.6.2.2, (new) 6.1.6.2.x, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 29.503 CR 0776 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible API changes. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | V1: - Introduced a new datatype ExtendedSteeringContainer to resolve the issue where UiccConfigurationParameter can only include "One of" the attributes, while two attributes, steeringContainer and new attribute sorCmci, needs to be sent together. - Changes to OpenAPI corresponding to the above change. - Changes to OpenAPI on the data type for SorCmci, to correspond the changed proposal to TS 29.503 CR 0776 (proposed change changed to different API) V2: - Correction to OpenAPI due to missing line. - Corrected coversheet on clauses affected (added missing impacted new clause) |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0..

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 29.503: "Unified Data Management Services"; Stage 3.

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[16]	3GPP TS 31.115: "Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications".

[17]	3GPP TS 29.509: "Authentication Server Services; Stage 3".

[18]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)"

[xx]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

* * * Next Change * * * *

6.1.6	Data Model

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nspaf service based interface protocol.

Table 6.1.6.1-1: Nspaf specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| UiccConfigurationParameter | 6.1.6.2.2 | UICC Configuration Parameters |  |
| RoutingId | 6.1.6.3.2 | Routing ID |  |
| ExtendedSteeringContainer | 6.1.6.2.x | Extended Steering Container (including the contents of Steering Container and SOR-CMCI) |  |

Table 6.1.6.1-2 specifies data types re-used by the Nspaf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nspaf service based interface.

Table 6.1.6.1-2: Nspaf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SecuredPacket | 3GPP TS 29.503 [14] | Secured Packet |  |
| ProblemDetails | 3GPP TS 29.571 [15] |  |  |
| SteeringInfo | 3GPP TS 29.509 [17] | Steering Information |  |
| SorCmci | 3GPP TS 29.503 [14] | Contains SOR-CMCI as defined in 3GPP TS 24.501 [xx] |  |

* * * Next Change * * * *

6.1.6.2.2	Type: UiccConfigurationParameter

Table 6.1.6.2.2-1: Definition of type UiccConfigurationParameter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| routingId | RoutingId | C | 0..1 | The Routing Id that needs to be updated in the USIM. |  |
| steeringContainer | array(SteeringInfo) | C | 1..N | List of PLMN/AccessTechnologies combinations that need to be updated in the USIM. |  |
| extendedSteeringContainer | ExtendedSteeringContainer | C | 0..1 | Extended Steering Container that includes list of PLMN/AccessTechnologies combinations with the SOR-CMCI that need to be updated in the USIM |  |
| Note: Exactly one attribute shall be present |  |  |  |  |  |

* * * Next Change * * * *

6.1.6.2.x	Type: ExtendedSteeringContainer

Table 6.1.6.2.x-1: Definition of type ExtendedSteeringContainer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| steeringContainer | array(SteeringInfo) | C | 1..N | List of PLMN/AccessTechnologies combinations that need to be updated in the USIM. |  |
| sorCmci | SorCmci | C | 0..1 | When present, provides the SOR-CMCI values as defined in 3GPP TS 24.501 [xx] |  |
| Note: At least one attribute shall be present. If no additional attributes than steeringContainer is included, then use of steeringContainer can be considered instead of using ExtendedSteeringContainer. |  |  |  |  |  |

* * * Next Change * * * *

A.2	Nspaf_SecuredPacket API

openapi: 3.0.0

info:

title: 'Nspaf_SecuredPacket'

version: '1.1.0-alpha.1'

description: |

Nspaf Secured Packet Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.544, SP-AF Services, version V17.0.0

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.544/

-----(snipped for clarity)-----

# COMPLEX TYPES:

UiccConfigurationParameter:

description: Represents UICC Configuration Parameters.

type: object

oneOf:

- required: [routingId ]

- required: [steeringContainer ]

- required: [extendedSteeringContainer ]

properties:

routingId:

$ref: '#/components/schemas/RoutingId'

steeringContainer:

type: array

items:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SteeringInfo'

minItems: 1

extendedSteeringContainer:

$ref: '#/components/schemas/ExtendedSteeringContainer'

ExtendedSteeringContainer:

description: Represents Extended Steering Containers.

type: object

properties:

steeringContainer:

type: array

items:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SteeringInfo'

minItems: 1

sorCmci:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SorCmci'

# SIMPLE TYPES:

RoutingId:

description: Represents a routing indicator.

type: string

pattern: '^[0-9]{1,4}$'

# ENUMS:

-----(snipped for clarity)-----

* * * End of Changes * * * *