3GPP TSG-CT3 Meeting #94e 	CP-213204

# E-Meeting, 13th – 15th Decemeber 2021							                 (revision of C3-216098)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0447 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Nnef_MBSSession service definition – OpenAPI part |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-12-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 has further progressed the Stage 2 normative work on 5MBS WI and specified in 3GPP TS 23.247 (clause 9.4) the new NEF services that need to be supported. Hence, the new Nnef_MBSSession service exposed by the NEF needs to be defined in TS 29.522. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to: - Define the OpenAPI part of the new Nnef_MBSSession service exposed by the NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - 5MBS requirements from Stage 2 on Nnef_MBSSession service is not implemented in Stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.zz (new Annex) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a new OpenAPI specification file for the new Nnef_MBSSession API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1 has following changes: - Removed “502” status code response from the POST method on the “/mbsSessions” resource. - Updated “MbsSession” and “MbsSessionId” data type as defined in TS 29.571 (instead of TS 29.522 in the definition of the PUT and PATCH methods), - The request body of the PATCH method updated to “MbsSessionPatch” data type instead of the “MbsSession” data type. - Removed unnecessary quotes “’’$ref:”. |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

## A.zz	Nnef_MBSSession API

openapi: 3.0.0

info:

title: 3gpp-mbs-session

version: 1.0.0-alpha.1

description: |

<API Name> Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V17.4.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

servers:

- url: '{apiRoot}/3gpp-mbs-session/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- 3gpp-mbs-session

paths:

/mbs-sessions:

post:

summary:  Create

tags:

- MBS sessions collection

operationId: Create

requestBody:

description: representation of the MBS session to be created in the NEF

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionCreateReq'

responses:

'201':

description: successful creation of an MBS session

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionCreateRsp'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-mbs-session/v1/mbssessions/{mbsSessionId}'

required: true

schema:

type: string

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-sessions/{mbsSessionId}:

parameters:

- name: mbsSessionId

in: path

description: Identifier of the MBS Session

required: true

schema:

type: string

put:

summary: Updates/replaces an existing MBS session resource

tags:

- Individual MBS Session

requestBody:

description: Parameters to update/replace the MBS Session

required: true

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSession'

responses:

'200':

description: OK (Successful update of the MBS Session resource)

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSession'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Updates/replaces an existing MBS Session resource

tags:

- Individual MBS Session

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MbsSessionPatch'

responses:

'200':

description: OK. The MBS Session resource was modified successfully.

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSession'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing MBS Session resource

tags:

- Individual MBS Session

responses:

'204':

description: No Content (Successful deletion of the existing MBS Session resource)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-sessions/subscriptions:

get:

summary: read all of the active MBS Sessions status subscriptions

tags:

- MBS Session Subscriptions

responses:

'200':

description: OK.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/MbsSessionSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new MBS Session subscription resource

tags:

- MBS Session subscriptions

requestBody:

description: Request to create a new MBS Session subscription resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionSubsc'

callbacks:

notificationUri:

'{request.body#/notificationUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionStatusNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation of subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionSubsc'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-sessions/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: Identifier of the MBS Session subscription resource

required: true

schema:

type: string

get:

summary: read an active MBS Session subscriptions for the SCS/AS and the subscription Id

tags:

- Individual MBS Session subscription

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

tags:

- Individual MBS Session Subscription

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

3gpp-mbs-session: Access to the Nnef-MBSSession API

schemas:

#

# STRUCTURED DATA TYPES

#

MbsSessionCreateReq:

description: Data within Create Request

type: object

properties:

afId:

type: string

mbsSession:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSession'

required:

- mbsSession

MbsSessionCreateRsp:

description: Data within Create Response

type: object

properties:

mbsSession:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSession'

tmgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tmgi'

inTunAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TunnelAddress'

required:

- mbsSession

MbsSessionPatch:

description: Represents the requested modifications to an Individual MBS Session resource

type: object

properties:

mbsSession:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSession'

required:

- mbsSession

MbsSessionSubsc:

description: Represents an MBS Session Subscription.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

mbsSessionStatus:

$ref: '#/components/schemas/MbsSessionStatus'

required:

- notificationUri

- mbsSessionId

- mbsSessionStatus

MbsSessionStatusNotif:

description: Represents an MBS Session Status notification.

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

mbsSessionStatus:

$ref: '#/components/schemas/MbsSessionStatus'

required:

- mbsSessionId

- mbsSessionStatus

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

MbsSessionStatus:

anyOf:

- type: string

enum:

- MBS_REL_TMGI_EXPIRY

- BROADCAST_STATUS

- type: string

description: >

Possible values are

- MBS_REL_TMGI_EXPIRY: The AF requests to be notified about MBS Session release due to TMGI expiry.

- BROADCAST_STATUS: The AF requests to be notified about MBS Session broadcast status.

* * * End of changes * * * *