3GPP TSG-CT3 Meeting #119-e	C3-216043

# Online, , 11th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.214 | CR | 1667 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of an invalid 3GPP-User-Location-Info value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2021-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The 3GPP-User-Location-Info attribute (re-used from 29.061) mentions the values "TAI and NCGI" as applicable, which does not exist in 29.061. The correct value is "5GS TAI and NCGI". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the (invalid) "TAI and NCGI" with "5GS TAI and NCGI". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.4.0	General

Table 5.4.0.1 lists the Diameter AVPs re-used by the Rx reference point from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their usage within the Rx reference point. Other AVPs from existing Diameter Applications, except for the AVPs from Diameter Base Protocol, do not need to be supported. The AVPs from Diameter Base Protocol are not included in table 5.4.0.1, but they are re-used for the Rx protocol. Unless otherwise stated, re-used AVPs shall maintain their ‘M’, ‘P’ and ‘V’ flag settings. Where 3GPP Radius VSAs are re-used, unless otherwise stated, they shall be translated to Diameter AVPs as described in RFC 4005 [12] with the exception that the ‘M’ flag shall be set and the ‘P’ flag may be set.

# Table 5.4.0.1: Rx re-used Diameter AVPs

| Attribute Name | Reference | Comments | Applicability (notes 1, 2) |
| --- | --- | --- | --- |
| 3GPP-MS-TimeZone | 3GPP TS 29.061 [34] | Indicates the offset between universal time and local time in steps of 15 minutes of where the MS currently resides. This AVP shall have the 'M' bit cleared. | NetLoc RAN-NAS-Cause Netloc-Trusted-WLAN |
| 3GPP-SGSN-MCC-MNC | 3GPP TS 29.061 [34] | Indicates the serving core network operator ID. For GPRS accesses the MCC and the MNC of the SGSN. For EPS the MCC and the MNC provided by the SGW or TWAG. This AVP shall have the 'M' bit cleared. | NetLoc, Netloc-Trusted-WLAN RAN-NAS-Cause |
| 3GPP-User-Location-Info | 3GPP TS 29.061 [34] | Indicates details of where the UE is currently located (e.g. SAI or CGI), Coding shall be done as defined in 3GPP TS 29.274 [33]. The values "NCGI" and "5GS TAI and NCGI" (which are not applicable in some specification using this AVP) shall be applicable. This AVP shall have the 'M' bit cleared. | NetLoc RAN-NAS-Cause |
| AN-GW-Address | 3GPP TS 29.212 [8] | Carries the IP address of the ePDG used as IPSec tunnel endpoint with the UE. This AVP shall have the ‘M’ bit cleared. |  |
| AN-Trusted | 3GPP TS 29.273 [39] | Indicates whether the access network is trusted or untrusted for the Non-3GPP access network. This AVP shall have the 'M' bit cleared. |  |
| Called-Station-Id | IETF RFC 4005 [12] | The PDN the user is connected to. For GPRS and EPS the APN. When used to contain the APN, the APN is composed of the APN Network Identifier only, or the APN Network Identifier and the APN Operator Identifier as specified in TS 23.003 [38], clause 9.1. The inclusion of the APN Operator Identifier can be configurable. | Rel8 |
| DRMP | IETF RFC 7944 [43] | Allows Diameter endpoints to indicate the relative priority of Diameter transactions. |  |
| Final-Unit-Action | IETF RFC 4006 [14] | The action applied by the PCEF when the user’s account cannot cover the service cost. | Rel8 |
| Framed-IP-Address | IETF RFC 4005 [12] | The valid routable Ipv4 address that is applicable for the IP Flows towards the UE at the PCEF. The PCRF shall use this address to identify the correct IP-CAN session (session binding). For example, the IP address may actually be that of the network interface of a NAT device between the UE and the GW. The values 0xFFFFFFFF and 0xFFFFFFFE are not applicable as described in RFC 4005 [12]. |  |
| Framed-Ipv6-Prefix | IETF RFC 4005 [12] | A valid full Ipv6 address that is applicable to an IP flow or IP flows towards the UE at the PCEF. The PCRF shall use this address to identify the correct IP-CAN session (session binding, refer to TS 29.213 [9]). For example, the IP address may actually be that of the network interface of a NAT device between the UE and the GW. The encoding of the value within this Octet String type AVP shall be as defined in RFC 3162 [20], clause 2.3. The "Reserved", "Prefix-Length" and "Prefix" fields shall be included in this order. The AF shall set the "Prefix Length" to 128 and encode the Ipv6 address of the UE within the "Prefix" field. |  |
| Granted-Service-Unit (NOTE 3) | IETF RFC 4006 [14] | The volume and/or time thresholds for sponsored data connectivity. Only CC-Total-Octets, one of the CC-Input-Octets and CC-Output-Octets, or CC-Time AVPs are reused. This AVP shall have the ‘M’ bit cleared. | SponsoredConnectivity, SCTimeBasedUM |
| IP-CAN-Type | 3GPP TS 29.212 [8] | IP-CAN type of the user. |  |
| Load | IETF RFC 8583 [51] | The AVP used to convey load information between Diameter nodes. This AVP and all AVPs within this grouped AVP shall have the 'M' bit cleared. |  |
| NetLoc-Access-Support | 3GPP TS 29.212 [8] | Indicates the level of support for NetLoc procedures provided by the current access network. | NetLoc Netloc-Trusted-WLAN NetLoc- Untrusted-WLAN |
| OC-OLR | IETF RFC 7683 [35] | Contains the necessary information to convey an overload report |  |
| OC-Supported-Features | IETF RFC 7683 [35] | Defines the support for the Diameter overload indication conveyence by the sending node |  |
| Pre-emption-Capability | 3GPP TS 29.212 [8] | Indicates whether a service data flow can get resources that were already assigned to another service data flow with a lower priority level. | MCPTT-Preemption |
| Pre-emption-Vulnerability | 3GPP TS 29.212 [8] | Indicates whether a service data flow can lose the resources assigned to it in order to admit a service data flow with higher priority level. | MCPTT-Preemption |
| RAN-NAS-Release-Cause | 3GPP TS 29.212 [8] | Indicates RAN and/or NAS release cause code information. TWAN release cause code information or untrusted WLAN release cause code information. | RAN-NAS-Cause |
| RAT-Type | 3GPP TS 29.212 [8] | Indicate which Radio Access Technology is currently serving the UE. | Rel8 |
| Reference-Id | 3GPP TS 29.154 [47] | Indicates the transfer policy stored in the SPR. |  |
| Reservation-Priority | 3GPP TS 183.017 [15] | The vendor-id shall be set to ETSI (13019) [15]. The support of this AVP shall be advertised in the capabilities exchange mechanisms (CER/CEA) by including the ETSI parameter in the Supported-Vendor-Id AVP. |  |
| Subscription-Id | IETF RFC 4006 [14] | The identification of the subscription (IMSI, MSISDN, etc.) |  |
| Supported-Features | 3GPP TS 29.229 [25] | If present, this AVP informs the destination host about the features that the origin host requires to successfully complete this command exchange. | Rel8 |
| TCP-Source-Port | 3GPP TS 29.212 [8] | Contains the TCP source port number in the case that a NAT and firewall are detected and the IKEv2 messages exchanged between the UE and the ePDG are transported using the firewall traversal tunnel as described in 3GPP TS 24.302 [50].This AVP shall have the ‘M’ bit cleared. | NetLoc-Untrusted-WLAN |
| TWAN-Identifier | 3GPP TS 29.061 [34] | Indicates the UE location in a Trusted WLAN or Untrusted WLAN Access Network. This AVP shall have the ‘M’ bit cleared. | Netloc-Trusted-WLAN RAN-NAS-Cause NetLoc- Untrusted-WLAN |
| ToS-Traffic-Class | 3GPP TS 29.212 [8] | Indicates the DSCP code to be used for packet filter. The first octet contains the DSCP code and the second octet contains the mask field set to 11111100. | DSCP |
| UDP-Source-Port | 3GPP TS 29.212 [8] | Contains the UDP source port number in the case that NAT is detected and the IKEv2 messages exchanged between the UE and the ePDG are encapsulated in the UDP messages according to IETF RFC 3948 [49]. This AVP shall have the ‘M’ bit cleared. | NetLoc-Untrusted-WLAN |
| UE-Local-IP-Address | 3GPP TS 29.212 [8] | Indicates the local IP address of the UE. This AVP shall have the ‘M’ bit cleared. | NetLoc-Untrusted-WLAN |
| Used-Service-Unit (NOTE 3) | IETF RFC 4006 [14] | The measured volume and/or time for sponsored data connectivity. Only CC-Total-Octets, one of the CC-Input-Octets and CC-Output-Octets, or CC-Time AVPs are reused. This AVP shall have the ‘M’ bit cleared. | SponsoredConnectivity SCTimeBasedUM |
| User-Equipment-Info | IETF RFC 4006 [14] | The identification and capabilities of the terminal (IMEISV, etc.) When the User-Equipment-Info-Type is set to IMEISV(0), the value within the User-Equipment-Info-Value shall be a UTF-8 encoded decimal. |  |
| User-Location-Info-Time | 3GPP TS 29.212 [8] | Indicates the time the UE was last known to be in the location. | NetLoc RAN-NAS-Cause NetLoc- Untrusted-WLAN |
| NOTE 1: AVPs marked with "Rel8" are applicable as described in clause 5.4.1. NOTE 2: AVPs marked with "SponsoredConnectivity" are applicable for sponsored data connectivity. NOTE 3: Volume Usage monitoring control functionality is applicable for SponsoredConnectivity supported feature. Time Based Usage monitoring control is applicable for SCTimeBasedUM supported feature. |  |  |  |

# * * * * End of changes * * * *