3GPP TSG-CT Meeting #97-e	CP-222207

# E-Meeting, 12th – 14th September 2022														was C3-224707

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0706 | rev | 2 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating the OpenAPI description to support MBS Session Authorization provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The stage 2 requirements for MBS Session Authorization information provisioning by an AF are specified in clause 7.2.9 of TS 23.247. It is however not yet specified in stage 3 for the parts under CT3's remit. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to: - Define MBS Session Authorization information provisioning by an AF by updating the Nnef_MBSSession API to implement the related stage 2 requirements on the Nnef_ParameterProvision API. This will avoid creating a brand new MBS parameters provisioning service API. - This CR defines the necessary updates to the OpenAPI description of the Nnef_MBSSession API. CRs C3-224421, C3-224422 and C3-224423 define the other parts (service description, resources, data model). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Missing definition of the stage 2 requirements on MBS Session Authorization information provisioning by an AF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible new feature to the MBSSession API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: Update the reference of "gpsisList" attribute in the OpenAPI. |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

A.18	MBSSession API

openapi: 3.0.0

info:

title: 3gpp-mbs-session

version: 1.0.0

description: |

API for MBS Session Management.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V17.6.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

servers:

- url: '{apiRoot}/3gpp-mbs-session/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials: []

paths:

/mbs-sessions:

post:

summary: Create an Individual MBS Session resource.

tags:

- MBS sessions collection

operationId: CreateMBSSession

requestBody:

description: Representation of the new MBS session to be created at the NEF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionCreateReq'

responses:

'201':

description: >

Created. Successful creation of a new Individual MBS session resource.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionCreateRsp'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/3gpp-mbs-session/v1/mbssessions/{mbsSessionRef}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-sessions/{mbsSessionRef}:

parameters:

- name: mbsSessionRef

in: path

description: Identifier of the Individual MBS Session resource.

required: true

schema:

type: string

patch:

summary: Request the modification of an existing Individual MBS Session resource.

tags:

- Individual MBS Session

requestBody:

required: true

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

responses:

'204':

description: >

No Content. The concerned Individual MBS Session resource was successfully modified.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an existing Individual MBS Session resource.

tags:

- Individual MBS Session

responses:

'204':

description: >

No Content. Successful deletion of the concerned Individual MBS Session resource.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-sessions/subscriptions:

get:

summary: Retrieve all of the active MBS Sessions subscriptions.

tags:

- MBS Session Subscriptions

responses:

'200':

description: >

OK. All the MBS Session Subscriptions resources managed by the NEF are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/MbsSessionSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new Individual MBS Session subscription resource.

tags:

- MBS Session subscriptions

requestBody:

description: Request the creation of a new MBS Session subscription resource.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionSubsc'

responses:

'201':

description: >

Created. Successful creation of a new Individual MBS Session subscription.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionSubsc'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notificationUri:

'{request.body#/notificationUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionStatusNotif'

responses:

'204':

description: No Content. Successful reception of the notification.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-sessions/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: Identifier of the MBS Session subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an Individual MBS Session subscription resource identified by the subscription Id.

tags:

- Individual MBS Session subscription

responses:

'200':

description: >

OK. Successful retrieval of the targeted Individual MBS Session subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessionSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an existing Individual MBS Session subscription resource.

tags:

- Individual MBS Session Subscription

responses:

'204':

description: >

No Content. Successful deletion of the existing Individual MBS Session subscription resource.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-pp:

get:

summary: Request to retrieve all the active MBS Parameters Provisioning resources at the NEF.

operationId: GetMBSParamsProvisionings

tags:

- MBS Parameters Provisionings

responses:

'200':

description: >

OK. All the active MBS Parameters Provisioning resources managed by the NEF are

returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/MbsPpData'

minitems: 1

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new MBS Parameters Provisioning.

tags:

- MBS Parameters Provisioning

operationId: CreateMBSParamsProvisioning

requestBody:

description: Representation of the new MBS Parameters Provisioning to be created at the NEF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPpData'

responses:

'201':

description: >

Created. Successful creation of a new Individual MBS Parameters Provisioning resource.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPpData'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/3gpp-mbs-session/v1/mbs-pp/{mbsPpId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-pp/{mbsPpId}:

parameters:

- name: mbsPpId

in: path

description: >

Represents the identifier of the Individual MBS Parameters Provisioning resource.

required: true

schema:

type: string

get:

summary: Request to retrieve an existing Individual MBS Parameters Provisioning resource.

operationId: GetIndMBSParamsProvisioning

tags:

- Individual MBS Parameters Provisioning

responses:

'200':

description: >

OK. Successful retrieval of the requested Individual MBS Parameters Provisioning.

resource.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPpData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual MBS Parameters Provisioning resource.

tags:

- Individual MBS Parameters Provisioning

operationId: UpdateIndMBSParamsProvisioning

requestBody:

description: Represents the updated Individual MBS Parameters Provisioning resource

representation.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPpData'

responses:

'200':

description: >

OK. The Individual MBS Parameters Provisioning resource is successfully updated and a

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPpData'

'204':

description: >

No Content. The Individual MBS Parameters Provisioning resource is successfully updated.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual MBS Parameters Provisioning resource.

tags:

- Individual MBS Parameters Provisioning

operationId: ModifyIndMBSParamsProvisioning

requestBody:

description: >

Contains the parameters to request the modification of the Individual Parameters

Provisioning resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MbsPpDataPatch'

responses:

'200':

description: >

OK. The Individual MBS Parameters Provisioning resource is successfully modified and a

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPpData'

'204':

description: >

No Content. The Individual MBS Parameters Provisioning resource is successfully

modified.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual MBS Parameters Provisioning resource.

tags:

- Individual MBS Parameters Provisioning

operationId: DeleteIndMBSParamsProvisioning

responses:

'204':

description: >

No Content. The Individual MBS Parameters Provisioning resource is successfully

deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

MbsSessionCreateReq:

description: Data within the MBS Session Create Request.

type: object

properties:

afId:

type: string

mbsSession:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSession'

required:

- mbsSession

MbsSessionCreateRsp:

description: Data within the MBS Session Create Response.

type: object

properties:

mbsSession:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSession'

required:

- mbsSession

MbsSessionSubsc:

description: Represents an MBS Session Subscription.

type: object

properties:

afId:

type: string

subscription:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionSubscription'

required:

- subscription

MbsSessionStatusNotif:

description: Represents an MBS Session Status notification.

type: object

properties:

eventList:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionEventReportList'

MbsPpData:

description: Represents MBS Parameters Provisioning data.

type: object

properties:

afId:

type: string

mbsSessAuthData:

$ref: '#/components/schemas/MbsSessAuthData'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

MbsSessAuthData:

description: Represents the MBS Session Authorization data.

type: object

properties:

extGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

gpsisList:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minProperties: 1

description: >

Represents the list of the GPSI(s) of the member UE(s) constituting the multicast MBS group. Any

value of type can be used as a key of the map.

mbsSessionIdList:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/5MbsAuthorizationInfo'

required:

- extGroupId

- mbsSessionIdList

MbsPpDataPatch:

description: >

Represents the requested modification to existing MBS Parameters Provisioning data.

type: object

properties:

mbsSessAuthData:

$ref: '#/components/schemas/MbsSessAuthData'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

* * * * End of changes * * * *