3GPP TSG-CT WG4 Meeting #114	C4-230023

# Athens, Greece, 27th Feb – 03rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 0977 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Memory Capacity for SMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SMS_SBI |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SMSF cannot send a UE memory available for SMS indication to the UDM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A memory available indication is added to SmsfRegistration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UDM cannot trigger alerting for SMS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.2.6, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.502... CR 3836... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections with impact to the following APIs: TS29503_Nudm_UECM.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.6.2.6	Type: SmsfRegistration

Table 6.2.6.2.6-1: Definition of type SmsfRegistration

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| smsfInstanceId | NfInstanceId | M | 1 | NF Instance Id of the SMSF |
| smsfSetId | NfSetId | C | 0..1 | This IE shall be present if the SMSF belongs to an SMSF SET. If present, it indicates the NF Set ID of SMSF Set. |
| supportedFeatures | SupportedFeatures | O | 0..1 | See clause 6.2.8 These are the features supported by the SMSF. |
| plmnId | PlmnId | M | 1 | Serving PLMN identity This IE shall include the PLMN ID of the serving network where the UE is registered from. |
| smsfMAPAddress | E164Number | C | 0..1 | International E.164 number of the SMSF; shall be present if the SMSF supports MAP (see 3GPP TS 29.002 [21]) |
| smsfDiameterAddress | NetworkNodeDiameterAddress | C | 0..1 | shall be present if the SMSF supports Diameter (see 3GPP TS 29.338 [22]) |
| registrationTime | DateTime | C | 0..1 | Time of SmsfRegistration. Shall be present when used on Nudr. |
| contextInfo | ContextInfo | C | 0..1 | This IE if present may contain e.g. the headers received by the UDM along with the SmsfRegistration. Shall be absent on Nudm and may be present on Nudr.. |
| smsfSbiSupInd | boolean | C | 0..1 | SBI Support Indication of the SMSF. it shall be present if the SMSF support SBI-based MT SM transmit (see clauses 5.3.2.2.5 and 5.3.2.2.6). Contains the indication on whether or not the SMSF is expecting to support SBI-based MT SM transmit. - true: the SMSF supports SBI for MT SM transmit. - false, or absence of this attribute: the SMSF do not support SBI for MT SM transmit. |
| dataRestorationCallbackUri | Uri | O | 0..1 | If present, it contains the URI where notifications about UDR-initiated data restoration shall be sent by UDM. |
| resetIds | array(string) | O | 1..N | May be present in registration response messages. The SMSF may decide to re-register at the UDM when receiving a data restoration notification containing a matching resetId. |
| udrRestartInd | boolean | O | 0..1 | May be present in request messages from the SMSF to the UDM. If present: - true: indicates that the registration message sent by the SMSF is due to a re-synchronization event, motivated by a previous reception at the SMSF of a Data Restoration Notification from the UDM - false (or absent): indicates that this is a normal registration message (i.e., not motivated by a data restoration notification event) |
| lastSynchronizationTime | DateTime | O | 0..1 | This IE is only applicable to the Nudm API and shall not be used on the Nudr API. It may only be included when "udrRestartInd" attribute is present and set to true. When present, it contains the timestamp (previously stored by SMSF locally, after successful registration at UDM) when profiles in the SMSF and in UDM/UDR were synchronized. |
| ueMemoryAvailableInd | boolean | C | 0..1 | When the SMSF detects that the UE has memory available for SMS, the SMSF shall send the registration message with ueMemoryAvailableInd set to true. Otherwise shall be absent. When present and true, the UDM shall trigger Alert for SMS. Storing the ueMemoryAvailableInd in the UDR is not applicable. |

* * * Next Change * * * *

# A.3	Nudm_UECM API

openapi: 3.0.0

********text not shown for clarity******

SmsfRegistration:

type: object

required:

- smsfInstanceId

- plmnId

properties:

smsfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smsfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

smsfMAPAddress:

$ref: '#/components/schemas/E164Number'

smsfDiameterAddress:

$ref: '#/components/schemas/NetworkNodeDiameterAddress'

registrationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

contextInfo:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ContextInfo'

dataRestorationCallbackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

resetIds:

type: array

items:

type: string

minItems: 1

smsfSbiSupInd:

type: boolean

default: false

udrRestartInd:

type: boolean

default: false

lastSynchronizationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ueMemoryAvailableInd:

type: boolean

enum:

- true

********text not shown for clarity******

* * * End of Change * * * *