3GPP TSG-CT WG4 Meeting #114	C4-230205

# Athens, Greece, 27th Feb – 03rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.500 | CR | 0371 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NF Set ID encoding for an AMF set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The format of the NF Set ID for an AMF set defined in TS 23.003 is not aligned with the pattern of the NfSetId common data type (that is used by 70 APIs in Rel-16 and 127 APIs as of now in Rel-18) defined in TS 29.571 since the latter does not include the AMF "region" and the <Set ID> pattern does not allow the usage of the period ".". TS 29.571: NfSetId; string; NF Set Identifier (see clause 28.12 of 3GPP TS 23.003 [7]), formatted as the following string: " set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC>", or "set<SetID>.<NFType>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC>" with […] <Set ID> encoded as a string of characters consisting of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit. Pattern: '^([A-Za-z0-9\-]*[A-Za-z0-9])$' Examples: "setxyz.smfset.5gc.mnc012.mcc345" "set12.pcfset.5gc.mnc012.mcc345". NfSetId: type: string description: > NF Set Identifier (see clause 28.12 of 3GPP TS 23.003), formatted as the following string "set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC>", or "set<SetID>.<NFType>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC>" with […] <Set ID> encoded as a string of characters consisting of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit. Clause 28.12 of TS 23.003: A NF Set Identifier shall be formatted as the following string: set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC> for a NF Set in a PLMN, or set<Set ID>.<nftype>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC> for a NF Set in a SNPN. […] - the Set ID shall be a NF type specific Set ID within the PLMN, chosen by the operator, that shall consist of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit; - the case of alphabetic characters is not significant (i.e. two NF Set IDs with the same characters but using different lower and upper cases identify the same NF Set). For an AMF set, the Set ID shall be set to "<AMF Set ID>.region<AMF Region ID>", with the AMF Region ID and AMF Set ID encoded as defined in 3GPP TS 29.571 [129]. EXAMPLE 1: setxyz.smfset.5gc.mnc012.mcc345 EXAMPLE 2: set12.pcfset.5gc.mnc012.mcc345 EXAMPLE 3: set001.region48.amfset.5gc.mnc012.mcc345 (for AMF Region 48 (hexadecimal) and AMF Set 1) EXAMPLE 4: setxyz.smfset.5gc.nid000007ed9d5 Aligning the encoding of the NfSetId pattern of TS 29.571 (stage 3) on the format of NF Set ID for an AMF set defined in TS 23.003 (stage 2) would impact 70 (Rel-16) to 127 (Rel-18) APIs and could cause backward compatibility issues. It is thus proposed to align the format of the NF Set ID for an AMF set defined in TS 23.003 on the encoding of the NF Set ID pattern defined in TS 29.571 (that is also used by all other NF types). |
| Summary of change: |  | A few examples of 3GPP custom HTTP headers encoding an NF set ID for an AMF set are aligned with the corrections of CR 23.003 #0661, i.e. correcting "<AMF Set ID>-region<AMF Region ID>" to "<AMF Set ID>.region<AMF Region ID>". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between TS 29.571 and TS 23.003 for the encoding of NF Set ID for an AMF set. JSON attributes defined with the NfSetId data type of TS 29.571 cannot encode an AMF set, causing numerous functional restrictions, or if they do so, causing interoperability issues due to the encoding not complying with the pattern of the NfSetId data type. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.5, 5.2.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.003 CR 0661 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.3.2.5	3gpp-Sbi-Routing-Binding

This header contains a Routing Binding Indication used to direct a service request to an HTTP server which has the targeted NF service resource context (see clause 6.12).

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Routing-Binding = "3gpp-Sbi-Routing-Binding" ":" OWS "bl=" blvalue 1*(";" OWS parameter)

blvalue	= "nf-instance" / "nf-set" / "nfservice-instance" / "nfservice-set"

parameter	=  parametername "=" token

parametername	= "nfinst" / "nfset" / "nfservinst" / "nfserviceset" / "servname" / "backupamfinst" / "backupnf"

The following parameters are defined:

-	bl (binding level): the value of this parameter (blvalue) indicates a preferred binding to a binding entity, i.e. either to an NF Instance, an NF set, an NF Service Instance or an NF Service Set. If the binding level is set to an NF Service Instance (nfservice-instance), then either NF Service Set ID or NF Instance ID shall also be present to unambiguously identify the NF Service Instance.

-	nfinst (NF instance): indicates an NF Instance ID, as defined in clause 5.2.2.2.2 in 3GPP TS 29.510 [8]. This parameter shall be present if the binding level is set to "nf-instance", or if the binding level is set to "nfservice-instance" and the nfserviceset parameter is not included.

-	nfset (NF set): indicates an NF Set ID, as defined in clause 28.12 in 3GPP TS 23.003 [15]. This parameter shall be present if the binding level is set to "nf-set". It may be present otherwise (see clause 6.12.1).

-	nfservinst (NF service instance): indicates an NF Service Instance ID. This parameter shall be present if the binding level is set to "nfservice-instance".

-	nfserviceset (NF service set): indicates an NF Service Set ID as defined in clause 28.13 in 3GPP TS 23.003 [15]. This parameter shall be present if the binding level is set to "nfservice-set". It shall also be present if the binding level is set to "nfservice-instance" and the NF service instance indicated by the nfservinst parameter is part of an NF service set (see clause 6.12.1).

-	servname (service name): indicates the name of a service, as defined in 3GPP TS 29.510 [8], or a custom service that handles a notification or a callback request. It may be present in a Routing Binding Indication in a notification or a callback request.

-	backupamfinst (backup NF Instance): indicates the NF Instance ID (as defined in clause 5.2.2.2.2 in 3GPP TS 29.510 [8]) of the backup NF, i.e. the backup AMF as specified in 3GPP TS 23.501 [3]. The backupamfinst may be present only when the binding level is nf-instance or nfservice-instance or nfservice-set. When backupamfinst is present, no binding entity corresponding to NF set shall be present. When the binding level is nf-set, backupamfinst shall not be present.

-	for the definition and encoding of the backupnf see clause 5.2.3.2.6.

See clause 3.2.6 of IETF RFC 7230 [12] for the "token" type definition. A token's value is a string, which contains a binding entity ID or a service name.

EXAMPLE 1:	Binding to SMF set 1 of MCC 345 and MNC 012: 

3gpp-Sbi-Routing-Binding: bl=nf-set; nfset=set1.smfset.5gc.mnc012.mcc345

EXAMPLE 2:	Binding to an SMF instance within SMF set of Example 1: 

3gpp-Sbi-Routing-Binding: bl=nf-instance; nfinst=54804518-4191-46b3-955c-ac631f953ed8; nfset=set1.smfset.5gc.mnc012.mcc345

EXAMPLE 3:	Binding to a SMF Service Set "xyz" within an SMF instance within SMF set of Example 1:

3gpp-Sbi-Routing-Binding: bl=nfservice-set; nfserviceset=setxyz.snnsmf-pdusession.nfi54804518-4191-46b3-955c-ac631f953ed8.5gc.mnc012.mcc345; nfset=set1.smfset.5gc.mnc012.mcc345

EXAMPLE 4:	Binding to AMF set 1 within AMF region 48 (hexadecimal):
3gpp-Sbi-Routing-Binding: bl=nf-set; nfset=set1-region48.amfset.5gc.mnc012.mcc345

EXAMPLE 5:	Binding for a subscription (i.e. notification requests) to AMF set 1 within AMF region 48 (hexadecimal) and Namf_Communication service:
3gpp-Sbi-Routing-Binding: bl=nf-set; nfset= set1-region48.amfset.5gc.mnc012.mcc345; servname=namf-comm

EXAMPLE 6:	Binding to the AMF Instance in addition with backup AMF, where the nfinst carries the Identity of the AMF to which the resource is bound and whose backup AMF is indicated in backupamfinst:
3gpp-Sbi-Routing-Binding: bl=nf-instance; nfinst=54804518-4191-46b3-955c-ac631f953ed7; backupamfinst=54804518-4191-46b3-955c-ac631f953ed8

* * * Next Change * * * *

##### 5.2.3.2.6	3gpp-Sbi-Binding

This header contains a comma-delimited list of Binding Indications from an HTTP server for storage and subsequent use by an HTTP client (see clause 6.12).

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Binding = "3gpp-Sbi-Binding" ":" 1#(OWS "bl=" blvalue 1*(";" OWS parameter) [";" OWS recoverytime] [";" OWS notif-receiver] [";" OWS "group=" groupvalue] [1*(";" OWS groupparameter)] [";" OWS "no-redundancy=" no-red-value])

blvalue	= "nf-instance" / "nf-set" / "nfservice-instance" / "nfservice-set"

parameter	=  parametername "=" token

parametername	= "nfinst" / "nfset" / "nfservinst" / "nfserviceset" / "servname" / "scope" / "backupamfinst" / "backupnf"

recoverytime = "recoverytime=" OWS DQUOTE date-time DQUOTE

notif-receiver = "nr=" URI	; URI production rule from IETF RFC 3986 [14], Appendix A

groupvalue = "true" / "false"

groupparameter = groupparametername "=" token

groupparametername = "oldgroupid" / "groupid" / "uribase" / "oldnfinst / "oldservset" / "oldservinst" / "guami"

no-red-value = "true"

The following parameters are defined:

-	scope: indicates the applicability of a Binding Indication in a service request other than a notification request, or in a notification or callback response. This may take one of the following values:

-	"other-service": the binding information applies to other service(s) that the NF Service Consumer may later on provide as an NF Service Producer (see clause 6.12.3);

-	"subscription-events": the binding information applies to subscription change event notifications (see clause 6.12.4);

-	"callback": the binding information applies to notification or callback requests (see clauses 6.12.4 and 6.12.5).

The absence of this parameter in a Binding Indication in a service request other than a notification request, or in a notification or callback response, shall be interpreted as "callback".

Two scope parameters may be present in a Binding Indication if the binding information applies to notification/callback requests and to other services.

-	servname (service name): indicates the name of a service, as defined in 3GPP TS 29.510 [8], or a custom service, i.e.:

-	the name of the service that handles a notification or a callback request, when present in a Binding Indication for a subscription or a callback, i.e. with a scope parameter absent or set to "callback"; or

-	the name of the other service(s) for which the binding applies, when present in a Binding Indication in a service request for the other services the NF Service Consumer can provide later on as an NF Service Producer, i.e. with the scope parameter set to "other-service". More than one servname parameter may be present to represent multiple such services. The absence of this parameter in a Binding Indication with the scope parameter set to "other-service" shall be interpreted as binding information that applies to all the services that the NF Service Consumer may provide later as an NF Service Producer.

-	recoverytime: indicates the recovery timestamp of the entity corresponding to the highest resiliency level supported for the resource, that is, the higher level binding entity indicated in the Binding Indication. See Table 6.3.1.0-1 of 3GPP TS 23.501 [3] and clause 6.1 of 3GPP TS 23.527 [38]. The date-time type is specified in IETF RFC 5322 [37] and clause 7.1.1.1 of IETF RFC 7231 [11].

-	nr: indicates the URI of the notification endpoint when this binding information is applicable; it applies to callback requests (see clause 6.12.4); if the notification URI does not contain a correlationID in the path (i.e. it is a common notification URI for multiple subscriptions), the correlationID shall be added as a fragment component of the URI (i.e. following the "#" character) at the end of the URI.

-	for the definition and encoding of the blvalue, nfinst, backupamfinst, nfset, nfservinst and nfserviceset see clause 5.2.3.2.5.

-	backupnf: indicates the backup NF service instance identifier and/or the backup NF identifier as defined in clause 5.2.2.2.2 or in 3GPP TS 29.510 [8], which shall be used when preferred binding entity is not reachable if supported.

-	group: it is a boolean indicating if the binding indication is for a group of resource/session contexts.

-	groupid (group id): indicates the group identifier allocated by the NF (service) instance, one ore more resource/session contexts are sharing the same groupid. The groupid is optional and it may be allocated when the resource/session context is created and then be updated afterwards. The groupid is global unique and it may be encoded using the same mechnaism for the NfInstanceId as specificed in 3GPP TS 29.571 [13].

-	oldgroupid (old group id): indicates the group identifier allocated by the NF (service) instance previously and to be replaced by the groupid, hence it shall only be present when to update a Binding Indication for multiple contexts.  When the if the oldgroupid is present, the groupid shall also be present to indicate the new groupid allocated.

-	uribase: identify the apiroot and path segments part in the resource URI or notification/callback URI which is common to multiple contexts. This parameter may only be present when to update a Binding Indication for multiple contexts and when the "group" is set to "true". When included, it indicates that all resources or notification contexts with this uribase will use the updated Binding Indication subsequently. More than one uribase may be present.

-	oldnfinst: indicates the NF Instance ID of the NF instance where the group of resource/session contexts are currently served (i.e. the Binding Indication allocated previously for the group of resource/session contexts includes the information of the NF instance), as defined in clause 5.2.2.2.2 in 3GPP TS 29.510 [8]. When included, it indicates that all the resource/session contexts served by this NF instance will use the updated Binding Indication subsequently.

-	oldservset: indicates the NF Service Set ID of the NF Service Set where the group of resource/session contexts are currently served (i.e. the Binding Indication allocated previously for the group of resource/session contexts includes the information of the NF Service Set), as defined in clause 5.2.2.2.2 in 3GPP TS 29.510 [8]. When included, it indicates that all the resource/session contexts served by this NF Service Set will use the updated Binding Indication subsequently.

-	oldservinst: indicates the NF Service Instance ID of the NF service instance where the group of resource/session contexts are currently served (i.e. the Binding Indication allocated previously for the group of resource/session contexts includes the information of the NF service instance), as defined in clause 5.2.2.2.2 in 3GPP TS 29.510 [8]. When included, it indicates that all the resource/session contexts served by this NF service instance will use the updated Binding Indication subsequently.

-	guami (GUAMI): indicates the GUAMI of the AMF currently serving UE contexts, as defined in clause 5.3.4.1 of 3GPP TS 29.571 [13]. When included, it indicates that all the UE contexts associated with the GUMAI will use the updated Binding Indication subsequently.

-	no-redundancy: it is a boolean set to true indicating that the resource is exclusively bound to the NF service instance identified in the binding indication. It may be present in a binding with any scope, i.e. "other-service", "subscription-events" or "callback", or with no scope parameter. When this parameter is present, the blvalue shall be set to "nfservice-instance", the nfservinst parameter shall be present and either the nfservset parameter or the nfinst parameter shall be present. The nfservset or nfinst parameter included in the binding indication shall only be used to identify the NF service instance and shall not be considered as a binding entity for reselection. The no-redundancy parameter shall only be signaled if the receiver of this information is known to support this parameter (see clause 6.12.1). Subsequently, when sending further requests targeting a resource with no-redundancy, the HTTP client shall not include any routing binding indication in the request message (to prevent the SCP from performing any reselection).

EXAMPLES 1 to 5:	Same as EXAMPLES 1 to 5 defined in clause 5.2.3.2.5, with the header name "3gpp-Sbi-Binding" instead of "3gpp-Sbi-Routing-Binding".

EXAMPLE 6:	Subscription request from one NF on behalf of another NF, with 2 binding indications:

3gpp-Sbi-Binding: bl= nf-set; nfset=set1.udmset.5gc.mnc012.mcc345; servname=nudm-ee;scope=subscription-events
3gpp-Sbi-Binding: bl= nf-set; nfset=set1.nefset.5gc.mnc012.mcc345; servname=nnef-event-exposure

EXAMPLE 7:	Service request with 2 binding indications, for callback requests and for other services the NF Service Consumer may provide later as an NF Service Producer:

3gpp-Sbi-Binding: bl=nf-instance; nfinst=54804518-4191-46b3-955c-ac631f953ed8; nfset=set1.smfset.5gc.mnc012.mcc345; servname=nsmf-pdusession
3gpp-Sbi-Binding: bl=nf-instance; nfinst=54804518-4191-46b3-955c-ac631f953ed8; nfset=set1.smfset.5gc.mnc012.mcc345; scope=other-service; servname=nsmf-event-exposure

EXAMPLE 8:	Service request with one binding indication applying to notification/callback requests and to any other services the NF Service Consumer may provide later as an NF Service Producer:

3gpp-Sbi-Binding: bl=nf-set; nfset=set1-region48.amfset.5gc.mnc012.mcc345; scope=callback; scope=other-service

EXAMPLE 9:	Service request with one binding indication applying to notification/callback requests together with a recovery time stamp associated with the NF Set indicated in the binding indication and with the binding level set to "nfset":
3gpp-Sbi-Binding: bl=nfset; nfset=set1-region48.amfset.5gc.mnc012.mcc345; scope=callback; recoverytime= "Tue, 04 Feb 2020 08:49:37 GMT"

EXAMPLE 10:	Service response with one binding indication applying to the session context with a recovery time stamp associated with the NF Set indicated in "nfset" in the binding indication and with the binding level set to "nfinstance":

3gpp-Sbi-Binding: bl= nfinstance; nfinst=54804518-4191-46b3-955c-ac631f953ed8; nfset=set1.smfset.5gc.mnc012.mcc345; recoverytime= "Tue, 04 Feb 2020 08:49:37 GMT"

EXAMPLE 11:	Service response with one binding indication applying to the session context with a recovery time stamp associated with the NF Instance included the binding indication and with the binding level set to nfserviceinstance:

3gpp-Sbi-Binding: bl=nfserviceinstance; nfservinst=xyz; nfinst=54804518-4191-46b3-955c-ac631f953ed8; recoverytime= "Tue, 04 Feb 2020 08:49:37 GMT"

EXAMPLE 12:	Service response with one binding indication applying to the resource context pertaining to a group identified by "54804518-4191-46b3-955c-ac631f953ed1" together with a backup nf:

3gpp-Sbi-Binding: bl= nfinstance; nfinst=54804518-4191-46b3-955c-ac631f953ed0; nfset=set1.smfset.5gc.mnc012.mcc345; groupid=54804518-4191-46b3-955c-ac631f953ed1; backupnf=54804519-4191-46b3-955c-ac631f953ed2

EXAMPLE 13:	A notification request message with one binding indication applying to the resource contexts with the oldgroup identifier "54804518-4191-46b3-955c-ac631f953ed1", where the preferred binding entity is changed to "nfinst=54804519-4191-46b3-955c-ac631f953ed0" together with a new group identifier "54804519-4191-46b3-955c-ac631f953ed3" allocated.

3gpp-Sbi-Binding: bl= nfinstance; nfinst=54804519-4191-46b3-955c-ac631f953ed0; nfset=set1.smfset.5gc.mnc012.mcc345; group=true; oldgroupid=54804518-4191-46b3-955c-ac631f953ed1; groupid=54804519-4191-46b3-955c-ac631f953ed3

EXAMPLE 14:	A notification request message with one binding indication applying to the resource contexts identified by an uribase, where the preferred binding entity is changed to "nfinst=54804519-4191-46b3-955c-ac631f953ed0":

3gpp-Sbi-Binding: bl= nfinstance; nfinst=54804519-4191-46b3-955c-ac631f953ed0; nfset=set1.smfset.5gc.mnc012.mcc345; group=true; uribase= http%3A%2F%2F10.10.10.10%2Fstringxyz

EXAMPLE 15:	A notification request message with one binding indication applying to the resource contexts served by the NF instance identified by "64804518-4191-46b3-955c-ac631f953ed8" where the preferred binding entity is changed to "nfinst=74804519-4191-46b3-955c-ac631f953ed0".

3gpp-Sbi-Binding: bl= nfinstance; nfinst=74804519-4191-46b3-955c-ac631f953ed0; nfset=set1.smfset.5gc.mnc012.mcc345; group=true; oldnfinst=64804518-4191-46b3-955c-ac631f953ed8

EXAMPLE 16:	Service request message with an updated binding indication applying to the UE contexts for GUAMI" <mnc(012)><mcc(345)><AmfId("abcd12")> where the backupamfinst is changed.

3gpp-Sbi-Binding: bl=nf-instance; nfinst=54804518-4191-46b3-955c-ac631f953ed7; backupamfinst=54804520-4191-46b3-955c-ac631f953ed8; scope=other-service; group=true; guami={"plmnId":{"mnc":"012","mcc":"345"},"amfId":"abcd12"}

EXAMPLE 17:	Service response with a binding indication applying to the resource context which is exclusively bound to a specific NF service instance.

3gpp-Sbi-Binding: bl=nfserviceinstance; nfservinst=xyz; nfinst=54804518-4191-46b3-955c-ac631f953ed8; no-redundancy= true

NOTE:	Examples 6 and 7 are formatted as two distinct headers (which improves the readability), but they can also be formatted as a single header with two Binding Indication values separated by a comma.

* * * End of Changes * * * *