3GPP TSG-CT Meeting #99	CP-230190

# Rotterdam, Netherlands, 20th – 21st March 2023

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	CT aspects of Gateway UE function for Mission Critical Communication

Acronym:	MCGWUE

Unique identifier:	990011

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| x | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| MCGWUE | SA6 | 930017 | Gateway UE function for Mission Critical Communication |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

# 3	Justification

SA6 WI MCGWUE has reached 100% and hence stage 3 can now be specified. This work covers the necessary procedures for an MC gateway UE to provide service access with the MC service system for multiple MC service clients operating on devices that have no MC UE capabilities and corresponding UICC capabilities.

# 4	Objective

The objective is to update the mission critical specifications to enable the MC gateway UE to support:

1)	Authorization  and disconnection procedures for the connections between MCgateway client and the MC service server via the MC GW UE

2)	Location management procedures

3)	MBMS support procedures

4)	Configuration supporting the procedures

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 24.281 | Procedures for connectivity, location and MBMS | CT#103 |  |
| 24.282 | Procedures for connectivity, location and MBMS | CT#103 |  |
| 24.379 | Procedures for connectivity, location and MBMS | CT#103 |  |
| 24.380 | Possible impacts from MBMS and location reporting | CT#103 |  |
| 24.483 | Configuration on the UE | CT#103 |  |
| 24.484 | Configuration for the MC GW UE | CT#103 |  |
| 24.581 | Possible impacts from MBMS and location reporting | CT#103 |  |
| 24.582 | Possible impacts from MBMS and location reporting | CT#103 |  |

# 6	Work item Rapporteur(s)

Axell, Jörgen, Ericsson, Jorgen DOT Axell AT ericsson DOT com

# 7	Work item leadership

CT1

# 8	Aspects that involve other WGs

None identified

# 9	Supporting Individual Members

Supporting IM name

Ericsson

Airbus

AT&T

BDBOS

FirstNet

Huawei

Motorola Solutions

Nokia

Nokia Shanghai Bell

Samsung

Sepura Ltd

UIC