3GPP TSG-CT Meeting #99	CP-230279

# Rotterdam, Netherlands, 20th – 21st March 2023							(Revision of C3-230848)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0819 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS group delivery procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-03-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS Group Message Delivery to deliver a payload to a group of UEs located in a particular geographical area when MBS broadcast is used is defined in TS 23.247. The corresponding implementation in 29.522 needs to be carried out according to the SA2#154AHe agreed TS 23.247 CR 0161 and TS 23.501 CR 3755. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedure descriptions for the group message delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Group message delivery procedures are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.29.7 (new), 4.4.29.7.1(new), 4.4.29.7.2 (new), 4.4.29.7.3 (new). 4.4.29.7.4 (new). 4.4.29.7.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.247 CR 0161 TS 23.501 CR 3755 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of C3-230848: - C3-230848 was based on the wrong version of the revised C3-230414. Revised to C3-230414r1 contents which is the version agreed between companies during the CT3#126 meeting. |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.4.29.7	Procedures for MBS Group Message Delivery Management

##### 4.4.29.7.1	General

The procedures described in the clauses below are used by an AF to request and manage MBS Group Message Delivery as defined in clauses 6.15 and 7.5 of 3GPP TS 23.247 [53].

Editor's Note:	The service description procedures in the below clauses are still FFS and will be updated based on the progress of the related stage 2 requirements.

##### 4.4.29.7.2	Procedure for MBS Group Message Delivery Creation

This procedure is used by an AF to request MBS Group Message Delivery.

In order to request MBS Group Message Delivery, an AF shall send a Nnef_MBSGroupMsgDelivery_Create request message to the NEF using the HTTP POST method, with the Request body including the MbsGroupMsgDelReq data structure that shall contain:

-	within the "mbsGroupMsgDelData" attribute, the group message delivery data; and

-	within the "suppFeat" attribute, the list of supported features, if feature negotiation needs to take place.

The NEF shall then check whether the AF is authorized to perform this operation or not, as defined in clause 6.1.1 of 3GPP TS 23.247 [53]. If the AF is authorized, the NEF performs MBS User Service and MBS User Data Ingest Session provisioning towards the MBSF using the Object Distribution Method, as specified in 3GPP TS 26.502 [65] and 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall send a Nnef_MBSGroupMsgDelivery_Create response to the AF with an HTTP "201 Created" status code and the response body including the MbsGroupMsgDelResp data structure that shall contain:

-	within the "mbsGroupMsgDelData" attribute, the representation of the created "Individual MBS Group Message Delivery" resource;

-	within the "delStatus" attribute, the status of the Group Message Delivery; and

-	within the "suppFeat" attribute, the list of supported features, if feature negotiation needs to take place.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.29.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.7.3	Procedure for MBS Group Message Delivery Update

This procedure is used by an AF to request the modification of an existing "Individual MBS Group Message Delivery" resource at the NEF.

In order to modify a previously submitted group message delivery, an AF shall send a Nnef_MBSGroupMsgDelivery_Update request message to the NEF using the HTTP PATCH method with the request body including the MbsGroupMsgDelPatch data structure.

The NEF shall then check whether the AF is authorized to perform this operation or not, as defined in clause 6.1.1 of 3GPP TS 23.247 [53]. If the AF is authorized the NEF may further interact with the MBSF to request the modification of the associated MBS User Service and MBS User Data Ingest Session instances.

Upon reception of a successful response from the MBSF confirming the modification is successfully performed, the NEF shall send a Nnef_MBSGroupMsgDelivery_Update response to the AF, with an HTTP "200 OK" containing the the MbsGroupMsgDelResp data structure, or an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.29.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.7.4	Procedure for MBS Group Message Delivery Deletion

This procedure is used by an AF to request the deletion of an existing "Individual MBS Group Message Delivery" resource at the NEF.

In order to delete a previously submitted group message delivery, an AF shall send a Nnef_MBSGroupMsgDelivery_DELETE request message to the NEF using the HTTP DELETE.

The NEF shall then check whether the AF is authorized to perform this operation or not, as defined in clause 6.1.1 of 3GPP TS 23.247 [53].

Upon reception of a successful response from the MBSF, the NEF shall send a Nnef_MBSGroupMsgDelivery_DELETE response to the AF, with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.29.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.7.5	Procedure for MBS Group Message Delivery Status Notification

This procedure is used by the NEF to notify an already subscribed AF of the status for the previously submitted group message delivery.

In order to notify an AF of the status for the previously submitted group message delivery, the NEF shall send a Nnef_MBSGroupMsgDelivery_StatusNotify request message to the AF using the HTTP POST method with the request body including the MbsGroupMsgDelStatusNotif data structure.

Upon reception of this notification request, the AF shall acknowledge its successful reception by sending a Nnef_MBSGroupMsgDelivery_StatusNotify response message with an HTTP "204 No Content" status code.

On failure, the AF shall take proper error handling actions, as specified in clause 5.7.7, and respond to the NEF with an appropriate error status code.

*** End of Changes ***