3GPP TSG-CT WG1 Meeting #140	C1-230280

# Athens, Greece, 27 February – 3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.526 | CR | 0169 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting reference and text |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc18 |  |  |  |  |  | Date: |  |  | 2023-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The refernce for the E-UTRAN Cell Global Identifier should be TS 36.413 and not TS 36.331. It is not clarified how the E-UTRAN cell identity (28 bits) and the NR cell identity (36 bits) are represented as a number octets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the reference. Added explanation how to zero- pad the E-UTRAN cell identity (28 bits) and the NR cell identity (36 bits) in order to represent them as a number of octets. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong reference and the unexplained text remain in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************ Next Change ************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.503: " Policy and Charging Control Framework for the 5G System; Stage 2".

[3]	3GPP TS 24.502: "Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN); Stage 3".

[4]	3GPP TS 23.003: "Numbering, addressing and identification".

[5]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[6]	Void

[6A]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access (E-UTRA); S1 Application Protocol (S1AP)".

[7]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[8]	IEEE Std 802.11™-2016: "Information Technology- Telecommunications and information exchange between systems-Local and metropolitan area networks-Specific requirements-Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[9]	Wi-Fi Alliance: "Hotspot 2.0 (Release 2) Technical Specification, version 1.0.0", 2014-08-08.

[10]	ITU-T Recommendation E.212: "The international identification plan for public networks and subscriptions", 2016-09-23.

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	IETF RFC 1035: "Domain names - implementation and specification".

[13]	ISO 8601:2004: "Data elements and interchange formats -- Information interchange -- Representation of dates and times".

[14]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[15]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[16]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[17]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[18]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[18A]	3GPP TS 24.555: "Proximity-services (ProSe) in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[19]	IEEE 1003.1-2004, Part 1: Base Definitions.

[20]	IEEE Std 802.1Q-2018: "IEEE Standard for Local and metropolitan area networks--Bridges and Bridged Networks".

[21]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[22]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting; Stage 3".

************************ Next Change ************************

### 5.3.2	Encoding of WLANSP

The purpose of the WLANSP field is to indicate the rules related to selection and reselection of a WLAN.

The WLANSP field is encoded as shown in figures 5.3.2.1 to 5.3.2.20 and table 5.3.2.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | ANDSP Info type={WLANSP} |  |  |  | octet 1 |
| Spare |  |  |  |  |  |  |  |  |
| Length of ANDSP info contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| ANDSP info contents={WLANSP contents } |  |  |  |  |  |  |  | octet 4 octet x |

Figure 5.3.2.1: ANDSP Info = {WLANSP}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WLANSP rule 1 |  |  |  |  |  |  |  | octet 4 octet u |
| WLANSP rule 2 |  |  |  |  |  |  |  | octet u+1* octet v* |
| … |  |  |  |  |  |  |  | octet v+1* octet w* |
| WLANSP rule n |  |  |  |  |  |  |  | octet w+1* octet x* |

Figure 5.3.2.2: WLANSP contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of WLANSP rule |  |  |  |  |  |  |  | octet 4 octet 5 |
| Rule identifier |  |  |  |  |  |  |  | octet 6 |
| Rule priority |  |  |  |  |  |  |  | octet 7 |
| Roaming | validity area ind | 3GPP loc ind | WLAN loc ind | Geo loc ind | time of day ind | 0 spare | 0 spare | octet 8 |
| Selection criteria |  |  |  |  |  |  |  | octet 9 octet r |
| Validity area |  |  |  |  |  |  |  | octet r+1* octet s* |
| Time of day |  |  |  |  |  |  |  | octet s+1* octet u* |

Figure 5.3.2.3: WLANSP rule

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of selection criteria |  |  |  |  |  |  |  | octet 9 octet 10 |
| number of selection criteria entries |  |  |  |  |  |  |  | octet 11 |
| Selection criteria entry 1 |  |  |  |  |  |  |  | octet 12 octet a |
| Selection criteria entry 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet c* |
| Selection criteria entry n |  |  |  |  |  |  |  | octet c+1* octet r* |

Figure 5.3.2.4: Selection criteria

| 8 | 7 |  | 6 |  | 5 |  | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of selection criteria entry |  |  |  |  |  |  |  |  |  |  | octet 12 octet 13 |
| Spare |  | MaxBSSload ind |  | Home network ind |  | Criteria priority |  |  |  |  | octet 14 |
| Maximum BSS load value |  |  |  |  |  |  |  |  |  |  | octet 15 octet 16 |
| Selection criteria set 1 |  |  |  |  |  |  |  |  |  |  | octet 17 octet t* |
| … |  |  |  |  |  |  |  |  |  |  | octet t+1* octet y* |
| Selection criteria set n (n <= 5) |  |  |  |  |  |  |  |  |  |  | octet y+1* octet a* |

Figure 5.3.2.4a: Selection criteria entry

| 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of selection criteria set |  |  |  |  |  |  |  |  | octet 17 octet 18 |
| Selection criteria set type {preferred SSID list, preferred roaming partner list, required protocol port tuple, SP exclusion list, minimum backhaul threshold} |  |  |  |  | Number of sub entries |  |  |  | octet 19 |
| Sub entry 1 |  |  |  |  |  |  |  |  | octet 20 octet aa |
| … |  |  |  |  |  |  |  |  | octet aa+1 octet bb |
| Sub entry n |  |  |  |  |  |  |  |  | octet cc+1 octet dd |

Figure 5.3.2.4b: Selection criteria set

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of sub entry {set type = preferred SSID list} |  |  |  |  |  |  |  |  |  | octet 20 |
| WLAN priority |  |  |  |  |  |  |  |  |  | octet 21 |
| 0 Spare |  |  |  |  |  |  | HESSID ind | SSID ind |  | octet 22 |
| SSID length |  |  |  |  |  |  |  |  |  | octet 23* |
| SSID |  |  |  |  |  |  |  |  |  | octet 24* octet ee* |
| HESSID |  |  |  |  |  |  |  |  |  | octet ee+1* octet ee+6* |

Figure 5.3.2.4c: Selection criteria sub entry {selection criteria set type = preferred SSID list}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of sub entry {set type = preferred roaming partner list} |  |  |  |  |  |  |  | octet 20 |
| Priority |  |  |  |  |  |  |  | octet 21 |
| FQDN_Match length |  |  |  |  |  |  |  | octet 22 |
| FQDN_Match |  |  |  |  |  |  |  | octet 23 octet ee* |
| Country length |  |  |  |  |  |  |  | octet ee+1 |
| Country |  |  |  |  |  |  |  | octet ee+2 octet ff* |

Figure 5.3.2.4d: Selection criteria sub entry {selection criteria set type = preferred roaming partner list}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of sub entry {set type = required protocol port tuple} |  |  |  |  |  |  |  | octet 20 |
| IP protocol |  |  |  |  |  |  |  | octet 21 |
| Length of port number |  |  |  |  |  |  |  | octet 22 |
| Port number |  |  |  |  |  |  |  | octet 23 octet ff |

Figure 5.3.2.4e: Selection criteria sub entry {selection criteria set type = required protocol port tuple}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of sub entry {set type = SP exclusion list} |  |  |  |  |  |  |  | octet 20 |
| SSID |  |  |  |  |  |  |  | octet 21 octet ff* |

Figure 5.3.2.4f: Selection criteria sub entry {selection criteria set type = SP exclusion list}

| 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spare |  |  |  |  | ULBI |  | DLBI |  | Network type |  | octet 20 |
| Downlink bandwidth |  |  |  |  |  |  |  |  |  |  | octet 21 octet 24 |
| Uplink bandwidth |  |  |  |  |  |  |  |  |  |  | octet 25 octet 28 |

Figure 5.3.2.4g: Selection criteria sub entry {selection criteria set type = minimum backhaul threshold}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| length of validity area |  |  |  |  |  |  |  | octet r+1 octet r+2 |
| number of location entries |  |  |  |  |  |  |  | octet r+3 |
| location entry 1 |  |  |  |  |  |  |  | octet r+4 octet d |
| …. |  |  |  |  |  |  |  | octet d+1* octet e* |
| location entry m |  |  |  |  |  |  |  | octet e+1* octet s* |

Figure 5.3.2.5: Validity area

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of location entry |  |  |  |  |  |  |  | octet r+4* octet r+5* |
| entry type {3GPP, WLAN, Geo} |  | number of sub entries |  |  |  |  |  | octet r+6* |
| sub entry contents |  |  |  |  |  |  |  | octet r+7* octet d* |

Figure 5.3.2.6: Location entry

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of location entry |  |  |  |  |  |  |  | octet r+4* octet r+5* |
| entry type= {3GPP location} |  | number of sub entries |  |  |  |  |  | octet r+6* |
| 3GPP location sub entry 1 |  |  |  |  |  |  |  | octet r+7 octet f |
| 3GPP location sub entry 2 |  |  |  |  |  |  |  | octet f+1* octet g* |
| … |  |  |  |  |  |  |  | octet g+1* octet h* |
| 3GPP location sub entry o |  |  |  |  |  |  |  | octet h+1* octet d* |

Figure 5.3.2.7: Location entry {entry type =3GPP location}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of location entry |  |  |  |  |  |  |  | octet r+4* octet r+5* |
| entry type= {WLAN location } |  | number of sub entries |  |  |  |  |  | octet r+6* |
| WLAN location sub entry 1 |  |  |  |  |  |  |  | octet r+7 octet f |
| WLAN location sub entry 2 |  |  |  |  |  |  |  | octet f+1* octet g* |
| … |  |  |  |  |  |  |  | octet g+1* octet h* |
| WLAN location sub entry p |  |  |  |  |  |  |  | octet h+1* octet d* |

Figure 5.3.2.8: Location entry {entry type =WLAN location}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of location entry |  |  |  |  |  |  |  | octet r+4* octet r+5* |
| entry type= {Geo location } |  | number of sub entries |  |  |  |  |  | octet r+6* |
| Geo location sub entry 1 |  |  |  |  |  |  |  | octet r+7 octet f |
| Geo location sub entry 2 |  |  |  |  |  |  |  | octet f+1* octet g* |
| … |  |  |  |  |  |  |  | octet g+1* octet h* |
| Geo location sub entry q |  |  |  |  |  |  |  | octet h+1* octet d* |

Figure 5.3.2.9: Location entry {entry type =Geo location}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of 3GPP location sub entry |  |  |  |  |  |  |  | octet r+7 octet r+8 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet r+9 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet r+10 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet r+11 |
| number of location fields |  |  |  |  |  |  |  | octet r+12* |
| 3GPP location field 1 |  |  |  |  |  |  |  | octet r+13* octet l* |
| … |  |  |  |  |  |  |  | octet l+1* octet m* |
| 3GPP location field n |  |  |  |  |  |  |  | octet m+1* octet f* |

Figure 5.3.2.10: Location sub entry {entry type= 3GPP location}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of location sub entry |  |  |  |  |  |  |  | octet r+7 octet r+8 |
| number of location fields |  |  |  |  |  |  |  | octet r+9* |
| WLAN or Geo location field 1 |  |  |  |  |  |  |  | octet r+10* octet l* |
| … |  |  |  |  |  |  |  | octet l+1* octet m* |
| WLAN or Geo location field n |  |  |  |  |  |  |  | octet m+1* octet f* |

Figure 5.3.2.10a: Location sub entry {entry type= WLAN location or Geo location}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor latitude |  |  |  |  |  |  |  | octet r+10* octet r+13* |
| Anchor longitude |  |  |  |  |  |  |  | octet r+14* octet r+17* |
| Radius |  |  |  |  |  |  |  | octet r+18* octet r+19* |

Figure 5.3.2.11a: Location field {entry type= Geo location}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of 3GPP location field |  |  |  |  |  |  |  | octet r+13* |
| 3GPP location field type |  |  |  |  |  |  |  | octet r+14 |
| 3GPP location field contents |  |  |  |  |  |  |  | octet r+15* octet l* |

Figure 5.3.2.11b: Location field {entry type= 3GPP location}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of WLAN location field |  |  |  |  |  |  |  | octet r+10 |
| WLAN location field type |  |  |  |  |  |  |  | octet r+11 |
| WLAN location field contents |  |  |  |  |  |  |  | octet r+12* octet l* |

Figure 5.3.2.11c: Location field {entry type= WLAN location}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of 3GPP location field |  |  |  |  |  |  |  | octet r+13* |
| field type = {TAC} |  |  |  |  |  |  |  | octet r+14 |
| TAC |  |  |  |  |  |  |  | octet r+15 |

Figure 5.3.2.12: 3GPP location field {field type = TAC}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of 3GPP location field |  |  |  |  |  |  |  | octet r+13* |
| field type = {EUTRA CI} |  |  |  |  |  |  |  | octet r+14 |
| EUTRA CI |  |  |  |  |  |  |  | octet r+15 octet r+16 |

Figure 5.3.2.13: 3GPP location field {field type = EUTRA CI}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of 3GPP location field |  |  |  |  |  |  |  | octet r+13* |
| field type = {NR CI} |  |  |  |  |  |  |  | octet r+14 |
| NR CI |  |  |  |  |  |  |  | octet r+15 octet r+17 |

Figure 5.3.2.14: 3GPP location field {field type = NR CI}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of WLAN location field |  |  |  |  |  |  |  | octet r+10* |
| field type = {HESSID} |  |  |  |  |  |  |  | octet r+11 |
| HESSID |  |  |  |  |  |  |  | octet r+12 octet r+17 |

Figure 5.3.2.14a: WLAN location field {field type = HESSID}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of WLAN location field |  |  |  |  |  |  |  | octet r+10* |
| field type = {SSID} |  |  |  |  |  |  |  | octet r+11 |
| SSID |  |  |  |  |  |  |  | octet r+12 octet l* |

Figure 5.3.2.14b: WLAN location field {field type = SSID}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of WLAN location field |  |  |  |  |  |  |  | octet r+10* |
| field type = {BSSID} |  |  |  |  |  |  |  | octet r+11 |
| BSSID |  |  |  |  |  |  |  | octet r+12 octet r+17 |

Figure 5.3.2.14c: WLAN location field {field type = BSSID}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of time of day |  |  |  |  |  |  |  | octet s+1 octet s+2 |
| number of time of day entries |  |  |  |  |  |  |  | octet s+3 |
| Time of day entry 1 |  |  |  |  |  |  |  | octet s+4 octet n1 |
| Time of day entry 2 |  |  |  |  |  |  |  | octet n1+1* octet n2* |
| … |  |  |  |  |  |  |  | octet n2+1* octet n3* |
| Time of day entry n |  |  |  |  |  |  |  | octet n3+1* octet u* |

Figure 5.3.2.15: Time of day

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of time of day entry |  |  |  |  |  |  |  | octet s+4 octet s+5 |
| number of sub fields |  |  |  |  |  |  |  | octet s+6* |
| ToD sub field 1 |  |  |  |  |  |  |  | octet s+7 octet z1 |
| ToD sub field 2 |  |  |  |  |  |  |  | octet z1+1* octet z2* |
| … |  |  |  |  |  |  |  | octet z2+1* octet z3* |
| ToD sub field y |  |  |  |  |  |  |  | octet z3+1* octet n1* |

Figure 5.3.2.16: Time of day sub field

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ToD sub field |  |  |  |  |  |  |  | octet s+7* |
| ToD sub field type |  |  |  |  |  |  |  | octet s+8* |
| ToD sub field contents |  |  |  |  |  |  |  | octet s+9 octet f |

Figure 5.3.2.17: ToD sub field

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ToD sub field |  |  |  |  |  |  |  | octet s+7* |
| ToD sub field type ={time start, time stop} |  |  |  |  |  |  |  | octet s+8* |
| ToD sub field contents |  |  |  |  |  |  |  | octet s+9 octet f |

Figure 5.3.2.18: ToD sub field {field type = "time start" or "time stop"}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ToD sub field |  |  |  |  |  |  |  | octet s+7* |
| ToD sub field type ={ date start, date stop } |  |  |  |  |  |  |  | octet s+8* |
| ToD sub field contents |  |  |  |  |  |  |  | octet s+9 octet f |

Figure 5.3.2.19: ToD sub field {field type = "date start" or "date stop"}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ToD sub field |  |  |  |  |  |  |  | octet s+7* |
| ToD sub field type ={ day of week} |  |  |  |  |  |  |  | octet s+8* |
| 1 | Mon | Tue | Wed | Thu | Fri | Sat | Sun | octet s+9 |

Figure 5.3.2.20: ToD sub field {field type = "day of the week"}

Table 5.3.2.1: WLANSP information element

| Value part of the WLANSP information element (octets 4 to x) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANDSP Info type (bit 1 to 4 of octet 1) shall be set to "0001" (WLANSP) |  |  |  |  |  |  |
| Bits 8 to 5 of octet 1 are spare and shall be encoded as zero. |  |  |  |  |  |  |
| Length of WLANSP contents (octets 2 to 3) |  |  |  |  |  |  |
| Length of WLANSP rule (octets 4 to 5) |  |  |  |  |  |  |
| Rule Identifier (octet 6) |  |  |  |  |  |  |
| This field contains the binary encoding of the WLANSP rule identifier |  |  |  |  |  |  |
| Rule priority (octet 7) |  |  |  |  |  |  |
| This field contains the binary encoding of the WLANSP rule priority |  |  |  |  |  |  |
| Spare bits and shall be encoded as zero (bits 1 to 2 of octet 8) |  |  |  |  |  |  |
| Time of day index (bit 3 of octet 8) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 3. 0; WLANSP rule does not include time of day information. 1; WLANSP rule includes time of day information. |
| Geo location index (bit 4 of octet 8) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 4. 0; WLANSP rule does not include Geo location information. 1; WLANSP rule includes Geo location information. |
| WLAN location index (bit 5 of octet 8) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 5. 0; WLANSP rule is not for WLAN location. 1; WLANSP rule is for WLAN location. |
| 3GPP location index (bit 6 of octet 8) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 6. 0; WLANSP rule is not for 3GPP location. 1; WLANSP rule is for 3GPP location. |
| Validity area index (bit 7 of octet 8) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 7. 0; WLANSP rule is not for validity area. 1; WLANSP rule is for validity area. |
| Roaming (bit 8 of octet 8) (NOTE 1) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 0 |  |  |  |  | WLANSP rule is only valid when the UE is not roaming |
| 1 |  |  |  |  | WLANSP rule is only valid when the UE is roaming |
| Selection criteria (octets 9 to r) |  |  |  |  |  |  |
| This field contains the binary encoding of the selection criteria for a particular WLANSP rule. |  |  |  |  |  |  |  |  |
| Selection criteria entry (octets 12 to a) (NOTE 2) Length of selection criteria entry (octets 12 to 13) indicates length of subsequent fields in the selection criteria entry. Criteria priority (bits 1-5 of octet 14): the lower value indicates the selection criteria having the higher priority among the selection criteria in the WLANSP rule. Home network ind (bit 6 of octet 14): (NOTE 3) Bit 6 0 all WLANs could match this selection criteria entry. 1 only the WLANs that are operated by the home operator could match this selection criteria entry. MaxBSSload ind (bit 7 of octet 14): Bit 7 0 maximum BSS load value (octets 15 to 16) not present 1 maximum BSS load value (octets 15 to 16) present Maximum BSS load value (octets 15 to 16) is as the node PerProviderSubscription/<X+>/Policy/MaximumBSSLoadValue defined in Hotspot 2.0 (Release 2) Technical Specification [9]. |  |  |  |  |  |  |  |
| Selection criteria set (octets 17 to dd) contains the contents of a specific criteria set. In this release of specification there can be 5 types of criteria sets. Selection criteria set type (bits 5-8 of octet 19) is coded as follows. Bits 8 7 6 5 0 0 0 1 preferred SSID list (NOTE 4) 0 0 1 0 preferred roaming partner list (NOTE 5) 0 0 1 1 required protocol port tuple 0 1 0 0 SP exclusion list 0 1 0 1 minimum backhaul threshold All other values are reserved. |  |  |  |  |  |  |  |
| Selection criteria sub entry (octets 20 to ee+6) when set type is "preferred SSID list" is coded as follows. Length of sub entry (octet 20) indicates length of subsequent fields in the selection criteria sub entry. WLAN priority (octet 21): the lower WLAN priority value indicates the WLAN having the higher priority among the WLANs in the preferred SSID list. SSID ind (bit 1 of octet 22): Bit 5 0 SSID field (octets 24 to ee) is not present. 1 SSID field (octets 24 to ee) is present. HESSID ind (bit 2 of octet 22): Bit 6 0 HESSID field (octets ee+1 to ee+6) is not present. 1 HESSID field (octet ee+1 to ee+6) is present. SSID length (octet 23) indicates the length of the SSID field. SSID field (octets 24 to ee) is an Octet String which shall have a maximum length of 32 octets (see IEEE Std 802.11 [8]). HESSID field (octets ee+1 to ee+6) is a 6 octet MAC address that identifies the homogeneous ESS (see IEEE Std 802.11 [8]). |  |  |  |  |  |  |  |
| Selection criteria sub entry (octets 20 to ff) when set type is "preferred roaming partner list" is coded as follows. Length of sub entry (octet 20) indicates length of subsequent fields in the selection criteria sub entry. Priority (octet 21): the lower priority value indicates the higher priority in the preferred roaming partner list. FQDN_Match length (octet 22) indicates the length of the FQDN_Match field. FQDN_Match field (octets 23 to ee) is as the node PerProviderSubscription/<X+>/Policy/PreferredRoamingPartnerList/<X+>/FQDN_Match defined in Hotspot 2.0 (Release 2) Technical Specification [9]. Country length (octet ee+1) indicates the length of the country field. Country field (octets ee+2 to ff) is as the node PerProviderSubscription/<X+>/Policy/PreferredRoamingPartnerList/<X+>/Country defined in Hotspot 2.0 (Release 2) Technical Specification [9]. Selection criteria sub entry (octets 20 to ff) when set type is "required protocol port tuple" is coded as follows. Length of sub entry (octet 20) indicates length of subsequent fields in the selection criteria sub entry. IP protocol field (octet 21) shall be present in the sub entry and refers to IP protocol field in IPv4 packets or the next header field in IPv6 packets. It is required by operator-supported application(s) on UE as specified in Hotspot 2.0 (Release 2) Technical Specification [9]. Length of port number (octet 22) indicates the length of port number field. Port number field (octets 23 to ff) is as the node PerProviderSubscription/<X+>/Policy/RequiredProtoPortTuple/<X+>/PortNumber defined in Hotspot 2.0 (Release 2) Technical Specification [9]. |  |  |  |  |  |  |  |
| Selection criteria sub entry (octets 20 to ff) when set type is "SP exclusion list" is coded as follows. Length of sub entry (octet 20) indicates length of subsequent fields in the selection criteria sub entry, i.e. the length of SSID field. SSID field (octets 21 to ff) is as the node PerProviderSubscription/<X+>/Policy/SPExclusionList/<X+>SSID defined in Hotspot 2.0 (Release 2) Technical Specification [9]. |  |  |  |  |  |  |  |
| Selection criteria sub entry (octets 20 to 28) when set type is "minmum backhaul threshold" is coded as follows. Network type (bit 1-2 of octet 20) is coded as follows according to the definition of the node PerProviderSubscription/<X+>/Policy/MinBackhaulThreshold/<X+>/NetworkType in Hotspot 2.0 (Release 2) Technical Specification [9]. Bits 2 1 0 0 home 0 1 roaming All other values are reserved. DLBI (bit 3 of octet 20): Bit 3 0 Downlink bandwidth field (octets 21 to 24) is not present. 1 Downlink bandwidth field (octets 21 to 24) is present. ULBI (bit 4 of octet 20): Bit 4 0 Uplink bandwidth field (octets 25 to 28) is not present. 1 Uplink bandwidth field (octets 25 to 28) is present. |  |  |  |  |  |  |  |
| Downlink bandwidth field (octets 21 to 24) is as the node PerProviderSubscription/<X+>/Policy/MinBackhaulThreshold/<X+>/DLBandwidth defined in Hotspot 2.0 (Release 2) Technical Specification [9]. Uplink bandwidth field (octets 25 to 28) is as the node PerProviderSubscription/<X+>/Policy/MinBackhaulThreshold/<X+>/ULBandwidth defined in Hotspot 2.0 (Release 2) Technical Specification [9]. |  |  |  |  |  |  |  |
| Validity area (octets r+1 to s) |  |  |  |  |  |  |
| This field contains the binary encoding of the validity area for a particular WLANSP rule. |  |  |  |  |  |  |  |  |
| Entry type (bits 7-8 of octet r+6) is coded as follows: Bits 8 7 0 1 3GPP location 1 0 WLAN location 1 1 Geo location All other values are reserved. |  |  |  |  |  |  |  |
| Length of 3GPP location sub entry (octets r+7 to r+8) |  |  |  |  |  |  |  |  |
| This field contains the length of the location entry when the WLANSP rule is for validity area of a 3GPP location. |  |  |  |  |  |  |  |  |
| MCC, Mobile country code (octet r+9, and bits 4 to 1 of octet r+10) |  |  |  |  |  |  |  |  |
| The MCC field is coded as in ITU-T Recommendation E.212 [10], annex A. |  |  |  |  |  |  |  |  |
| MNC, Mobile network code (bits 8 to 5 of octet r+10, and octet r+11) |  |  |  |  |  |  |  |  |
| The encoding of this field is the responsibility of each administration but BCD encoding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator may decide to use only two digits in the MNC over the radio interface, MNC digit 3 shall be encoded as "1111". |  |  |  |  |  |  |  |  |
| When the location entry type is "geo location", the location field in this entry has fixed length as shown in figure 5.3.2.11a. Anchor latitude (octets r+10 to r+13) is defined in clause 6.1 of 3GPP TS 23.032 [7]. Anchor longitude (octets r+14 to r+17) is defined in clause 6.1 of 3GPP TS 23.032 [7]. Radius (octets r+18 to r+19) is given in meters and is defined in clause 6.6 of 3GPP TS 23.032 [7]. |  |  |  |  |  |  |  |  |
| Location field type (octet r+14) when entry type is 3GPP location, or Location field type (octet r+11) when entry type is WLAN location. This field indicates the type of location field. Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 1 TAC 0 0 0 0 0 0 1 0 EUTRA CI 0 0 0 0 0 1 0 0 NR CI 1 0 0 0 0 0 0 1 HESSID 1 0 0 0 0 0 1 0 SSID 1 0 0 0 0 1 0 0 BSSID All other values are reserved. |  |  |  |  |  |  |  |  |
| When 3GPP location field type is set to "TAC", the TAC field is as defined in 3GPP TS 23.003 [4]. TAC has a length of 3 octets. When 3GPP location field type is set to "EUTRA CI", the EUTRA CI field is set to the E-UTRAN cell identity part of the E-UTRAN Cell Global Identifier, as defined in 3GPP TS 36.413 [6A]. E-UTRAN cell identity has a length of 28 bits and is represented by 4 octets by zero-padding the most 4 significant bits of the 4th octet. When 3GPP location field type is set to "NR CI", the NR CI field is set to the NR cell identity part of the NR Cell Global Identifier as defined in 3GPP TS 38.413 [14]. NR cell identity has a length 36 bits and is represented by 5 octets by zero-padding the most 4 significant bits of the 5th octet. When WLAN location field type is set to "HESSID", the HESSID field is set to a 6 octet MAC address that identifies the homogeneous ESS (see IEEE Std 802.11 [8]). When WLAN location field type is set to "SSID", the SSID field is set to an Octet String which shall have a maximum length of 32 octets (see IEEE Std 802.11 [8]). When WLAN location field type is set to "BSSID", the BSSID field is set to an Octet String which shall be 6 octets long (see IEEE Std 802.11 [8]). |  |  |  |  |  |  |  |  |
| Time of day (octets s+1 to u) |  |  |  |  |  |
| This field contains the binary encoding of the time of day condition for a particular WLANSP rule. |  |  |  |  |  |  |  |
| ToD sub field type ={time start, time stop, date start, date stop, day of week} (octet s+8) Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 1 time start 0 0 0 0 0 0 1 0 time stop 0 0 0 0 0 1 0 0 date start 0 0 0 0 1 0 0 0 date stop 0 0 0 1 0 0 0 0 day of the week All other values are reserved. when field type is set to "time start" or "time stop", the value of this ToD sub field contents is time of the day represented in string format, as defined in ISO 8601:2004 [13] When field type is set to "date start" or "date stop", the value of this ToD sub field contents is a date represented in string format, as defined in ISO 8601:2004 [13]. When field type is set to "day of the week", the value of this ToD sub field contents is an 8-bit integer formatted as a bitmap representing days of the week. The most significant bit is set to one. The remaining bits represent days of the week. |  |  |  |  |  |  |  |  |
| NOTE 1: The value of roaming is valid only if the WLANSP rule is provided by the H-PCF. NOTE 2: The group of selection criteria as described in clause 4.3.2.1 is encoded as selection criteria entry. NOTE 3: The home network indication shall not be set by V-PCF. NOTE 4: If the home network indication bit is set to "1", the preferred SSID list shall not be present. NOTE 5: If the home network indication bit is set to "1", the preferred roaming partner list shall not be present. The preferred roaming partner list is provided by H-PCF only. |  |  |  |  |  |  |

************************ End of Changes ************************