3GPP TSG-CT Meeting #99	CP-230291

Rotterdam, Netherlands, 20th – 21st March 2023					(revision of C3-230126, 0637)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.520 | CR | 0624 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PFD Determination Analytics for Nnwdaf_AnalyticsInfo API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Ericsson, China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | KDDI, Ericsson, China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-03-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PFD Determination Analytics was implemented in clause 16 of TS 23.288, hence need to be implemented in this specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add new feature, event, data types and procedures for NWDAF supporting PFD Determination analytics in Nnwdaf_AnalyticsInfo API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 requirement is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1, 4.3.1.2, 4.3.1.3.2, 4.3.2.1, 4.3.2.2.2, 5.2.6.1, 5.2.6.2.2, 5.2.6.2.3, 5.2.6.3.3, 5.2.8, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR 0625 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is a backwards compatible feature in the OpenAPI file of the Nnwdaf_AnalyticsInfo API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes from C3-230637: - Remove the "pfdDetermInfos" attribute in the "AnalyticsData" data type in the OpenAPI file. - Add a editor's note to indicate the implement the "pfdDetermInfos" attribute in the OpenAPI file is FFS. |  |  |  |  |  |  |  |  |

# *** 1st Changes ***

4.3.1.1	Overview

The Nnwdaf_AnalyticsInfo service as defined in 3GPP TS 23.501 [2], 3GPP TS 23.288 [17] and 3GPP TS 23.503 [4], is provided by the Network Data Analytics Function (NWDAF).

This service:

-	allows NF service consumers to request and get different type of analytic event information; and

-	allows NF service consumers to request and get context information related to analytics subscriptions.

The types of observed events include:

-	Slice load level information;

-	Network slice instance load level information;

-	Service experience;

-	NF load;

-	Network performance;

-	Abnormal behaviour;

-	UE mobility;

-	UE communication;

-	User data congestion;

-	QoS sustainability;

-	SM congestion control experience;

-	Dispersion;

-	Redundant transmission experience;

-	WLAN performance;

-	DN performance; and

-	PFD determination.

# *** 2nd Change ***

4.3.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [17]. The Network Data Analytics signalling flows are defined in 3GPP TS 29.552 [25], the Policy and Charging related 5G architecture is also described in 3GPP TS 23.503 [4] and 3GPP TS 29.513 [5].

The Nnwdaf_AnalyticsInfo service is part of the Nnwdaf service-based interface exhibited by the Network Data Analytics Function (NWDAF).

Known consumers of the Nnwdaf_AnalyticsInfo service are:

-	Policy Control Function (PCF)

-	Network Slice Selection Function (NSSF)

-	Access and Mobility Management Function (AMF)

-	Session Management Function (SMF)

-	Network Exposure Function (NEF) or Network Exposure Function (Packet Flow Description Function) (NEF (PFDF))

-	Application Function (AF)

-	Operation, Administration, and Maintenance (OAM)

-	Network Data Analytics Function (NWDAF)

-	Data Collection Coordination Function (DCCF)

The PCF accesses the Nnwdaf_AnalyticsInfo service at the NWDAF via the N23 Reference point. The NSSF accesses the Nnwdaf_AnalyticsInfo service at the NWDAF via the N34 Reference point.

Figure 4.3.1.2-1: Reference Architecture for the Nnwdaf_AnalyticsInfo Service; SBI representation

Figure 4.3.1.2-2: Reference Architecture for the Nnwdaf_AnalyticsInfo Service: reference point representation

NOTE:	When the NEF subscribes the PFD Determination Analytics to the NWDAF, the NEF needs to support PFDF function as NEF (PFDF).

# *** 3rd Change ***

4.3.1.3.2	NF Service Consumers

The Policy Control Function (PCF):

-	supports taking analytics information for slice load level information from the NWDAF;

-	supports taking analytics information for service experience related network data from the NWDAF;

-	supports taking analytics information for network performance from the NWDAF;

-	supports taking analytics information for abnormal UE behaviour from the NWDAF;

-	supports taking analytics information for user data congestion from the NWDAF.

-	supports taking analytics information for dispersion from the NWDAF;

-	supports taking analytics information for WLAN performance from the NWDAF; and

-	supports taking one or more above input from NWDAF into consideration for policies on assignment of network resources and/or for traffic steering policies.

NOTE:	How this information is used by the PCF is not standardized in this specification.

The Network Slice Selection Function (NSSF):

-	supports taking slice load level information or network slice instance load level information from the NWDAF into consideration for slice selection;

-	supports taking analytics information for service experience related network data from the NWDAF; and

-	supports taking analytics information for dispersion at the slice from the NWDAF.

The Access and Mobility Management Function (AMF):

-	supports taking SMF load information from the NWDAF into consideration for SMF selection;

-	supports taking expected UE behaviour information (UE mobility and/or UE communication) from the NWDAF into consideration for monitoring UE behaviour;

-	supports taking abnormal UE behaviour information from the NWDAF into consideration for adjustment of UE mobility related network parameters to solve the abnormal risk;

-	supports taking slice load level information or network slice instance load level information from NWDAF into consideration for slice selection;

-	supports taking analytics information for service experience related network data from the NWDAF; and

-	supports taking analytics information for dispersion at the slice from the NWDAF.

The Session Management Function (SMF):

-	supports taking UPF load information from the NWDAF into consideration for UPF selection;

-	supports taking expected UE behaviour information (UE mobility and/or UE communication) from the NWDAF into consideration for monitoring UE behaviour;

-	supports taking UE mobility information from the NWDAF into consideration for UPF selection;

-	supports taking abnormal UE behaviour information from the NWDAF into consideration for adjustment of UE mobility related network parameters to solve the abnormal risk;

-	supports taking analytics information for SM congestion control experience from the NWDAF into consideration for determining back-off timer provided to UE;

-	supports taking analytics information for redundant transmission experience from the NWDAF to consider whether redundant transmission shall be performed, or (if it had been activated) shall be stopped; and

-	supports taking analytics information for DN performance from the NWDAF into consideration for user plane performance.

The Network Exposure Function (NEF):

-	supports forwarding UE mobility information from the NWDAF to the AF when it is untrusted;

-	supports forwarding UE communication information from the NWDAF to the AF when it is untrusted;

-	supports forwarding expected UE behavioural information (UE mobility and/or UE communication) from the NWDAF to the AF when it is untrusted;

-	supports forwarding abnormal behaviour information from the NWDAF to the AF when it is untrusted;

-	supports forwarding user data congestion information from the NWDAF to the AF when it is untrusted;

-	supports forwarding network performance information from the NWDAF to the AF when it is untrusted;

-	supports forwarding QoS Sustainability information from the NWDAF to the AF when it is untrusted;

-	supports forwarding Dispersion information from the NWDAF to the AF when it is untrusted;

-	supports forwarding DN performance information from the NWDAF to the AF when it is untrusted; and

-	supports forwarding Observed Service Experience information from NWDAF to the AF when it is untrusted.

-	supports taking the analytics information for NWDAF assisted PFD Determination from the NWDAF when support PFDF function as the Network Exposure Function (Packet Flow Description Function) (NEF (PFDF)).

The Application Function (AF):

-	supports receiving UE mobility information from the NWDAF or via the NEF;

-	supports receiving UE communication information from the NWDAF or via the NEF;

-	supports receiving expected UE behavioural information (UE mobility and/or UE communication) from the NWDAF or via the NEF;

-	supports receiving abnormal behaviour information from the NWDAF or via the NEF;

-	supports receiving user data congestion information from the NWDAF or via the NEF;

-	supports receiving network performance information from the NWDAF or via the NEF;

-	supports receiving QoS Sustainability information from the NWDAF or via the NEF;

-	supports receiving Dispersion information from the NWDAF or via the NEF;

-	supports receiving DN performance information from NWDAF or via the NEF; and

-	supports receiving Observed Service Experience information from NWDAF or via the NEF.

The Operation, Administration, and Maintenance (OAM):

-	supports receiving slice load level information from the NWDAF;

-	supports receiving observed service experience from the NWDAF;

-	supports receiving NF load information from the NWDAF;

-	supports receiving network performance information from the NWDAF;

-	supports receiving UE mobility information from the NWDAF;

-	supports receiving UE communication information from the NWDAF;

-	supports receiving expected UE behaviour information (UE mobility and/or UE communication) from the NWDAF; and

-	supports receiving abnormal UE behaviour information from the NWDAF.

The Network Data Analytics Function (NWDAF):

-	supports receiving information for all types of network data analytics from the NWDAF; and

-	supports receiving context information related to analytics subscriptions from the NWDAF.

The Data Collection Coordination Function (DCCF):

-	supports receiving information for all types of network data analytics from the NWDAF.

# *** 4th Change ***

4.3.2.1	Introduction

Table 4.3.2.1-1: Operations of the Nnwdaf_AnalyticsInfo Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Nnwdaf_AnalyticsInfo_Request | This service operation is used by an NF to request and get specific analytics from NWDAF. | NF consumer (PCF, NSSF, AMF, SMF, NEF or NEF (PFDF), AF, OAM, NWDAF, DCCF) |
| Nnwdaf_AnalyticsInfo_ContextTransfer | This service operation is used by an NF to request and get context information related to analytics subscriptions from NWDAF. | NF consumer (NWDAF) |

# *** 5th Change ***

4.3.2.2.2	Request and get from NWDAF Analytics information

Figure 4.3.2.2.2-1 shows a scenario where the NF service consumer (e.g. PCF) sends a request to the NWDAF to request and get from the NWDAF analytics information (as shown in 3GPP TS 23.288 [17]).

Figure 4.3.2.2.2-1: Requesting a NWDAF Analytics information

The NF service consumer (e.g. PCF) shall invoke the Nnwdaf_AnalyticsInfo_Request service operation when requesting the NWDAF analytics information. The NF service consumer shall send an HTTP GET request on the resource URI "{apiRoot}/nnwdaf-analyticsinfo/<apiVersion>/analytics" representing the "NWDAF Analytics" (as shown in figure 4.3.2.2.2-1, step 1), to request analytics data according to the query parameter value of the "event-id" attribute. In addition, the following information may be provided:

-	common reporting requirement in the "ana-req" attribute as follows:

1)	identification of time window for the requested analytics data applies via identification of date-time(s) in the "startTs" and "endTs" attributes;

2)	preferred level of accuracy of the analytics in "accuracy" attribute;

3)	percentage of sampling among impacted UEs in the "sampRatio" attribute;

4)	maximum number of objects in the "maxObjectNbr" attribute;

5)	maximum number of SUPIs expected for an analytics report in the "maxSupiNbr" attribute;

6) 	identification of time when analytics information is needed in the "timeAnaNeeded" attribute if the feature "EneNA" is supported;

7)	indication of which analytics metadata is requested to be delivered with the response in the "anaMeta" attribute if the feature "Aggregation" is supported;

8)	requested values for the analytics metadata information to be used for the generation of the analytics in the "anaMetaInd" attribute if the feature "Aggregation" is supported; and/or

9)	preferred accuracy level per analytics subset in the "accPerSubset" attribute if the "listOfAnaSubsets" attribute is present and the EneNA feature is supported.

For different event types:

-	if the event is "LOAD_LEVEL_INFORMATION", it shall provide the event specific filter information within "event-filter" attribute including identification(s) of the network slice via:

1)	identification of network slice(s) in the "snssais" attribute; or

2)	any slices indication in the "anySlice" attribute;

-	if the feature "NsiLoad" is supported and the event is "NSI_LOAD_LEVEL", it shall provide the event specific filter information within "event-filter" attribute including identification(s) of the network slice via:

1)	identification of network slice(s) and the optionally associated instance(s) if available, in the "nsiIdInfos" attribute; or

NOTE 1:		The network slice instance of a PDU session is not available in the PCF.

2)	any slices indication in the "anySlice" attribute;

and may include:

1)	a list of analytics subsets carried by "listOfAnaSubsets" attribute with value(s) only applicable to "NSI_LOAD_LEVEL" event, if the "EneNA" feature is supported;

-	if the feature "NfLoad" is supported and the event is "NF_LOAD", it shall provide:

1)	identification of target UE(s) to which the request applies by "supis" or "anyUe" in the "tgt-ue" attribute; and

NOTE 2:	Only NF instances of type AMF and SMF which are serving the UE can be determined using a SUPI in "supis" attribute.

NOTE 3:	If a list of the NF Instance IDs (or respectively of NF Set IDs) is provided, the NWDAF needs to provide the analytics for each designated NF instance (or respectively for each NF instance belonging to each designated NF Set). In such case the target UE(s) of the Analytics Reporting need be ignored.

-	the "event-filter" attribute may provide:

a)	either list of NF instance IDs in the "nfInstanceIds" attribute or list of NF set IDs in the "nfSetIds" attribute if the identification of target UE(s) applies to all UEs;

b)	list of NF instance types in the "nfTypes" attribute;

c)	identification of network slice(s) in the "snssais" attribute;

d)	optional area of interest by "networkArea" attribute; and/or

e)	an optional list of analytics subsets by "listOfAnaSubsets" attribute with value(s) only applicable to NF_LOAD event, if the "EneNA" feature is supported;

-	if the feature "UeMobility" is supported and the event is "UE_MOBILITY", it shall provide:

1)	identification of target UE(s) to which the request applies by "supis" or "intGroupIds" attribute in the "tgt-ue" attribute;

-	and may provide:

1)	event specific filter information in the "event-filter" attribute:

a)	identification of network area to which the request applies via identification of network area by "networkArea" attribute; and/or

b)	if the feature "UeMobilityExt" is supported,

i)	identification of LADN DNN in the "ladnDnns" attribute;

ii)	visited Area(s) of Interest as the "visitedAreas" attirbute;

NOTE 1:	For LADN service, the consumer (e.g. SMF) provides the LADN DNN to refer the LADN service area as the AOI.

-	if the feature "UeCommunication" is supported and the event is "UE_COMM", it shall provide:

1)	identification of target UE(s) to which the request applies by "supis" or "intGroupIds" attribute in the "tgt-ue" attribute;

-	and may provide:

1)	event specific filter information in the "event-filter" attribute:

a)	identification of the application as "appIds" attribute;

b)	identification of network area to which the request applies via identification of network area by "networkArea" attribute;

c)	identification of DNN in the "dnns" attribute;

d)	identification of network slice(s) in the "snssais" attribute; and/or

e)	a list of analytics subsets carried by "listOfAnaSubsets" attribute with value(s) only applicable to "UE_COMM" event, if the "EneNA" feature is supported;

-	if the feature "NetworkPerformance" is supported and the event is "NETWORK_PERFORMANCE", it shall provide:

1)	identification of target UE(s) to which the request applies by "supis", "intGroupIds" or "anyUe" attribute in the "tgt-ue" attribute;

2)	event specific filter information in the "event-filter" attribute which shall provide:

a)	the network performance types via "nwPerfTypes" attribute;

the "event-filter" attribute may provide:

a)	identification of network area to which the request applies via identification of network area(s) by "networkArea" attribute (mandatory if "anyUe" attribute is set to true); and/or

b)	a list of analytics subsets carried by "listOfAnaSubsets" attribute with value(s) only applicable to "NETWORK_PERFORMANCE" event, if the "EneNA" feature is supported;

-	if the feature "ServiceExperience" is supported and the event is "SERVICE_EXPERIENCE", it shall provide:

1)	identification of target UE(s) to which the request applies by "supis", "intGroupIds" or "anyUe" attribute in the "tgt-ue" attribute;

2)	event specific filter information in the "event-filter" attribute which shall provide:

a)	any slices indication in the "anySlice" attribute or identification of network slice(s) together with the optionally associated network slice instance(s) if available, via the "nsiIdInfos" attribute; and

NOTE 4:		The network slice instance of a PDU session is not available in the PCF.

the "event-filter" attribute may provide:

a)	identification of application(s) to which the request applies via "appIds" attribute;

b)	identification of DNN via identification of Dnn(s) by "dnns" attribute;

c)	identification of user plane accesses to one or more DN(s) where applications are deployed via "dnais" attribute;

d)	identification of network area to which the request applies via identification of network area(s) by "networkArea" attribute (mandatory if "anyUe" attribute is set to true);

e)	if "appIds" attribute is provided, the bandwidth requirement of each application by "bwRequs" attribute;

f)	identication of all the RAT types and/or all the frequencies that the NWDAF received for the application or specific RAT type(s) and/or frequency(ies) and the service experience threshold value(s) for the RAT Type(s) and/or Frequency value(s) where the UE camps on by "ratFreqs" attribute if the feature "ServiceExperienceExt" is also supported;

g)	a list of analytics subsets carried by "listOfAnaSubsets" attribute with value(s) only applicable to "SERVICE_EXPERIENCE" event, if the "EneNA" feature is supported;

h)	the identification of the UPF as the "upfInfo" attribute if the feature "ServiceExperienceExt" is also supported; and/or

i)	IP address(s)/FQDN(s) of the Application Server(s) as the "appServerAddrs" attribute if the feature "ServiceExperienceExt" is also supported;

-	if the feature "QoSSustainability" is supported and the event is "QOS_SUSTAINABILITY", it shall provide:

1)	event specific filter information in the "event-filter" attribute which shall provide:

a)	identification of network area to which the request applies via identification of network area by "networkArea" attribute; and

b)	QoS requirements via "qosRequ" attribute;

2)	identification of target UE(s) to which the request applies by "anyUe" in the "tgt-ue" attribute;

the "event-filter" attribute may provide:

a)	identification of network slice(s) by "snssais" attribute;

-	if the feature "AbnormalBehaviour" is supported and the event is "ABNORMAL_BEHAVIOUR", it shall provide:

1)	identification of target UE(s) to which the request applies by "supis", "intGroupIds" or "anyUe" attribute in the "tgt-ue" attribute; and

2)	event specific filter information in the "event-filter" attribute which shall provide

a)	either the expected analytics type via "exptAnaType" attribute or a list of exception Ids via "excepIds" attribute. If the expected analytics type via "exptAnaType" attribute is provided, the NWDAF shall derive the corresponding Exception Ids from the received expected analytics type as follows:

-	if "exptAnaType" attribute sets to "MOBILITY", the corresponding list of Exception Ids are "UNEXPECTED_UE_LOCATION", "PING_PONG_ACROSS_CELLS", "UNEXPECTED_WAKEUP" and "UNEXPECTED_RADIO_LINK_FAILURES";

-	if "exptAnaType" attribute sets to "COMMUN", the corresponding list of Exception Ids are "UNEXPECTED_LONG_LIVE_FLOW", "UNEXPECTED_LARGE_RATE_FLOW", "SUSPICION_OF_DDOS_ATTACK", "WRONG_DESTINATION_ADDRESS" and "TOO_FREQUENT_SERVICE_ACCESS";

-	if "exptAnaType" attribute sets to "MOBILITY_AND_COMMUN", the corresponding list of Exception Ids includes all above derived exception Ids.

The derived list of Exception Ids are used by the NWDAF to notify the NF service consumer when UE's behaviour is exceptional based on one or more Exception Ids within the list.

If the "anyUe" attribute in the "tgt-ue" attribute sets to "true":

a)	the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via "excepIds" attribute shall not be requested for both mobility and communication related analytics at the same time;

b)	if the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via "excepIds" attribute is mobility related, at least one of identification of network area by "networkArea" attribute and identification of network slice(s) by "snssais" attribute should be provided; and

c)	if the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via "excepIds" attribute is communication related, at least one of identification of network area by "networkArea" attribute, identification of application(s) by "appIds" attribute, identification of DNN(s) in the "dnns" attribute and identification of network slice(s) by "snssais" attribute should be provided;

the "event-filter" attribute may provide:

a)	expected UE behaviour via "exptUeBehav" attribute;

-	if the feature "UserDataCongestion" is supported and the event is "USER_DATA_CONGESTION", it shall provide one of the following attributes:

1)	identification of target UE(s) via "supis" "gpsis" (if feature "UserDataCongestionExt" is supported)  or "anyUe" attribute within "tgt-ue" attribute;

and may provide:

1)	event specific filter information in the "event-filter" attribute which may provide:

a)	identification of network slice(s) by "snssais" attribute;

b)	identification of network area to which the request applies via identification of network area by "networkArea" attribute (mandatory if "anyUe" attribute is set to true);

c)	if the feature "UserDataCongestionExt" is also supported, request a list of top applications with maximum number that contribute the most to the traffic in uplink and/or downlink directions bythe "maxTopAppUlNbr" attribute and/or the "maxTopAppDlNbr" attribute; and/or

d)	a list of analytics subsets carried by "listOfAnaSubsets" attribute with value(s) only applicable to "USER_DATA_CONGESTION" event, if the "EneNA" feature is supported;

-	if the feature "SMCCE" is supported and the event is "SM_CONGESTION", it shall provide:

1)	event specific filter information in the "event-filter" attribute which shall provide:

a)	identification of DNN in the "dnns" attribute; and/or

b)	identification of network slice(s) in the "snssais" attribute; and

2)	identification of target UE(s) via "supis" attribute in the "tgt-ue" attribute where the target UE(s) are one have the PDU Session for the DNN and/or S-NSSAI indicated by the event specific filter information;

and may include:

1)	a list of analytics subsets carried by "listOfAnaSubsets" attribute with value(s) only applicable to "SM_CONGESTION" event, if the "EneNA" feature is supported;

NOTE 5:	The predictions are not applicable for Session Management Congestion Control Experience analytics.

-	if the feature "Dispersion" is supported and the event is "DISPERSION", shall provide:

1)	identification of target UE(s) applies by "supis", "intGroupIds" or "anyUe" attribute within "tgt-ue" attribute, "anyUe" attribute is only supported in combination with "snssais" attribute, "networkArea" attribute and/or "disperClass" attribute;

and may include:

1)	identification of network area applies via identification of network area by "networkArea" attribute;

2)	identification of network slice(s) by "snssais" attribute;

3)	application identifier(s) in "appIds" attribute;

4)	dispersion analytics requirements in "disperReqs" attribute, which for the requested dispersion type may include dispersion class, ranking, ordering and/or accuracy requirments; and/or

5)	an optional list of analytics subsets by "listOfAnaSubsets" attribute with value(s) only applicable to DISPERSION event;

-	if the feature "RedundantTransmissionExp" is supported and the event is "RED_TRANS_EXP", shall provide:

1)	identification of target UE(s) applies by "supis", "intGroupIds" or "anyUe" attribute within "tgt-ue" attribute;

and may include:

1)	identification of network area applies via identification of network area by "networkArea" attribute, if the "supis" attribute or "intGroupIds" attribute is included in the "tgt-ue" attribute;

2)	identification of network slice(s) by "snssais" attribute;

3)	identification of DNN in the "dnns" attribute;

4)	other redundant transmission experience analysis requirements in "redTransReqs" attribute, which may include preferred order of results for the list of Redundant Transmission Experience; and/or

5)	an optional list of analytics subsets by "listOfAnaSubsets" attribute with value(s) only applicable to RED_TRANS_EXP event, if the "EneNA" feature is supported;

-	if the feature "WlanPerformance" is supported and the event is "WLAN_PERFORMANCE", shall provide:

1)	identification of target UE(s) by "supis", "intGroupIds" or "anyUe" attribute in the "tgt-ue" attribute. If "anyUe" attribute is included in the "tgt-ue" attribute, then any of "networkArea" attribute, "ssIds" or "bssIds" attribute shall be present in the "wlanReqs" attribute;

and may include:

1)	identification of network area to which the request applies via identification of network area by "networkArea" attribute;

2)	other WLAN performance analytics requirements in "wlanReqs" attribute, which may include SSID(s), BSSID(s), preferred order of results for the list of WLAN performance information and/or accuracy per analytics subset; and/or

3)	an optional list of analytics subsets by "listOfAnaSubsets" attribute with value(s) only applicable to WLAN_PERFORMANCE event, if the "EneNA" feature is supported;

-	if the feature "DnPerformance" is supported and the event is "DN_PERFORMANCE", shall provide:

1)	identification of target UE(s) to which the request applies by "supis", "intGroupIds" or "anyUe" attribute in the "tgt-ue" attribute;

and may include:

1)	identification of network area to which the request applies via identification of network area by "networkArea" attribute;

2)	identification of network slice(s) in the "snssais" attribute;

3)	identification of network slice and the optionally associated network slice instance(s) if available, via the "nsiIdInfos" attribute or any slices indication in the "anySlice" attribute;

4)	application identifier(s) in "appIds" attribute;

5)	an identification of DNN in the "dnns" attribute;

6)	identification of a user plane access to one or more DN(s) where applications are deployed by "dnais" attribute;

7)	the identification of the UPF as the "upfInfo" attribute;

8)	IP address(s)/FQDN(s) of the Application Server(s) as the "appServerAddrs" attribute;

9)	DN performance analytics requirements in "dnPerfReqs" attribute, which may include the preferred order of results for the list of DN performance information and/or the reporting threshold of each analytics subset; and/or

10)	an optional list of analytics subsets by "listOfAnaSubsets" attribute with value(s) only applicable to "DN_PERFORMANCE" event, if the "EneNA" feature is supported.

-	if the feature "PfdDetermination" is supported and the event is "PFD_DETERMINATION", shall provide:

1)	a list of application identifier(s) in the "appIds" attribute.

-	and may include:

1)	an identification of DNN in the "dnns" attribute;

2)	identification of network slice(s) in the "snssais" attribute;

NOTE: PFD Determination analytics do not have a target UE, they are always for any UE.

Upon the reception of the HTTP GET request, the NWDAF shall:

-	analyse the requested analytic data according to the requested event.

If the HTTP request message from the NF service consumer is accepted, the NWDAF shall respond with "200 OK" status code with the message body containing the analytics with parameters as relevant for the requesting NF service consumer. The AnalyticsData data structure in the response body shall include:

-	analytics with the corresponding information as described in clause 4.2.2.4.2.

If the requested NWDAF Analytics data does not exist, the NWDAF shall respond with "204 No Content" status code.

If the "timeAnaNeeded" attribute within EventReportingRequirement is provided during the request, if the time is reached but the requested analytics information is not ready, the consumer does not need to wait for the analytics information any longer, the NWDAF may send a "500 Internal Server Error" status code to the NF service consumer. In addition, if the EneNA feature is supported, the NWDAF may provide, within the ProblemDetailsAnalyticsInfoRequestdata in the response, the corresponding failure reason via a "problemDetails" attribute with the "cause" attribute set to "UNSATISFIED_REQUESTED_ANALYTICS_TIME" and a minimum time interval recommended by the NWDAF via a "rvWaitTime" attribute which is used by the NF service consumer to determine the time when analytics information is needed in similar future analytics requests.

If the analytics target period provided in the body of the HTTP GET request includes the start time in the past and the end time in the future, the NWDAF shall reject the request with an HTTP "400 Bad Request" response including the "cause" attribute set to "BOTH_STAT_PRED_NOT_ALLOWED".

If the statistics in the past are requested but the necessary data to perform the service is unavailable, the NWDAF shall reject the request with an HTTP "500 Internal Server Error" response including the "cause" attribute set to "UNAVAILABLE_DATA".

If the user consent has not been checked by the NF service consumer and is required for the requested analytics collection depending on local policy and regulations, then the NWDAF shall check user consent for the targeted UE(s) by retrieving the user consent subscription data via the Nudm_SDM service API of the UDM as described in clause 5.2.2 of 3GPP TS 29.503 [23]. If the NWDAF receive the response from the UDM that it is not granted for the impacted user(s), then the NWDAF shall send an HTTP "403 Forbidden" error response including the "cause" attribute set to "USER_CONSENT_NOT_GRANTED".

NOTE 6:	When the target of reporting is a SUPI or a GPSI then the subscription can be rejected, e.g. because user consent is not granted, and the error is sent to the consumer. When the target of reporting is an Internal Group Id, or a list of SUPIs/GPSI(s) or any UE, and the user consent is not granted for a subset of the impacted users, then no error is sent, but a subset of the SUPIs/GPSIs is skipped if user consent is not granted.

If an error occurs when processing the HTTP GET request, the NWDAF shall send an HTTP error response as specified in clause 5.2.7.

# *** 6th Change ***

5.2.6.1	General

This clause specifies the application data model supported by the API.

Table 5.2.6.1-1 specifies the data types defined for the Nnwdaf_AnalyticsInfo service based interface protocol.

Table 5.2.6.1-1: Nnwdaf_AnalyticsInfo specific Data Types

| Data type |  |  |  | Section defined | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| AdditionInfoAnalyticsInfoRequest |  |  | 5.2.6.2.5 | Contains more details (not only the ProblemDetails) in case an Nnwdaf_AnalyticsInfo request is rejected. | EneNA |  |
| AdrfDataType |  |  | 5.2.6.3.5 | Represents a type of data that is stored in the ADRF. | AnaCtxTransfer |  |
| AnalyticsData |  |  |  | 5.2.6.2.2 | Describes analytics with parameters indicated in the request. |  |
| ContextData |  |  | 5.2.6.2.6 | Contains context information related to analytics subscriptions corresponding with one or more context identifiers. | AnaCtxTransfer |  |
| ContextElement |  |  | 5.2.6.2.7 | Contains context information corresponding with a specific context identifier. | AnaCtxTransfer |  |
| ContextIdList |  |  | 5.2.6.2.8 | Contains list of context identifiers of context information of analytics subscriptions. | AnaCtxTransfer |  |
| ContextType |  |  | 5.2.6.3.4 | Identfies the type of analytics context information. | AnaCtxTransfer |  |
| DnPerfInfo |  |  | 5.1.6.2.45 | Represents DN performance information | DnPerformance |  |
| DnPerformanceReq |  | 5.1.6.2.66 |  | Represents the DN performance requirements. | DnPerformance |  |
| EventFilter |  | 5.2.6.2.3 |  |  | Represents the event filters used to identify the requested analytics. |  |
| EventId |  | 5.2.6.3.3 |  |  | Describes the type of analytics. |  |
| HistoricalData |  | 5.2.6.2.9 |  | Contains historical data related to an analytics subscription. | AnaCtxTransfer |  |
| ProblemDetailsAnalyticsInfoRequest |  | 5.2.6.4.1 |  |  | Data type that extends ProblemDetails. | EneNA |
| RequestedContext |  |  | 5.2.6.2.11 | Contains types of analytics context information. | AnaCtxTransfer |  |
| SmcceInfo |  |  | 5.2.6.2.12 | Represents the analytics of Session Management congestion control experience information. | SMCCE |  |
| SmcceUeList |  |  | 5.2.6.2.13 | Represents the List of UEs classified based on experience level of Session Management congestion control. | SMCCE |  |
| SpecificAnalyticsSubscription |  |  | 5.2.6.2.10 | Represents an existing subscription for a specific type of analytics to a specific NWDAF. | AnaCtxTransfer |  |
| SpecificDataSubscription |  |  | 5.2.6.2.14 | Represents an existing data collection subscription to a specific data source NF. | AnaCtxTransfer |  |

Table 5.2.6.1-2 specifies data types re-used by the Nnwdaf_AnalyticsInfo service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnwdaf service based interface.

Re-used data types of clause 5.1.6 refer here to requests instead of subscriptions.

Table 5.2.6.1-2: Nnwdaf_AnalyticsInfo re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AbnormalBehaviour | 5.1.6.2.15 | Represents the abnormal behaviour information. | AbnormalBehaviour |
| AnalyticsContextIdentifier | 5.1.6.2.43 | Contains information about the available analytics contexts. | AnaCtxTransfer |
| AnalyticsMetadataInfo | 5.1.6.2.37 | Contains analytics metadata information required for analytics aggregation. | Aggregation |
| AnalyticsSubset | 5.1.6.3.18 | Contains information about the analytics subsets provided in the subscription request. | EneNA |
| AnySlice | 5.1.6.3.2 |  |  |
| ApplicationId | 3GPP TS 29.571 [8] | Identifies the application. | ServiceExperience UeCommunication AbnormalBehaviour DnPerformance PfdDetermination |
| BwRequirement | 5.1.6.2.25 |  | ServiceExperience |
| DataNotification | 3GPP TS 29.575 [27] | Describes Notifications about data collection events that occurred. | EneNA |
| DataSubscription | 3GPP TS 29.575 [27] | Represents data subscription from data source (e.g. AMF, SMF, UDM, NEF, AF). | EneNA |
| DateTime | 3GPP TS 29.571 [8] | Identifies the time. |  |
| DispersionRequirement | 5.1.6.2.50 | Dispersion analytics requirement. | Dispersion |
| DispersionInfo | 5.1.6.2.53 | Dispersion analytics information. | Dispersion |
| Dnai | 3GPP TS 29.571 [8] | Identifies a user plane access to one or more DN(s). | ServiceExperience DnPerformance |
| Dnn | 3GPP TS 29.571 [8] | Identifies the DNN. | ServiceExperience AbnormalBehaviour UeCommunication SMCCE DnPerformance PfdDetermination |
| DnPerfInfo | 5.1.6.2.45 | Represents DN performance information | DnPerformance |
| DnPerformanceReq | 5.1.6.2.66 | Represents the DN performance requirements. | DnPerformance |
| DurationSec | 3GPP TS 29.571 [8] |  |  |
| EventNotification | 5.1.6.2.5 | Describes Notifications about analytics events that occurred. | AnaCtxTransfer |
| EventReportingRequirement | 5.1.6.2.7 |  |  |
| ExceptionId | 5.1.6.3.6 |  | AbnormalBehaviour |
| ExpectedUeBehaviourData | 3GPP TS 29.503 [23] |  | AbnormalBehaviour |
| ExpectedAnalyticsType | 5.1.6.3.11 |  | AbnormalBehaviour |
| ModelInfo | 5.1.6.2.42 | The information of the ML models. | AnaCtxTransfer |
| NetworkAreaInfo | 3GPP TS 29.554 [18] | The network area information. | UeMobility UeCommunication NetworkPerformance QoSSustainability ServiceExperience UserDataCongestion AbnormalBehaviour NsiLoadExt Dispersion RedundantTransmissionExp WlanPerformance DnPerformance |
| NetworkPerfInfo | 5.1.6.2.23 |  | NetworkPerformance |
| NetworkPerfType | 5.1.6.3.10 | Represents the network performance types. | NetworkPerformance |
| NfLoadLevelInformation | 5.1.6.2.31 | Represents load level information of a given NF instance. | NfLoad |
| NfInstanceId | 3GPP TS 29.571 [8] | Identifies an NF instance | NfLoad |
| NfSetId | 3GPP TS 29.571 [8] | Identifies an NF Set instance. | NfLoad |
| NFType | 3GPP TS 29.510 [12] | Indentifies a type of NF. | NfLoad |
| NsiIdInfo | 5.1.6.2.33 | Identify the S-NSSAI and the associated Network Slice Instance(s). | ServiceExperience NsiLoad DnPerformance |
| NsiLoadLevelInfo | 5.1.6.2.34 | Represents the load level information for an S-NSSAI and the associated network slice instance. | NsiLoad |
| NnwdafEventsSubscription | 5.1.6.2.2 | Represents an Individual NWDAF Event Subscription resource. | AnaCtxTransfer |
| PfdDeterminationInfo | 5.1.6.2.71 | Represents PFD Determination information. | PfdDetermination |
| ProblemDetails | 3GPP TS 29.571 [8] | Used in error responses to provide more detailed information about an error. |  |
| QosRequirement | 5.1.6.2.20 |  | QoSSustainability |
| QosSustainabilityInfo | 5.1.6.2.19 |  | QoSSustainability |
| RatFreqInformation | 5.1.6.2.67 | Represents the RAT type and/or Frequency information | ServiceExperienceExt |
| RedundantTransmissionExpInfo | 5.1.6.2.57 | Redundant transmission experience analytics information. | RedundantTransmissionExp |
| RedundantTransmissionExpReq | 5.1.6.2.56 | Redundant transmission experience analytics requirement. | RedundantTransmissionExp |
| ServiceExperienceInfo | 5.1.6.2.24 |  | ServiceExperience |
| Supi | 3GPP TS 29.571 [8] | Identifies the UE. | ServiceExperience, NfLoad NetworkPerformance UserDataCongestion UeMobility UeCommunication AbnormalBehaviour SMCCE Dispersion RedundantTransmissionExp WlanPerformance |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.2.8-1. |  |
| Snssai | 3GPP TS 29.571 [8] |  |  |
| SliceLoadLevelInformation | 5.1.6.2.6 |  |  |
| TargetUeInformation | 5.1.6.2.8 | Identifies the target UE information. | ServiceExperience NfLoad NetworkPerformance UserDataCongestion UeMobility UeCommunication AbnormalBehaviour QoSSustainability Dispersion RedundantTransmissionExp WlanPerformance SMCCE DnPerformance |
| UeCommunication | 5.1.6.2.13 |  | UeCommunication |
| UeMobility | 5.1.6.2.10 |  | UeMobility |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned Integer, i.e. only value 0 and integers above 0 are permissible. |  |
| UpfInformation | 3GPP TS 29.508 [29] | The information of the UPF serving the UE. | ServiceExperienceExt DnPerformance |
| UserDataCongestionInfo | 5.1.6.2.17 |  | UserDataCongestion |
| WlanPerformanceInfo | 5.1.6.2.60 | WLAN performance analytics information. | WlanPerformance |
| WlanPerformanceReq | 5.1.6.2.59 | WLAN performance analytics requirement. | WlanPerformance |

# *** 7th Change ***

5.2.6.2.2	Type AnalyticsData

Table 5.2.6.2.2-1: Definition of type AnalyticsData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| start | DateTime | O | 0..1 | It defines the start time of which the analytics information will become valid. (NOTE 1) |  |
| expiry | DateTime | O | 0..1 | It defines the expiration time after which the analytics information will become invalid. (NOTE 1) |  |
| timeStampGen | DateTime | C | 0..1 | It defines the timestamp of analytics generation. (NOTE 3) |  |
| anaMetaInfo | AnalyticsMetadataInfo | C | 0..1 | Contains information about analytics metadata required to aggregate the analytics. It shall be present if the "anaMeta" attribute was included in the request, containing the information indicated by the "anaMeta" attribute. | Aggregation |
| sliceLoadLevelInfos | array(SliceLoadLevelInformation) | C | 1..N | The slices and the load level information. Shall be present when the requested event is "LOAD_LEVEL_INFORMATION". |  |
| nsiLoadLevelInfos | array(NsiLoadLevelInfo) | C | 1..N | Each element identifies the load level information for an S-NSSAI and the optionally associated network slice instance. Shall be presented when the requested event is "NSI_LOAD_LEVEL" | NsiLoad |
| nwPerfs | array(NetworkPerfInfo) | C | 1..N | The network performance information. Shall be present when the requested event is "NETWORK_PERFORMANCE". | NetworkPerformance |
| nfLoadLevelInfos | array(NfLoadLevelInformation) | C | 1..N | The NF load information. When the requestedevent is "NF_LOAD", the nfLoadLevelInfos shall be included. | NfLoad |
| qosSustainInfos | array(QosSustainabilityInfo) | C | 1..N | The QoS sustainability informations in the certain geographic areas. It shall be present if the requested eventis "QOS_SUSTAINABILITY". (NOTE 2) | QoSSustainability |
| ueMobs | array(UeMobility) | C | 1..N | The UE mobility information. When the requested event is "UE_MOBILITY", the "ueMobs" attribute shall be included. | UeMobility |
| ueComms | array(UeCommunication) | C | 1..N | The UE communication information. When the requested event is "UE_COMM", the "ueComms" attribute shall be included. | UeCommunication |
| userDataCongInfos | array(UserDataCongestionInfo) | C | 1..N | The user data congestion information. Shall be present when the requested event is "USER_DATA_CONGESTION". | UserDataCongestion |
| suppFeat | SupportedFeatures | C | 0..1 | List of Supported features used as described in clause 5.2.8. This parameter shall be supplied by NWDAF in the reply of GET request that request the analytics resource, if the consumer includes "supported-features" in the GET request. |  |
| svcExps | array(ServiceExperienceInfo) | C | 1..N | The service experience information. Shall be present when the requested event is "SERVICE_EXPERIENCE". | ServiceExperience |
| abnorBehavrs | array(AbnormalBehaviour) | C | 1..N | The abnormal behaviour information. Shall be present when the requested event is "ABNORMAL_BEHAVIOUR". | AbnormalBehaviour |
| smccExps | array(SmcceInfo) | C | 1..N | The Session Management congestion control experience information. Shall be present when the requested event is "SM_CONGESTION". | SMCCE |
| disperInfos | array(DispersionInfo) | C | 1..N | The Dispersion information. Shall be present when the requested event is "DISPERSION". | Dispersion |
| redTransInfos | array(RedundantTransmissionExpInfo) | C | 1..N | The Redundant Transmission Experience analytics information. Shall be present when the requested event is "RED_TRANS_EXP". | RedundantTransmissionExp |
| wlanInfos | array(WlanPerformanceInfo) | C | 1..N | The WLAN performance related information. When requested event is "WLAN_PERFORMANCE", the "wlanInfos" attribute shall be included. | WlanPerformance |
| dnPerfInfos | array(DnPerfInfo) | C | 1..N | The DN performance information. Shall be present when the requested event is "DN_PERFORMANCE". | DnPerformance |
| pfdDetermInfos | array(PfdDeterminationInfo) | C | 1..N | Represents the PFD Determination information for a known application identifier. Shall be present when the requested event is "PFD_DETERMINATION". | PfdDetermination |
| NOTE 1: If the "start" attribute and the "expiry" attribute are both provided, the DateTime of the "expiry" attribute shall not be earlier than the DateTime of the "start" attribute. NOTE 2: The "qosFlowRetThd" and "ranUeThrouThd" attributes in QosSustainabilityInfo data type are not applicable. NOTE 3: This attribute shall be included when ADRF is deployed. |  |  |  |  |  |

Editor's Note:	 The definition of "pfdDetermInfos" in the OpenAPI file is FFS.

# *** 8th Change ***

5.2.6.2.3	Type EventFilter

Table 5.2.6.2.3-1: Definition of type EventFilter

| Attribute name | Data type |  | P | Cardinality |  | Description | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| anySlice | AnySlice |  | C | 0..1 |  | Default is "FALSE". (NOTE 1) |  |  |
| appIds | array(ApplicationId) |  | C | 1..N |  | Represents the Application Identifier(s). The absence of appIds means applicable to all applications. (NOTE 4) (NOTE 10) | ServiceExperience UeCommunication AbnormalBehaviour Dispersion DnPerformance PfdDetermination |  |
| dnns | array(Dnn) |  | C | 1..N |  | Represents the DNN(s). Each DNN is a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. The absence of dnns means applicable to all DNNs. (NOTE 4) | ServiceExperience UeCommunication AbnormalBehaviour SMCCE DnPerformance RedundantTransmissionExp PfdDetermination |  |
| dnais | array(Dnai) |  | C | 1..N |  | Represents the Data Network Access Identifier(s) of user plane accesses to DN(s) where applications are deployed. It may be included when event-id is "SERVICE_EXPERIENCE" or "DN_PERFORMANCE". | ServiceExperience DnPerformance |  |
| ladnDnns | array(Dnn) |  | O | 1..N |  | Represents the LADN DNN(s) to indicate the LADN service area(s) as the AoI(s). | UeMobilityExt |  |
| snssais | array(Snssai) |  | C | 1..N |  | Identification(s) of network slice(s). (NOTE 1), (NOTE 4) |  |  |
| nfInstanceIds | array(NfInstanceId) |  | O | 1..N |  | Identification(s) of NF instance(s). | NfLoad |  |
| nfSetIds | array(NfSetId) |  | O | 1..N |  | Identification(s) of NF instance set(s). | NfLoad |  |
| nfTypes | array(NFType) |  | O | 1..N |  | Identification(s) of NF type(s). (NOTE 8) | NfLoad NsiLoadExt |  |
| networkArea | NetworkAreaInfo |  | C | 0..1 |  | This IE represents the network area where the NF service consumer wants to know the analytics result. (NOTE 2), (NOTE 4) | UeMobility UeCommunication NetworkPerformance QoSSustainability ServiceExperience UserDataCongestion AbnormalBehaviour NsiLoadExt NfLoadExt Dispersion RedundantTransmissionExp WlanPerformance DnPerformance |  |
| visitedAreas | array(NetworkAreaInfo) |  | O | 1..N |  | Identification(s) of network area(s) which the UEs had previously been in at least one of the Visited Area(s) of Interest. (NOTE 9) | UeMobilityExt |  |
| maxTopAppUlNbr | Uinteger |  | O | 0..1 |  | Indicates the requested maximum number of top applications that contribute the most to the traffic in Uplink direction. Minimum = 1. May be included when one of the elements in the "listOfAnaSubsets" attribute is set to LIST_OF_TOP_APP_UL. | UserDataCongestionExt |  |
| maxTopAppDlNbr | Uinteger |  | O | 0..1 |  | Indicates the requested maximum number of top applications that contribute the most to the traffic in Downlink direction. Minimum = 1. May be included when one of the elements in the "listOfAnaSubsets" attribute is set to LIST_OF_TOP_APP_DL. | UserDataCongestionExt |  |
| nsiIdInfos | array(NsiIdInfo) |  | O | 1..N |  | Each element identifies the S-NSSAI and the optionally associated network slice instance(s). May be included when subscribed event is "NSI_LOAD_LEVEL", "SERVICE_EXPERIENCE" or "DN_PERFORMANCE". (NOTE 1) | ServiceExperience NsiLoad DnPerformance |  |
| nwPerfTypes | array(NetworkPerfType) |  | C | 1..N |  | Represents the network performance types. This attribute shall be included when event-id is "NETWORK_PERFORMANCE". | NetworkPerformance |  |
| qosRequ | QoSRequirement |  | C | 0..1 |  | Represents the QoS requirements. This attribute shall be included when event-id is "QOS_SUSTAINABILITY". | QoSSustainability |  |
| bwRequs | array(BwRequirement) |  | O | 1..N |  | Represents the media/application bandwidth requirement for each application. It may only be present if "appIds" attribute is provided. | ServiceExperience |  |
| excepIds | array(ExceptionId) |  | C | 1..N |  | Represents a list of Exception Ids. (NOTE 3), (NOTE 4) | AbnormalBehaviour |  |
| exptAnaType | ExpectedAnalyticsType |  | C | 0..1 |  | Represents expected UE analytics type. (NOTE 3), (NOTE 4) | AbnormalBehaviour |  |
| exptUeBehav | ExpectedUeBehaviourData |  | O | 0..1 |  | Represents expected UE behaviour. | AbnormalBehaviour |  |
| ratFreqs | array(RatFreqInformation) | O |  | 1..N | Identification(s) of the RAT type(s) and/or frequency(ies) of UE's serving cell(s) which the request applies. (NOTE 5) |  |  |  |
| disperReqs | array(DispersionRequirement) | O |  | 1..N | Represents the dispersion analytics requirements. |  |  | Dispersion |
| redTransReqs | array(RedundantTransmissionExpReq) | O |  | 1..N | Represents the redundant transmission experience analytics requirements. |  |  | RedundantTransmissionExp |
| wlanReqs | array(WlanPerformanceReq) | O |  | 1..N | Represents other WLAN performance analytics requirements. If the attribute contains no content, may take default handling action. |  |  | WlanPerformance |
| listOfAnaSubsets | array(AnalyticsSubset) | O |  | 1..N | The list of analytics subsets used to indicate the content of the analytics. |  |  | EneNA |
| upfInfo | UpfInformation | O |  | 0..1 | Identifies the UPF. (NOTE 7) |  |  | ServiceExperienceExt DnPerformance |
| appServerAddrs | array(AddrFqdn) | C |  | 1..N | Each element represents the Application Server Instance (IP address/FQDN of the Application Server). (NOTE 6) |  |  | ServiceExperienceExt DnPerformance |
| dnPerfReqs | array(DnPerformanceReq) | O |  | 1..N | Represents the DN performance requirements. This attribute shall be included when event-id is "DN_PERFORMANCE". |  |  | DnPerformance |
| NOTE 1: The "anySlice" attribute is not applicable to features "UeMobility" and "NetworkPerformance". The NOTE 1: The "anySlice" attribute is not applicable to features "UeMobility" and "NetworkPerformance". The "snssais" attribute is not applicable to features "ServiceExperience", "NsiLoad", "UeMobility" and "NetworkPerformance". When event-id in the request is "LOAD_LEVEL_INFORMATION", the identifications of network slices, either information about slice(s) identified by the "snssais" attribute, or "anySlice" set to "TRUE", shall be included. When requested event is "NSI_LOAD_LEVEL" or "SERVICE_EXPERIENCE", either the "nsiIdInfos" attribute or anySlice set to "TRUE" shall be included. When requested event is "QOS_SUSTAINABILITY", "NF_LOAD", "UE_COMM", "ABNORMAL_BEHAVIOUR", "USER_DATA_CONGESTION", "DISPERSION" "RED_TRANS_EXP" or "PFD_DETERMINATION", the identifications of network slices identified by the "snssais" attribute is optional. NOTE 2: For "NETWORK_PERFORMANCE", "SERVICE_EXPERIENCE" or "USER_DATA_CONGESTION" event, this attribute shall be provided if the event applied for all UEs (i.e. "anyUe" attribute set to true). For "QOS_SUSTAINABILITY", this attribute shall be provided. NOTE 3: Either "excepIds" or "exptAnaType" shall be provided if event-id in the request is "ABNORMAL_BEHAVIOUR". NOTE 4: For "ABNORMAL_BEHAVIOUR" event with "anyUe" attribute in "tgt-ue" attribute sets to true, - at least one of the "networkArea" and the "snssais" attribute should be included, if the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via the "excepIds" attribute is mobility related; - at least one of the "networkArea", "appIds", "dnns" and "snssais" attribute should be included, if the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via the "excepIds" attribute is communication related; - the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via "excepIds" attribute shall not be requested for both mobility and communication related analytics at the same time. NOTE 5: If both the "allFreq" attribute and the "allRat" attributes in RatFreqInformation data type are present, then the only one instance of the RatFreqInformation data type shall be present to indicate for all the RAT type and Frequency value the NWDAF has received for the application. NOTE 6: This parameter shall be provided when a consumer requires analytics for an edge application over a UP path. NOTE 7: This parameter may be provided when a consumer requires analytics for an edge application over a UP path. NOTE 8: When event-id in the request is "NSI_LOAD_LEVEL" and the NsiLoadExt feature is supported, and the NF service consumer provides the "nfTypes" attribute, then the NWDAF accounts only for the resource usage of the NF types included in "nfTypes" to derive the output analytics. NOTE 9: If this attribute is provided, the analytics target period shall be a past time period (i.e. only statistics is supported). NOTE 10: When event-id in the request is "PFD_DETERMINATION" and the PfdDetermination feature is supported, the "appIds" attribute shall be included. |  |  |  |  |  |  |  |  |

NOTE:	Care needs to be taken to avoid excessive signalling.

# *** 9th Change ***

5.2.6.3.3	Enumeration: EventId

Table 5.2.6.3.3-1: Enumeration EventId

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LOAD_LEVEL_INFORMATION | Represents the analytics of load level information of corresponding network slice. |  |
| NETWORK_PERFORMANCE | Represents the analytics of network performance information | NetworkPerformance |
| NF_LOAD | Represents the analytics of NF Load information. | NfLoad |
| QOS_SUSTAINABILITY | Represents the analytics of QoS sustainability in the certain area. | QoSSustainability |
| SERVICE_EXPERIENCE | Represents the analytics of service experience of corresponding application and/or network slice. | ServiceExperience |
| UE_MOBILITY | Represents the analytics of UE mobility. | UeMobility |
| UE_COMM | Represents the analytics of UE communication. | UeCommunication |
| USER_DATA_CONGESTION | Represents the analytics of the user data congestion in the certain area. | UserDataCongestion |
| ABNORMAL_BEHAVIOUR | Represents the analytics of abnormal behaviour information. | AbnormalBehaviour |
| NSI_LOAD_LEVEL | Represents the analytics of load level information of Network Slice and the optionally associated Network Slice Instance | NsiLoad |
| SM_CONGESTION | Represents the analytics of Session Management congestion control experience information for specific DNN and/or S-NSSAI. | SMCCE |
| DN_PERFORMANCE | Represents the analytics of DN performance. | DnPerformance |
| DISPERSION | Represents the analytics of dispersion. | Dispersion |
| RED_TRANS_EXP | Represents the analytics of Redundant Transmission Experience. | RedundantTransmissionExp |
| WLAN_PERFORMANCE | Represents the analytics of WLAN performance. | WlanPerformance |
| PFD_DETERMINATION | Represents the analytics of PFD Determination information for known application identifier(s). | PfdDetermination |

# *** 10th Change ***

5.2.8	Feature negotiation

The optional features in table 5.2.8-1 are defined for the Nnwdaf_AnalyticsInfo API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [6].

Table 5.2.8-1: Supported Features

| Feature number |  | Feature Name |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | UeMobility |  | This feature indicates the support of analytics based on UE mobility information. |  |
| 2 |  | UeCommunication |  | This feature indicates the support of analytics based on UE communication information. |  |
| 3 |  | NetworkPerformance |  | This feature indicates the support of analytics based on network performance. |  |
| 4 |  | ServiceExperience |  | This feature indicates support for the event related to service experience. |  |
| 5 |  | QoSSustainability |  | This feature indicates support for the event related to QoS sustainability. |  |
| 6 |  | AbnormalBehaviour |  | This feature indicates support for the event related to abnormal behaviour information. |  |
| 7 |  | UserDataCongestion |  | This feature indicates the support of the analytics related on user data congestion. |  |
| 8 |  | NfLoad |  | This feature indicates the support of the analytics related to the load of NF instances. |  |
| 9 |  | NsiLoad |  | This feature indicates the support of the analytics related to the load level of Network Slice and the optionally associated Network Slice Instance. |  |
| 10 |  | EneNA |  | This feature indicates support for the enhancements of network data analytics requirements. |  |
| 11 |  | UserDataCongestionExt |  | This feature indicates support for the extensions to the event related to user data congestion, including support of GPSI and/or list of Top applications. Supporting this feature also requires the support of feature UserDataCongestion. |  |
| 12 |  | Aggregation |  | This feature indicates support for analytics aggregation. |  |
| 13 |  | NsiLoadExt |  | This feature indicates support for the extensions to the event related to the load level of Network Slice and the optionally associated Network Slice Instance, including support of area of interest, NF load information and number of UE or number of PDU Session. Supporting this feature also requires the support of feature NsiLoad. |  |
| 14 |  | ServiceExperienceExt |  | This feature indicates support for the extensions to the event related to service experience, including support of RAT type and/or Frequency. Supporting this feature also requires the support of feature ServiceExperience. |  |
| 15 |  | SMCCE |  | This feature indicates support for the event related to SM congestion control experience. |  |
| 16 |  | NfLoadExt |  | This feature indicates support for the extensions to the event related to the load of NF instances, including NF load over area of interest. Supporting this feature also required the support of feature NfLoad. |  |
| 17 |  | Dispersion |  | This feature indicates support for the event related to dispersion analytics information. |  |
| 18 |  | RedundantTransmissionExp |  | This feature indicates support for the event related to redundant transmission experience analytics information. |  |
| 19 |  | WlanPerformance |  | This feature indicates support of the event related to WLAN performance analytics information. |  |
| 20 |  | UeMobilityExt |  | This feature indicates support for extensions to the event related to UE mobility, including support of LADN DNN to refer the LADN service area as the AOI. Supporting this feature also requires the support of feature UeMobility. |  |
| 21 |  | DnPerformance |  | This feature indicates the support of the analytics related to DN performance. |  |
| 22 |  | AnaCtxTransfer |  | This feature indicates the support of analytics context transfer. |  |
| 23 |  | UserConsent |  | Indicates the support of detailed handling of user consent, e.g. error responses related to the lack of user consent. |  |
| 24 |  | PfdDetermination |  | Indicates the support of the analytics related to PFD Determination information of known application identifier(s). |  |  |

# *** 11th Change ***

A.3	Nnwdaf_AnalyticsInfo API

openapi: 3.0.0

info:

version: 1.3.0-alpha.1

title: Nnwdaf_AnalyticsInfo

description: |

Nnwdaf_AnalyticsInfo Service API.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.520 V18.0.0; 5G System; Network Data Analytics Services.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.520/'

security:

- {}

- oAuth2ClientCredentials:

- nnwdaf-analyticsinfo

servers:

- url: '{apiRoot}/nnwdaf-analyticsinfo/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

paths:

/analytics:

get:

summary: Read a NWDAF Analytics

operationId: GetNWDAFAnalytics

tags:

- NWDAF Analytics (Document)

parameters:

- name: event-id

in: query

description: Identify the analytics.

required: true

schema:

$ref: '#/components/schemas/EventId'

- name: ana-req

in: query

description: Identifies the analytics reporting requirement information.

required: false

content:

application/json:

schema:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/EventReportingRequirement'

- name: event-filter

in: query

description: Identify the analytics.

required: false

content:

application/json:

schema:

$ref: '#/components/schemas/EventFilter'

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: tgt-ue

in: query

description: Identify the target UE information.

required: false

content:

application/json:

schema:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/TargetUeInformation'

responses:

'200':

description: >

Containing the analytics with parameters as relevant for the requesting NF service

consumer.

content:

application/json:

schema:

$ref: '#/components/schemas/AnalyticsData'

'204':

description: No Content. The requested NWDAF Analytics data does not exist.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Indicates that the NWDAF Analytics resource does not exist.

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

description: >

The request is rejected by the NWDAF and more details (not only the ProblemDetails) are

returned.

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetailsAnalyticsInfoRequest'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/context:

get:

summary: Get context information related to analytics subscriptions.

operationId: GetNwdafContext

tags:

- NWDAF Context (Document)

parameters:

- name: context-ids

in: query

description: Identifies specific context information related to analytics subscriptions.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ContextIdList'

- name: req-context

in: query

description: >

Identfies the type(s) of the analytics context information the consumer wishes to receive.

required: false

content:

application/json:

schema:

$ref: '#/components/schemas/RequestedContext'

responses:

'200':

description: >

Contains context information related to analytics subscriptions corresponding with one or

more context identifiers.

content:

application/json:

schema:

$ref: '#/components/schemas/ContextData'

'204':

description: >

No Content. (\No context information could be retrieved for the requested context

Identifiers.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnwdaf-analyticsinfo: Access to the Nnwdaf_AnalyticsInfo API

schemas:

AnalyticsData:

description: >

Represents the description of analytics with parameters as relevant for the requesting NF

service consumer.

type: object

properties:

start:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeStampGen:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

anaMetaInfo:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/AnalyticsMetadataInfo'

sliceLoadLevelInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/SliceLoadLevelInformation'

minItems: 1

description: The slices and their load level information.

nsiLoadLevelInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NsiLoadLevelInfo'

minItems: 1

nfLoadLevelInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NfLoadLevelInformation'

minItems: 1

nwPerfs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NetworkPerfInfo'

minItems: 1

svcExps:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ServiceExperienceInfo'

minItems: 1

qosSustainInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/QosSustainabilityInfo'

minItems: 1

ueMobs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/UeMobility'

minItems: 1

ueComms:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/UeCommunication'

minItems: 1

userDataCongInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/UserDataCongestionInfo'

minItems: 1

abnorBehavrs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/AbnormalBehaviour'

minItems: 1

smccExps:

type: array

items:

$ref: '#/components/schemas/SmcceInfo'

minItems: 1

disperInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/DispersionInfo'

minItems: 1

redTransInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/RedundantTransmissionExpInfo'

minItems: 1

wlanInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/WlanPerformanceInfo'

minItems: 1

dnPerfInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/DnPerfInfo'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

EventFilter:

description: Represents the event filters used to identify the requested analytics.

type: object

properties:

anySlice:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/AnySlice'

snssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

description: Identification(s) of network slice.

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

dnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

ladnDnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

description: Identification(s) of LADN DNN to indicate the LADN service area as the AOI.

networkArea:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

visitedAreas:

type: array

items:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

minItems: 1

maxTopAppUlNbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxTopAppDlNbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

nfInstanceIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

nfSetIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

minItems: 1

nfTypes:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

minItems: 1

nsiIdInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NsiIdInfo'

minItems: 1

qosRequ:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/QosRequirement'

nwPerfTypes:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NetworkPerfType'

minItems: 1

bwRequs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/BwRequirement'

minItems: 1

excepIds:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ExceptionId'

minItems: 1

exptAnaType:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ExpectedAnalyticsType'

exptUeBehav:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExpectedUeBehaviourData'

ratFreqs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/RatFreqInformation'

minItems: 1

disperReqs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/DispersionRequirement'

minItems: 1

redTransReqs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/RedundantTransmissionExpReq'

minItems: 1

wlanReqs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/WlanPerformanceReq'

minItems: 1

listOfAnaSubsets:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/AnalyticsSubset'

minItems: 1

upfInfo:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/UpfInformation'

appServerAddrs:

type: array

items:

$ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AddrFqdn'

minItems: 1

dnPerfReqs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/DnPerformanceReq'

minItems: 1

not:

required: [anySlice, snssais]

ProblemDetailsAnalyticsInfoRequest:

description: >

Extends ProblemDetails to indicate more details why the analytics request is rejected.

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

- $ref: '#/components/schemas/AdditionInfoAnalyticsInfoRequest'

AdditionInfoAnalyticsInfoRequest:

description: Indicates additional information why the analytics request is rejected.

type: object

properties:

rvWaitTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

ContextData:

description: >

Contains context information related to analytics subscriptions corresponding with one or

more context identifiers.

type: object

properties:

contextElems:

type: array

items:

$ref: '#/components/schemas/ContextElement'

minItems: 1

description: >

List of items that contain context information corresponding with a context identifier.

required:

- contextElems

ContextElement:

description: Contains context information corresponding with a specific context identifier.

type: object

properties:

contextId:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/AnalyticsContextIdentifier'

pendAnalytics:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/EventNotification'

minItems: 1

description: >

Output analytics for the analytics subscription which have not yet been sent to the

analytics consumer.

histAnalytics:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/EventNotification'

minItems: 1

description: Historical output analytics.

lastOutputTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

aggrSubs:

type: array

items:

$ref: '#/components/schemas/SpecificAnalyticsSubscription'

minItems: 1

description: >

Information about analytics subscriptions that the NWDAF has with other NWDAFs to perform

aggregation.

histData:

type: array

items:

$ref: '#/components/schemas/HistoricalData'

minItems: 1

description: Historical data related to the analytics subscription.

adrfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

adrfDataTypes:

type: array

items:

$ref: '#/components/schemas/AdrfDataType'

minItems: 1

description: Type(s) of data stored in the ADRF by the NWDAF.

aggrNwdafIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

description: >

NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating

multiple analytics subscriptions.

modelInfo:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ModelInfo'

minItems: 1

description: >

Contains information identifying the ML model(s) that the consumer NWDAF is currently

subscribing for the analytics.

required:

- contextId

ContextIdList:

description: >

Contains a list of context identifiers of context information of analytics subscriptions.

type: object

properties:

contextIds:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/AnalyticsContextIdentifier'

minItems: 1

required:

- contextIds

HistoricalData:

description: Contains historical data related to an analytics subscription.

type: object

properties:

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

subsWithSources:

type: array

items:

$ref: '#/components/schemas/SpecificDataSubscription'

minItems: 1

description: Information about subscriptions with the data sources.

data:

type: array

items:

$ref: 'TS29575_Nadrf_DataManagement.yaml#/components/schemas/DataNotification'

minItems: 1

description: Historical data related to the analytics.

required:

- data

SpecificAnalyticsSubscription:

description: >

Represents an existing subscription for a specific type of analytics to a specific NWDAF.

type: object

properties:

subscriptionId:

type: string

producerId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

producerSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

nwdafEvSub:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscription'

allOf:

- oneOf:

- required: [producerId]

- required: [producerSetId]

- required: [subscriptionId]

- required: [nwdafEvSub]

RequestedContext:

description: Contains types of analytics context information.

type: object

properties:

contexts:

type: array

items:

$ref: '#/components/schemas/ContextType'

minItems: 1

description: List of analytics context types.

required:

- contexts

SmcceInfo:

description: Represents the Session Management congestion control experience information.

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

smcceUeList:

$ref: '#/components/schemas/SmcceUeList'

required:

- smcceUeList

SmcceUeList:

description: >

Represents the List of UEs classified based on experience level of Session Management

congestion control.

type: object

properties:

highLevel:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

mediumLevel:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

lowLevel:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

anyOf:

- required: [highLevel]

- required: [mediumLevel]

- required: [lowLevel]

SpecificDataSubscription:

description: >

Represents an existing subscription for data collection to a specific data source NF.

type: object

properties:

subscriptionId:

type: string

producerId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

producerSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

dataSub:

$ref: 'TS29575_Nadrf_DataManagement.yaml#/components/schemas/DataSubscription'

allOf:

- oneOf:

- required: [producerId]

- required: [producerSetId]

- required: [subscriptionId]

- required: [dataSub]

EventId:

anyOf:

- type: string

enum:

- LOAD_LEVEL_INFORMATION

- NETWORK_PERFORMANCE

- NF_LOAD

- SERVICE_EXPERIENCE

- UE_MOBILITY

- UE_COMMUNICATION

- QOS_SUSTAINABILITY

- ABNORMAL_BEHAVIOUR

- USER_DATA_CONGESTION

- NSI_LOAD_LEVEL

- SM_CONGESTION

- DISPERSION

- RED_TRANS_EXP

- WLAN_PERFORMANCE

- DN_PERFORMANCE

- PFD_DETERMINATION

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- LOAD_LEVEL_INFORMATION: Represent the analytics of load level information of corresponding network slice.

- NETWORK_PERFORMANCE: Represent the analytics of network performance information.

- NF_LOAD: Indicates that the event subscribed is NF Load.

- SERVICE_EXPERIENCE: Represent the analytics of service experience information of the specific applications.

- UE_MOBILITY: Represent the analytics of UE mobility.

- UE_COMMUNICATION: Represent the analytics of UE communication.

- QOS_SUSTAINABILITY: Represent the analytics of QoS sustainability information in the certain area.

- ABNORMAL_BEHAVIOUR: Indicates that the event subscribed is abnormal behaviour information.

- USER_DATA_CONGESTION: Represent the analytics of the user data congestion in the certain area.

- NSI_LOAD_LEVEL: Represent the analytics of Network Slice and the optionally associated Network Slice Instance.

- SM_CONGESTION: Represent the analytics of Session Management congestion control experience information for specific DNN and/or S-NSSAI.

- DISPERSION: Represents the analytics of dispersion.

- RED_TRANS_EXP: Represents the analytics of Redundant Transmission Experience.

- WLAN_PERFORMANCE: Represents the analytics of WLAN performance.

- DN_PERFORMANCE: Represents the analytics of DN performance.

- PFD_DETERMINATION: Represents the analytics of PFD Determination information for known application identifier(s).

ContextType:

anyOf:

- type: string

enum:

- PENDING_ANALYTICS

- HISTORICAL_ANALYTICS

- AGGR_SUBS

- DATA

- AGGR_INFO

- ML_MODELS

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- PENDING_ANALYTICS: Represents context information that relates to pending output analytics.

- HISTORICAL_ANALYTICS: Represents context information that relates to historical output analytics.

- AGGR_SUBS: Represents context information about the analytics subscriptions that an NWDAF has with other NWDAFs that collectively serve an analytics subscription.

- DATA: Represents context information about historical data that is available.

- AGGR_INFO: Represents context information that is related to aggregation of analytics from multiple NWDAF subscriptions.

- ML_MODELS: Represents context information about used ML models.

AdrfDataType:

anyOf:

- type: string

enum:

- HISTORICAL_ANALYTICS

- HISTORICAL_DATA

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- HISTORICAL_ANALYTICS: Indicates that historical analytics are stored in the ADRF.

- HISTORICAL_DATA: Indicates that historical data are stored in the ADRF.

# *** End of Changes ***