3GPP TSG-CT WG1 Meeting #101	C1-165055

# Reno, NV (USA), 14-18 November 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 2711 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to EMM-REGISTERED.UPDATE-NEEDED state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SAES5 |  |  |  |  |  |  | Date: |  | 2016-11-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | We have observed that the specification leads to misinterpretation to some UE designers with regards to use of tracking area updating (TAU) as a paging response. The sub-clause 5.1.3.2.4.6 EMM-REGISTERED.UPDATE-NEEDED states quote: No EMM procedure except tracking area updating or combined tracking area updating or service request as a response to paging shall be initiated by the UE in this substate. The above text can be interpreted as the UE can initiate either TAU or service request procedure as paging response when in the EMM- REGISTERED.UPDATE-NEEDED state. This raises the following points; how does the UE decide to use TAU or service request procedure as a paging response in this sub-state? If TAU procedure is used as a paging response in this sub-state, then this procedure should include the "active" flag. The fact is that though the TAU procedure can be used as a paging response this is only allowed for the case the UE is in EMM-REGISTERED.ATTEMPTING-TO-UPDATE state. However, there is no trigger in the TAU procedure as a paging response specified for EMM-REGISTERED.UPDATE-NEEDED state. In addition to this, Annex D does not cover the case of triggering TAU procedure as a paging response in EMM-REGISTERED.UPDATE-NEEDED state. In short, we believe that the problem (misinterpretation) comes due to the wording of the current sub-clause 5.1.3.2.4.6 is not good which needs correction to clear out any misinterpretation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of the EMM-REGISTERED.UPDATE-NEEDED state is clarified so that it is clear that only service request procedure can be used as a paging response in this sub-state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification creates problem to designers which leads to misinterpretation of the specification requirements. This can result in wrong implementations in the market in the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.2.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

###### 5.1.3.2.4.6	EMM-REGISTERED.UPDATE-NEEDED

The UE has to perform a tracking area updating or combined tracking area updating procedure, but access to the current cell is barred. This state can be entered if the access class is blocked due to access class control, or if the network rejects the NAS signalling connection establishment.

No EMM procedure except:

-	tracking area updating;

-	combined tracking area updating; or

-	service request as a response to paging

shall be initiated by the UE in this substate.