3GPP TSG-CT WG1 Meeting #117	C1-193294

# Reno (NV), USA, 13-17 May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 1204 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on follow-on request indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc16 |  |  |  |  |  | Date: |  |  | 2019-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the case b) for starting T3540, it covers the condition “2) the UE has set the Follow-on request indicator to 0 in the REGISTRATION REQUEST message”. However, there is no Follow-on indicator provided by the AMF in the REGISTRATION accept message. Also even the UE has not set the Follow-on request indicator in the REGISTRATION REQUEST message, once the there are downlink pending signalling and user data needs to be transported to the UE, the AMF will also prolong the NAS signalling connection. But the UE does not know this situation happened at the network side. In this case, if the UE still keeps the T3540 running, it will locally release the current NAS signalling connection at expiry of T3540 and then result in the ongoing downlink signalling and user data failure. Hence, in this case, the UE should stop the the running T3540 and respond to the network-initiated DL procedure via the existing NAS signalling connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It proposes that in case of the UE has not set the Follow-on request indicator in the REGISTRATION REQUEST message and started T3540, the the UE should stop the the running T3540 and respond to the network-initiated DL procedure via the existing NAS signalling connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | At the expiry of T3540, it will locally release the current NAS signalling connection and then result in the ongoing downlink signalling and user data failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.3.1.3	Release of the N1 NAS signalling connection

The signalling procedure for the release of the N1 NAS signalling connection is initiated by the network.

In N1 mode, upon indication from lower layers that the access stratum connection has been released, the UE shall enter 5GMM-IDLE mode and consider the N1 NAS signalling connection released.

If the UE in 3GPP access is configured for eCall only mode as specified in 3GPP TS 31.102 [22] then:

-	if the N1 NAS signalling connection that was released had been established for eCall over IMS, the UE shall start timer T3444; and

-	if the N1 NAS signalling connection that was released had been established for a call to an HPLMN designated non-emergency MSISDN or URI for test or terminal reconfiguration service, the UE shall start timer T3445.

To allow the network to release the N1 NAS signalling connection, the UE:

a)	shall start the timer T3540 if the UE receives any of the 5GMM cause values #7, #11, #12, #13, #15, #27, or #72;

b)	shall start the timer T3540 for a UE in 3GPP access if:

1)	the UE receives a REGISTRATION ACCEPT message;

2)	the UE has set the Follow-on request indicator to 0 in the REGISTRATION REQUEST message;

3)	the UE has not included the Uplink data status IE in the REGISTRATION REQUEST message, or the UE has included the Uplink data status IE in the REGISTRATION REQUEST message but the REGISTRATION ACCEPT message indicates that no user-plane resources of any PDU sessions are to be re-established;

4)	the UE has not included the Allowed PDU session status IE or has included the Allowed PDU session status IE indicating there is no PDU session(s) for which the UE allowed the user-plane resource to be re-established over 3GPP access in the REGISTRATION REQUEST message, or the UE has included the Allowed PDU session status IE in the REGISTRATION REQUEST message but the REGISTRATION ACCEPT message does not indicate that any user-plane resources of any PDU sessions are to be re-established;

5)	the registration procedure has been initiated in 5GMM-IDLE mode; and

6)	the user-plane resources for PDU sessions have not been set up;

NOTE 1:	The lower layers indicate when the user-plane resources for PDU sessions are successfully established or released.

c)	shall start the timer T3540 if the UE receives a REGISTRATION REJECT message indicating:

the 5GMM cause value #9 or #10;

d)	shall start the timer T3540 if the UE receives a SERVICE REJECT message indicating:

the 5GMM cause value #9 or #10;

e)	shall start the timer T3540 if:

1)	the UE receives a CONFIGURATION UPDATE COMMAND message containing the Configuration update indication IE with the Registration bit set to "registration requested" and with:

i)	either new allowed NSSAI information or new configured NSSAI information or both included;

ii)	the network slicing indication; or

iii)	no other parameters;

2)	the user-plane resources for PDU sessions have not been set up; and

3)	no emergency PDU session has been established; or

f)	shall start the timer T3540 if:

1)	the UE receives a SERVICE ACCEPT message;

2)	the UE has not included the Uplink data status IE in the SERVICE REQUEST message, or the UE has included the Uplink data status IE in the SERVICE REQUEST message but the SERVICE ACCEPT message indicates that no user-plane resources of any PDU sessions are to be re-established;

3)	the UE has not included the Allowed PDU session status IE or has included the Allowed PDU session status IE indicating there is no PDU session(s) for which the UE allowed the user-plane resource to be re-established over 3GPP access in the SERVICE ACCEPT message, or the UE has included the Allowed PDU session status IE in the SERVICE REQUEST message but the SERVICE ACCEPT message does not indicate that any user-plane resources of any PDU sessions are to be re-established;

4)	the service request procedure has been initiated in 5GMM-IDLE mode; and

5)	the user-plane resources for PDU sessions have not been set up.

NOTE 2:	The lower layers indicate when the user-plane resources for PDU sessions are successfully established or released.

Upon expiry of T3540,

-	in cases a), b) and f), the UE shall locally release the established N1 NAS signalling connection;

-	in cases c) and d) the UE shall locally release the established N1 NAS signalling connection and the UE shall initiate the registration procedure as described in subclause 5.5.1.3.5 or  5.6.1.5; or

-	in case e), the UE shall locally release the established N1 NAS signalling connection and perform a new registration procedure as specified in subclause 5.5.1.3.2.

In case a),

-	upon receiving a request from upper layers to send NAS signalling associated with emergency services fallback only for a UE in 3GPP access or establishing an emergency PDU session, the UE shall stop timer T3540 and shall locally release the N1 NAS signalling connection, before proceeding as specified in subclause 5.5.1.

In case b) and f),

-	upon an indication from the lower layers that the user-plane resources for PDU sessions are set up, the UE shall stop timer T3540 and may send uplink signalling via the existing N1 NAS signalling connection or user data via user plane. If the uplink signalling is associated with emergency services fallback only for a UE in 3GPP access or establishing an emergency PDU session, the UE shall stop timer T3540 and send the uplink signalling via the existing N1 NAS signalling connection;

-	upon receipt of a DEREGISTRATION REQUEST message, the UE shall stop timer T3540 and respond to the network-initiated de-registration request via the existing NAS signalling connection as specified in subclause 5.5.2.3;

-	upon receipt of a message of a network-initiated 5GMM common procedure, the UE shall stop timer T3540 and respond to the network-initiated 5GMM common procedure via the existing NAS signalling connection as specified in subclause 5.4;

-	upon receipt of a NOTIFICATION message over 3GPP access, the UE shall stop timer T3540 and respond to the NOTIFICATION message via the existing NAS signalling connection as specified in subclause 5.6.3;

-	upon receipt of a message of a network-requested 5GSM procedure, the UE shall stop timer T3540 and respond to the network-requested 5GSM procedure via the existing NAS signalling connection as specified in subclause 6.3;

-	upon receiving a request from upper layers to send NAS signalling associated with emergency services fallback only for a UE in 3GPP access or establishing an emergency PDU session, the UE shall stop timer T3540 and shall locally release the N1 NAS signalling connection, before proceeding as specified in subclause 5.6.1; or

-	upon receiving a request from upper layers to send NAS signalling not associated with emergency services fallback only for a UE in 3GPP access or establishing an emergency PDU session, the UE shall wait for the local release of the established N1 NAS signalling connection upon expiry of timer T3540 or wait for timer T3540 being stopped, before initiating NAS signalling.

In case c) and d),

-	upon an indication from the lower layers that the access stratum connection has been released, the UE shall stop timer T3540 and perform a new registration procedure as specified in subclause 5.5.1.3.5 or 5.6.1.5.

-	upon receiving a request from upper layers to send NAS signalling associated with emergency services fallback only for a UE in 3GPP access or establishing an emergency PDU session, the UE shall stop timer T3540 and shall locally release the N1 NAS signalling connection, before proceeding as specified in subclause 5.5.1.

In case e),

-	upon an indication from the lower layers that the access stratum connection has been released, the UE shall stop timer T3540 and perform a new registration procedure as specified in subclause 5.5.1.3.2.

-	upon an indication from the lower layers that the user-plane resources for PDU sessions are set up, the UE shall stop timer T3540 and may send user data via user plane.

-	upon receiving a request from upper layers to send NAS signalling associated with emergency services fallback only for a UE in 3GPP access or establishing an emergency PDU session, the UE shall stop timer T3540 and shall locally release the N1 NAS signalling connection, before proceeding as specified in subclause 5.6.1.

* * * End of Change * * * *