3GPP TSG-CT WG1 Meeting #122-e	C1-200808

- Electronic meeting; 20-28 February 2020											   was C1-200449

Source:	Samsung

Title:	Obtain list of users based on location

Spec:	3GPP TS 24.545 v0.2.0

Agenda item:	16.2.20

Document for:	Agreement

1. Introduction

This p-CR provides procedure to obtain list of users based on location for 3GPP TS 24.544 specification.

2. Reason for Change

The procedure has been defined in stage-2 specification 3GPP TS 23.434.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.545 v0.2.0.

* * * First Change * * * *

### 6.2.x	Query list of users based on location

#### 6.2.x.1	Client procedure

The procedure defined in this clause can be used by SGM-S to query list of users based on given geolocation area.

In order to query the list of users based on given geolocation area, the SGM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 2616 [6]. In the HTTP POST request message, the SGM-S:

a)	shall set the Request-URI to the URI corresponding to the identity of the SLM-S;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

c)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

1)	shall include an <identity> element with a <VAL-user-id> child element set to the identity of the SGM-S querying list of users; and

2)	shall include an <location-based-query> element with a <polygon-area> child element or an <ellipsoid-arc-area> child element.

* * * Next Change * * * *

#### 6.2.x.2	Server procedure

Upon reception of an HTTP POST request containing:

- a)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

b)	an application/vnd.3gpp.seal-location-info+xml MIME body with a < location-based-query> element included in the <location-info> root element;

the SLM-S:

a)	shall authorize the identity of the sender of the received HTTP POST request; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to obtain list of users based on given geolocation area, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

b)	shall generate the list of users who are currently available in requested geographical area; and

c)	shall send an HTTP 200 (OK) response message to SLM-C. In the HTTP 200 (OK) response message, the SLM-S:

1)	 shall generate an application/vnd.3gpp.seal-location-info+xml MIME body containing:

i)	an <identity> element with a <VAL-user-id> child element set to the identity of the SGM-S querying list of users; and

ii)	a <location-based-response> element which shall include:

A)	an <identities-list> element with one or more <VAL-user-id> child elements set to the identities of the VAL users to be queried;

* * * End of Change * * * *