3GPP TSG-CT WG1 Meeting #122-e	C1-200562

- Electronic meeting; 20-28 February 2020

Source:	Huawei, HiSilicon

Title:	MBMS bearer announcement over MBMS bearer procedure

Spec:	3GPP TS 24.548 v0.2.0

Agenda item:	16.2.20

Document for:	Agreement

1. Introduction

This p-CR defines the MBMS bearer announcement over MBMS bearer procedure for 3GPP TS 24.548.

2. Reason for Change

The specification needs to define the stage 3 details of the location information subscription procedure defined in 3GPP TS 23.434 clause 14.3.4.4.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.548 v0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[4]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[5]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[6]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[7]	IETF RFC 3428: "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[8]	IETF RFC 3841: "Caller Preferences for the Session Initiation Protocol (SIP)".

[9]	IETF RFC 4566: "Session Description Protocol".

[10]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[11]	OMA OMA-TS-XDM_Group-V1_1_1-20170124-A: "Group XDM Specification".

[r5795]	IETF RFC 5795: "The Robust Header Compression (ROHC) Framework".

* * * Next Change * * * *

#### 6.2.3.3	MBMS bearer announcement over MBMS bearer procedure

* * * Next Change * * * *

##### 6.2.3.3.1	General

The availability of a MBMS bearer is announced to SNRM-Cs by means of an MBMS bearer announcement message. One or more MBMS bearer announcement elements are included in an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body.

An MBMS bearer announcement message can contain new MBMS bearer announcements, updated MBMS bearer announcements or cancelled MBMS bearer announcements or a mix of all of them at the same time in an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body. Each initial MBMS bearer announcement message announces one MBMS bearer intended to carry a general purpose MBMS subchannel used for application level multicast signalling in a specified MBMS service area and additionally, the message could also announce zero or more extra MBMS bearers intended to carry media and media control.

NOTE 1:	A new MBMS bearer announcement does not implicitly remove previously sent MBMS bearer announcements if the previously sent MBMS bearer announcement is not included in an MBMS bearer announcement message.

NOTE 2:	The SNRM-C will use the same identity which has been authenticated by VAL service with SIP core using SIP based REGISTER message. If VAL service do not support SIP protocol, then HTTP based method needs to be used.

When and to whom the SNRM-S sends the MBMS bearer announcement is based on local policy in the SNRM-S.

* * * Next Change * * * *

##### 6.2.3.3.2	Server procedure

###### 6.2.3.3.2.1	MBMS bearer announcement procedure

For each SNRM-C that the SNRM-S is sending an MBMS bearer announcement to, the SNRM-S:

a)	shall generate an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body with the <version> element set to "1" and one or more <announcement> elements associated with the pre-activated MBMS bearers. Each set of an <announcement> element:

1)	shall include a <TMGI> element set to a TMGI value;

NOTE 1:	The same TMGI value can only appear in one <announcement> element. The TMGI value is also used to identify the <announcement> when updating or cancelling the <announcement> element.

NOTE 2:	The security key active for the general purpose MBMS subchannel on which the mapping (i.e. the Map Group To Bearer message) of media or media control to this MBMS bearer was indicated, is used for MBMS subchannels on this MBMS bearer, unless a different key or an indication of not using encryption is in place.

2)	may include an <alternative-TMGI> element set to a list of additional alternative TMGI used in roaming scenarios;

3)	may include the QCI value in the <QCI> element;

4)	shall include one or more MBMS service area IDs in <mbms-service-area-id> elements in the <mbms-service-areas> element;

NOTE 3:	Initial mappings of groups to MBMS subchannels on an MBMS bearer for the purpose of carrying media or media control can occur only where the MBMS service area for this bearer and the MBMS service area for the bearer carrying the general purpose MBMS subchannel on which the Map Group To Bearer message is sent intersect. However, once media or media control were successfully mapped to this bearer, the reception by the SNRM-C can continue (until Unmap Group To Bearer is received or until timeout) throughout the entire MBMS service area of this bearer.

5)	if multiple carriers are supported, shall include the frequency to be used in the <frequency> element;

NOTE 4:	In the current release if the <frequency> element is included, the frequency in the <frequency> element is the same as the frequency used for unicast.

6)	shall include a <seal-mbms-sdp> element set to the SDP with media and application control information applicable to groups that can use this bearer;

7)	may include a <monitoring-state> element set to "monitoring" or "not-monitoring" used to control if the client is actively monitoring the MBMS bearer quality or not;

8)	may include an <announcement-acknowlegement> element set to "true" or "false" indicating if the NRM server requires an acknowledgement of the MBMS bearer announcement;

9)	may include an <unicast-status> element set to "listening" or "not-listening" indicating if the listening status of the unicast bearer is requested;

10)	if the packet headers are compressed with ROHC specified in RFC 5795 [r5795] in this MBMS bearer, shall include a <seal-mbms-rohc> element;

6.2.3.3.2.1.1	SIP based procedure

If the VAL service supports SIP, the SNRM-S shall generate an SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [7] with the constructed application/vnd.3gpp.seal-mbms-usage-info +xml MIME body. In the SIP MESSAGE request, the SNRM-S:

a)	shall set the Request-URI to the URI received in the To header field in a third-party SIP REGISTER request;

b)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media-feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.seal" along with parameters "require" and "explicit" according to IETF RFC 3841 [8];

c)	shall include a P-Asserted-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.seal";

d)	shall include the MBMS public service identity of the SNRM-S in the P-Asserted-Identity header field;

e)	shall include in a MIME body with Content-Type header field set to "application/vnd.3gpp.seal-info+xml", the <seal-request-uri> element set to the VAL user ID of the user; and

f)	shall send the SIP MESSAGE request towards the SNRM-C according to 3GPP TS 24.229 [4];

6.2.3.3.2.1.2	HTTP based procedure

If the VAL service does not support SIP, the SNRM-S shall generate an HTTP POST request message in accordance with IETF RFC 2616 [6] with the constructed application/vnd.3gpp.seal-mbms-usage-info +xml MIME body. In the HTTP POST request message, the SNRM-S:

a)	shall set the Request-URI to the URI corresponding to the identity of the SNRM-C;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-mbms-usage-info +xml";

c)	shall include in a MIME body with Content-Type header field set to "application/vnd.3gpp.seal-info+xml", the <seal-request-uri> element set to the VAL user ID of the user; and

d)	shall send the HTTP POST request towards the SNRM-C according to IETF RFC 2616 [6].

#### 6.2.3.3.2.2	MBMS bearer de-announcement procedure

When the SNRM-S wants to cancel an MBMS bearer announcement associated with an <announcement> element, the SNRM-S sends an MBMS bearer announcement as specified in clause 6.2.3.3.2.1 where the SNRM-S in the <announcement> element to be cancelled. The SNRM-S:

a)	shall include the same TMGI value as in the <announcement> element to be cancelled in the <TMGI> element; and

b)	shall not include an <mbms-service-areas> element.

* * * Next Change * * * *

#### 6.2.3.3.3	Client procedure

Upon receiving a SIP MESSAGE request containing:

a)	a P-Asserted-Service header field containing the "urn:urn-7:3gpp-service.ims.icsi.seal"; and

b)	an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body containing one or more <announcement> element(s);

or an HTTP POST request message containing:

- a Content-Type header field set to "application/vnd.3gpp.seal-mbms-usage-info +xml"; and

b)	an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body containing one or more <announcement> element(s);

the SNRM-C for each <announcement> element in the application/vnd.3gpp.seal-mbms-usage-info+xml MIME body:

a)	if the <mbms-service-areas> element is present:

1)	if an <announcement> element with the same value of the <TMGI> element is already stored:

i)	shall replace the old <announcement> element with the <announcement> element received in the application/vnd.3gpp.seal-mbms-usage-info+xml MIME body;

2)	if there is no <announcement> element with the same value of the <TMGI> element stored:

i)	shall store the received <announcement> element;

3)	shall store the MBMS public service identity of the SNRM-S received in the P-Asserted-Identity header field and associate the MBMS public service identity with the new <announcement> element;

4)	if there is an <announcement-acknowlegement> element set to "true", shall send an acknowledgement of the MBMS bearer to the SNRM-S;

5)	shall check the condition for sending a listening status report; and

b)	if no <mbms-service-areas> element is present:

1)	shall discard a previously stored <announcement> element identified by the value of the <TMGI>; and

2)	check the condition for sending a listening status report.

* * * End of Change * * * *