3GPP TSG-CT WG1 Meeting #122	C1-200961

- Sophia Antipolis (France); 24-28 February 2020

Source:	Ericsson

Title:	Management object correction, MuD

Spec:	3GPP TS 24.175 V1.0.1

Agenda item:	16.3.3

Document for:	Agreement

1. Introduction

<Introduction part >

2. Reason for Change

An editor's note exist regarding the need for dynamic interior nodes. The analysis is that the MuD object does not need such nodes now as it is only one address foreseen for the call log URI.

For the MiD object it seems better to decouple the SharedIdentity and DelegatedIdentity so that the number of these leaves becomes independent of each other.

3. Conclusions

Reallocate the dynamic nodes in figure 4-1 to have no dynamic nodes for multi-device and have one each for the SharedIdentity and DelegatedIdentity leaves.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.175 V1.0.1.

* * * First Change * * * *

# 4	Multi-Device and Multi-Identity management object

The MO for Multi-Device and Multi-Identity in IMS is used to manage settings of the UE for Multi-Device and Multi-Identity in IMS. Figure 4-1 gives an overview of the configuration parameters of this MO. The conventions used in this specifications are as defined in OMA-TS-DM_TND [4] with the following modifications:

-	interior nodes are drawn as rectangles, not rectangles with rounded corners; and

-	leaf nodes are plain text, not rectangles.

The MO for Multi-Device and Multi-Identity in IMS covers configuration parameters for a UE supporting Multi-Device and Multi-Identity in IMS specified in TS 24.174 [3].

The MO identifier is: urn:oma:mo:ext-3gpp-mudmid:1.0.

Figure 4-1: MO for Multi-Device and Multi-Identity in IMS

* * * Next Change * * * *

## 5.4	/<X>/MultiIdentity/<X>

This interior node contains the multi-identity settings.

-	Occurrence: ZeroOrMore

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.5	/<X>/MultiIdentity/<X>/SharedIdentity

This leaf node contains a shared identity the UE can use.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: N/A

## 5.6	/<X>/MultiIdentity/<X>

This interior node contains the multi-identity settings.

-	Occurrence: ZeroOrMore

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.7	/<X>/MultiIdentity/<X>/DelegatedIdentity

This leaf node contains a delegated identity that is allowed to use the native identity.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: N/A

## 5.8	/<X>/MultiDevice

This interior node contains the multi-device parameters.

-	Occurrence: One

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.9	/<X>/MultiDevice/CallLogUri

This leaf node contains a URI the UE can use to access the call log.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Access Types: Get, Replace

-	Values: N/A

* * * Next Change * * * *

Annex A (informative) DDF of MO for Multi-Device and Multi-Identity in IMS

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version.

<?xml version="1.0" encoding="UTF-8"?>

<!DOCTYPE MgmtTree PUBLIC "-//OMA//DTD-DM-DDF 1.2//EN"

"http://www.openmobilealliance.org/tech/DTD/DM_DDF-V1_2.dtd">

<MgmtTree>

<VerDTD>1.2</VerDTD>

<Man>--The device manufacturer--</Man>

<Mod>--The device model--</Mod>

<Node>

<NodeName/>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<Description>Configuration parameters for the Multi-Device and Multi-Identity in IMS</Description>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<OneOrMore/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>The Management Object (MO) for Multi-Device and Multi-Identity in IMS.</DFTitle>

<DFType>

<DDFName>urn:oma:mo:ext-3gpp-mudmid:1.0</DDFName>

</DFType>

</DFProperties>

<Node>

<NodeName>MultiIdentity</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>Interior node containing multi-identity parameters</DFTitle>

<DFType><DDFName/></DFType>

</DFProperties>

<Node>

<NodeName/>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrMore/>

</Occurrence>

<Scope>

<Dynamic/>

</Scope>

<DFTitle>This interior node contains a list of shared identities</DFTitle>

<DFType><DDFName/></DFType>

</DFProperties>

<Node>

<NodeName>SharedIdentity</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>This leaf node contains a shared identity the UE can use</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

</Node>

<Node>

<NodeName/>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrMore/>

</Occurrence>

<Scope>

<Dynamic/>

</Scope>

<DFTitle>This interior node contains the multi-identity settings</DFTitle>

<DFType><DDFName/></DFType>

</DFProperties>

<Node>

<NodeName>DelegatedIdentity</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>This leaf node contains a delegated identity allowed to use the native identity</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

</Node>

</Node>

<Node>

<NodeName>MultiDevice</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>Interior node containing multi-device parameters</DFTitle>

<DFType><DDFName/></DFType>

</DFProperties>

<Node>

<NodeName>CallLogUri</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<Scope>

<Dynamic/>

</Scope>

<DFTitle>This leaf node contains the URI the UE needs to access the Call Log</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

</Node>

</Node>

</MgmtTree>

* * * Next Change * * * *

<Proposed change in revision marks>