3GPP TSG-CT WG1 Meeting #122-e	C1-20xxxx

# Electronic meeting; 20-28 February 2020

Source:	Motorola Mobility, Lenovo

Title:	MA PDU session is not supported

Spec:	3GPP TS 24.193

Agenda item:	16.2.5

Document for:	Agreement

1. Introduction

Subclause 4.1 of TS 24.193 is modified to include recent changes brought in to TS 23.501 via CR 2036. Approval of this document has dependency to CR 2036 against TS 23.501 and CR 1826 against TS 24.501.

2. Reason for Change

SA2 in Meeting #136-AH introduced a new requirement for the UE that the UE cannot initiate any ATSSS procedure unless the network has indicated to the UE its support for the ATSSS.

3. Conclusions

Subclause of TS 24.193 is modified to include the recent changes according to CR 2036 against 23.501.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.193.

* * * First Change * * * *

## 4.1	Introduction

ATSSS is an optional feature that can be supported by the UE and the 5GC network to route a data traffic across 3GPP access and non-3GPP access networks. An ATSSS capable UE establishes an MA PDU session supporting multi-access connectivity over 3GPP access and non-3GPP access networks as described in clause 4.2. The ATSSS capable UE may support ATSSS-LL and/or MPTCP steering functionality as described in clause 4.3, with associated steering modes, i.e. active-standby, smallest delay, load balancing, priority based. The ATSSS capable UE indicates the steering functionality and associated steering modes to the 5GC network.

When the ATSSS capable UE registers to a registration area, it receives an indication from the AMF if the network supports the ATSSS. The procedure for how the AMF indicates the UE about its ATSSS support is specified in 3GPP TS 24.501 [6]. The UE capable ATSSS and the network supporting ATSSS exchange access performance measurements as described in clause 4.4. Clause 4.5 describes the traffic distribution over 3GPP access and non-3GPP access networks. Clause 4.6 provides a description for interworking with EPS network. Clause 4.7 describes ATSSS when 5G-RG is interconnected with EPS.

The architecture reference model for ATSSS support is described in clause 4.2.10 of 3GPP TS 23.501 [2].

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>