3GPP TSG-CT WG1 Meeting #122-e	C1-201014

- Electronic meeting; 20-28 February 2020

Source:	MediaTek Inc.

Title:	Correction of release of user-plane resources

Spec:	3GPP TS 24.193 V1.0.1

Agenda item:	16.2.5

Document for:	Agreement

1. Introduction

In the current sub-clause 5.2.3, there are 3 bulltes. Bullet a) is network initiated releae procedure, bullet b) is UE initiated release procedure, but bullet c) is the common UE handling based on both bullet a) and b). The current structure and logic may cause confusion.

Also, based on the current design of the Access type IE, it indicates either 3GPP access or non-3GPP access, but not both. Thus the corresponding requirements in the current TS 24.193 need updates.

2. Reason for Change

- To make the release procedure clear, it is proposed to describe “release the MA PDU session” and “release the UP resources on an access type of the MA PDU session” independently.

  - Both the UE and the network can initiate the procedure for “release the MA PDU session”.

  - Only the network can initiate the procedure for “release the UP resources on an access type of the MA PDU session”.

- Based on the current design of Access type IE, it only indicates one access type (i.e., either 3GPP access or non-3GPP access).

3. Proposal

It is proposed to agree the following changes to 3GPP TS 3GPP TS 24.193 v1.0.1..

* * * First Change * * * *

### 5.2.3	Release of user-plane resources

In order to release the MA PDU session:

a)	the SMF shall initiate the network-requested PDU session release procedure as specified in clause 6.3.3.2 of 3GPP TS 24.501 [6] over 3GPP access or non-3GPP access, by sending the PDU SESSION RELEASE COMMAND message to the UE. Over which access to initiate this network-requested PDU session release procedure is SMF implementation specific; or

b)	the UE shall initiate the UE-requested PDU session release procedure as specified in clause 6.4.3.2 of 3GPP TS 24.501 [6] over 3GPP access or non-3GPP access by sending the PDU SESSION RELEASE REQUEST message to the network. Over which access to initiate this UE-requested PDU session release procedure is UE implementation specific.

When the UE receives the PDU SESSION RELEASE COMMAND message, the UE shall behave as specified in 3GPP TS 24.501 [6] clause 6.3.3.3.

In order to release the MA PDU session's user-plane resources on either 3GPP access or non-3GPP access,

the SMF shall initiate the network-requested PDU session release procedure as specified in clause 6.3.3.2 of 3GPP TS 24.501 [6] over 3GPP access or non-3GPP access, by sending the PDU SESSION RELEASE COMMAND message with the Access type IE indicating of which access the user-plane resources are released to the UE, e.g. when the AMF indicates to the SMF that the UE is deregistered over an access or when S-NSSAI of the MA PDU session is not in the Allowed NSSAI over an access. Over which access to initiate this network-requested PDU session release procedure is SMF implementation specific.

When the UE receives the PDU SESSION RELEASE COMMAND message, the UE shall behave as specified in 3GPP TS 24.501 [6] clause 6.3.3.3, and consider that the user plane resources on the access indicated in the Access type IE of the MA PDU session have been released.