3GPP TSG-CT WG1 Meeting #122-e	C1-200293

# Electronic meeting, 20-28 February 2020

Source:	Ericsson

Title:	Updates of configuration parameters for V2X communication over Uu

Spec:	3GPP TS 24.587 v1.0.1

Agenda item:	16.2.13

Document for:	Agreement

2. Reason for Change

The configuration parameters specified in 24.587 lack some of configuration parameters specified in 24.386, which were added to 24.386 as part of stage-3 improvements. Particularly:

1) In 24.386, the configuration parameters for the V2X communication over LTE-Uu (with exception of parameters for PDN connection establishment) are provided per PLMN. This is not possible in 24.587.

2) In 24.386, the configuration parameters for the V2X communication over LTE-Uu for transfer of a V2X message of a V2X service NOT identified by a V2X service identifier can be provided with or without dependency on geographical area. The possibility to provide them without dependency on geographical area is not available in 24.587.

3) In 24.386, the configuration parameters for the V2X communication over LTE-Uu for transfer of a V2X message of a V2X service identified by a V2X service identifier can be provided with or without dependency on geographical area. The possibility to provide them with dependency on geographical area is not available in 24.587.

4) In 24.386, the configuration parameters for the V2X communication over LTE-Uu for transfer of a V2X message of a V2X service identified by a V2X service identifier also contain a possiblity to provide a default V2X application server with or without dependency on geographical area. This is not available in 24.587.

5) In 24.386, the configuration parameters for the V2X communication over LTE-Uu for transfer of a V2X message of a V2X service identified by a V2X service identifier also contain a possiblity to use existing unicast routing. This is not available in 24.587.

6) In 24.386 and in 23.287, V2X application server for the V2X communication over LTE-Uu (or Uu) for transfer of a V2X message of a V2X service NOT identified by a V2X service identifier is identified by IP address or FQDN only (i.e. no port).

Aligning 24.386 and 24.587 would enable a simple handling when the UE supports V2X communication over Uu in EPS and in 5GS, uses the V2X communication over Uu in EPS and performs inter-system change from S1 mode to N1 mode (and vice versa).

Additionally:

7) there is no need to indicate whether to use TCP or UDP, if a UDP or TCP port is included.

8) both TCP and UDP ports can be provided for V2X application server for a V2X service identified by a V2X service identifier in 23.287.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.587.

Changes from C1-200293:

- "authorized" -> "configured"

***** change *****

### 5.2.4	Configuration parameters for V2X communication over Uu

The configuration parameters for V2X communication over Uu consist of:

a)	an expiration time for the validity of the configuration parameters for V2X communication over Uu to 5GCN;

Editor’s note:	The expiration time as part of the configuration parameters can be re-evaluated when the reply to the LS in C1-196967 arrives.

b)	a list of V2X service identifier to PDU session parameters mapping rules. Each mapping rule contains one or more V2X service identifiers of a the V2X service and one or more parameters for establishment of a PDU session for V2X communication over Uu for the V2X services:

1)	one of the "IPv4", "IPv6", "IPv4v6" or "Unstructured" PDU session types;

2)	an SSC mode;

3)	a list of zero or more S-NSSAIs; and

4)	a list of zero or more DNNs; and

c)	a list of PLMNs in which the UE is configured to use V2X communication over Uu. For each PLMN, the list contains:

1)	for transfer of a V2X message of a V2X service identified by a V2X service identifier:

i)	a list of V2X service identifier to V2X application server address mapping rules, applicable when the UE is registered to the PLMN. Each mapping rule contains:

A)	one or more V2X service identifiers;

B)	a V2X application server address for unicast consisting of:

-	an FQDN, or an IP address; and

-	a UDP port for uplink transport, a UDP port for downlink transport, a TCP port for bidirectional transport or any combination of them; and

C)	optionally a geographical area;

ii)	optionally per type of data (IP and non-IP) and V2X message family (in case of non-IP) and optionally a geographical area, one or more default V2X application server addresses for the unicast V2X communication over Uu applicable when the UE is registered to the PLMN. Each V2X application server address consists of:

i)	an FQDN, or an IP address; and

ii)	a UDP port for uplink transport, a UDP port for downlink transport, a TCP port for bidirectional transport or any combination of them; and

iii)	a list of V2X service identifiers of the V2X services configured for V2X communication over Uu using existing unicast routing; and

2)	for transfer of a V2X message of a V2X service not identified by a V2X service identifier:

i)	a list of the V2X application servers per optional geographical area where usage of those V2X application servers applies, applicable when the UE is registered to the PLMN. Each entry of the list contains:

A)	a V2X application server address consisting of

an FQDN, or an IP address; and

B)	optionally a geographical area;

.