3GPP TSG-CT WG1 Meeting #122-e	C1-20xxxx

# Electronic meeting, 20-28 February 2020	(was C1-200381)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 100 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct reference in 8.3.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16 |  |  |  |  |  | Date: |  |  | 24 February 2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 8.3.2.6 step 10 references clause 9.2.3.2 when discussing construction of the application/pidf+xml MIME body. However, it is clause 8.4.1 that covers construction of the application/pidf+xml MIME type. Clause 9.2.3.2 covers client procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the reference in 8.3.2.6 step 10 from 9.2.3.2 to 8.4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect reference in the spec and possible confusion on construction of the MIME type. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changed work item code. |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

#### 8.3.2.6	Sending affiliation status change towards MCData server owning MCData group procedure

NOTE 1:	Usage of one SIP PUBLISH request to carry information about change of affiliation state of several MCData users served by the same MCData server is not supported in this version of the specification.

In order:

-	to send an affiliation request of a served MCData ID to a handled MCData group ID;

-	to send an de-affiliation request of a served MCData ID from a handled MCData group ID; or

-	to send an affiliation request of a served MCData ID to a handled MCData group ID due to near expiration of the previously published information;

the MCData server shall generate a SIP PUBLISH request according to 3GPP TS 24.229 [5], IETF RFC 3903 [34] and IETF RFC 3856 [39]. In the SIP PUBLISH request, the MCData server:

1)	shall set the Request-URI to the public service identity of the controlling MCData function associated with the handled MCData group ID;

2)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body. In the application/vnd.3gpp.mcdata-info+xml MIME body, the MCData server:

a)	shall include the <mcdata-request-uri> element set to the handled MCData group ID; and

b)	shall include the <mcdata-calling-user-id> element set to the served MCData ID;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Asserted-Service header field according to IETF RFC 6050 [7];

4)	if sending an affiliation request, shall set the Expires header field according to IETF RFC 3903 [34], to 4294967295;

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [4].

5)	if sending an de-affiliation request, shall set the Expires header field according to IETF RFC 3903 [34], to zero;

6)	shall include an P-Asserted-Identity header field set to the public service identity of the MCData server according to 3GPP TS 24.229 [5];

7)	shall set the current p-id to a globally unique value;

8)	shall consider an MCData user information entry such that:

a)	the MCData user information entry is in the list of MCData user information entries described in subclause 8.3.2.2; and

b)	the MCData ID of the MCData user information entry is equal to the served MCData ID;

as the served MCData user information entry;

9)	for each MCData group information entry such that:

a)	the MCData group information entry has the "affiliating" affiliation status, the MCData group ID set to the handled MCData group ID, the expiration time has not expired yet and the affiliating p-id is not set;

b)	the MCData group information entry is in the list of the MCData group information entries of an MCData client information entry; and

c)	 the MCData client information entry is in the list of the MCData client information entries of the served MCData user information entry;

shall set the affiliating p-id of the MCData group information entry to the current p-id; and

10)	shall include an application/pidf+xml MIME body indicating per-group affiliation information constructed according to subclause 8.4.1. The MCData server shall indicate all served MCData client IDs, such that:

a)	the affiliation status is set to "affiliating" or "affiliated", and the expiration time has not expired yet in an MCData group information entry with the MCData group ID set to the handled MCData group;

b)	the MCData group information entry is in the list of the MCData group information entries of an MCData client information entry;

c)	the MCData client information entry has the MCData client ID set to the served MCData client ID; and

d)	the MCData client information entry is in the list of the MCData client information entries of the served MCData user information entry.

The MCData server shall set the <p-id> child element of the <presence> root element to the current p-id.

The MCData server shall not include the "expires" attribute in the <affiliation> element.

The MCData server shall send the SIP PUBLISH request according to 3GPP TS 24.229 [5].

If timer F expires for the SIP PUBLISH request sent for a (de)affiliation request of served MCData ID to the MCData group ID or upon receiving a SIP 3xx, 4xx, 5xx or 6xx response to the SIP PUBLISH request, the MCData server:

1)	shall remove each MCData group ID entry such that:

a)	the MCData group information entry has the MCData group ID set to the handled MCData group ID;

b)	the MCData group information entry is in the list of the MCData group information entries of an MCData client information entry; and

c)	 the MCData client information entry is in the list of the MCData client information entries of the served MCData user information entry.

* * * * * * * END CHANGES * * * * * * *