3GPP TSG-CT WG1 Meeting #122e	C1-200568

- Electronic meeting; 20-28 February 2020

Source:	CATT

Title:	Supplementary UE initiated location services operations

Spec:	3GPP TS 24.571 v1.0.0

Agenda item:	5G_eLCS

Document for:	Agreement

1. Introduction

Several UE initiated locaton services operations are still missing in subclause 5.2.2, 3GPP TS 24.571 v1.0.0.

2. Reason for Change

Increment the remaining UE initiated location services operations triggered by the UE, such as UE initiated Positioning Information Transport and EventReport operations.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.571 v1.0.0.

* * * First Change * * * *

## 5.1	General

This clause defines the NG-RAN LCS operations (subclause 5.2) and the format and coding of the messages (subclause 5.3).

The messages defined in this subclause can be transported in the UL/DL NAS Transport message defined in 3GPP TS 24.501 [3].

* * * Next Change * * * *

- 5.2.2.x	UE initiated Positioning Information Transport

The UE sends LPP message and the associated Routing Identifier in the UL NAS Transport message (refer to 3GPP TS 24.501 [3] and 3GPP TS 23.273 [2] clause 6.11.1]). Figure 5.2.2.x-1 illustrates an example of the NAS signalling transport for uplink LPP messages.

Figure 5.2.2.x-1: NAS signalling transport for uplink LPP messages

* * * Next Change * * * *

- 5.2.2.y	UE initiated Event Reporting Procedure

##### 5.2.2.y.1	General

The supplmentary services EventReport operation enables the UE to report the periodic or triggered location event invoked by the LMF via LCS PeriodicTriggered Invoke operation as described in clause 6.3.1 of 3GPP TS 23.273 [2] when some certain events are detected in the UE. The supplmentary services EventReport message is transferred to the LMF via the serving AMF in a UL NAS TRANSPORT message defined in 3GPP TS 24.501 [3]. A response from the LMF may be returned to the UE via the serving AMF and be transferred to the UE in a DL NAS TRANSPORT message.

Figure 5.2.2.y.1-1 illustrates an example of the NAS signalling transport for EventReport messages,

Figure 5.2.2.y.1-1: NAS signalling transport for EventReport messages

##### 5.2.2.y.2	Normal operation

The UE invokes an EventReport procedure by sending a REGISTER message containing an EventReport component to the LMF as defined in 3GPP TS 24.080 [5].

The LMF shall return a supplementary services acknowledgement of event report if it can handle this event report.

The LMF shall then return a RELEASE COMPLETE message containing an EventReport Acknowledge component (see Figure 5.2.2.y.2-1).

If the LMF is unable to process the request received from the UE or can not identify the ongoing periodic or triggered location to be terminated, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = LCS-EventReport (eventType, referenceNumberExt, h-gmlc-callBackUri, lcs-QoS, locationInfo, supportedGADShapes, multiplePositioningProtocolPDUs, terminationCause))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = LCS-EventReport Acknowledgement)

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.y.2-1: EventReport

NOTE:	Only the following IEs defined in EventReport operations in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	eventType

-	referenceNumberExt

-	h-gmlc-callBackUri

-	lcs-QoS

-	locationInfo

-	supportedGADShapes

-	multiplePositioningProtocolPDUs

-	terminationCause

* * * End of Changes * * * *