3GPP TSG-CT WG1 Meeting #123-e	C1-202298

- Electronic meeting; 16-24 April 2020

Source:	Huawei, HiSilicon

Title:	Request for modification of unicast resources procedure

Spec:	3GPP TS 24.548 v1.0.0

Agenda item:	16.2.20

Document for:	Agreement

1. Introduction

This p-CR defines the request for modification of unicast resources procedure for 3GPP TS 24.548.

2. Reason for Change

The specification needs to define the stage 3 details of the request for modification of unicast resources procedure defined in 3GPP TS 23.434 clause 14.3.3.2.2.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.548 v1.0.0.

* * * First Change * * * *

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[4]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[5]	3GPP TS 24.486: "Vehicle-to-Everything (V2X) Application Enabler (VAE) layer; Protocol aspects; Stage 3".

[6]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[7]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[8]	3GPP TS 29.549: "Service Enabler Architecture Layer (SEAL); Application Programming Interface (API) specification".

[9]	IETF RFC 3428: "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[10]	IETF RFC 3841: "Caller Preferences for the Session Initiation Protocol (SIP)".

[11]	IETF RFC 4566: "Session Description Protocol".

[12]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[13]	IETF RFC 5795: "The Robust Header Compression (ROHC) Framework".

[14]	OMA OMA-TS-XDM_Group-V1_1_1-20170124-A: "Group XDM Specification".

[r2616]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

* * * Next Change * * * *

#### 6.2.2.3	Request for modification of unicast resources procedure with SIP core

##### 6.2.2.3.1	VAL server procedure

To modify unicast bearers, the VAL server shall generate an HTTP POST request according to procedures specified in IETF RFC 2616 [r2616]. In the HTTP POST request message, the VAL server:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SNRM-S;

b)	shall include an Accept header fideld set to "application/vnd.3gpp.seal-unicast-info+xml";

c)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-unicast-info +xml";

d)	shall include an application/vnd.3gpp.seal-unicast-info+xml MIME body and in the <unicast-info> root element:

1)	shall include a <modification> element which shall include:

i)	a <requester-identity> element set to the identity of the VAL server performing the request;

ii)	an <identity> element set to the identity of the VAL user or VAL UE which requests the VAL service communication; and

iii)	an <requirement-info> element set to the modified unicast resource information; and

e)	shall send the HTTP POST request message towards the VAL server according to IETF RFC 2616 [r2616].

NOTE:	Before terminating connection due to no response from the SNRM-S, the VAL server allows sufficient time for the SNRM-S to reserve resources and respond. It is up to implementation to decide how long the VAL server waits for receiving response.

* * * Next Change * * * *

##### 6.2.2.3.1	Server procedure

Upon receiving an HTTP POST request message containing:

a)	an Accept header fideld set to "application/vnd.3gpp.seal-unicast-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-unicast-info +xml"; and

c)	an application/vnd.3gpp.seal-unicast-info+xml MIME body with a <modification> element in the <unicast-info> root element;

the SNRM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 6.2.1.1, and:

1)	if the identity of the sender of the received HTTP POST request is not authorized to modify unicast resource, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; and

2)	shall support handling an HTTP POST request from a VAL server according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	if the media bearer modification is not required, shall generate an HTTP 200 (OK) response message according to IETF RFC 2616 [r2616]. In the HTTP 200 (OK) response message, the SNRM-S:

1)	shall include a Request-URI set to the URI corresponding to the identity of the VAL server;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-unicast-info+xml";

3)	shall include an application/vnd.3gpp.seal-unicast-info+xml MIME body and in the <unicast-info> root element:

i)	shall include a <modification-result> element set to “failure” indicating failure of the resource modification request operation; and

4)	shall send the HTTP 200 (OK) response message towards the VAL server according to IETF RFC 2616 [r2616]; and

c)	if the media bearer modification is required, shall send a SIP MESSAGE request containing the modified parameters of the unicast bearer according to procedures specified in 3GPP TS 29.214 [6] for EPS and/or 3GPP TS 29.514 [7] for 5G.

Upon receiving a SIP 200 (OK) response to the SIP MESSAGE request, the SNRM-S:

a)	shall generate an HTTP 200 (OK) response message according to IETF RFC 2616 [r2616]. In the HTTP 200 (OK) response message, the SNRM-S:

1)	shall include a Request-URI set to the URI corresponding to the identity of the VAL server;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-unicast-info+xml"; and

3)	shall include an application/vnd.3gpp.seal-unicast-info+xml MIME body and in the <unicast-info> root element:

i)	shall include a <modification-result> element set to “success” indicating success of the resource modification request operation; and

b)	shall send the HTTP 200 (OK) response message towards the VAL server according to IETF RFC 2616 [r2616].

* * * End of Change * * * *