3GPP TSG-CT WG1 Meeting #124-e	C1-203569

- Electronic meeting; 2-10 June 2020

Source:	Huawei, HiSilicon

Title:	On-network dynamic group creation procedure

Spec:	3GPP TS 24.486 v1.1.0

Agenda item:	16.2.12

Document for:	Agreement

1. Introduction

This p-CR defines the on-network dynamic group creation procedure for 3GPP TS 24.486.

2. Reason for Change

The specification needs to define the stage 3 details of the on-network dynamic group creation procedure defined in 3GPP TS 23.286 clause 9.12.3.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.486 v1.1.0.

* * * First Change * * * *

### 6.10.1	On-network dynamic group creation procedure

#### 6.10.1.1	V2X application specific server procedure

In order to create a V2X group, the V2X application specific server shall generate an HTTP POST request message according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the V2X application specific server:

a)	shall include a Request-URI set to the URI corresponding to the identity of the VAE-S;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info +xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <configure-dynamic-group-request> element in the <VAE-info> root element which shall include:

1)	a <dynamic-group-info> element which shall include:

i)	a <dynamic-group-id> element set to the identity of the dynamic group;

ii)	a <group-leader-id> element set to the identity of the group leader; and

iii) a <group-definition> element indicating the conditions for creating the group; and

2)	an <endpoint-info> element set to the endpoint information to which the configure dynamic group notification has to be sent; and

d)	shall send the HTTP POST request message towards the VAE-S according to IETF RFC 2616 [19].

#### 6.10.1.2	Server procedure

Upon receiving an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info +xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with a <configure-dynamic-group-request> element in the <VAE-info> root element;

the VAE-S:

a)	shall assign a ProSe Layer-2 Group ID to the received dynamic group information from the available ProSe Layer-2 Group ID pool and generate an HTTP 200 (OK) response message according to procedures specified in IETF RFC 2616 [19]. In the HTTP 200 (OK) response message, the VAE-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info +xml";

2)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <configure-dynamic-group-result> element in the <VAE-info> root element indicating "success" or "failure" of the dynamic group creation;

3)	shall send the HTTP 200 (OK) response message towards the V2X application specific server according to IETF RFC 2616 [19].

Then the VAE-S shall generate an HTTP PUT request message according to procedures specified in IETF RFC 2616 [19]. In the HTTP PUT request message, the VAE-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the VAE-C of the group leader;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info +xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <layer2-group-id-mapping> element in the <VAE-info> root element which shall include:

1)	a <dynamic-group-info> element which shall include:

i)	a <dynamic-group-id> element set to the identity of the dynamic group;

ii)	a <group-leader-id> element set to the identity of the group leader; and

2)	a <prose-layer2-group-id> element corresponding to the dynamic group information; and

d)	shall send the HTTP PUT request message towards the VAE-C according to IETF RFC 2616 [19].

#### 6.10.1.3	Client procedure

Upon receiving an HTTP PUT request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info +xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with a <layer2-group-id-mapping> element in the <VAE-info> root element;

the VAE-C shall store the content of the <layer2-group-id-mapping> element and may further announce the dynamic group information including the corresponding ProSe Layer-2 Group ID to the other VAE clients within the PC5 communication proximity on a PC5 channel dedicated for V5-AE communications, enabling more V2X UEs to join the dynamic group.

* * * End of Change * * * *