3GPP TSG-CT WG1 Meeting #124-e	C1-203571

- Electronic meeting; 2-10 June 2020

Source:	Huawei, HiSilicon

Title:	On-network dynamic group notification procedure

Spec:	3GPP TS 24.486 v1.1.0

Agenda item:	16.2.12

Document for:	Agreement

1. Introduction

This p-CR defines the on-network dynamic group notification procedure for 3GPP TS 24.486.

2. Reason for Change

The specification needs to define the stage 3 details of the on-network dynamic group notification procedure defined in 3GPP TS 23.286 clause 9.12.5.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.486 v1.1.0.

* * * First Change * * * *

## 6.10	Dynamic group management procedure

* * * Next Change * * * *

### 6.10.2	On-network dynamic group notification procedure

#### 6.10.2.1	Client procedure

Once the on-network dynamic group is created as defined in subclause 6.10.1, if the group changes (i.e. UE joins or leaves the group), the VAE-C shall generate an HTTP POST request message according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the VAE-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the VAE-S;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info +xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with an <id-list-notification> element in the <VAE-info> root element which shall include:

1)	a <dynamic-group-id> element set to the identity of the dynamic group; and

2)	one or more <UE-id> element(s), each of which set to the identity of the joined or left V2X UE and with a <group-scope> attribute that has the value "joined" or "left"; and

d)	 shall send the HTTP POST request message towards the VAE-S according to IETF RFC 2616 [19].

* * * Next Change * * * *

#### 6.10.2.2	Server procedure

Upon receiving an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info +xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with an <id-list-notification> element in the <VAE-info> root element;

the VAE-S shall generate an HTTP POST request message according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the VAE-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the V2X application specific server;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info +xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <configure-dynamic-group-notification> element in the <VAE-info> root element which shall include:

1)	a <dynamic-group-id> element set to the identity of the dynamic group; and

2)	one or more <UE-id> element(s), each of which set to the identity of the joined or left V2X UE and with a <group-scope> attribute that has the value "joined" or "left"; and

d)	 shall send the HTTP POST request message towards the V2X application specific server according to IETF RFC 2616 [19].

* * * End of Change * * * *