3GPP TSG-CT WG1 Meeting #124-e	C1-203621

- Electronic meeting; 2-10 June 2020

Source:	Samsung

Title:	VAE specific extension of UE configuration

Spec:	3GPP TS 24.486 V1.1.0

Agenda item:	16.2.12

Document for:	Agreement

1. Introduction

This pCR provides VAE specific extension of SEAL UE configuration. The VAE client will receive VAE specific configuration from SEAL configuration management server.

2. Reason for Change

The SEAL configuration management specification 3GPP TS 24.546 provides common configuration parameters.

In 3GPP TS 23.286, Annex A.2 specifies VAE specific configurations which will be configured over CM-Uu interface. Following text and table are from Annex A.2 of 3GPP TS 23.286:

Data in table C.2-1 can be configured using the CM-UU reference point as specified in 3GPP TS 23.434 [6].

Table C.2-1: VAE client related configuration data

| Reference | Parameter description |
| --- | --- |
| Clause 8.1 and clause 9.1.2.2 | V2X user/UE identity |
| Clause 9.1.2.2 | V2X service ID |
| Clause 9.1.2.2 | V2X server USD |
| Clause 9.2.3 and Clause 9.1.2.3 | IP address and port information of the initial VAE server serving the VAE client |
| Clause 9.3.3 | GEO ID information |

The CM-Uu reference point is between SEAL Configuration Management Server (SCM-S) and SEAL Configuration Management Client (SCM-C). So, VAE specific configuration specifed in above table will be configured by SCM-S to SCM-C. So, it is required to provide VAE specific extension of SEAL UE Configuration document.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.486 v1.1.0.

* * * First Change * * * *

# X	VAE related configuration

## X.1	General

This clause specifies VAE specific configurations to be used along with common configurations defined in 3GPP TS 24.546 [11].

## X.2	VAE Client UE Configuration

### X.2.1	General

This clause specified the extension of the SEAL UE configuration document as defined in 3GPP TS 24.546 [11]. The procedure to retrieve configuration document is also specified in 3GPP TS 24.546 [11].

### X.2.2	Application unique ID

The AUID shall be set to the VAE service ID as specified in specified in ETSI TS 102 965 [17] or ISO TS 17419 [20].

### X.2.3	Structure

The VAE client configuration document structure is described in the clause 7.2 of 3GPP TS 24.546 [11] with the VAE specific clarifications specified in this subclause.

The <on-network> element of the <seal-UE-configuration> element specified in clause 7.2 of 3GPP TS 24.546 [11]:

a)	shall include a <VAE-server-ip> element;

b)	shall include a <VAE-server-transport-port> element;

c)	may include an <announcement> element as specified in clause 8; and

d)	may include a <geo-id> element as specified in clause 8.

### X.2.4	XML schema

#### X.2.4.1	General

The V2X UE configuration document is composed according the XML schema described in the clause 7.2 of 3GPP TS 24.546 [11], and extended with extensions from the XML schema defined in subclause X.2.4.2.

#### X.2.4.2	XML schema for V2X specific extensions

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns="urn:3gpp:ns:seal:V2XUEConfig:1.0"

targetNamespace="urn:3gpp:ns:seal:V2XUEConfig:1.0"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:v2xuec="urn:3gpp:ns:seal:v2xUEConfig:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified">

<!--V2X specific "on-network" child elements -->

<xs:element name="VAE-server-ip" type="xs:string"/>

<xs:element name="VAE-server-transport-port" type="xs:unsignedInt"/>

</xs:schema>

### X.2.5	Data semantics

The <VAL-UE-id> element in <seal-UE-configuration> element is V2X UE ID.

The <VAL-Service-id> element in <seal-UE-configuration> element is V2X service ID.

The <VAE-server-ip> element in <on-network> element of <seal-UE-configuration> element is IP address information of the initial VAE server serving the VAE client.

The <VAE-server-transport-port> element in <on-network> element of <seal-UE-configuration> element is port information of the initial VAE server serving the VAE client.

The <announcement> element contains V2X server USD as specified in clause 8.

The <geo-id> element contains GEO ID information as specified in clause 8.

### X.2.6	MIME types

The MIME type for the VAE UE configuration document shall use the MIME type as specified in the clause 7.2.6 of 3GPP TS 24.546 [11].

* * * End of Change * * * *