- 3GPP TSG CT WG1 Meeting 124-e	TDoc C1-203862

- Electronic meeting, 2-10 June 2020

Title:	LS on mandate to provide "any PLMN" entry in the non-3GPP access node selection information

Release:	Rel-16

Work Item:	5GProtoc16

Source:	CT1#124-e

To:	SA2

Contact person:	John-Luc Bakker

JBakker@blackberry.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N.A.

# 1	Overall description

Some companies in CT1 have discussed the requirement that mandates the inclusion of the "any PLMN" entry in the non-3GPP access node selection information (TS 23.501, clause 6.3.6).

The equivalent of the "any PLMN" entry in the ePDG selection information is optional (see TS 23.402).

According to the N3IWF selection procedure described in clause 6.3.6 of TS 23.501, a mandatory inclusion of the "any PLMN" entry in the N3AN selection information makes impossible some roaming scenarios involving HPLMN's non-3GPP access (e.g. the scenarios presented in Figures 4.2.8.2.2-2 and 4.2.8.2.3-2 of TS 23.501). An example of conditions where a roaming scenario involving HPLMN's non-3GPP access would be impossible:

-	N3AN node selection for IMS service is performed;

-	the UE is not located in its home country;

-	the UE attached to EPS of a VPLMN X; and

-	the N3AN node configuration information is provisioned;

-	an N3AN node selection information entry for the VPLMN X is absent in the N3AN node selection information of the N3AN node configuration information;

-	the DNS server function is able to resolve N3IWF FQDN or ePDG FQDN constructed according to the 'Any_PLMN' N3AN node selection information entry of the N3AN node configuration information.

For such scenarios, unless selection of the N3IWF or ePDG in the visited country is required, using the HPLMN's non-3GPP access should be enabled and controlled by the HPLMN's policy. This is impossible to achieve with the N3IWF selection procedure described in clause 6.3.6 of TS 23.501.

# 2	Actions

To SA2

ACTION: 	3GPP TSG CT WG1 asks SA2 group to clarify the purpose of the mandatory presence of the "any PLMN" entry in the non-3GPP access node selection information and comment on the scenarios disabled by that.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 125	USA	24-28 August 2020

TSG CT WG1 Meeting 126	India	12-16 October 2020