3GPP TSG-CT WG1 Meeting #125-e	C1-205250

Electronic meeting, 20-28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6430 | rev | 3 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 24.229 P-CSCF and UE MPS priority upgrade |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Perspecta Labs, CISA ECD, AT&T, T-Mobile USA, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS2 |  |  |  |  |  | Date: |  |  | 2020-08-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR supports the following stage 1 requirements: TS 22.153 clause 9.1.2.1: The system shall support means for a Service User using a UE with an MPS subscription to: - initiate MPS for an MMTEL voice call or an MPS MMTEL voice conference call, or - request upgrade of an established MMTEL voice call, or an established MMTEL voice conference call to MPS. The system shall support means for a Service User using a UE with an MPS subscription to: - initiate MPS for an MMTEL voice call, or an MPS MMTEL voice conference call when roaming within the home country and outside the home country, or - requests upgrade of an established MMTEL voice call, or an established MMTEL voice conference call to MPS when roaming within the home country and outside the home country. TS 22.153 clause 9.2.2.1: The system shall support means for a Service User using a UE with an MPS subscription to: - initiate MPS for an MMTEL video call or an MPS MMTEL video conference call, or - request upgrade of an established MMTEL video call, or an established MMTEL video conference call to MPS. The system shall support means for a Service User using a UE with an MPS subscription to: - initiate MPS for an MMTEL video call, or an MPS MMTEL video conference call when roaming within the home country and outside the home country, or - request upgrade of an established MMTEL video call, or an established MMTEL video conference call to MPS when roaming within the home country and outside the home country. Currently, TS 24.229 does not support receiving a change to the RPH value during a session. Changes to the P-CSCF are required to support the feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First change: For the UE-originating case, the following statement is added to the P-CSCF Subsequent Requests clause: "The P-CSCF shall adjust the priority treatment of transactions or dialogs according to the most recently received Resource-Priority header field value." Second change: For the UE-terminating case, the P-CSCF needs to expect a changing RPH during a dialog. The following statement is added: "The P-CSCF shall adjust the priority treatment of transactions or dialogs according to the most recently received Resource-Priority header field value and set the backwards indication accordingly." |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs will not receive a priority upgrade during a call. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.9.1, 5.2.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 5.2.9.1	UE-originating case

The P-CSCF shall respond to all reINVITE requests with a 100 (Trying) provisional response.

For a reINVITE request or UPDATE request from the UE within the same dialog, the P-CSCF shall include the updated access-network-charging-info parameter from P-Charging-Vector header field when sending the SIP request to the S-CSCF. See subclause 5.2.7.4 for further information on the access network charging information.

For an ACK request from the UE sent on a dialog where a 200 (OK) has been received, the P-CSCF shall include the access-network-charging-info parameter from the P-Charging-Vector header field when updated access-network-charging-info is available when sending the ACK request to the S-CSCF. See subclause 5.2.7.4 for further information on the access network charging information.

If priority is supported, the P-CSCF shall adjust the priority treatment of transactions or dialogs according to the most recently received authorized Resource-Priority header field value.

Editor's note:	[WI: MPS2, CR#6530] More study is needed as to how the changed priority is signalled to the P-CSCF.

***** Second change *****

#### 5.2.9.2	UE-terminating case

The P-CSCF shall respond to all reINVITE requests with a 100 (Trying) provisional response.

For a reINVITE request or UPDATE request destined towards the UE within the same dialog, when the P-CSCF sends 200 (OK) response (to the INVITE request or UPDATE request) towards the S-CSCF, the P-CSCF shall include the updated access-network-charging-info parameter in the P-Charging-Vector header field. See subclause 5.2.7.4 for further information on the access network charging information.

If priority is supported, the P-CSCF shall adjust the priority treatment of transactions or dialogs according to the most recently received authorized Resource-Priority header field value and set the backwards indication accordingly.

Editor's note:	[WI: MPS2, CR#6530] More study is needed as to how the changed priority is signalled to the P-CSCF.