- 3GPP TSG CT WG1 Meeting 125-e	TDoc C1-20xxxx

- Electronic meeting, 20-28 August 2020

Title:	LS on handling OVERLOAD START message in the N3IWF

Response to:	-

Release:	Rel-17

Work Item:	5GProtoc17

Source:	CT1

To:	RAN3

Cc:	-

Contact person:	Sung Hwan WON

sung.won@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	C1-20xxxx

# 1	Overall description

CT1 has agreed the attached CR including the following sentence with an understanding that RAN3 could possibly update TS 29.413 accordingly.

After the N3IWF receives from the UE an IKE_AUTH request message, the N3IWF shall construct an IKE_AUTH response message including a CONGESTION Notify payload as defined in subclause 9.2.4.2 and a N3GPP_BACKOFF_TIMER Notify payload as defined in subclause 9.3.1.7 if the N3IWF decides to not accept the IKE SA and signalling IPsec SA establishment based on the OVERLOAD START message received from the AMF(s) as specified in 3GPP TS 29.413 [39].

It is CT1’s view that the N3IWF operation which is slightly different from the NG-RAN operation specified in clause 8.7.7 of TS 38.413 should be clarified in TS 29.413 considering that available values for the establishment cause for non-3GPP access are (see clause 9.2.2 of TS 24.502):

- emergency;

- highPriorityAccess;

- mo-Signalling;

- mo-Data;

- mps-PriorityAccess; and

- mcs-PriorityAccess.

# 2	Actions

To RAN3

ACTION: 	CT1 respectfully request RAN3 to review if update in TS 29.413 is required and update TS 29.413 (if needed).

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 126e	TBD

TSG CT WG1 Meeting 127e	TBD