3GPP TSG-CT WG1 Meeting #125-e	C1-20wxyz

Electronic meeting, 20-28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | Spec# | CR | CR# | rev | - | Current version: | Version# |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New use case for MuD and MiD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MuDe |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | New release of this work item allows new features and flexibilities for terminating the calls. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new fucntionalities such as caller preferencse and callee capabilties. Added GRUU functionalities. Added new call flows |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New use cases are not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3.1, 4.5.3.6, A.3.X (new), A.3.X.1 (new), A.3.X.2 (new), A.3.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------ Next Change ---------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[3]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[4]	3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[5]	IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[6]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Network Asserted Identity within Trusted Networks".

[7]	3GPP TS 24.623: "Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[8]	IETF draft-ietf-stir-passport-divert-08 (March  2020): "PASSporT Extension for Diverted Calls".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[9]	OMA-TS-CPM_Message_Storage_Using_RESTFul_API-V1_0-20181025-D: "CPM Message Store using RESTFul API, Draft Version 1.0 – 25 Oct 2018",
http://member.openmobilealliance.org/ftp/Public_documents/COM/COM-CPM/Permanent_documents/OMA-TS-Message_Storage_Using_RESTFul_API-V1_0-20181025-D.zip.

[10]	OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C: "RESTful Network API for Network Message Storage, Candidate Version 1.0 – 28 May 2019",
http://member.openmobilealliance.org/ftp/Public_documents/ARCH/Permanent_documents/OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C.zip.

[11]	3GPP TS 24.629: "Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[12]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[13]	3GPP TS 24.175: "Management Object (MO) for Multi-Device and Multi-Identity in IMS; Stage 3".

[14]	3GPP TS 23.003: "Numbering, addressing and identification".

[XX]	IETF RFC 3841: "Caller Preferences for the Session Initiation Protocol (SIP)".

[YY]	IETF RFC 5627: "Obtaining and Using Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol (SIP)".

------------------------------------------ Next Change ---------------------------------------

#### 4.5.3.1	Actions at the UE of user A

If user A wishes to use a native identity, the UE shall include in the outgoing INVITE or MESSAGE request the From header field and may include the P-Preferred-Identity header field(s) as specified in TS 24.229 [3]. The From header field and the P-Preferred-Identity header field (if included) shall contain the native identity.

If user A wishes to use an alternative identity or a virtual identity that the UE has registered, the UE shall include in the outgoing INVITE or MESSAGE request the From header field and the P-Preferred-Identity header field. The P-Preferred-Identity header field and the From header field shall contain the alternative identity or the virtual identity.

If user A wishes to reach an identity with certain capabilities, the UE shall use Accept-Contact header field and Reject-Contact header field as described in IETF RFC 3841 [XX].

If user A wishes to use the identity C, the UE shall include in any outgoing INVITE or MESSAGE requests, an Additional-Identity header field, defined in TS 24.229 [3], set to the selected identity and shall include the native identity in the From header field. The UE can learn the identity C by device configuration as specified in TS 24.175 [13] or by using the service configuration in clause 4.8.

The UE may support being configured with the identities to be used in the "MultiIdentity" leaf node of TS 24.175 [13].

When establishing an emergency session and performing the emergency related procedures defined in TS 24.229 [3], the UE shall only use the native identity.

A UE supporting the MuD service may synchronize the local call log with the network stored call log as specified in OMA-TS-CPM_Message_Storage_Using_RESTFul_API [9] and OMA-TS-REST_NetAPI_NMS [10]. If the served user in the "From" header field is an identity not registered by the UE, the UE shall deduce that the call was originated using the Additional-Identity header field.

------------------------------------------ Next Change ---------------------------------------

#### 4.5.3.6	Actions at the UE of user B

A UE supporting the MiD service shall support the receipt of the Additional-Identity header field, defined in TS 24.229 [3], in SIP requests initiating a dialog or standalone transaction.

NOTE:	The UE finds a targeted external alternative identity in the Additional-Identity header field.

A UE supporting the MuD service may synchronize the local call log with the network stored call log as specified in OMA-TS-CPM_Message_Storage_Using_RESTFul_API [9] and OMA-TS-REST_NetAPI_NMS [10]. If the served user in the "To" header field is an identity not registered by the UE, the UE shall deduce that the call was originated using the Additional-Identity header field.

A UE supporting MuD and MiD services shall support GRUU and GRUU management as defined in IETF RFC 5627 [YY].

A UE supporting MuD and MiD services shall support callee capabilities registration as defined in IETF RFC 3841 [XX].

------------------------------------------ Next Change ---------------------------------------

## A.3.X	UE-B reached by native identity on multiple devices and multiple identities

------------------------------------------ Next Change ---------------------------------------

#### A.3.X.1	Devices are registered the same contact

This call flow illustrates the handling of the terminating call case when the request is forwarded towards the user who has subscribed to the MuD and MiD services with two devices registered the same contact and using identities B and C as native identities for those devices. The requested identity is B in this call flow.

Figure A.3.X.1-1: UE-B with same contact as UE-C, reached by a native identity on multiple devices and multiple identities

1.	S-CSCF-B receives a SIP INVITE request from PLMN-A, for an example see table A.3.X.1-1

The Request-URI identifies user B as B-native.

Table A.3.X.1-1: SIP INVITE request (PLMN-A to I/S/P-CSCF-B)

INVITE tel:+11112222 SIP/2.0

To: <tel:+11112222>

From: <tel:+11111111>;tag=4fa3

P-Asserted-Identity: <sip:+11111111@plmnA.net;user=phone>, <tel:+11111111>

Other SIP header fields and SDP according to 3GPP TS 24.229 [3]

2.	S-CSCF-B forwards the SIP INVITE request to AS-B.

The AS-B determines that it shall forward the SIP INVITE request towards a UE-B for which the identity in Request-URI is a native identity.

3.	AS-B forwards the SIP INVITE request to S-CSCF-B.

The S-CSCF-B replaces B identity with the UE contact in the Request-URI and adds a P-Called-Party-ID header field set to identity B.

4.	S-CSCF-B forwards the INVITE request to UE-B.

Multiple devices with the same contact receive the SIP INVITE request and determines the call is to be terminated by the device using identity B due to P-Called-Party-ID header field being set to B.

5.	UE-B responds with SIP 200 (OK) response.

6.	S-CSCF-B forwards the SIP 200 (OK) response to AS-B.

7.	AS-B forwards the SIP 200 (OK) response to the S-CSCF-B.

8.	S-CSCF-B forwards the SIP 200 (OK) response to PLMN-A.

9.	S-CSCF-B receives SIP ACK request from PLMN-A.

10.	S-CSCF-B forwards the SIP ACK request to AS-B.

11.	AS-B forwards the SIP ACK request to S-CSCF-B.

12.	S-CSCF-B forwards the ACK request to UE-B.

------------------------------------------ Next Change ---------------------------------------

#### A.3.X.2	Devices are registered different instances of the same contact

This call flow illustrates the handling of the terminating call case when the request is forwarded towards the user who has subscribed to the MuD and MiD services with two devices registered by 2 instances of the same contact and using the same identity B as native identities for those devices. The requested identity is B in this call flow.

Figure A.3.X.2-1: An instance of UE-B contact reached by a native identity on multiple devices and multiple identities

1.	S-CSCF-B receives a SIP INVITE request from PLMN-A, for an example see table A.3.X.2-1

The Request-URI identifies user B as B-native.

Table A.3.X.2-1: SIP INVITE request (PLMN-A to I/S/P-CSCF-B)

INVITE tel:+11112222 SIP/2.0

To: <tel:+11112222>

From: <tel:+11111111>;tag=4fa3

P-Asserted-Identity: <sip:+11111111@plmnA.net;user=phone>, <tel:+11111111>

Other SIP header fields and SDP according to 3GPP TS 24.229 [3]

2.	S-CSCF-B forwards the SIP INVITE request to AS-B.

The AS-B determines that it shall forward the SIP INVITE request towards a UE-B for which the identity in Request-URI is a native identity.

3.	AS-B forwards the SIP INVITE request to S-CSCF-B.

UE instance 1 with identity B, UE-B instance 1, registered instance 1 of UE contact according to IETF RFC 56427 [YY] with an instance-id based on GSMA IMEI URN and a q-value between 0 and 1. UE-B instance 1 registered certain capabilities as described in IETF RFC 3841 [XX]. Registered capabilities or the q-value allows the S-CSCF-B to forward the SIP INVITE request towards UE-B instance 1. The S-CSCF-B replaces B identity with the registered instance of the UE contact in the Request-URI and adds a P-Called-Party-ID header field set to identity B.

UE instance 2 with identity B, UE-B instance 2, registered instance 2 of UE contact according to IETF RFC 56427 [YY] with another instance-id based on GSMA IMEI URN and a q-value between 0 and 1. UE-B instance 2 registered certain capabilities as described in IETF RFC 3841 [XX]. Registered capabilities or the q-value does not allow the S-CSCF-B to forward the SIP INVITE request towards UE-B instance 2. The S-CSCF-B replaces B identity with the registered instance of the UE contact in the Request-URI and adds a P-Called-Party-ID header field set to identity B.

4.	S-CSCF-B forwards the INVITE request to UE-B instance 1.

5.	UE-B instance 1 responds with SIP 200 (OK) response.

6.	S-CSCF-B forwards the SIP 200 (OK) response to AS-B.

7.	AS-B forwards the SIP 200 (OK) response to the S-CSCF-B.

8.	S-CSCF-B forwards the SIP 200 (OK) response to PLMN-A.

9.	S-CSCF-B receives SIP ACK request from PLMN-A.

10.	S-CSCF-B forwards the SIP ACK request to AS-B.

11.	AS-B forwards the SIP ACK request to S-CSCF-B.

12.	S-CSCF-B forwards the ACK request to UE-B instance 1.

------------------------------------------ Next Change ---------------------------------------

#### A.3.X.3	Devices are registered different contacts

This call flow illustrates the handling of the terminating call case when the request is forwarded towards the user who has subscribed to the MuD and MiD services with two devices registered by 2 contacts UE1 or UE 2 and using identities B and C as alternative identities for those devices. The requested identity is B in this call flow.

Figure A.3.X.3-1: UE-B with different contact from UE-C, reached by an alternate identity on multiple devices and multiple identities

1.	S-CSCF-B receives a SIP INVITE request from PLMN-A, for an example see table A.3.X.3-1

The Request-URI identifies user B as B-native.

Table A.3.X.3-1: SIP INVITE request (PLMN-A to I/S/P-CSCF-B)

INVITE tel:+11112222 SIP/2.0

To: <tel:+11112222>

From: <tel:+11111111>;tag=4fa3

P-Asserted-Identity: <sip:+11111111@plmnA.net;user=phone>, <tel:+11111111>

Other SIP header fields and SDP according to 3GPP TS 24.229 [3]

2.	S-CSCF-B forwards the SIP INVITE request to AS-B.

The AS-B determines that it shall forward the SIP INVITE request towards a UE-B for which the identity in Request-URI is an alternative identity.

3.	AS-B forwards the SIP INVITE request to S-CSCF-B.

UE1 with identity B, UE1-B, registered a q-value between 0 and 1 and certain capabilities as described in IETF RFC 3841 [XX]. Registered capabilities or the q-value allows the S-CSCF to forward the SIP INVITE request towards UE1-B. The S-CSCF-B replaces B identity with the registered instance of the UE1 contact in the Request-URI and adds a P-Called-Party-ID header field set to identity B.

UE2 with identity C, UE1-C, registered a q-value between 0 and 1 and certain capabilities as described in IETF RFC 3841 [XX]. Registered capabilities or the q-value also allows the S-CSCF-B to forward the SIP INVITE request towards UE1-C. The S-CSCF-B replaces B identity with the registered instance of the UE2 contact in the Request-URI and adds a P-Called-Party-ID header field set to identity B.

4.	S-CSCF-B forwards the INVITE request to UE1-B and UE2-C.

UE1-B with identity B terminates the call since P-Called-Party-ID header field being set to B.

UE2-C with identity C does not terminate the call since P-Called-Party-ID header field being set to B.

NOTE:	S-CSCF-B forwards the SIP INVITE request to UE2-C since B is an alternative identity for UE2.

5.	UE1-B responds with SIP 200 (OK) response.

6.	S-CSCF-B forwards the SIP 200 (OK) response to AS-B.

7.	AS-B forwards the SIP 200 (OK) response to the S-CSCF-B.

8.	S-CSCF-B forwards the SIP 200 (OK) response to PLMN-A.

9.	S-CSCF-B receives SIP ACK request from PLMN-A.

10.	S-CSCF-B forwards the SIP ACK request to AS-B.

11.	AS-B forwards the SIP ACK request to S-CSCF-B.

12.	S-CSCF-B forwards the ACK request to UE1-B.

------------------------------------------ End of Change ---------------------------------------