- 3GPP TSG CT WG1 Meeting 125-e	TDoc C1-205223

- Electronic meeting, 20-28 August 2020

Title:	[Draft] LS on LS on resume procedure on CAG cell

Release:	Release 16

Work Item:	Vertical_LAN

Source:	CT1

To:	SA2

Cc:	RAN2

Contact person:	Kundan Tiwari

k.tiwari@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT1 has discussed following resume scenario on a CAG cell. CTI would like to clarify how the UE is updated with latest Allowed CAG list in this scenario to avoid paging failure.

- A UE is in RRC_INACTIVE state.

- The UE performs resume procedure on a CAG which CAG ID is in the UE “Allowed CAG list” but this CAG ID subscription has expired and the UE and network are not in sync.

- The NG-RAN will reject the Resume procedure.

- The UE moves to RRC IDLE state and remains on the CAG cell as the CAG ID is still in Allowed CAG list.

- The network will not page the UE in the CAG cell as it is not in the allowed CAG list.

# 2	Actions

To SA

ACTION: 	3GPP TSG CT WG1 seeks SA2 guidance how a UE is update with the latest Allowed CAG list so that the UE moves to suitable cell.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 126e	TBD

TSG CT WG1 Meeting 127e	TBD