3GPP TSG-CT WG1 Meeting #126-e	C1-206313

Electronic meeting, 15-23 October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 2774 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage of initial CAG information list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc17 |  |  |  |  |  | Date: |  |  | 2020-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed during CT1#125-e, there is a deployment demand to enable a UE supporting CAG but without CAG information list to access a CAG cell in automatic network selection mode to get the up-to-date CAG information list: 1. Special industrial MEs cannot be pre-configured with CAG information list when leaving factory, as it’s an operator dependent information. 2. These special industrial MEs cannot camp on a non-CAG cell to get CAG information list, as machine-only zones are not permitted to deploy non-CAG cell and thus non-CAG cell are not available to these MEs. 3. These special industrial MEs arrive in batches irregularly.The dates are business sensitive information.The customers disagree to notify the operator to do the configuration of these MEs for every purchase. 4. Manual selection mode is seen as unsafe factor by customers. The above demand can be solved by: - enabling HPLMN to configure the initial CAG information list in the USIM; and - enabling the ME to configure the CAG information list stored in the ME with the initial CAG information list stored in the USIM when a new USIM is inserted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When a new USIM is inserted, if the "initial CAG information list" is stored in the USIM, the ME supporting CAG configures the "CAG information list" stored in the ME with the "initial CAG information list" stored in the USIM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The deployment demand not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.14.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** change *****

### 4.14.3	Public network integrated non-public network (PNI-NPN)

A PNI-NPN is made available by means of e.g. dedicated DNNs or by one or more S-NSSAIs allocated for it. A CAG can be optionally used in order to prevent UEs not allowed to access a PNI-NPN from accessing the PNI-NPN. The key enablers for the CAG in the NAS layer are as follows:

a)	CAG selection (see 3GPP TS 23.122 [5]); and

b)	provisioning of a "CAG information list" as specified in 3GPP TS 23.122 [5], from network to UE via the generic UE configuration update procedure, the registration procedure, and the service request procedure. These procedures shall not impact "CAG information list" stored in the USIM.

The UE supporting CAG may perform the initial registration for emergency services via a non-CAG cell in a PLMN for which the UE has an "indication that the UE is only allowed to access 5GS via CAG cells" or via a CAG cell that is not included in the "Allowed CAG list" (see 3GPP TS 23.122 [5]) for the selected PLMN. If a UE supporting CAG having an emergency PDU session is camping on:

a)	a CAG cell and none of the CAG-IDs of the CAG cell are included in the "Allowed CAG list" for the current PLMN in the UE's subscription; or

b)	a non-CAG cell in a PLMN for which the UE's subscription contains an "indication that the UE is only allowed to access 5GS via CAG cells";

the AMF shall release all non-emergency PDU sessions associated with 3GPP access, if any. The AMF shall not release the emergency PDU session.

NOTE:	The emergency services for CAG only UE can be subject to local regulation.