3GPP TSG-CT WG1 Meeting #127bis-e	C1-210xxx

- Electronic meeting; 25-29 January 2021												(was C1-210116)

Source:	Qualcomm Incorporated

Title:	Solution to MINT Key Issue #7 (Prevention of signalling overload in PLMNs without Disaster Condition) – Alternative 1: providing disaster roaming assistance information to distribute roamers, and congestion mitigation

Spec:	TR 24.811 v0.1.0

Agenda item:	17.2.9

Document for:	Agreement

1. Introduction

At CT1#127-e, Key Issue #7 on prevention of signalling overload in PLMNs without Disaster Condition was agreed into MINT TR 24.811. The purpose of this contribution is to propose a solution for this Key Issue.

2. Reason for Change

Key Issue #7 includes the following questions to be studied:

1)	How to distribute the subscribers of the PLMN with Disaster Condition between the PLMNs without Disaster Condition available in the area where the Disaster Condition applies, so as to share the load as evenly as possible between the PLMNs without Disaster Condition;

2)	How to stagger the arrival of UEs in the PLMNs without Disaster Condition, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit;

3)	How to use new Access Identity 3 for the purpose of Disaster Inbound Roamer access control and signalling overload prevention in the PLMNs without Disaster Condition; and

4)	How to enable a PLMN without Disaster Condition to efficiently prevent Disaster Inbound Roamers from attempting registration on the PLMN when the PLMN can no longer accept Disaster Inbound Roamers due to congestion.

This solution addresses questions 1), 2) and 4).

To address question 1):

It is proposed to specify a mechanism to distribute the Disaster Inbound Roamers among the available  PLMNs that can accommodate Disaster Inbound Roamers, consisting of providing the roamers with assistance information enabling them to pick a specific PLMN for disaster inbound roaming. The assistance information is composed of either:

- a prioritized list of PLMNs;

- a weighted list of PLMNs, where each PLMN is assigned a weight that can e.g. be conmensurate to its capactiy to accommodate roamers; or

- an indication of the capacity of PLMNs without Disaster to accept Disaster Inbound Roamers.

To address question 2):

It is proposed to specify that upon selecting a new PLMN following the notification that a Disaster Condition applies to the registered PLMN, the Disaster Inbound Roamers wait for a randomized amount of time within a pre-configured range before initiating registration on the selected PLMN.

Using a randomly drawn timer value avoids requiring the network to configure a different timer value for each UE, and drawing from a range ensures that the UE does not wait longer than a specific upper bound, to keep the interruption of service within a known limit.

To address question 4):

It is proposed to specify that if a PLMN accommodating Disaster Inbound Roamers encounters a congestion situation, the PLMN can turn off the indication that it can accommodate Disaster Inbound Roamers, as well as reject the registration requests or service requests from Disaster Inbound Roamers which have already selected the PLMN, either with an existing 5GMM cause value accompanied by back-off timer T3346, or with a new 5GMM cause value indicating to the UE that the UE shall attempt to select another PLMN for disaster roaming.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811 v0.1.0.

* * * First Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| B |  |  |  |  |  |  | X |  |

* * * Next Change * * * *

## 6.B	Solution #B: Solution for prevention of signalling overload in PLMNs without Disaster Condition by providing disaster roaming assistance information to distribute roamers, and congestion mitigation

### 6.B.1	Introduction

This is a solution for Key Issue #7 (Prevention of signalling overload in PLMNs without Disaster Condition).

The solution addresses the first, second and fourth questions to be studied under Key Issue #7, namely:

-	How to distribute the subscribers of the PLMN with Disaster Condition between the PLMNs without Disaster Condition available in the area where the Disaster Condition applies, so as to share the load as evenly as possible between the PLMNs without Disaster Condition;

-	How to stagger the arrival of UEs in the PLMNs without Disaster Condition, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit; and

-	How to enable a PLMN without Disaster Condition to efficiently prevent Disaster Inbound Roamers from attempting registration on the PLMN when the PLMN can no longer accept Disaster Inbound Roamers due to congestion.

The solution assumes that PLMNs without Disaster Condition which indicate that they can accept Disaster Inbound Roamers will accept Disaster Inbound Roamers from any PLMN with Disaster Condition, up until the point when they reach a congestion situation.

### 6.B.2	Detailed description

#### 6.B.2.1	Distribution of subscribers of the PLMN with Disaster Condition between the PLMNs without Disaster Condition

The UE can be provisioned with disaster roaming assistance information, which the UE will use to select a PLMN upon being notified that a Disaster Condition applies to the registered PLMN.

The disaster roaming assistance information can be:

a)	pre-configured in the ME;

b)	pre-configured in the USIM;

c)	sent to the UE by the network using the UE parameters update procedure (before a Disaster Condition applies);

d)	sent to the UE by the network using the steering of roaming procedure (before a Disaster Condition applies); or

e)	signalled to the UE by the PLMNs without Disaster Condition (when a Disaster Condition applies).

Only the disaster roaming assistance information provisioned by the network is used by the UE, if both a disaster roaming assistance information provisioned by the network and a pre-configured disaster roaming assistance information are present. If no disaster roaming assistance information is provisioned by the network, and the UE has a pre-configured disaster roaming assistance information in both the USIM and the ME, then only the pre-configured disaster roaming assistance information in the USIM is used.

The disaster roaming assistance information can consist of:

a)	a prioritized list of PLMNs. In this case, the UE attempts registration on the available PLMNs from the list in priority order;

NOTE 1:	The prioritized list of PLMN can be UE-specific, so as to direct a first group of UEs to PLMN 1, a second group of UEs to PLMN 2, and so on.

b)	a weighted list of PLMNs. In this case, the UE performs a weighted random draw among the available PLMNs from the list; or

NOTE 2:	The weight assigned to each PLMN can be commensurate to the size of the PLMN or the capacity of the PLMN to accommodate Disaster Inbound Roamers.

c)	an indication of the capacity of PLMNs without Disaster to accept Disaster Inbound Roamers e.g broadcast by the PLMNs which indicate that they can accommodate Disaster Inbound Roamers. In this case, the UE performs a weighted random draw among the available PLMNs taking into account the capacity of each PLMN.

Editor's note: Extension of broadcast signalling is subject to agreement of RAN WGs.

Editor's note:	Input from SA3 is needed regarding potential security risks resulting from using broadcast signalling to convey information related to disaster roaming.

#### 6.B.2.2	Staggering of arrival of Disaster Inbound Roamers

The UE can be provisioned with a disaster roaming wait range, which the UE will use to determine how long to wait before initiating registration on the selected PLMN after performing a PLMN selection triggered by a notification that a Disaster Condition applies to the registered PLMN.

The disaster roaming wait range can be:

a)	pre-configured in the ME;

b)	pre-configured in the USIM;

c)	sent to the UE by the network using the UE parameters update procedure (before a Disaster Condition applies); or

d)	sent to the UE by the network using the steering of roaming procedure (before a Disaster Condition applies).

Editor's note:	How the network can determine the disaster roaming wait range value before a Disaster Condition applies is FFS.

Only the disaster roaming wait range value n provisioned by the network is used by the UE, if both a disaster roaming wait range value provisioned by the network and a pre-configured disaster roaming wait range value are present. If no disaster roaming wait range value is provisioned by the network, and the UE has a pre-configured disaster roaming wait range value in both the USIM and the ME, then only the pre-configured disaster roaming wait range value in the USIM is used.

Upon selecting a candidate PLMN following a notification that a Disaster Condition applies to the registered PLMN, the UE shall generate a random number within the disaster roaming wait range by applying a hash function to its IMSI and start a timer set to the generated random number. While the timer is running, the UE shall not initiate registration. Upon expiration of the timer, the UE shall initiate registration on the selected PLMN. If no disaster roaming wait range is provisioned to the UE, the UE shall use a default disaster roaming wait range.

Editor's note:	The default disaster roaming wait range is FFS.

#### 6.B.2.3	Congestion mitigation in PLMNs without Disaster Condition

Upon encountering congestion due to the arrival of Disaster Inbound Roamers, a PLMN without Disaster Condition indicating that it can accommodate Disaster Inbound Roamers may perform one or more of the following:

a)	turn off the indication (e.g. SIB flag) that the PLMN can accommodate Disaster Inbound Roamers, so as to prevent new roamers from considering the PLMN as PLMN selection candidate;

Editor's note: Extension of broadcast signalling is subject to agreement of RAN WGs.

Editor's note:	Input from SA3 is needed regarding potential security risks resulting from using broadcast signalling to convey information related to disaster roaming.

b)	reject the Disaster Inbound Roamers’ registration or service requests with 5GMM cause #22 "congestion" and back-off timer T3346; or

c)	reject the Disaster Inbound Roamers’ registration or service requests with 5GMM cause #YYY "disaster inbound roamers not allowed".

Editor's note:	5GMM cause #YYY "disaster inbound roamers not allowed" can result into a UE trying in sequence to register with all PLMNs offering Disaster Roaming to the UE and getting rejected in all those PLMNs. It is FFS how to avoid such situation.

Editor's note:	It is FFS whether 5GMM cause #22 "congestion" is sufficient instead of using 5GMM cause #YYY "disaster inbound roamers not allowed".

Upon receiving a REGISTRATION REJECT or SERVICE REJECT with 5GMM cause #YYY "disaster inbound roamers not allowed", the UE shall perform PLMN selection as specified in the solution(s) for Key Issue #5 (PLMN selection when a "Disaster Condition" applies) with the addition that the UE shall not consider the selected PLMN as candidate for PLMN selection.

The UE should maintain a list of PLMNs where 5GMM cause # YYY "disaster inbound roamers not allowed" was received:

-	upon receiving 5GMM cause # YYY "disaster inbound roamers not allowed", the UE should add the identity of the PLMN to the list of PLMNs where 5GMM cause # YYY "disaster inbound roamers not allowed" was received and should start timer TX if timer TX is not already running. The number of PLMNs that the UE can store where 5GMM cause # YYY "disaster inbound roamers not allowed" was received is implementation specific, but it shall be at least one. The value of timer TX is UE implementation specific;

-	in automatic PLMN selection the UE shall not consider PLMNs where 5GMM cause # YYY "disaster inbound roamers not allowed" was received as PLMN selection candidates, unless no other PLMN is available; and

-	the UE shall delete stored information on PLMNs where 5GMM cause # YYY "disaster inbound roamers not allowed" was received when the USIM is removed, timer TX expires or the UE is notified that the Disaster Condition no longer applies as specified in solution(s) to Key Issue #6 (Notification that Disaster Condition is no longer applicable to the UEs).

### 6.B.3	Impacts on existing nodes and functionality

UE

-	support for handling of disaster roaming assistance information;

-	support for handling of disaster roaming wait range; and

-	support for 5GMM cause #YYY "disaster inbound roamers not allowed".

UDM of HPLMN

-	optionally, support for providing disaster roaming assistance information; and

-	optionally, support for providing disaster roaming wait range.

AMF of registered PLMN (with Disaster Condition)

- optionally, support for providing disaster roaming assistance information; and

- optionally, support for providing disaster roaming wait range.

AMF of PLMNs indicating that they can accept Disaster Inbound Roamers:

-	optionally, support for providing disaster roaming assistance information; and

-	optionally, support for 5GMM cause #YYY "disaster inbound roamers not allowed".

* * * End of Changes * * * *