3GPP TSG-CT WG1 Meeting #127-bis-e 	C1-210231

Electronic meeting, 25-29 January 2021

Source:	Thales, BlackBerry UK Ltd.

Title:	Solution 2 description enhancement

Spec:	3GPP TR 24.821

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

In TR 24.821, Key Issue 2, CT1 agreed to address the following issues:

1)	how the UE determines which PLMN(s) can be selected in determined country of its physical location (if it is determined that the UE is not in an international area);

2)	what information could be available to the UE to help in selecting, in a given country, a PLMN that meets the LI requirements of that country:

a)	pre-configured in the UE; and

b)	provided by PLMN;

3)	what modifications are needed, if any, to the PLMN selelction procedure to take the LI requirement into account.

In the TR solutions 2 and solution 3 are proposed in a complementary way to tackle the issues with options 2)a) and 2)b).

2. Reason for Change

It is necessary to complement solution 2 to specify how the indications provided by PLMN may interact with information pre-configured in the UE.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.821 v0.2.0.

* * * First Change * * * *

- 6.3	Solution 2: Determining PLMNs that can be selected in the country of UE location based on USIM configuration

- 6.3.1	Target key issue

This is a candidate solution to Key Issue 2. It is relevant to the deployment scenario A and B (B1 through B3).

- 6.3.2	Solution Description

It is assumed that the UE knows in which country it is located. How this is achieved is a topic in Key Issue 1.

For each PLMN in the "HPLMN selector with Access Technology" and "User Controlled PLMN Selector with Access Technology" and "Operator Controlled PLMN Selector with Access Technology" files in USIM, there can be configured a list of countries in which the PLMN can be selected. This list is optional for each PLMN. If the list is not configured for a PLMN, the PLMN is considered as selectable only in the country of its MCC, unless the PLMN has a shared PLMN ID (MCC 9xx), in which case the PLMN is considered as selectable in all countries.

NOTE:	How the list of countries in which the PLMN can be selected is encoded in USIM is left to CT6 to discuss. The encoding should suppport all the relevant deployment scenarios.

Below is an example of the proposed USIM configuration. In this example, the new proposed list of countries is implemented as an additional column D in the existing Operator Controlled PLMN Selector with Access Technology file. How exactly it ends up being implemented would be up to CT6 to decide.

Table 6.3-1: HPLMN selector with Access Technology file in USIM according to the solution

| A. Priority | B. PLMN ID | C. RAT(s) supported | D. Allowed in following countries (MCC) |
| --- | --- | --- | --- |
| 1 | 310 070 (AT&T USA) | E-UTRA, NR, NR_GEO | Canada (302), Mexico (334) (NOTE 1) |
| NOTE 1: A PLMN is always allowed in its home country |  |  |  |

Table 6.3-2: Operator Controlled PLMN Selector with Access Technology file in USIM according to the solution

| A. Priority | B. PLMN ID | C. RAT(s) supported | D. Allowed in following countries (MCC) |
| --- | --- | --- | --- |
| 1 | 310 270 (Verizon USA) | E-UTRA, NR | Empty (NOTE 1) (NOTE 2) |
| 2 | 302 720 (AT&T Canada) | E-UTRA, NR, NR_GEO | USA (310) (NOTE 1) |
| 3 | 334 50 (AT&T Mexico) | E-UTRA, NR, NR_GEO | USA (310) (NOTE 1) |
| 4 | 901 44 (AT&T global) | NR_GEO | Empty (NOTE 3) |
| 5 | 901 88 (XYZ) | NR_GEO | China (460) (NOTE 1) |
| NOTE 1: A PLMN is always allowed in its home country NOTE 2: Column D may not be applicable to PLMNs that do not support NTN access NOTE 3: If column D is empty, a PLMN with a shared MCC is allowed in all countries |  |  |  |

Editor's note:	The following behavior aims to specify the impact of (S2-2009484) onto PLMN selection procedure.

In case the network (AMF) rejects the registration, the AMF may provide a list of possible MCCs that could be used during subsequent PLMN selection, given the UE location.

If the UE receives in the registration reject an optional list of possible MCCs for use during subsequent PLMN selection, the UE shall consider this list together with USIM files content as valid for subsequent NTN PLMN selection procedures, until next registration reject or a registration accept.

For the next PLMN selections following a registration reject and until another registration reject or a registration accept, existing PLMN procedures as defined in TS 23.122 are applied, considering also the list of MCCs when selecting an NTN PLMN, that is adding the condition that MCC shall be proposed as possible by the network.

Editor's note:	the complete conditions for the UE to use and discard the MCCs received from AMF are FFS.

- 6.3.3	Impacts

UE impacts:

-	New optional UE configuration in USIM indicating the countries in which each PLMN can be selected.

Network impacts:

- Determination of the MCC list by ad-hoc SA WGs