3GPP TSG-CT WG1 Meeting #129-e	C1-212239

- Electronic meeting; 19th – 23rd April

Source:	Qualcomm Incorporated

Title:	Evaluations for Solutions for Key Issue 2

Spec:	3GPP TR 24.821

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

TR 24.821 contains four solution for key issue #2. No new solutions for this key issue is expected to be added to the TR.

2. Reason for Change

There is a need for evalution of the solutions for key issue #2.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.821 v1.0.0.

* * * First Change * * * *

## 7.2	Evaluation of Solutions for Key Issue #2

Table 7.2-1 Evaluation of the solutions to KI#2

| Solution | Summary | Evaluation |
| --- | --- | --- |
| 2 | For each PLMN in the "HPLMN selector with Access Technology" and "User Controlled PLMN Selector with Access Technology" and "Operator Controlled PLMN Selector with Access Technology" files in USIM, there can be configured a list of countries in which the PLMN can be selected. This list is optional for each PLMN. If the list is not configured for a PLMN, the PLMN is considered as selectable only in the country of its MCC, unless the PLMN has a shared PLMN ID (MCC 9xx), in which case the PLMN is considered as selectable in all countries. In other aspects, the UE follows the legacy PLMN selection procedure. | HPLMN continues to control the PLMN selection for satellite access via USIM. It allows for various deployment scenarios for a PLMN (country-specific MCC, extraterritorial MCC, global/shared MCC). The UE impact is new optional USIM configuration. In other aspects, the UE follows the legacy PLMN selection procedure. This solution is not compatible with solutions 3 and 19. |
| 3 | The mapping (a country to one or more MCCs) information provisioned in the UE is used to determine the MCC(s) which can be selected from the country. PLMN(s) with such MCC(s) are allowed to operate and can be selected in the country. How to provision this information to the UE is outside the scope of 3GPP in this solution. Key differences compared to the current PLMN selection procedure are as follows. 1) The UE prioritizes PLMNs with the MCC(s) corresponding (based on the provisioned mapping) to the country of its physical location. 2) While a UE is in not only in a VPLMN but also in the (E)HPLMN, if the UE detects change in the country of its present location, the UE needs to attempt to operate as specified in bullet 1) in addition to periodically scanning a higher priority PLMN with the same MCC as the currently serving PLMN's MCC. | The solution allows for various deployment scenarios for a PLMN (country-specific MCC, extraterritorial MCC, global/shared MCC). The UE impact is new optional UE configuration (outside of scope of 3GPP). It is not specified who controls the UE provisioning for PLMN selection for satellite access or how the HPLMN controls it. If country A wants to allow a PLMN with MCC B (country B) to operate in country A, it will automatically allow all PLMNs with MCC B to operate in country A. This solution is not compatible with solution 2. |
| 19 | If the UE receives an optional list of MCCs from the AMF for use during subsequent PLMN selection, the UE shall consider this list together with USIM files content as valid for subsequent selection of PLMNs providing satellite NG-RAN. PLMN procedures as defined in 3GPP TS 23.122 [3] are applied. If the UE receives an optional list of MCCs from the AMF for use during subsequent PLMN selection, the UE shall exclude PLMNs whose MCC is not present in the list received from the AMF, except for the shared/global PLMN. At the end of a time period, UE shall discard any list of MCCs received from the AMF. | The solution limits the selected PLMN to either the MCC of the country of the UE location as indicated by the AMF or shared MCC 9xx. The solution does not describe how the UE determines whether a PLMN with MCC 9xx can be selected at the UE location or not. This solution is not compatible with solution 2. |

Questions for discussion:

1. Should the UE configuration to aid the UE in determining which PLMNs are allowed at the UE location be:

a) Configured in USIM;

b) Provisioned in the UE (by means outside of scope of 3GPP)

* * * No more changes * * * *