3GPP TSG-CT WG1 Meeting #129-e	C1-212284

Electronic meeting, 19-23 April 2021

Source:	Ericsson, Samsung, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, OPPO, Apple, InterDigital, ZTE

Title:	Transfer of PDU session after end of Disaster Condition

Spec:	3GPP TR 24.811 v1.0.0

Agenda item:	17.2.9

Document for:	Agreement

2. Reason for Change

When the Disaster Condition ends and the UE returns to the PLMN formerly with Disaster Condition:

-	if a PDU session of the UE is transferred, then the UE needs to send solely 5GMM signalling (mobility registration update indicating the PDU session).

-	if a PDU session of the UE is lost, then the UE needs to send 5GSM signalling to re-establish the PDU session, in addition to 5GMM signalling.

Thus, loosing of a PDU session would generate additional unnecessary load on the PLMN formerly with Disaster Condition.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811.

* * * Change * * *

## 4.1	Architectural Assumptions

The following architectural assumptions apply:

- 	The PDU sessions of the UE can be transferred when a UE moves from a PLMN with Disaster Condition to a PLMN providing Disaster Roaming and when a UE moves from the PLMN providing Disaster Roaming to the PLMN previously with Disaster Condition.

-	In principle, the Disaster Inbound Roamers can receive the same services in a PLMN without Disaster Condition as non-disaster inbound roamers can receive in the PLMN without Disaster Condition, subject to agreement between HPLMN of the UE and the PLMN without Disaster Condition, regulations of the country, and constraints of the PLMN without Disaster Condition.

-	The network functions except one or more RAN nodes of the PLMN with Disaster Condition can be assumed to be still operational. One or more RAN nodes of the PLMN with Disaster Condition are non-operational.

-	Architecture defined in 3GPP TS 23.501 [4] is used as basis architecture for supporting minimization of service interruption.